CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4002A Manage occupational health and safety in the building and construction workplace cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Patrick Lee |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Patrick Lee |
| --- | --- |
| Participant signature |  |
| Date | 13/06/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 28 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

For each of the common building and construction jobs listed below, identify one (1) hazard specific to that job.

| Answer | Job | Risks |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Electrocution |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Eye damage |  |
|  | Crane use | Objects falling from above |  |
|  | Demolition | Unplanned structural collapse |  |
|  | Formwork | Formwork falling on workers |  |
|  | Scaffolding use | Failure of scaffolding structure causing collapse |  |
|  | Work at heights | Falls |  |
|  | Welding | Burns |  |

Using the risk table below, determine the level of risk for each of the hazards identified in Q1. Identify how each risk level was determined, by identifying the likelihood and consequences of each hazard.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

| Answer | Job | Level of risk |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | C3, Medium |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | B3, High |  |
|  | Crane use | A3, High |  |
|  | Demolition | A3, High |  |
|  | Formwork | B3, High |  |
|  | Scaffolding use | A3, High |  |
|  | Work at heights | A3, High |  |
|  | Welding | C3, Medium |  |

Identify an appropriate control measure for each of the hazards identified in Q1.

| Answer | Job | Control measure |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Ensure safety wear and tool is checked |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Wear appropriate eye wear and have warning safety signage |  |
|  | Crane use | Create exclusion zones |  |
|  | Demolition | Structural surveys to assess the strength and stability of the structure and adjacent structures |  |
|  | Formwork | Ensure design of formwork is performed by competent person |  |
|  | Scaffolding use | Ensure scaffolding used are not damaged and in good condition |  |
|  | Work at heights | Safety mesh |  |
|  | Welding | Wear fire resistance clothing |  |

True or False (Tick your response): Risk control options can come from both internal sources (e.g. policies and procedures) and external sources (e.g. codes and standards).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Name two (2) workplace personnel you may speak to for safety advice.

| Answer | Supervisors and WHS representative |  |
| --- | --- | --- |

Referring to the WHS Regulations, explain how the following documents contribute towards identifying hazards and risks in the workplace.

| Answer | Document | Description |  |
| --- | --- | --- | --- |
|  | Safe Work Method Statements | Safe Work Method Statements must be prepared prior taking into account all relevant matters related to the circumstances at the workplace that may affect the way in which the work is carried out |  |
|  | Site Safety Plan / WHS Management Plan | Site Safety Plan / WHS Management Plan must be prepared for the workplace before work commences. The plan identified health and safety responsibilities of all persons at the work place, arrangements for managing any work health and safety incidents, and site specific health and safety rules |  |

Identify two (2) advisory standards you might refer to safely undertake building and construction work and briefly summarise their requirements.

| Answer | Standard name | Requirements |  |
| --- | --- | --- | --- |
|  | Steel construction - Code of Practice 2004 | This code provides advice aimed at preventing the risk of injury or death to persons undertaking steel work construction and other persons at the workplace |  |
|  | Formwork Code of Practice 2016 | This code provides practical guidance to persons conducting a business or undertaking on how to comply with their health and safety duties when carrying out construction work involving formwork including high risk construction work |  |

Building and construction industry contracts require building legislation and the National Construction Code to be observed. Refer to these documents when responding to Q8. to Q10.

Identify the requirements in the National Construction Code Volume 1 with regards to air pressurisation for fire-isolated exits. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | E2.2 General Requirements (d) | A smoke detection system must be installed in accordance with Clause 6 of Specification E2.2a to operate AS1668.1 systems that are provided for zone pressurisation and automatic air pressurisation for fire-isolated exits |  |

Identify the requirements in the National Construction Code Volume 2 with regards to smoke alarms. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | Part 3.7.5.2 Smoke alarms requirements | Smoke alarms to be located in Class 1a and 1b buildings, comply with AS 3786, powered from consumer mains source and be interconnected where there is more than one alarm. |  |

Name the building legislation (Act and Regulations) that apply within your jurisdiction.

| Answer | Queensland Building and Construction Commission Act 1991, Building Act 1975, Building Regulation 2006 |  |
| --- | --- | --- |

Which of the following should be considered when developing a risk control plan? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | How and when controls will be implemented |  |
|  |  | Evaluation processes for implemented controls |  |
|  |  | Consultation with individuals and parties |  |
|  |  | Responsibilities for implementation |  |

True or False (Tick your response): If control measures do not adequately control risks, they must be revised.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following are appropriate techniques, tools and processes for identifying hazards and risks? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Workplace inspections |  |
|  |  | Compensation claim information |  |
|  |  | Safe Work Method Statements |  |
|  |  | Information issued by WHS regulators |  |

Which of the following can be used to assist with workplace inspections? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Hazard and risk checklists |  |
|  |  | Job Safety Analysis (JSA) |  |
|  |  | Manifests and registers |  |
|  |  | Surveys |  |
|  |  | Workplace walk-through |  |

Which of the following are limitations of using a generic hazard/risk assessment checklist? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Specifics of the workplace are not considered |  |
|  |  | Not all checklist points will be applicable |  |
|  |  | Such checklists/processes must be completed on a computer |  |
|  |  | You always have to purchase these checklists |  |

Identify the applicable WHS Act in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Act name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and Safety Act 2011 Section 19 Primary Duty of Care | A PCBU must ensure, so far as reasonably practicable, the health and safety of workers engaged and workers whose activities in carrying out the work are influenced or directed by the person while at work in the business or undertaking |  |

Identify the applicable WHS Regulation in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Regulation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and Safety Regulation 2011 Section 34 Duty to identify hazards | A duty holder, in managing risks to health and safety, must identify reasonable foreseeable hazards that could give rise to risks to health and safety |  |

Identify a relevant policy in your organisation with regards to identifying hazards and risks and briefly describe its requirements.

| Answer | Policy name | Requirements |  |
| --- | --- | --- | --- |
|  | Hazard Identification and Control Policy | To ensure there is a formal process for hazard identification and risk assessment which will effectively manage the hazards that may occur within the workplace |  |

Identify the statutory requirements in your jurisdiction with regards to the completion of inspection reports.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and Safety Act 2011 Section 103E Assessment Reports | A work health and safety officer must prepare a report for the assessment of risks to health and safety arising from work carried out as part of the conduct of the business and a work health and safety criteria for the business |  |

Which of the following individuals and parties may require information about workplace hazards and proposed risk controls? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Health and Safety Committees |  |
|  |  | PCBU |  |
|  |  | All members of the public |  |
|  |  | Workers |  |

Identify the requirements in the Code of Practice with regards to consulting parties when managing risks.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety consultation, cooperation and coordination Code of Practice 2021, Section 2.1 | Consultation with workers and who will be affected by the decision either directly or through their health and safety representative is required when identifying hazards, assessing risks and deciding on measures to eliminate or minimise those risks |  |

Identify the requirements in the Code of Practice with regards to consulting parties when developing procedures for communicating WHS policy and practice requirements.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety consultation, cooperation and coordination Code of Practice 2021 section 3.5 | You must consult on health and safety matters so far as is reasonably practicable with workers who carry out work for you and who are (or are likely to be) directly affected |  |

Name three (3) parties who should consult to determine effective strategies for communicating WHS policy and practice.

| Answer | Employees, contractors and subcontractors. Also to consult Management, health and safety representatives, and health and safety committees |  |
| --- | --- | --- |

Name two (2) communication strategies that may be used in the building and construction industry.

| Answer | Issue site specific instructions and signage and written communication like emails |  |
| --- | --- | --- |

Name two (2) education programs that may be implemented in the building and construction industry.

| Answer | Site specific induction training and specialist and targeted training |  |
| --- | --- | --- |

Fill in the blanks for the statement below:

consulted

training

The ‘How to manage work health and safety risks Code of Practice 2011’ identifies that control measures are more effective when ______________________ is provided and when workers are _____________________.

| Answer |  |  |
| --- | --- | --- |

Explain why it is important to review the effectiveness of communications and educational programs.

| Answer | To ensure that the effectiveness of communication and educational programs has improved the likelihood of the work health and safety hazards or risks occurring |  |
| --- | --- | --- |

List three (3) methods for reviewing the effectiveness of communication and educational programs.

| Answer | Conducting post training surveys, evaluating training programs for effectiveness for example reviewing incident report trends, and reviewing training materials for accuracy of content, currency and suitability |  |
| --- | --- | --- |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Patrick Lee |
| --- | --- |
| Participant signature |  |
| Date | 13/06/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to conduct and report on a WHS risk analysis, including workplace inspection. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - A copy of a Site Specific Safety Plan or Construction Safety Plan, including: - Copies of Risk Assessments completed on a project - Copies of Safety Work Method Statements used on a project - A copy a workplace safety inspection checklist that has been used on a project The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Patrick Lee |  |  |
| Name of observer: | Jason Wang |  |  |
| Role of observer (Confirm suitability to sign) | General Manager |  |  |
| Email address of observer | Jason.w@ckon.com.au |  |  |
| Signature of observer: |  |  |  |
| Date: | 13/06/2021 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Conduct a workplace inspection to evaluate construction site safety for the range of occupations in the workplace according to WHS legislation and company policies, including: - Identifying construction hazards and potential risk areas - Identifying and prioritising hazards - Identifying and prioritising risks |  |  |  |
| - Seek expert advice as appropriate, including: - Advice from workplace personnel regarding the application of building codes and standards - Consulting with relevant parties regarding compliance with statutory requirements (including SWMS and site safety plans) - Consulting with appropriate personnel to determine effective strategies for communicating WHS policy. |  |  |  |
| - Complete an inspection report in accordance with best practice and statutory obligations: including: - Recommendations from findings - Required approaches to remediation |  |  |  |
| - Implement agreed control measures in conjunction with relevant workplace personnel, implementing negotiation and conflict resolution skills, as necessary. |  |  |  |
| - Monitor and review control measures, updating records with findings |  |  |  |
| - Establish communication strategies and educational programs specific to the building and construction industry in accordance with statutory requirements and best practice. |  |  |  |
| - Review the effectiveness of communication and educational programs, updating records with findings. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments

Patrick has demonstrated the ability to conduct and report on Work Health and Safety risk analysis including workplace inspection. Due to company policies, not all documents can be released but Patrick has provided a few examples of safety management plans and safe work methods statements demonstrating this.