# Quality Management Plan Template

| # Project Name: Maroochydore private hospital |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Prepared by: Rex Henderson |  |  |  |  |  |  |
| # Date: 30/3/2025 |  |  |  |  |  |  |
| # Description of project: Maroochy Private Hospital is in the heart of the Maroochydore Central Business District “Sun Central”, a 53-hectare Greenfield site. The building consists of: • Ground Floor – Lobby, Loading Dock, Plant Rooms, Car Park Entry, Medical and Retail Tenancies • Level 1 – Car park, End of Trip Facilities, Plant Rooms • Level 2 to 4 - Car park • Level 5 to 7 – Consulting Tenancies (Medical) • Level 8 & 9 – Private Orthopaedic Hospital with Operating Theatres, CSSD Facility, Pre & Post op rooms and In-Patient Ward • Level 10 (Roof) – Plant, Kitchen, Indoor Lounge, Outdoor Terrace and End-of-trip Facilities • B10 storey plus services basement and plant deck • Circa 9400m2 Net Lettable (NLA) of the works • 156 no Carparks spaces • 18 Motorcycle parks • 5 Star Green Star – Design & As Built • 5 Star NABERS (National Australian Building Environmental Rating System) • Regional PCA Grade A (Commercial Building) # • Ground Floor Commercial & Retail Tenancies |  |  |  |  |  |  |
| Description of Project Quality System: A Construction Project Quality System (QPQS) ensures construction projects meet client expectations, comply with regulations, and deliver outcomes that are safe, durable, and within budget. This system provides a structured approach to managing quality from planning through to construction and close-out By implementing a robust quality system, construction managers can identify and address potential issues early, prevent defects, improve efficiency, and increase client satisfaction. Quality assurance, continuous improvement, and effective document control are the core components of any successful construction quality management system. |  |  |  |  |  |  |
| Describe in as much detail as needed specifically what will be required in each of the following areas to manage quality on this project: |  |  |  |  |  |  |
| Organisational Structure: The Organizational Structure for a Quality Management Plan (QMP) in a construction project outlines the roles, responsibilities, and relationships between the individuals and teams involved in ensuring that quality standards are met throughout the project. Below is a typical organizational structure for a QMP in a construction project, ensuring effective quality management across all phases. |  |  |  |  |  |  |
| Roles and Responsibilities: In a Quality Management Plan (QMP) for a construction project, clearly defined roles and responsibilities are crucial for ensuring that all quality-related activities are executed effectively and in alignment with project objectives. Below is a breakdown of the key roles and their associated responsibilities: |  |  |  |  |  |  |
| Stakeholders and responsibilities: In a Quality Management Plan (QMP) for a construction project, stakeholders play critical roles in ensuring that the project meets quality standards, complies with regulations, and satisfies client expectations. Below is a detailed list of typical stakeholders in a construction project, along with their specific responsibilities regarding quality management. |  |  |  |  |  |  |
| Consultation: Consultation in a Quality Management Plan (QMP) for a construction project refers to the process of engaging key stakeholders, team members, and experts to ensure that the quality management practices and processes are well-defined, feasible, and aligned with the project's goals and objectives. Effective consultation ensures that all aspects of the quality plan are addressed thoroughly, and that key players understand their roles in achieving quality outcomes. Consultation is an essential component in achieving stakeholder buy-in, ensuring regulatory compliance, and managing risks. Here’s how consultation typically plays out in the context of a Quality Management Plan (QMP): |  |  |  |  |  |  |
| Procedures: A Quality Management Plan (QMP) for a construction project outlines the procedures, processes, and guidelines to ensure the project meets specified quality standards. It typically includes the following key procedures: Quality Objectives Measurement Criteria Tools Used Timing |  |  |  |  |  |  |
| Processes: The Quality Management Plan (QMP) for a construction project outlines the overall structure, processes, and strategies needed to ensure that the project meets the required quality standards throughout its lifecycle. The following are the key processes typically included in a Quality Management Plan for a construction project: |  |  |  |  |  |  |
| Resources: In a Quality Management Plan (QMP) for a construction project, resources refer to the people, equipment, materials, and systems that will be utilized to ensure the project meets its quality standards. Proper resource management is crucial for maintaining consistency, efficiency, and overall quality throughout the project. Below are key resources typically included in a QMP: |  |  |  |  |  |  |
| Quality objectives aligned to workplace objectives and procedures |  |  |  |  |  |  |
| Quality items | Measures for each quality item |  | Units to measure each item | Existing project tools that will provide quality data |  | Timing of measurement |
| Ensuring compliance with regulation and standards | Understand Relevant Regulations and Standards |  | Employee Productivity and Performance. | Microsoft Project, Regulatory Compliance Software for Industry-Specific Standards |  | All stages of project |
| Meeting client expectations | Clients expect projects, products, service to be delivered on time |  | Meeting dates and deadlines. | Regular meetings. Setting targets. Order of importance. |  | Critical Path And noncritical path. |
| Reducing defect and reworks | Ensuring constant monitoring of progress |  | Physical records of task progress. | Photo’s Meetings. |  | Critical Path And noncritical path |
| Improving safety performance | Regular training, Interactive training, Safety protocols. |  | Use workshop emergency processes and real-world procedures | Safety management systems (SMS) |  | All stages of project |
| Optimizing resources utilization | Conduct and Audit. Identify bottlenecks. |  | Productivity of each person. Labor Cost per Unit of Output. | Microsoft Project, Smartsheet, Time Tracking Tools. |  | All stages of project |
| Fostering continuous improvement | Workforce Productivity and Performance, Construction Material Quality, safety standards |  | Labor Productivity Rate. Material Strength and Durability. Incident Rate ​ | Building Information Modelling (BIM) Procore |  | All stages of project |
| Communication procedures and methods to be applied |  |  |  |  |  |  |
| Communication to management: Email, Procore, Weekly Meetings, Toolbox talks, Mobile devices. |  |  |  |  |  |  |
| Communication to work teams: Email, Procore, Weekly Meetings, Toolbox talks, Mobile devices. |  |  |  |  |  |  |
| Issues to be addressed and consultation |  |  |  |  |  |  |
| Issue | Who to consult |  | Improvements for consideration | Feedback following consideration |  | Supporting Resources |
| Concrete block fill blowout resulting in copper pipes covered in concrete | Block layers Plumbers Site team |  | Ensure blockworks are sealed before filling | Double checks |  | Toolbox talks |
| Water tank access ladder not 1 meter out of tank opening | Bladder installer HSC Site team |  | Custom ladder for entry before works start. | Plan to ensure works are not delayed |  | Company Quality Alert |
| Wall penetration cut wrong orientation, Landscape not portrait | Block layer Aircon sub trade |  | Drawings Architect | Ensure plans are interpreted correctly |  | Plan from Architect |
| Double lined wall missed inside ductwork riser | Block layer Aircon sub trade |  | Double check plans Don’t assume same as previous floor | Double check from all parties |  | Plan from Architect |
| Describe how each of the following aspects of quality management will be addressed on this project: |  |  |  |  |  |  |
| Quality Control: To effectively address Quality Control (QC) in the project, it is important to integrate these aspects into a Quality Management plan. Continuous monitoring, testing, and analysis throughout the project lifecycle help ensure that the project remains on track to meet its quality goals. By using proactive strategies like audits, root cause analysis, SPC, and stakeholder feedback, quality can be systematically managed and improved, resulting in high-quality deliverables and successful project completion. |  |  |  |  |  |  |
| Control item |  | Measure |  |  | Project Stage |  |
| Concrete strength test |  | Pot test |  |  | As instructed before next process (before props are removed) |  |
| Water tank Prep |  | Check all surfaces, walls, Floors for sharps that could penetrate liner |  |  | Before liner install |  |
| Structural steel outriggers |  | Engineer Checks |  |  | Before roof lining goes on |  |
| Reo in block work correct size and placement. (As per plans) |  | Engineer checks |  |  | Before block fill |  |
| Lining of walls (villa board) |  | Check fixing spacing as per NCC |  |  | As works are being completed |  |
| Quality Assurance processes used |  |  |  |  |  |  |
| Training: necessary competence for personnel performing work affecting conformity to product. requirements through selection of appropriately training people for the task at hand. extensive learning and development training program than undertakes to provide employees. with the opportunity to achieve the necessary competence. |  |  |  |  |  |  |
| Monitoring and maintenance of information sources: The Site Manager/Foreman shall continually monitor the quality of the workmanship of the Subcontractors on site through informal visual inspections as well as via formal Inspection Test Plans. Site audits shall be undertaken at key milestones identified by the HSEQ Manager and/or the Site Management Team. Audit reports shall be issued to the Client as requested. Specific audit criteria shall be prepared from approved Specifications and Construction-Issue drawings. |  |  |  |  |  |  |
| Resource availability and access within timeframe: Resource available on company network and available at any time |  |  |  |  |  |  |
| Records management: We shall maintain records of every concrete pour via a Concrete Pour Record and accompanying Inspection. and Test Plans. This documentation shall be accompanied by records of all batch receipts and applicable test. reports. Submission of material documents shall be in accordance with technical specifications provided by the Client. |  |  |  |  |  |  |
| Quality Improvement / amendments to be instigated: Quality improvement can be made by... ITP Toolbox talks Observation with regular closeouts Quality alerts a Live on a register which can be accessed at any time |  |  |  |  |  |  |

Quality Audit Template

| # Project Name: Maroochy private hospital |  |  |  |
| --- | --- | --- | --- |
| Prepared by: Ric Henderson |  |  |  |
| # Date: 31/3/2025 |  |  |  |
| Project Phase: Execution Phase | Overall Project Status: Current project on track |  |  |
| Audit Date: 31/3/2025 | Audit Number: 23502 |  | Audit Leader: Quality business Partner |
| Audit Team: Construction manager Project manager |  |  |  |
| Goal(s) of This Specific Audit: A Quality Audit for construction is a systematic process that assesses whether a construction project is being executed according to the set standards, regulations, and contractual requirements. The goal is to ensure that all processes, materials, workmanship, and outcomes meet the necessary quality standards for safety, functionality, and compliance. |  |  |  |
| Audit of Management of Project: |  |  |  |
| 1 Planning the Audit |  | Assessment: Define scope and objectives | Comment: Determine the specific areas of the project to be audited. |
| 2. Document Review |  | Assessment: Examine Documentation | Comment: Review key project document |
| 3 Site Inspection |  | Assessment: Physical inspection | Comment: Conduct an onsite inspection to verify the quality of work |
| 4. Assessing quality control process |  | Assessment: Quality control process | Comment: Verify that appropriate quality control procedures are being followed throughout project |
| Overall Assessment of Management of Project: An overall assessment of a construction project involves evaluating the project from various perspectives to ensure it meets the desired outcomes. This assessment typically covers aspects such as quality, cost, schedule, safety, and stakeholder satisfaction. Here’s how you can conduct a comprehensive overall assessment: |  |  |  |
| Recommended Action(s)/Lessons Learned Regarding Management of the Project: 1. Enhanced Planning and Scheduling 2. Improved Communication and Coordination 3. Better Risk Management |  |  |  |
| Document control: Effective Construction Document Control ensures that all project documents are systematically organized, easily accessible, properly archived, and compliant with relevant regulations. By using the right tools and adhering to best practices, construction teams can improve efficiency, minimize risks, and enhance collaboration. Whether you're working on a small residential build or a large-scale commercial project, document control is essential for smooth project execution. |  |  |  |