CPCCBC4012B

Read and interpret plans and specifications

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4012B Read and interpret plans and specifications cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Corey Passey |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Corey Passey |
| --- | --- |
| Participant signature |  |
| Date | 20/06/2022 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 87 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

- Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

- Which compass direction is always shown on plans and drawings?

| Answer | North |  |
| --- | --- | --- |

- What information is provided on a location plan?

| Answer | Location Plan A location plan is required so that you can correctly identify your building site. A location plan is necessary for a big development so that the relative location of the site in a particular street is clear. A location plan details the: - Lot number/s - Deposit plan numbers - North point - Scale (usually 1:5000) |  |
| --- | --- | --- |

- What detail does a site or block plan contain?

| Answer | A Site or Block Plan is usually drawn at 1:500 and gives the following information: - A datum - reference height to other site levels - Building location and orientation to north point - A Site or Block Plan is usually gives the following information: - Outline of site boundaries showing location of proposed building and existing dwellings - Street frontage names - Major geological and topographical features (trees and vegetation) - Datum points, contours and slopes - Driveways, stormwater drainage, easements and right of carriage - Soil and surface water drains, pipe sizes - Service runs from the house to mains - utility services location (sewer, water, gas, electricity) - Details of paving and retaining walls |  |
| --- | --- | --- |

- What do floor plans identify?

| Answer | - Floor plans: Shows detail of the proposed structure, such as rooms, positions of fixtures, floor coverings - the floor plan it shows the size and thickness of structural members and relationships between spaces - Floor plans have all the dimensions of room sizes |  |
| --- | --- | --- |

- What is a landscape plan and what information might it provide?

| Answer | - Landscape plan: Shows details of plants, paths, garden layouts and retaining walls |  |
| --- | --- | --- |

- What are elevation views? What do they provide to the plan reader?

| Answer | Elevation views View of the proposed structure from its sides (i.e. from north, south, east and west). Each side is what is called an elevation view. |  |
| --- | --- | --- |

- What is a section?

| Answer | Section views In most cases elevation and plan views cannot show sufficient information to enable a builder or tradesman to see how various structural parts are to be built or assembled. The section is a view of the house after it has been cut vertically at some point. Like the floor plan it shows the size and thickness of structural members and relationships between spaces. |  |
| --- | --- | --- |

- What are detailed drawings?

| Answer | Preliminary and final drawings and plans Final drawings and plans are drawings that show overall sizes, levels and references to assembly drawings. The location drawings, which can be plans, elevation or sections, are numbered consecutively. Typically, final drawings will include: - Site plans - Floor plans - Sections through the entire building - Elevations - Landscape plans - Detailed drawings |  |
| --- | --- | --- |

- What are specifications?

| Answer | General items contained within Building Specifications include a: - Full description of work - Precise list of materials - Details of all fittings, fixtures and finishes (appliances – make and model) - Precise list of products (e.g. types, brands, grades, thickness, colour, model) - Key features included within building specifications include: - Levels and survey information - Materials lists - Performance data and material technical data - Schedules of quantities - Stress, load and bearing calculations |  |
| --- | --- | --- |

- Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bull nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Block |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Glass |  |
|  | CJ | Ceiling Joist |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold water tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Proof Course |  |
|  | DSB | Distribn Switch Board |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion Joint |  |
|  | FPBD | Fibrous Plaster Board |  |
|  | FFL | Finished Floor Level |  |
|  | FHR | Fire Hose Reel |  |
|  | GPO | General Purpose Outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

- Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Double brick walls |  |  |
|  | Brick and frame wall |  |  |
|  | Internal Frame wall |  |  |
|  | Window external wall |  |  |
|  | Window |  |  |
|  | Door |  |  |
|  | Double doors |  |  |
|  | Cavity slider |  |  |
|  | External slider |  |  |
|  | Single power point |  |  |

- Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Hinged at side |  |  |
|  | Awning hinged at top |  |  |
|  | Sliding horizonatally |  |  |
|  | Fixed |  |  |

- What does a clouded area on a plan indicate?

| Answer | On plans, revisions or amendments are represented by a clouded area |  |
| --- | --- | --- |

- An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

- On what date was an amendment made to the window details?

| Answer | 5/10/2011 |  |
| --- | --- | --- |

- True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What amendment occurred on 08/11/2011?

| Answer | Bed 3 Robe deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

- What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

- What is the Lot number of this site?

| Answer | 16 |  |
| --- | --- | --- |

- What is the size of this lot?

| Answer | 3653m2 |  |
| --- | --- | --- |

- Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

- True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What size is the easement on the eastern boundary of the site?

| Answer | 5m |  |
| --- | --- | --- |

- True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

- What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

- Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

- Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26.?

| Answer | Yes Two story dwelling, slab on ground, corrugate iro roof with lightweight cladding to upper floor and brick veneer to lower level. With verandahs |  |
| --- | --- | --- |

- Who is the client?

| Answer | H and M Smith |  |
| --- | --- | --- |

- Where is this project to be built?

| Answer | Lot 16 Glenvalley Road Richmond |  |
| --- | --- | --- |

- What are the contact details for the draftsman?

| Answer | Joe Foreman Ph.03 9514 2311 M. 0413 113 113 18 Fairview Road Richmond 3216 |  |
| --- | --- | --- |

- What sheet number is this drawing?

| Answer | 5 of 7 |  |
| --- | --- | --- |

- What timber is required for loading and non-load bearing walls?

| Answer | F5 45x90 Load Bearing and F5 45035 x 90 non load bearing |  |
| --- | --- | --- |

- What spacing is recommended for common studs?

| Answer | 600mm maximum Stud spacing |  |
| --- | --- | --- |

- What type of roof truss is to be installed?

| Answer | Scissor trusses |  |
| --- | --- | --- |

- What centres are the trusses positioned at?

| Answer | 900mm spacing |  |
| --- | --- | --- |

- What size are the veranda posts?

| Answer | 90 x 90 DAR Oregon |  |
| --- | --- | --- |

- What timber species are the veranda posts to come from?

| Answer | DAR Oregon |  |
| --- | --- | --- |

- What pitch is the roof?

| Answer | 30 degree pitch |  |
| --- | --- | --- |

- What size are the noggings and at what spacing?

| Answer | F5 35 x 70mm and 1350mm spacing |  |
| --- | --- | --- |

- What size are the ceiling joists?

| Answer | 450mm spacing 35 x 70mm |  |
| --- | --- | --- |

- What timber is used for roof battens and what spacing?

| Answer | 35 x 70 FG pine battens at 900mm spacing |  |
| --- | --- | --- |

- What thickness is the plasterboard sheeting?

| Answer | 10mm |  |
| --- | --- | --- |

- What type of flooring is to be installed?

| Answer | 15mm plywood flooring |  |
| --- | --- | --- |

- What is the roof cladding material?

| Answer | Colorbond corrugated metal roofing |  |
| --- | --- | --- |

- Where will you find information about the footings?

| Answer | On the foundation plan |  |
| --- | --- | --- |

- What do the letters ‘DAR’ refer when timber ordering?

| Answer | Dressed all round , finished size of the post |  |
| --- | --- | --- |

- Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | Laminated Veneer Lumber, means the size of the joists |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

- What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

- What is the Job Number?

| Answer | OOR-112 |  |
| --- | --- | --- |

- What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

- What does the drawing show?

| Answer | Elevations and cross section |  |
| --- | --- | --- |

- Which view shows the front of the house?

| Answer | East elevation |  |
| --- | --- | --- |

- How many motor vehicles can be parked in the garage?

| Answer | 2 |  |
| --- | --- | --- |

- What are the window and doors made from?

| Answer | Aluminum framed windows and timber door, colourbond roller doors |  |
| --- | --- | --- |

- What is the size of the eaves?

| Answer | 450mm |  |
| --- | --- | --- |

- What are the eaves lined with?

| Answer | Hardiflex |  |
| --- | --- | --- |

- What type of roof structure is the roof built from?

| Answer | Timber framed roof trusses |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the requirements for the roof structure in Q59.?

| Answer | Part 3.4.3 |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | AS 1720.1 |  |
| --- | --- | --- |

- What is the roof cladding materials made from?

| Answer | Concrete Tile |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Part 3.5.2 |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | AS 2050 |  |
| --- | --- | --- |

- What is the roof pitch?

| Answer | 22.5 degrees |  |
| --- | --- | --- |

- Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | Local Council |  |
| --- | --- | --- |

- Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | All plans, specifications and any special soil reports etc and engineering required for the job and forms filled out are submitted. The process can take a few weeks to up to 10 months dependant on the size of the job and the shire or councils timing. Also depends on the work load of the councils. |  |
| --- | --- | --- |

- What is a prime cost (PC) item? Give an example.

| Answer | A PC item is: - An amount of money included in a contract sum to purchase a specified item such as tiles, taps, doors or bathroom fittings - An agreed estimated amount is included at contract signing but the specific products are not selected until a later stage - A builder must estimate the cost of such items at or above the lowest amount these items could reasonably cost - For example if a client selects a more expensive door than allowed by the PC item, say a $500 door compared with a $440 PC amount, a variation will be required to cover the additional $60. If the PC item varies, so will the final contract sum |  |
| --- | --- | --- |

- What is a provisional sum (PS) item? Give an example.

| Answer | A provisional sum is: - An amount of money included in contract sum to cover work and/or materials, not detailed when entering the contract - Typically builders will include a PS for site work costs - The builder is legally required to take reasonable steps to ensure an accurate site works cost estimate but unforeseen events mean that the provisional sum is exceeded which is added to the contract sum For example a building site may seem sandy and clean with minimal siteworks required, however, there may be large limestone rocks/tree trunks concealed below the surface which only emerge once siteworks commence, causing cost overrun. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

- True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify two (2) materials being used for the floors?

| Answer | Concrete, carpet and vinyl |  |
| --- | --- | --- |

- True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- How much does the contracted plasterer cost?

| Answer | $12,900 |  |
| --- | --- | --- |

- How much do the kitchen appliances cost?

| Answer | $2700 |  |
| --- | --- | --- |

- What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

- What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

- How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

- If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $275.00 2.75 / stud |  |
| --- | --- | --- |

- How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 |  |
| --- | --- | --- |

- What material has the highest total cost?

| Answer | Pine 90 x 45 MGP10 long lengths |  |
| --- | --- | --- |

- How many metres are required of the material in Q80. ?

| Answer | 322.5m |  |
| --- | --- | --- |

- What beam is identified as being a better option than F17?

| Answer | E14 beams |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

- Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter road construction $107.40 per hour |  |
| --- | --- | --- |

- If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $214.80 |  |
| --- | --- | --- |

- Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar bender and fixer $214.30 |  |
| --- | --- | --- |

- If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $1,288.80 |  |
| --- | --- | --- |

- Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | - Building Act 1975 (Qld) - Building Regulation 2021 (Qld) |  |
| --- | --- | --- |

CPCCBC4012B

Read and interpret plans and specifications

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Corey Passey |
| --- | --- |
| Participant signature |  |
| Date | 22/06/2022 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4012B

Read and interpret plans and specifications

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to read and interpret plans and specifications in order to inform estimation, planning and supervisory activities. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - Any drawing with a cloud or mark-up you have inserted. - Copy of the correspondence regarding mark up and any site management/authorising engineer response. The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Drawings (including site plan), Specifications, Tender documentation, Telephone, Facsimile machine, Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Corey Passey |  |  |
| Name of observer: | Mark Harris |  |  |
| Role of observer (Confirm suitability to sign) | GM of Stroud Homes and business owner |  |  |
| Email address of observer | Mark.harris@stroudhomes.com.au |  |  |
| Signature of observer: |  |  |  |
| Date: | 22/06/2022 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| 1 Interpret drawings |  |  |  |
| - Identify purpose and advantage of different types of drawings. |  |  |  |
| - Identify different aspects of drawings. |  |  |  |
| - Identify, understand and apply: - Commonly used symbols and abbreviations - Common building and construction terms on drawings |  |  |  |
| - Identify building site from location drawings. |  |  |  |
| - Identify on the site plan: - True north - Building orientation - Key features in relation to building and other structures - Levels - Contours - Service entry points - Site features to be removed or retained |  |  |  |
| - Identify key features of plans, elevations and sections. |  |  |  |
| - Identify client requested variations to standard plans on drawings. |  |  |  |
| 2 Interpret specifications |  |  |  |
| - Identify and correctly apply Provisional Sum (PS) and Prime Cost (PC) values. |  |  |  |
| - Identify and confirm customer variations to standard specifications and communicate changes to organisational personnel. |  |  |  |
| - Apply and communicate correct interpretations of essential elements to estimation, planning and supervisory tasks (including labour hours/costs and material quantities/costs). |  |  |  |
| - Identify Building Codes or standards affecting the work to be undertaken (including references to Australian Standards and the National Construction Code). |  |  |  |
| - Identify key features of products included in the specification, including the design, purpose, aesthetics and cost relationships. |  |  |  |
| 3 Communicate with others |  |  |  |
| - Consult with industry professionals, clients, organisational personnel and appropriate local authorities. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Using technology (including telephone, facsimile, email) |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments

Corey understands and has a good knowledge of the tasks in this module. I have observed him throughout the last year and he is very competent with these topics and knows his way around a set of plans and specifications and also pricing and BOQ’s and getting contracts signed and in and out of council. Corey has run his own building franchise in NZ for 10 years and has built over 400 homes so it’s clear he knows what he is doing with regards to reading and interpreting plans and specifications. Kind Regards Mark Harris