CPCCBC4006B

Select, procure and store construction materials for low rise projects

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Haydn Morriss |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 8/05/2019 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 44 |  | Yes | No |

Assessor Comments

Which of the following documents and/or people would you refer to when selecting materials for a building project? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Australian Standards |  |
|  |  | Contract |  |
|  |  | Drawings |  |
|  |  | Manufacturer |  |
|  |  | National Construction Code |  |
|  |  | Specifications |  |
|  |  | Tolerances |  |

Identify the Volume and Parts of the NCC that relate to suitability of materials for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Vol2 Sec 1 Part 1.2.1 | Vol 1 A2.2 |  |

Identify the assessment methods used to verify a performance solution.

| Answer | Evidence of suitability A verification method Expert judgment Comparison with the Deemed to satisfy solution |  |
| --- | --- | --- |

Which of the following solutions for material selection often references the Australian Standards? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Deemed-to-Satisfy solution |  |
|  |  | Performance solution |  |

Identify five (5) materials commonly used for low rise buildings in your region. For each material, identify the following:

- Its name

- Properties of the material

| Answer | # | Material name | Properties |  |
| --- | --- | --- | --- | --- |
|  | 1 | Rock | Very Dense Long lasting |  |
|  | 2 | Wood | Flexible under loads Strong when compressed vertically |  |
|  | 3 | Brick | Flame retardant Strong |  |
|  | 4 | Concrete | Formibility Longevity |  |
|  | 5 | Metal | Strong Flexible |  |

For each of the materials in Q5. identify the following:

- Quality of the material (i.e. durability, serviceability, cost effectiveness)

- Compatibility and non-compatibility of the material (i.e. what it would/would not be suitable for)

| Answer | # | Quality | Compatibility |  |
| --- | --- | --- | --- | --- |
|  | 1 | Durable | Compatible with wall construction |  |
|  | 2 | Durabe & Cost effective | Compatible for most situations as long as correct species of timber is chosen |  |
|  | 3 | Durable, long lasting & Cost effective | Compatible for most situations |  |
|  | 4 | Durable, costly | May need to be reinforced to be compatible |  |
|  | 5 | Durable | Compatible for larger buildings |  |

Identify five (5) physical characteristics that may be considered when evaluating potential building materials.

| Answer | Density Strength Moisture absorbency Acoustics Electrical Conductivity |  |
| --- | --- | --- |

Match the terms and definitions below by writing the appropriate letter next to each number in the centre column.

| Answer | Term |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Carbon footprint | 1 - B | A | The amount of energy required to manufacture a product |  |
|  | 2 | CO2e | 2 - E | B | Total GHD emissions produced during manufacture construction and use |  |
|  | 3 | Embodied energy | 3 - A | C | Greenhouse gas emissions |  |
|  | 4 | Embodied water | 4 - D | D | The amount of water required to manufacture a product |  |
|  | 5 | GHG | 5 - C | E | CO2equivalent tonnes emitted into the atmosphere |  |

Which of the following materials would be considered as ‘low energy content’? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Bricks |  |
|  |  | By-products of other industrial processes |  |
|  |  | Cement |  |
|  |  | Glass |  |
|  |  | Metals |  |
|  |  | Plastics |  |
|  |  | Sand |  |
|  |  | Stone |  |
|  |  | Timber |  |

Refer to the table below and identify the wall material type that has the smallest embodied energy and emission impact.

| Answer | Walls timber frame fibro cladding |  |
| --- | --- | --- |

Identify the Volume and Parts of the NCC that relate to energy efficiency of materials for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Vol 2 Part 2.6 | Vol 1 Sec J |  |

Identify the relevant section of the Standards and Tolerances Guide and the associated Australian Standard that addresses residential slabs and footings.

| Answer | Standards and Tolerances Guide | Australian Standard |  |
| --- | --- | --- | --- |
|  | Part 3.0 | AS 2870-2011 |  |

Identify the relevant section of the Standards and Tolerances Guide and the associated Australian Standard that addresses roof tiles.

| Answer | Standards and Tolerances Guide | Australian Standard |  |
| --- | --- | --- | --- |
|  | 7.4 | 2049-2002 |  |

Identify the Volume and Parts of the NCC that relate to safety barriers for swimming pools for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Vol 2 Part 3.9.2 | Vol 1 J7 |  |

In some states, barriers and fences for swimming pools are not entirely regulated by the NCC, as noted in the Ancillary Provisions and State and Territory Variations. Identify the applicable Act and Regulation that regulate safety barriers for swimming pools in your jurisdiction.

| Answer | QLD Development Code Mandatory Part 3.4 |  |
| --- | --- | --- |

Identify the Australian Standard that relates to safety barriers for swimming pools.

| Answer | AS 1926.1 |  |
| --- | --- | --- |

True or False (Tick your response): According to the Standard in Q16. all gates must be fitted with a self-closing device that will return the gate to a closed position from any position without the use of manual force.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify the Volume and Parts of the NCC that relate to fire-resistance of building elements for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Vol 2 Part 2.3 | Vol 1 Part C1 |  |

Identify the Australian Standard that relates to fire-resistance tests for elements of construction.

| Answer | 1530.4-2014 |  |
| --- | --- | --- |

For all NCC Classes, where the ground or subfloor space is excessively damp or subject to frequent flooding, the subfloor framing must which durability Class timbers?

| Answer | Class 1 or 2 timbers or H3 preservative treated timbers |  |
| --- | --- | --- |

Identify the Australian Standards that must be observed when meeting the requirements for Q20.

| Answer | AS 1684 Parts 2,3 or 4 |  |
| --- | --- | --- |

True or False (Tick your response): Where materials are not used as intended (or without considering NCC requirements), this can make a significant difference to the life or serviceability of the structure.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Explain why durability and serviceability impact on the cost effectiveness of chosen materials.

| Answer | If materials deteriorate it may cause failure of the structure requiring repair or replacement |  |
| --- | --- | --- |

Provide an example of short term degradation of materials and describe its effects on a building on structure.

| Answer | Timber is prone to short term degradation which in turn can lead to rotting, leaving a structure unsafe |  |
| --- | --- | --- |

Provide an example of long term degradation of materials and describe its effects on a building on structure.

| Answer | Metal can corrode as they form a compound with oxygen in the air causing failure of the structure |  |
| --- | --- | --- |

You have spoken to a supplier who has indicated that some of the client’s desired materials are unavailable. When considering alternative building materials what must be taken into consideration (i.e. other than the client’s requirements)?

| Answer | Material availability Environmental issues Life cycle assessment methodology Need for it to be performance tested |  |
| --- | --- | --- |

Identify two (2) things that you should look for when receiving a delivery of materials.

| Answer | Confirm correct quantity Confirm the condition of the delivery |  |
| --- | --- | --- |

If you encounter issues with delivery, what two (2) options do you have?

| Answer | Accept shipment with conditions Reject the entire shipment |  |
| --- | --- | --- |

Which of the following should be assessed before shifting materials? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | What needs to be relocated (e.g. size, weight) |  |
|  |  | The relocation area, site layout and obstacles |  |
|  |  | Distance and time to relocate load |  |
|  |  | Whether a team is available and/or appropriate |  |
|  |  | Whether PPE must be worn (e.g. for hazardous materials) |  |

What document should you refer to for guidance on how to handle and store hazardous chemicals?

| Answer | Safety data sheet |  |
| --- | --- | --- |

Name three (3) ways you can reduce the risk of injury if a load is too heavy for one person to carry, push or pull.

| Answer | Team lift Lighten the load Use a mechanical aid |  |
| --- | --- | --- |

In the diagram below a person is lifting a tool box.
Filling in the table, indicate how each body part should be used/placed.

F. Hands

E. Knees

D. Shoulders

C. Feet

B. Back

A. Neck

| Answer | Area of body (Letter) | How it should be placed |  |
| --- | --- | --- | --- |
|  | D | Level |  |
|  | A | Upright |  |
|  | F | Good grip |  |
|  | B | Natural curve |  |
|  | E | Bent |  |
|  | C | Flat, facing load |  |

How should construction materials and components be handled? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Sorted to suit material type and size, separating hazardous material |  |
|  |  | Stacked or stockpiled for ease of identification and retrieval |  |
|  |  | Stored in access ways for ease of retrieval |  |
|  |  | Stored clear of access ways |  |
|  |  | Handled and positioned ready for installation |  |

Identify the Australian Standard that relates to the storage and handling of LP gas.

| Answer | AS/NZS 1596-2014 |  |
| --- | --- | --- |

List one (1) testing method for each of the building materials listed below.

| Answer | Material name | Properties |  |
| --- | --- | --- | --- |
|  | Concrete | Hydraulic Press |  |
|  | Metals | Repeated loading |  |
|  | Timber | Visual grading |  |

What Code gives detailed tables which state the required grade of timber for specific sizes and uses?

| Answer | Australian Timber framing Code |  |
| --- | --- | --- |

Name two (2) ways that defects may be seen in timber.

| Answer | Visual Stress grading |  |
| --- | --- | --- |

Identify two (2) tests that may be performed to determine the durability of stone.

| Answer | Acid Immersion test Visual test |  |
| --- | --- | --- |

Name two (2) ways that defects may be seen in stone.

| Answer | Cracks Fissures |  |
| --- | --- | --- |

True or False (Tick your response): Under a contract, builders and other specialist tradespersons are the ones most likely to rectify or remedy problems that arise from incorrect installation, application or placement of materials.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Name two (2) established methods that may be used to preserve timber.

| Answer | Mechanical Protection Painting |  |
| --- | --- | --- |

Identify two (2) established methods that may be used to protect ferrous metals.

| Answer | Galvanising Painting |  |
| --- | --- | --- |

Explain why the corrosion resistance of non-ferrous metals is superior to that of ordinary steel.

| Answer | This is a result of the production of a ceramic coating of oxide that protects the metal from further corrosion |  |
| --- | --- | --- |

When should aluminium be coated with bituminous paint? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | At all times |  |
|  |  | When it is buried in soil |  |
|  |  | When it comes in contact with chemical materials |  |
|  |  | When it is immersed in water |  |
|  |  | When it is used on bitumen |  |