| Project No: | A511 | NCR No: | 2 |
| --- | --- | --- | --- |
| Project Name: | Gold Coast Secure Mental Health Unit | Date: | 14/05/24 |
| Issued by: | Ross Snelgrove | Issued to: | Padstar |
| Title | Damage to protective coatings due to installation of safety mesh |  |  |
| Trade | Roofing |  |  |
| Description of Non-Conformance | The roof structure is designed with PFC’s trimming the perimeter of the roof. As roof safety mesh needs to be fixed to these perimeter members the subcontractor has drilled through the flange of the PFC. This was selected in line with AS/NZS 4389:2015 as opposed to the alternative methods which would have resulted in greater damage to the protective coatings. Due to the action of drilling these holes some damage to the PFC protective coating/minor surface rust has occurred. |  |  |
| Location / Area | PFC’s trimming the Lower Ground roof structure |  |  |
| Root Cause | Process: Design Issue. Safety mesh not considered in design |  |  |
| Respond By | May 2024 |  |  |
| Proposed Remedial Action | Touch up any damaged protective coatings with the suppliers recommended site repair product to reinstate the durability. |  |  |
| Preventive Action | As the safety mesh needs to extend to the perimeter of the roof structure, holes will continue to be drilled into the PFC flange. Protective coatings will be inspected & touched up as required prior to concealing with roof sheeting. |  |  |