CPCCBC4004

Identify and produce estimated costs for building and construction projects

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4004 Identify and produce estimated costs for building and construction projects cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Justin Matheson |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. | Satisfactory | Not Satisfactory |
| Workbook | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for CPCCBC4004 | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Theory Exam Workbook Practical Assessment Compliance sheet (HO1) Project Details (HO2) Materials and consumables costing (HO3) | Off-site production cost estimation (HO4) Labour cost estimation (HO5) Physical resources cost estimation (HO6) Risk mitigation sheet (HO7) Costings estimate spreadsheet template (HO8) Building Construction estimate spreadsheet (HO9) |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

CPCCBC4004

Identify and produce estimated costs for building and construction projects

THEORY EXAM

| Participant name | Justin Matheson |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within two (2) hours, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

How is the construction industry in Australia regulated?

| Answer | The Australian Government collaborates with state and territory governments to promote nationally consistent regulations through the National Construction Code – (NCC). This is done through the Building Ministers’ Meeting and the Australian Building Codes Board – (ABCB). |  |
| --- | --- | --- |

How do you determine which Australian Standards are mandatory and therefore enforceable?

| Answer | 1.By Legislation and Regulations -Checking relevant legislation and regulations at the federal, state, or territory level, documents will often explicitly reference or mandate compliance with specific Australian Standards. 2. Visit the websites of government departments or agencies responsible for regulating the industry or sector in question. They often provide information on standards that are mandatory for compliance. 3. Industry Codes of Practice – Some industries have specific codes of practice that reference or adopt certain standards. These codes are often developed in consultation with industry stakeholders and may be endorsed or mandated by regulatory bodies. |  |
| --- | --- | --- |

Discuss how builders and others on construction sites and ensure the safety of all on site.

| Answer | - All workers must hold a current white card before they commence work on site. - All workers and visitors in the construction site should wear the required PPE to reduce exposure to various hazards on the worksite. - Be mindful and follow signs - Provide clear instructions - Keep site tidy - Organise and store tools properly - Use the right equipment for the right task - Have an emergency response plan - Set up safeguards - Report issues immediately |  |
| --- | --- | --- |

Why do construction projects have to abide by environmental protection legislation and regulations?

| Answer | Construction projects must abide by environmental protection legislation and regulations for several reasons. - Preserve ecosystems, keeping in mind disruption of habitats, deforestation, and the disturbance of wildlife. - Water quality protection is also important to control erosion, sedimentation and to help protect rivers and lakes etc. - Construction processes and materials can contribute to greenhouse gas emissions, which is a major factor in climate change. - Construction activities can also generate dust, emissions, and other airborne pollutants that may have adverse effects on air quality. - Noise control is also important as sites can be noisy, disrupting the peace and well-being of surrounding areas. - Construction projects can generate significant amount of waste, including materials like concrete, wood, and metals. Environmental regulations require proper waste management practices, such as recycling and responsible disposal, to minimize the environmental impact and promote sustainability. |  |
| --- | --- | --- |

Describe the Environmental Protection Authority (EPA) and how environmental protection is monitored in the state/territory you work.

| Answer | . The Environmental Protection Authority is an independent statutory body delegated to protect the environment and prevent and control environmental harm. They are in charge of enforcing regulations with an environmental concern. |  |
| --- | --- | --- |

Explain why workplace policies and procedures are important in relation to legislation.

| Answer | Policies and procedures can fulfil employers’ obligations and responsibilities under certain legislation such as work health and safety and discrimination legislation. Policies and procedures provide employees with a clear understanding of what is expected of them. |  |
| --- | --- | --- |

Why should you understand the ‘scope’ of the workplace when producing estimated costs for building and construction projects?

| Answer | Clearly defining and documenting the scope of a project helps you account for all the work that needs to be completed so you can accurately estimate costs and set a well-thought-out project plan. |  |
| --- | --- | --- |

Explain the importance of construction schedules in relation to contracting tradespeople, material resourcing and inspections.

| Answer | A construction schedule is used to control the comings and goings of contractors whilst being able to source materials before contractors are on site and giving you key dates for inspections. Having a detailed, handy, simple-to-use construction schedule means your suppliers know exactly when and what to deliver, your subcontractors know when to book their labourers and tradesmen, your general contractor and other stakeholders can monitor the project, and the project owner can view the overall progress. |  |
| --- | --- | --- |

Discuss why builders should keep and maintain records of subcontractors, site supervisors and workers qualifications, competencies and licences.

| Answer | 1.To comply with regulations 2.For quality assurance 3.To help with safety and risk management 4. For project efficiency 5.Insurance requirements 6. Subcontractor selection 7. To assist with career development and training 8. Documentation and accountability |  |
| --- | --- | --- |

Why is it important to identify the site conditions during planning?

| Answer | Identifying site conditions during planning is crucial for several reasons – 1.Project feasibility 2.Budgeting and cost estimation 3.Scheduling 4.Risk management and mitigation 5.Environmental and regulatory compliance 6.Design considerations 7.Resource allocation 8.Safety precautions |  |
| --- | --- | --- |

The ‘Managing the work environment and facilities Code of Practice’ states that a PCBU must so far as reasonably practicable provide adequate and accessible facilities for the welfare of workers. Discuss what must be organised to meet this requirement.

| Answer | The PCBU should actively consult with workers to understand their needs and preferences, to provide risk assessment to identify potential hazards and risks to the facilities and determine the most appropriate location for said facilities. |  |
| --- | --- | --- |

Why should you determine the temporary boundary fencing requirements for the site?

| Answer | Temporary boundary fencing restricts access to the site from unauthorised personnel, helps secure contain building materials and equipment, and prevent construction activities from affecting nearby areas. AS 4687 describes the standards for constructing and installing temporary fencing to protect the public and workers on site. |  |
| --- | --- | --- |

Why is it important to establish communication protocols for the site?

| Answer | It is important to establish communication protocols for the site to ensure the coordination of all construction activities from planning to the finished building delivery. |  |
| --- | --- | --- |

Discuss what must be determined when identifying waste removal requirements.

| Answer | When identifying waste removal requirements you must consider the types of waste, the volume and quantity, waste sources, legal and regulatory requirements, recycling opportunities, hazardous waste and the environmental impact. |  |
| --- | --- | --- |

What are the different costs to be considered when determining waste removal requirements for the site?

| Answer | * Disposal/dumping fees * Transportation * Equipment rental * Labour costs |  |
| --- | --- | --- |

Discuss additional project-specific statutory, approvals or compliance costs that must be met.

| Answer | After identifying the building site features, type of building, method of construction, building materials, and all the requirements the construction project must comply with, you should now identify the different statutory, approvals, or compliance costs that apply to the construction project. Statutory, approvals, or compliance costs may come from various sources, such as permit applications and fees. To identify what these costs are, you will need to refer to the building and construction regulations that the construction project must comply with or inquire at your jurisdiction's building administration |  |
| --- | --- | --- |

Discuss what a quantity take off is.

| Answer | The process of estimating how much of each building material is needed for a construction project can be done by looking at the construction drawings and specifications is called quantity take off. A list of the materials and consumables with the quantities of each required for the construction project is produced during quantity take off. The measurement of the materials, whether in number, length, volume, area, or weight, is also noted in the quantity list. |  |
| --- | --- | --- |

How can you make use of modern technology when producing a take off?

| Answer | Today's technology makes it possible to produce a materials and consumables quantity list by merely scanning construction drawings or inputting a project's specifications into software such as AutoCAD. Building materials calculators used to estimate the appropriate quantity of certain building materials have also grown in popularity to eliminate the need for manual calculations. Other digital technologies that aid in estimating quantities needed for a project include the following: Electronic take off It is possible to produce estimates of quantities with just the click of a button using certain digital software. Quantity take off done through digital means is called electronic take off. - Electronic bidding/estimate prep Some estimation software is synced with take-off systems so that quantities are automatically linked to labour assignments. With this technology, you can account for what-if scenarios and adjust estimates accordingly |  |
| --- | --- | --- |

Discuss how you would estimate the cost of precast concrete components.

| Answer | - Suppliers provide the materials and consumables for a construction project. They are often contracted by the organisation in charge of the construction project. The process of obtaining a price from the supplier may vary between organisations; however, it is often done by submitting a copy of the quantity list to them and asking for an estimate of how much each item in the list would cost. |  |
| --- | --- | --- |

Briefly discuss what is needed for the safe operation of the following physical resources at a current and active worksite in your state or territory.

| Answer | Physical resource | Requirement for operation |  |
| --- | --- | --- | --- |
|  | Bulldozer | 1.Operator Training and Certification 2.Pre-Operation Inspection 3.PPE 4. Rollover Protection System 5. Emergency Procedures |  |
|  | Excavator | 1.Operator Training and Certification 2. Pre-Operation Inspection 3.PPE 4.Falling Object Protection 5.Load Limits 6.Emergency Procedures |  |
|  | Hoist | 1.Operator Training and Certification 2.Pre-Operation Inspection 3.Load Capacity 4. Rigging and lifting equipment 5.PPE 6.Emergency Procedures 7.Follow regulations regarding hoist operations |  |
|  | Forklift | 1.Operator training and certification 2.Pre-operation inspection 3.Load capacity 4.Load placement and stability 5.PPE 6.Forklift attachments are adequately maintained |  |
|  | Mobile Crane | 1.Operator training and certification 2. Pre-operation inspection 3. Load capacity 4.Rigging and lifting equipment in good order 5.PPE 6.Clear communication 7. Operator positioning |  |

Discuss what you need for the safe and effective operation of a compressor, generator or a pump at the work site.

| Answer | A compressor, generator or pump needs: - A good location for plenty of ventilation - To be on stable ground with a level surface - Properly grounded - Regularly inspections - Clean air filters - Follow manufacturer’s maintenance schedule for routine checks and servicing - Set and monitor the pressure within safe limits to prevent equipment damage and ensure safety - Operator training |  |
| --- | --- | --- |

Describe how you can obtain the supplier prices for physical resources such as the hire of earthmoving equipment or smaller equipment such as generators.

| Answer | - Research and identify potential suppliers in your region - Consider their reputation, reliability and customer views - Phone or email, clearly communicating your requirements, including the type and duration of equipment rental - Prepare a detailed Request for Quotation (RFQ) that includes: - Equipment specifications, rental duration, delivery of pick up details - Negotiate terms and conditions - Check for additional costs - Compare quotes |  |
| --- | --- | --- |

Discuss how you confirm additional transport costs that may apply to the hire of plant and equipment.

| Answer | 1.Clearly define transport requirements 2.Request detailed quotations 3.Inquire about distance and accessibility 4. Verify fuel surcharges 5.Understand delivery options 6.Check for return transport costs 7.Ask for a written confirmation |  |
| --- | --- | --- |

Discuss what is meant by ‘overhead costs’ and why these must be included when calculating and estimating costs for a job.

| Answer | The materials, consumables, labour, and physical resources are not the only costs considered when producing the project's estimated cost. You must also consider the costs that your organisation makes when running their business, including rent for their office, utilities, supplies, time spent during and other things that cannot be directly added to the bill. These are called overhead costs. |  |
| --- | --- | --- |

How is an overhead margin calculated?

| Answer | . Organisations typically want to make an overhead margin to make up for these costs. They often apply a set mark-up percentage to their quoted prices called the overhead recovery rate to achieve this margin. The overhead margin is the percentage difference between the direct costs (i.e. material, labour, physical resources) and the total project cost that an organisation wants to meet to make up for its overhead costs. |  |
| --- | --- | --- |

Discuss risks associated with estimating project costs that you must manage and mitigate.

| Answer | Risks to estimating costs include unreliable data, over-optimism, and unrealistic assumptions. These risks may come from underestimating how much of a resource is needed to complete a project. For example if a task is estimated to only take 72 hours, but in fact takes 90 hours, the project may result in a loss instead of a profit. |  |
| --- | --- | --- |

Describe how you manage and mitigate the risks associated with estimating costs?

| Answer | These risks can be managed by following your organisation’s procedures for estimating project costs, as they are likely based on historical data from previous projects. Another way to manage risks associated with estimating project costs is to have a thorough understanding of the construction project. Understanding what the project needs, and how and when it needs them make for more precise data for a more accurate cost estimate. Understanding the risks associated with the construction project and contingencies required to put in place for these risks allows for a cost estimate that is more ready to accommodate variations that the project cost may take. |  |
| --- | --- | --- |

CPCCBC4004

Identify and produce estimated costs for building and construction projects

WORKBOOK

| Participant name | Justin Matheson |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written activities as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - The timeframe for these tasks will be established by your Assessor or Training Plan, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

NOTE

At times, you will be provided with a specific business context to which you must respond, otherwise it is anticipated that you will use a workplace of your choosing (your own, or hypothetical) to provide the context to the task.

At all times, the responses in your assessment must reference Australian (State/ Territory or Federal) documentation, including legislation, regulations, codes of practice, standards and guidance publications when required.

Access the internet and locate the project plan below: Case Study: Caloundra, Queensland After reviewing the project plan, answer the following questions: - What is the size of the land? - Based on the floor plan, how many bedrooms does the house have? - In which cardinal direction is the living room oriented? - What type of slab is required? - What type of building is shown for this project? - Specify the materials used to support the sustainability of this construction.

- What is the size of the land?

320m2

- Based on the floor plan, how many bedrooms does the house have?

3

- In which cardinal direction is the living room oriented?

North facing

- What type of slab is required?

Concrete slab

- What type of building is shown for this project?

Low density

- Specify the materials used to support the sustainability of this construction

Concrete slab, reverse brick veneer, lightweight construction, insulation, energy saving windows, energy-efficient appliances, solar photovoltaic system, rainwater tank.

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Following organisational procedure, you have contacted the appropriate supplier to enquire about the price to hire a bulldozer and an excavator for a construction project across a 30 day period. You have clarified workplace requirements that the supplier transport the plant from the supplier to the construction site and collect after use. The supplier has responded to with the following quote: Plant Hire; Cost /per day; Transport cost (Combined delivery and pick-up). 1 x Track-mounted excavator; $120.60; $234.50. 1 x Track-mounted bulldozer; $131.70. Complete the costing sheets below for each item.

| Plant for hire: | 1 x track mounted excavator |  |  |
| --- | --- | --- | --- |
| Timeframe: | 30 day period |  |  |
| Operational Costs: | $9455.00 |  |  |
| Associated Costs: |  | Cost per day of use | Total |
| Fuel |  | $42.60 | $1278.00 |
| Operator’s wage |  | $84.20 | $2526.00 |
| Maintenance |  | $18.00 | $540.00 |
| Hire costs |  | $158.70 | $4761.00 |
| Transport cost |  |  | $350.00 |
| Total cost |  | $9455.00 |  |

| Plant for hire: | 1 x track mounted bulldozer |  |  |
| --- | --- | --- | --- |
| Timeframe: | 30 days |  |  |
| Operational Costs: | $8711.00 |  |  |
| Associated Costs: |  | Cost per day of use | Total |
| Fuel |  | $42.60 | $1278.00 |
| Operator’s wage |  | $84.20 | $2526.00 |
| Maintenance |  | $18.00 | $540.00 |
| Hire costs |  | $133.90 | $4017.00 |
| Transport cost |  | $11.66 | $350.00 |
| Total cost |  | $8711.00 |  |

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Swayze Construction recently built a new bedroom as an extension to an already-existing bungalow. These are the specifications and features of the new room: - The room has a mat foundation that is 2 m deep. - The room has four masonry walls: - Two walls are 4 m wide and 3.5 m tall. - Two walls are 3 m wide and 3.5 m tall. - All walls are painted eggshell white. - The steel tray roofing is 20 m2 large. Laying the mat foundation cost $30,000.00. This includes labour and materials. Putting up all the masonry walls cost $3,500.00. This includes labour and materials. Laying the steel tray roofing cost $5,000.00. This includes labour and materials. Painting the masonry walls cost $2,000.00. This includes labour and materials. Given the information above, calculate for the following: - Cost of laying the foundation per metre of depth. - Cost of building the masonry walls per square metre. - Cost of laying the steel tray roofing per square metre. - Cost of painting the masonry walls per square metre. You must show the calculations to your answers. Round up your answers to the nearest dollar.

- Cost of laying the foundation per metre of depth

Square metre of footprint for foundation is 4m x 3m =12 square metres Depth of foundation is 2m. 12m2 x 2m = 24 square metres Laying foundation cost is $30,000 $30,000 divided by 24 square metres = $1250 per square metre

- Cost of building the masonry walls per square metre

Wall 4m x 3.5m x 2 = 28m2 Wall 3m x 3.5m x 2 = 21m2 Wall total is 49m2 Cost of total walls = $3,500 $3,500 divided by 49m2 = $71.42 per square metre

- Cost of laying the steel tray roofing per square metre.

Roof 20m2 Roof cost $5000 $5000 divided by 20m2 = $250 per square metre

- Cost of painting the masonry walls per square metre.

Total wall area – 49m2 Cost to paint wall - $2000 $2000 divided by 49m2 = $40.82 per square metre

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Swayze Construction recently built a new bathroom as an extension to an already existing bungalow. These are the specifications and features of the new bathroom: - The foundation slabs cover an area of 12 m2 - A total of 20 m of pipes were installed as part of the bathroom’s plumbing. - There are eight new pieces of sanitary ware in the bathroom: - One toilet - Two washbasins - Two sink taps - Two shower taps - One shower outlet - The floor area of the bathroom is 9 m2. All of the bathroom floor is tiled. - The bathroom has four walls. - Each wall is 3 m wide and 3 m tall. - Only half the height of the wall is tiled. - Laying the slabs cost $3,000.00. This includes labour and materials. - Installing the pipes cost $6,000.00. This includes labour and materials. - Installing all the sanitary ware cost $18,000.00. This includes labour and materials. - Tiling the tiled sections of the bathroom (i.e. all of the bathroom floor and half the height of each wall) cost $1,000.00. This includes labour and materials. Given the information above, calculate for the following: - Cost of laying of the slabs per square metre. - Cost of installing the pipes per square metre. - Cost of installing sanitary ware per unit. - Cost of tiling per square metre. You must show the calculations to your answers. Round up your answers to the nearest dollar.

- Cost of laying of the slabs per square metre

Total slab cost $3000 Foundation slab covers area of 12m2. $3,000 divided by 12m2 = $250 per square metre

- Cost of installing the pipes per square metre.

Pipes cannot be quoted on square metreage, so calculations below will be for cost per metre. 20 metres of pipe laid Total cost of pipes laid = $6000 $6000 divided by 20 metres = $300 per metre

- Cost of installing sanitary ware per unit.

There are 8 pieces of sanitary units. Total cost to install is $18000. $18000 divided by 8 = $2250 per unit

- Cost of tiling per square metre.

Total cost of tiling is $1000. Floor covers an area of 9m2. Walls are 3m x 1.5m x 4 = 18m2 Total square metreage of tiling = 27m2 $1000 divided by 27m2 = $37.04 per square metre

| Satisfactory Outcome |  |  |
| --- | --- | --- |

CPCCBC4004

Identify and produce estimated costs for building and construction projects

PRACTICAL ASSESSMENT

| Participant name | Justin Matheson |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel, unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 | 26/01/2024 | Workplace Simulation |  |  |  |

ASSESSOR COMMENTS: GENERAL

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 1 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Provide detail |
| 1.1A. | Specify drawings, specifications and plans reviewed for compliance: Student reviewed Revisions Schedule, making sure all plans were up to date. Site plans reviewed – includes all drawings and specifications that have been completed by engineers and architects and stamped for construction. |
| 1.2A. 1.3C. | Specify how supplier list was determined: Resourced product materials and consumables through the internet. Researched suppliers reputation for product and services. Took prices into consideration. |
| 1.2C. 1.2D. | Specify how contractors, sub-contractors and employees were selected: All contractors, sub-contractors and employees were selected on their experience, work ethics, attention to detail, consistency, and availability |

CPCCBC4004

Identify and produce estimated costs for building and construction projects

| Practical Task 1 |  |
| --- | --- |
| Description of task: The participant must demonstrate the ability and knowledge to effectively estimate the cost of one (1) residential OR commercial building and construction project, ideally, the task will be undertaken for a workplace project, using workplace documentation, however, own templates can be used for cost calculations. Some templates are available for use as needed if workplace documents are not available. The assessor will be looking for evidence of: estimate project costing from construction drawings, specifications and written information list physical resources appropriate for the construction of the project produce project cost using an appropriate software program. - Establish project requirements - Read construction drawings and plans in conjunction with local regulations, relevant codes and standards to determine and document compliance requirements. (template available – HO1) - Confirm site features, type of building and method of construction, and specified building materials. (template available – HO2) - Determine and document site facilities, communication and temporary fencing requirements. (template available – HO2) - Determine and document waste removal costs including waste management site fees. (template available – HO2) - Determine and document other site specific statutory, approvals, permits or compliance costs. (template available – HO2) - Calculate materials and labour costs. - Produce a materials and consumables quantity list and obtain a price from supplier. (template available – HO3) - Determine and estimate off-site production costs, including delivery. (template available – HO4) - Determine numbers of contractors, sub-contractors and employees appropriate for the project and work rates. (template available – HO5) - Estimate labour hours and calculate labour costs, including on-costs and worker insurance costs. (template available – HO5) - Identify type and cost of physical resources. - Determine physical resources required for the project. (template available – HO6) - Determine limitations, conditions, operational costs and timeframes for hire of physical resources. (template available – HO6) - Obtain supplier prices, including transport of physical resources. (template available – HO6) - Identify, itemise and cost plant, equipment and machinery. (template available – HO6) - Produce estimated project costs. - Review and document materials, consumables and off-site production costs and delivery charges. - Document labour costs inclusive of rates and entitlements. (template available – HO5, HO9) - Document costs of physical resources. (template available – HO6, HO9) - Apply organisational overhead recovery and margins. (excel template available- HO8,HO9) - Produce estimated project costs for inclusion in a tender or bill. (excel template available- HO8,HO9) - Manage and mitigate risks associated with estimating project costs. (template available – HO7) These tasks may be simulated or real. The Participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. The Observer is to provide detailed comments on the tasks completed by the Participant (on the Observer Comments, overleaf). The Assessor is to verify the Observer’s comments, noting their findings in the Assessor Comments (on the back of the Practical Assessment cover page) and provide the assessment outcome. |  |
| Resources required: Workplace documents including current construction drawings and specifications, HR contractor files and rates, insurance information, Access to PC to research compliance requirements from NCC, council, state legislation including WHS and environmental protection requirements. Access to take-off software or similar technology to achieve requirements. Templates (workplace or RTO). |  |
| Observer details |  |
| Name of observer: |  |
| Role of observer (Confirm suitability to sign) |  |
| Email address of observer |  |
| Signature of observer: |  |
| Date: | Select Date. |

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |
| --- | --- |
| 1 Establish project requirements. |  |
| Read construction drawings and plans in conjunction with local regulations, relevant codes and standards to determine and document compliance requirements. (template available – HO1) |  |
| Confirm site features, type of building and method of construction, and specified building materials. (template available – HO2) |  |
| Determine and document site facilities, communication and temporary fencing requirements. (template available – HO2) |  |
| Determine and document waste removal costs including waste management site fees. (template available – HO2) |  |
| Determine and document other site specific statutory, approvals, permits or compliance costs. (template available – HO2) |  |
| 2 Calculate materials and labour costs. |  |
| Produce a materials and consumables quantity list and obtain a price from supplier. (template available – HO3) |  |
| Determine and estimate off-site production costs, including delivery. (template available – HO4) |  |
| Determine numbers of contractors, sub-contractors and employees appropriate for the project and work rates. (template available – HO5) |  |
| Estimate labour hours and calculate labour costs, including on-costs and worker insurance costs. (template available – HO5) |  |
| 3 Identify type and cost of physical resources. |  |
| Determine physical resources required for the project. (template available – HO6) |  |
| Determine limitations, conditions, operational costs and timeframes for hire of physical resources. (template available – HO6) |  |
| Obtain supplier prices, including transport of physical resources. (template available – HO6) |  |
| Identify, itemise and cost plant, equipment and machinery. (template available – HO6) |  |
| 4 Produce estimated project costs. |  |
| Review and document materials, consumables and off-site production costs and delivery charges. |  |
| Document labour costs inclusive of rates and entitlements. (template available – HO5, HO9) |  |
| Document costs of physical resources. (template available – HO6, HO9) |  |
| Apply organisational overhead recovery and margins. (excel template available- HO8, HO9) |  |
| Produce estimated project costs for inclusion in a tender or bill. (excel template available- HO8, HO9) |  |
| Manage and mitigate risks associated with estimating project costs. (template available – HO7) |  |

Observer comments: