Building and Construction Regulations

| Regulation |  |
| --- | --- |
| Queensland Planning Act 2016 | Development Approval granted for the carrying out of "Building Work" in accordance with this Footings with allowable bearing pressure Footings will be placed centrally under walls and columns |
| Water Supply Services Legislation Amendment Act 2014 | It integrates land use planning and planning for water and sewerage infrastructure and includes a distributor-retailer’s connection policy. The Bill makes minor amendments to the current process for making or amending a water netserv plan, including clarifying requirements about public consultation and endorsement by participating local governments and the Planning Minister before its adoption by the distributor-retailer. |
| Building Act 1975 | It informs us what building work is accepted development, and the requirements as per Act. It provides who is responsible for carrying out the building assessment work and building applications It provides for the giving of final inspection certificates and inspection documentation It makes provision for fire safety It provides matters for swimming pool safety and fencing It provides for the designation of land as a transport noise corridor |
| Building Regulations 2021 | The process under the Building Regulation 2021 is that where an appointed competent person issues a Non-compliance Notice, the certifier must then follow-up and issue their own certificate and the builders obligation to ensure stage complies with development approval |
| NCC | A building, plumbing or drainage solution will comply with the NCC if it satisfies the Performance Requirements, which are the mandatory requirements of the NCC. The Performance Requirements are also supported by General Requirements. There are three ways to comply with the NCC's Performance Requirements. These include Performance Solution and Deemed-to-Satisfy (DTS) Solution |
| D Development code part 3.4 – pool safety barrier and gate | Australian Standard AS 1926.1 and AS 1926.2 and fence which is not a permanent structure, has at least one gate, otherwise complies with this part and is securely fixed to resist reasonably foreseeable actions to which it may be subjected. |
| Work Health and Safety Act | Including safety in design report covering: Safe material handling and lifting Handling hazardous material Compliance of materials to match specifications Reducing trip, slips, fall hazards Safe use of equipment to comply with procedures Safe and suitable equipment and PPE Formwork erection and use Scaffolding erection and its use Temporary bracing installation Excavations outside property boundaries or near services Worker training Use of suitably qualified tradesmen/women/contractors/subcontractors Safe undertaking of compliance inspections Ensure construction loads no exceeded |

State/Territory EPA Regulations

| Regulation |  |
| --- | --- |
| QDC MP4.4 Noise Category – Transport Noise | Construction elements must comply with the requirements set out in Table 4 of SEG report and to ensure habitable rooms of particular residential buildings located in transport noise corridors are designed and constructed to reduce the extent to which transport noise intrudes into those rooms. |
| Environmental protection regulation 2019 and Environmental Protection Act 1994 | It manages and monitors environmental risk through a range of assessment, compliance, investigation, enforcement activities and waste management to protect our environment and ensuring no harm to the nature and environment around and within the construction site. |

Performance Requirements

| Performance Requirements | Description of how it complies |
| --- | --- |
| Stormwater pipes | To be 90mm class 6 UPVC ; One 90mm UPVC pipe per 100 sqm of roof area laid to 1:100 min grade. |
| Pad cut | To allow ground water to drain away from dwelling all round at 1:20 fall. |
| Pool safety barrier and gate | Swimming pools must have a barrier which complies with Development code part 3.4 - pool: (a) is continuous for the full extent of the hazard; and (b) is of a strength and rigidity to withstand the foreseeable impact of people; and (c) restricts the access of young children to the pool and the immediate pool surrounds, including access from class 1, 2 or 3 buildings or class 4 parts of buildings located within or outside the pool area; and (d) has any gates fitted with latching devices not readily operated by young children, and constructed to automatically close and latch; and (e) except for indoor swimming pools, does not incorporate any doors providing access to or from a building. |
| Control joint | Articulation joints to be installed - positions are at 6m intervals starting at full height window or door. |
| Smoke detectors | To comply with NCC class 1 and class 10 parts 3.7.2 to AS 3786 and hard wired to electrical system by a qualified electrician and to ensure at least one smoke alarm is equipped on each storey of their homes where there is a room used as living accommodation |
| Fixed barrier screen or mechanical restrictors | installed to all openable windows to first floor to prevent falls |
| Truss Manufacturer | leave space for fan coil unit |
| M12 rods | 13kN capacity connection at max. 1200 crs |
| All lintels supporting girder trusses | to be certified by timber supplier |
| Wind category N2 – vertical bracing required | 79kN |
| Wind category N2 – horizontal bracing required | 62kN |
| Surface drainage - slab | External surface falls to be established and maintained to ensure at least 50mm fall over the first metre from dwelling. Gully pits or strip drains to be installed and connected to the stormwater system to collect surface water and direct to the legal point of discharge. |
| Plumbing requirements | Articulated/flexible joints are required |
| Footings | All footings to be founded a minimum of 200 mm into suitable founding material. |
| Internal point and distributed loads | Extra thickenings to be installed under internal point and distributed loads in additional to under ply bracing and similar fixings. |
| Residential loading | Residential Loading Only of 1.5 kPa in general and 2.5kPa for garage / shed. |
| Piers | For Piers >2m deep 1-N16 Bar and for piers >2m deep, 4-N12 bars with R6 spiral ligs at 300 crs |
| Minimum embedment into founding material | To be 500mm into dense sand; 500mm into stiff clay; 200 mm into weathered rock |
| Minimum pier reinforcement | To be no REO < 1000mm; 1-N16 <2000mm; 3-N!@ bars and R6 spiral ligs <3000mm; 4-N!@ bars and R6 sprial ligs < 4500mm |
| Council services | Min 1200 to council services |
| Private services | Min 450 to private services |
| Surface drainage | Min 50mm fall to be maintained for the first 1000mm from the dwelling perimeter |
| Plumbing | Minimum lagging 40mm Min expansion joint length 150mm Under slab vertical joints – nil Swivel jts – Y Allowable joint rotation 15 degrees Downpipe expansion joints Y Hose cock and AC unit pits Y |
| Rolled fill | Will not exceed : 600mm compacted in layers not more than 300mm for sand material or 300mm max rolled in 150mm layers for other materials |
| Clay fill | Should be moist during compacting |
| Prevention of shrinkage crack | Ensure max 20mm top cover to fabric reinforcement in order to minimise the cracks. Damage may be reduced by using flexible mortars and glues for fixing tiles. |
| Masonry walls | Min 10mm articulation joints to be installed to external masonry walls |
| Flexible masonry anchors | To be installed across the jt at every fourth course |
| Wall ties | To be installed 200mm each side of the joint at each third course to the timber frame behind |
| Articulation joint | To be installed at the position where wall heights changes by > 20%; at no more than 5000mm centres where openings are > 900sq occurs; where walls change in thickness; at control jts in footing slab; at junctions of walls with compressible material; within 300mm of vertical support element |
| Concrete surface cured | Cured for min 7 days commencing immediately after initial set. If curing compounds are used, they should be applied within 2 hrs of finishing concrete and for walls and columns, straight as per removal of formwork |
| Cement type | Type A, max size 20mm and slump 100mm |
| Surface drainage and slab | The finished height of any slab be a min of 150mm above the finished ground level after landscaping |
| Top of overflow relief gully | To be a min of 150mm below the lowest grate in the slab and 75mm above the finished ground level |
| Pathways, driveways and finished surface of any ground | To be graded min of 50mm away from any footing, slab or basement reating wall over th first meter |
| Drainage pits and pipes | Drainage pits to be installed to allow surface water to drain away in areas where the distance from a footing to a boundary is less than 1m |
| Retaining walls | To be installed at the base of cut and fill batters where batter slopes exceed 1:3 and where cutting below the base of an existing retaining wall |
| Visqueen lay | Lay 0.2mm visqueen or similar on min 50mm sand fill under slab min 225mm above ground |
| Brick veneer walls | Brick veneer walls to have ties in timber frame at 600 x 600crs |
| Plaster board | PB to walls 10mm and to ceiling 10mm unspan fixed. Villaboard to all wet areas |
| Mid rail height | Mid rail height to upper level windows to be 865mm above sill |
| External timber | All external timber to be min durability 1 & 2 or H3 treated and to be scaled to edge |
| Backfill | Backfill to fall away from building at min gradient 1:20 |
| Pipework grades on ‘M’, ‘H’, ‘E’ and ‘P’ sites | Should have a minimum grade of 1:30 within 15 m of the building and 1:60 elsewhere. Grades in flexible fittings to be set at the min grade |
| Frontage crossover max widths | Frontage >7.5m but <10m – 4m; Frontage <7.5m – 3.5m |
| Minor roads crossover | To be a min 10m from minor intersection; 20m from major intersection |

Standards

| Standards | Description of how it complies |
| --- | --- |
| AS 3660.0 parts A & B | Termite treatment is in accordance with AS 3660.0 parts A & B and allows for the use of integrated systems for total termite protection of the dwelling - for example, a monolithic slab could be incorporated with a penetration protection, say, graded stone, and the slab perimeter could be protected with stainless steel mesh. |
| BCC policy & BCA housing provisions | All cut/fill embankments, retaining walls shown and constructed are to comply. |
| NCC class 1 and class 10 parts 3.7.2 to AS 3786 | smoke detectors hard wired to electrical system by a qualified electrician and to comply with NCC class 1 and class 10 parts 3.7.2 to AS 3786. To comply with NCC class 1 and class 10 parts 3.7.2 to AS 3786 and hard wired to electrical system by a qualified electrician and to ensure at least one smoke alarm is equipped on each storey of their homes where there is a room used as living accommodation |
| AS1684 - Table 8.18 (h) | Value 6.4kN/m @ min 600mm |
| AS1684 - Table 8.18 (h) | Value 6.0kN/m @ min 900mm |
| NCC 2019 Volume Two - Part 3.5.3 - Gutters & Downpipes | 40sqm catchment area per downpipe - Slotted with min 1200mm. opening per metre of gutter |
| NCC vol 2 - Table 3.5.3.2a - Gutters | 40sqm catchment area per downpipe - Gutter size and downpipe selection - A or D being 255mm/h |
| NCC vol 2 - Table 3.5.3.2b - Gutters | A- Medium rectangular min 6500mm. or D - 125mm D gutter min 6300mm. |
| NCC vol 2 - Table 3.5.3.1d - Downpipes | Average recurrence interval 20 Years (Brisbane - Victoria Point) - 234mm/h - 245mm/h |
| NCC vol 2 - Table 3.5.3.2c - Downpipes | A - (75mm dia, 100mmx50mm, 90mm dia, 100mmx75mm) D - (75mm dia, 100mmx50mm, 90mm dia, 100mmx75mm) |
| NCC vol 2 – overflow - Table 3.5.3.1d | Average recurrence interval 100 Years (Brisbane - Victoria Point) - 305mm/h - 320mm/h |
| NCC vol 2 – overflow - Table 3.5.3.3a | Overflow Volume 325mm/h |
| NCC vol 2 – overflow - Table 3.5.3.4a | < 5m Ridge to gutter length use front face slotted > 5m Ridge to gutter length use controlled back gap |
| Section 3 AS2870-2011 | Class P - Design Slab Class - H1. Pier Founding Material (PFL) is Very Stiff Clay >150kPa. Edge Beam Founding Material is Silty Sand or Silty Clay. |
| AS3500 | External plumbing (notes available under relevant performance requirements) |
| AS 2870-2011 | Fill used in the construction of slab to ground will consist of controlled fill or rolled fill |
| AS4773.1 | Table 13.1.2 and 13.1.3 for maximum joint spacing for wall construction where opening >900m, 5m spacing; straight wall >12m, 8m spacing |
| AS3600 | Concrete material and works in accordance to: slab internal 20mm top and 50 bottom slab external 30mm top and 50 bottom footings 65mm beams internal 30mm top and 50 bottom slab on ground 20mm top and 50 bottom internal 20mm top and 50 bottom external 30mm top and 50 bottom perimeter beam 50mm in acid sulphate soil |
| NCC vol 2 surface drainage and slab | The finished height of any slab be a min of 150mm above the finished ground level after landscaping |
| AS/NZ3500.2-2003 | Top of overflow relief gully be a min of 150mm below the lowest grate in the slab and 75mm above the finished ground level |
| AS/NZS 3500.3-2003 | Pits and Pipes to be constructed in accordance to: Drainage pits to be installed to allow surface water to drain away in areas where the distance from a footing to a boundary is less than 1m |
| AS2890.1-2004 | Finished floor level of any garage or carport will be established to allow driveway slopes complying to. |
| AS/NZS 3500 part 5 – stormwater systems | Storm water systems that collect roof water and surface water are designed and constructed in accordance with this. |
| AS 1280 and AS1415 | Expandable joint and swivel specs to be manufactured and comply with this and to be installed accordingly. Drainage pipes founded within the filled section of the building pad under the slab are to be hung from slab reinforcement with stainless steel straps. |
| AS 3500 Sewer pipes | Sewer pipes founded within uncontrolled filled under the slab are to be hung from slab reinforcement with metal straps or similar |
| AS 1684-2 | All timber framing to comply with this covering the design, building practice, tie-downs, bracing and span. (refer to Performance requirements table) |
| AS 3600 | Perimeter and penetration to comply with this |
| BCA part 2.9.2 ? NCC part 3.9.2 | Balustrades and handrails to comply with this. Where an alternative barrier is proposed as a Performance Solution to that described in Part 3.9.2, that proposal must comply with— Performance RequirementP2.5.2; and the relevant Performance Requirements determined in accordance with A2.4(3). Where an alternative handrail is proposed as a Performance Solution to that described in Part 3.9.2, that proposal must comply with— Performance RequirementP2.5.1(b)(i); and the relevant Performance Requirements determined in accordance with A2.2(3) and A2.4(3) as applicable. |
| AS 1604 | All external timber to be min durability 1 & 2 or H3 treated in accordance with this |
| AS 1684 | All H3 treated material to be scaled to edge |
| AS 1684 table 8.18 | Type D bracing capacity 3kN/m |
| AS 1684-2 clause 8.3.6.9 | All internal bracing walls to be fixed to the ceiling or roof frame and/or to the external wall frame |
| AS 2870 section 5, clause 5.6 and section 6, clause 6.6 | For slab and strip footings on highly and extremely reactive sites the following to apply: Drains attached to or emerging from underneath the building shall incorporate flexible joints immediately outside the footing and commencing within 1m of the building perimeter to accommodate a total range of differential movement in any direction equal to the estimated characteristic surface movement of the site. In the absence of specific design requirements, the fittings or other devices that are provided to allow for the movement shall be set at the mid position of their range of possible movement at the time of installation, so as to allow for movement equal to 0.5ys in any direction from the initial setting. this requirement applies to all stormwater and sanitary plumbing drains and discharge pipes. |
| AS 3500 clause 4.10 | Plumbing and drainage under the slab should be avoided and Branch drains connected to jump-ups shall be independently supported where the unsupported length exceeds 500 mm. |

Building Site Features

Boundary of the site as outlined and site area 538sqm and will have retaining wall at the right side of the property, about half of the back boundary and around the pool.

Site is surrounded by neighbour houses of which two are existing highest houses and two are existing lowest houses

House adjacent to main road and subject to Transport Noise

P Power pole on left side of property next to main road

Contours ranging from 29.75 to mostly 30 along the driveway and 30.50 from front left side of the proposed site gradually sloping towards the back lefty side to 29.75, therefore being a slight sloping land.

E Entrance to the house is West facing

Specifications of materials to be used

Timber

Bricks

Steel beams

Timber posts – 90 x 90

St Steel posts 75 x 75 x 3.2

Truss

Tiles - floor

Tiles - wall

Tiles - splashback

Colorbond roof sheeting

Aluminium balustrade

Cladding

Plasterboard

Support posts

Floor sheet

Ply brace sheet 600mm Direction X kN3.8

Ply brace sheets 700mm Direction X kN 4.5

Ply brace sheets 900mm Direction X kN 27

Ply brace sheets 2700mm Direction X kN 48.6

Ply brace sheet 600mm Direction Y kN 3.8

Ply brace sheets 900mm Direction Y kN 27

Ply brace sheets 1800mm Direction Y kN 43.2

Ply brace sheets 900mm Direction X kN 32.4

Ply brace sheets 700mm Direction Y kN 4.5

Ply brace sheets 900mm Direction Y kN 21.6

Ply brace sheets 1800mm Direction X kN 10.8

Filler block

PFC board 250

SHS Post 75 x 75

Floor beam

Downpipes

Gutters

Wall ties

Foam backing Rod 10mm

Stud frame

Architrave

Masonry expansion tie

Articulation joints

Edge beam

Internal beam

Common beam

Patio beam

Widened rib

Strip footing

Fill Piers 450 dia

Tree Piers 450 dia

Service Piers 450 dia

SL92 Mesh 20 top cover 300 pods

SL92 Mesh 40 top cover 600 pods

2-N12 ‘Z’ bars

Underslab bedding

Trench mesh

N12 corner bars – 600 legs

Steel roof battens

Top plates 2/35 x 70 MGP12

Colourbond roof sheeting

Cyclone strap

Trusses

Gyprock ceiling

Gyprock wall lining

Sarking

Metal fascia and gutter

Cornice

FC sheets

Timber beading 42 x 11

Cladding

Breather foil

DPC

Brick ties

R1.5 Batts

Skirting

Particle board flooring

Floor joists

Studs (load bearing) 70 x 35 MGP12

Studs (Non loadbearing) 70 x 35 MGP10

Bottom plate on slab 35 x 70 MGP12

Bottom plate on floor joists 2/35 x 70 MGP12

Noggings

Lintel

1/10 55mm Roof Zip Batten screw

2/10 55mm Roof Zip batten screws

1/10mm powers screw bold

900mm plywood sheet

Villaboard TB3A

Steel angle brace

4.5mm F11 or 7mm F8 structural ply

Downpipes

ConcreteSL72 welded mesh

Turf

Swivel joints

PVC joist jackets

6 mm backing rod

Fibre reinforced waterproof membrane

10 x 20 aluminium angle

Nails 30 x 3.14 diameter flat head

Nailing plates

30mm x 2.8mm dia nails

30mm x 0.8mm tensioned galvanized metal strap

Galvanized flat head nails

Screw bolts

Backfill

Termite barrier

Garage door

M12 bolts

Vanity

Mirror

Bathtub 1500

Bathtub 1700

Undermount kitchen sink

900 canopy rangehood

Upright cooker

Hanging rail

Glass for balustrade

AC duct shaft

Window

Doors

flyscreen

Temporary Boundary Fencing Requirements

All boundary clearance and cut out dimensions to be verified prior to commencement of work

Site sign to be installed on the site boundary

Fencing the site off with temporary fencing and allow access to only authorized personnel

Do not place or stack materials close to fence lines or fences

Temporary fencing around excavation areas to prevent unauthorized personnel from entering

Pool safety fencing

Workplace Safety Requirements

Safe material handling and lifting

Handling hazardous materi8al

Compliance of materials to match specs

Reducing trip, slip and fall hazards

Safe use of equipment to comply with procedures

Safe and suitable equipment

Formwork erection and use

Scaffolding erection and its application

Temporary bracing installation procedures

Excavations – site prep, clearing, levelling before construction; soil testing ground design before construction

Excavations outside property boundaries

Excavation near services

Ensure construction loads not exceeded

Pool safety barrier

Waste Removal Requirements

Rubbish removal – loading trucks, waste bins and rubbish skips with demolition waste

Waste Removal Procedures

Ensure you identify waste areas for collection, movement, storage and disposal of the waste for safety purposes and all understand this

Progressively remove the waste to prevent build up and ensure ground conditions are maintained as required

Entry and exit points to be clean and clear of demolition waste and not stacked near the excavation site or entry/exit points

Ensure vehicle and trucks and material loads are accessing with safety measures in place for the people and vehicle movements and material loading/offloading.

Always ensure the underground services are not affected

Excavated material should be placed in such a manner that it channels rainwater and any other run-off water away from the excavation site.

Identify waste bins , rubbish or recycling bins and dedicated waste placement points

Ensure you identify waste areas for collection, movement, storage and disposal of the waste for safety purposes and all understand this

Progressively remove the waste to prevent build up

Entry and exit points to be clean and clear of materials and waste

Ensure construction materials not allowed to fall freely unless in a confined space eg) chute

Ensure the bins are emptied and replaced as required

SUBMISSION OF APPLICATIONS PLAN

We identified the project stages that require approval from local authority and dates for submission and below is the process undertaken for the records:

The project stages in a residential build that require approvals may vary depending on the specific location and regulations, but the following are the stages undertaken:

- Planning Approval: Before any construction work can begin, we need to obtain planning approval from the local council or relevant government authority. This approval will consider factors such as the proposed building's size, location, and impact on the environment and surrounding community.

- Building Approval: Once you have received planning approval, you will need to obtain building approval from the local council or a private certifier. This approval will consider whether the building complies with relevant building codes and standards.

- Plumbing Approval: We would need to obtain approval for any plumbing work that is being carried out as part of the construction process. This approval will ensure that the plumbing work is compliant with relevant standards and regulations.

- Electrical Approval: Similarly, we need to obtain approval for any electrical work that is being carried out as part of the construction process. This approval will ensure that the electrical work is compliant with relevant standards and regulations.

Submission Plan:

To create a submission plan for these approvals, we considered the following steps:

- We Identified the specific approvals that are required for the project based on the regulations in QLD.

- Researched the requirements and criteria for each approval and determined the necessary documentation that needs to be submitted.

- Created a checklist of the documents required for each approval.

- Developed a timeline for the submission of each approval, considering the time required for processing and any potential delays.

- Ensured that all documentation is completed accurately and in compliance with the relevant standards and regulations.

- Submitted the documentation to the relevant authorities as per the timeline, and followed up to ensure that the approvals are processed in a timely manner.

- Once all the necessary approvals are obtained, we can begin the construction process according to the approved plans and standards.

Submission plan:

A submission drawing, also known as an architectural drawing, is a technical drawing of a building (or any project) that falls under the definition of Design or architecture Design.

The following items are included in a submission/passing drawing:

- Every Floor Plan.

- One section is required.

- One elevation is required.

- Site map.

- The main strategy

- North is the direction.

- Calculation of the built-up area.

Stages and Dates for Submission of Applications:

- Pre-application Stage: This is the initial stage where you can obtain preliminary advice and information on the application process and requirements from the local council or private certifier. 5/1/2023

- Application Stage: The application stage involves submitting the completed application form and supporting documents to the local council or private certifier. The timeframe for this stage can vary depending on the complexity of the application and the local council's or private certifier's workload. 5/2/2023

- Assessment Stage: The assessment stage involves the council or private certifier assessing the application against the relevant planning scheme and building codes. This stage may take several weeks or months, depending on the complexity of the application. 25/2/2023

- Decision Stage: The decision stage involves the council or private certifier making a decision on the application. This stage may take up to 20 business days for a private certifier or up to 40 business days for a council. 5/3/2023

Get building approval

To get building approval, we need to do the following:

- Assign a licensed building surveyor to serve as your certifier.

- Fill out and submit the necessary forms.

- Pay the applicable fees.

We provided the certifier with plans that adhere to the relevant codes and design guidelines.

Commencement of building work

- Before starting construction, we must apply to the building certifier for a commencement notice and sign the commencement notice application form.

- Construction work can begin once the certifier has issued a commencement notice and all other necessary approvals and requirements are in place.

Conditions of building

We must be aware of the following:

- any project-related conditions in the notice of decision about your development approval, such as that certain conditions must be met before any construction or building work can begin.

- any lease provisions