CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4002A Manage occupational health and safety in the building and construction workplace cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | FEI XU |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | FEI XU |
| --- | --- |
| Participant signature |  |
| Date | 3/08/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 28 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

For each of the common building and construction jobs listed below, identify one (1) hazard specific to that job.

| Answer | Job | Risks |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Lacerations/amputation due to contact with blade |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | eye damage can be caused by lasers |  |
|  | Crane use | Falling goods can hurt or kill |  |
|  | Demolition | Asbestos breath-in |  |
|  | Formwork | Sunburn |  |
|  | Scaffolding use | Falling from heights |  |
|  | Work at heights | Falling from heights |  |
|  | Welding | Burns |  |

Using the risk table below, determine the level of risk for each of the hazards identified in Q1. Identify how each risk level was determined, by identifying the likelihood and consequences of each hazard.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

| Answer | Job | Level of risk |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | H Level, Lost time injury and Very likely |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | E level, Death or permanent disability and Likely |  |
|  | Crane use | H level, Death or permanent disability and Possible |  |
|  | Demolition | H level, Serious injury and Likely |  |
|  | Formwork | M LEVEL,Serious injury and Likely |  |
|  | Scaffolding use | E Level, Death or permanent disability and Very likely |  |
|  | Work at heights | E Level, Death or permanent disability and Very likely |  |
|  | Welding | C Level, Serious injury and Very Likely |  |

Identify an appropriate control measure for each of the hazards identified in Q1.

| Answer | Job | Control measure |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Ensure guards are fitted and working |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Ensure that the workers operating the laser equipment are trained in the proper operation of the equipment. |  |
|  | Crane use | fall prevention devices applied |  |
|  | Demolition | Better training and wearing masks |  |
|  | Formwork | SPF 50+ rated work clothes, hat, sunglasses, sunscreen apply |  |
|  | Scaffolding use | Do not overload a scaffold beyond its maximum load capacity |  |
|  | Work at heights | Scaffolding, edge protection, harness |  |
|  | Welding | Storing flammable solvents well away from welding areas |  |

True or False (Tick your response): Risk control options can come from both internal sources (e.g. policies and procedures) and external sources (e.g. codes and standards).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Name two (2) workplace personnel you may speak to for safety advice.

| Answer | RISK MANAGER HEALTH PROFESSIONALS |  |
| --- | --- | --- |

Referring to the WHS Regulations, explain how the following documents contribute towards identifying hazards and risks in the workplace.

| Answer | Document | Description |  |
| --- | --- | --- | --- |
|  | Safe Work Method Statements | - identify the work that is high risk construction work; and - state hazards relating to the high risk construction work and risks to health and safety associated with those hazards; and (c) describe the measures to be implemented to control the risks; and (d) Describe how the control measures are to be implemented, monitored and reviewed. |  |
|  | Site Safety Plan / WHS Management Plan | - the names, positions and health and safety responsibilities of all persons at the workplace whose positions or roles involve specific health and safety responsibilities in connection with the project; - the arrangements in place, between any persons conducting a business or undertaking at the workplace where the construction project is being undertaken, for consultation, cooperation and the coordination of activities in relation to compliance with their duties under the Act and this regulation; - the arrangements in place for managing any work health and safety incidents that occur; - any site-specific health and safety rules, and the arrangements for ensuring that all persons at the workplace are informed of these rules; (e) the arrangements for the collection and any assessment, monitoring and review of safe work method statements at the workplace. |  |

Identify two (2) advisory standards you might refer to safely undertake building and construction work and briefly summarise their requirements.

| Answer | Standard name | Requirements |  |
| --- | --- | --- | --- |
|  | AS/NZS 1200:2000 Pressure equipment | This Standard is a ‘parent’ document for use by the pressure equipment industry in Australia and New Zealand. It covers the design, materials, manufacture, examination, testing, installation, conformity assessment, commissioning, operation, inspection, maintenance, repair, alteration and disposal of pressure equipment (boilers, pressure vessels and pressure piping), but excluding gas cylinders and other equipment as set out in Appendix A. This Standard specifies detailed requirements for various pressure equipment by direct reference to a range of Australian, New Zealand and other Standards. This Standard also sets out general administrative and interpretation provisions which apply to the specified Standards. |  |
|  | AS/NZS 1269.1:2005 Occupational noise management | This Standard sets out requirements for, and provides guidance on, the types of noise assessments which may be necessary and suitable noise measuring instruments to carry them out. The procedures for noise measurement are also included. The type and detail of assessments carried out will depend on the use that is to be made of the information obtained. The type of measuring instruments and procedures to be used for the measurement of noise levels will depend on such factors as the type of workplace, type of noise, work patterns and information required. |  |

Building and construction industry contracts require building legislation and the National Construction Code to be observed. Refer to these documents when responding to Q8. to Q10.

Identify the requirements in the National Construction Code Volume 1 with regards to air pressurisation for fire-isolated exits. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | E2.2 General requirements | AS 1668.1 |  |

Identify the requirements in the National Construction Code Volume 2 with regards to smoke alarms. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | 3.7.5.2 Smoke alarm requirements | AS 3786 AS 1670.1 |  |

Name the building legislation (Act and Regulations) that apply within your jurisdiction.

| Answer | Queensland Building and Construction Commission Act 1991 Queensland Building and Construction Commission Regulation 2018 |  |
| --- | --- | --- |

Which of the following should be considered when developing a risk control plan? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | How and when controls will be implemented |  |
|  |  | Evaluation processes for implemented controls |  |
|  |  | Consultation with individuals and parties |  |
|  |  | Responsibilities for implementation |  |

True or False (Tick your response): If control measures do not adequately control risks, they must be revised.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following are appropriate techniques, tools and processes for identifying hazards and risks? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Workplace inspections |  |
|  |  | Compensation claim information |  |
|  |  | Safe Work Method Statements |  |
|  |  | Information issued by WHS regulators |  |

Which of the following can be used to assist with workplace inspections? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Hazard and risk checklists |  |
|  |  | Job Safety Analysis (JSA) |  |
|  |  | Manifests and registers |  |
|  |  | Surveys |  |
|  |  | Workplace walk-through |  |

Which of the following are limitations of using a generic hazard/risk assessment checklist? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Specifics of the workplace are not considered |  |
|  |  | Not all checklist points will be applicable |  |
|  |  | Such checklists/processes must be completed on a computer |  |
|  |  | You always have to purchase these checklists |  |

Identify the applicable WHS Act in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Act name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and Safety Act 2011 103B | identify hazards and risks to health and safety arising from the work carried out as part of the conduct of the business or undertaking |  |

Identify the applicable WHS Regulation in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Regulation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and Safety Regulation 2011 s34 Duty to identify hazards | A duty holder, in managing risks to health and safety, must identify reasonably foreseeable hazards that could give rise to risks to health and safety. |  |

Identify a relevant policy in your organisation with regards to identifying hazards and risks and briefly describe its requirements.

| Answer | Policy name | Requirements |  |
| --- | --- | --- | --- |
|  | How to manage work health and safety risks Codeof Practice 2011 | Identifying hazards in the workplace involves finding things and situations that could potentially cause harm to people. Hazards generally arise from the following aspects of work and their interaction:  physical work environment  equipment, materials and substances used  work tasks and how they are performed  work design and management. |  |

Identify the statutory requirements in your jurisdiction with regards to the completion of inspection reports.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and Safety Act 2011, Par 5A, 103D | Assessment reports (1) The work health and safety officer must— (a) prepare a report (an assessment report) for the assessment carried out under section 103D(1); and (b) if the assessment identifies any risks to health and safety arising from the work carried out as part of the conduct of the business or undertaking—include in the report recommendations about managing the risks. (2) The work health and safety officer must, within 30 days after the assessment has been carried out, give a copy of the assessment report to— (a) if the work health and safety officer is not the person conducting the business or undertaking—the person conducting the business or undertaking; and (b) if there is a health and safety committee at the business or undertaking—the committee. |  |

Which of the following individuals and parties may require information about workplace hazards and proposed risk controls? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Health and Safety Committees |  |
|  |  | PCBU |  |
|  |  | All members of the public |  |
|  |  | Workers |  |

Identify the requirements in the Code of Practice with regards to consulting parties when managing risks.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety consultation, co-operation and co-ordination, Code of Practice 2011,S 2.1 | Consultation is required when identifying hazards, assessing risks and deciding on measures to control those risks. In deciding how to control risks, you must consult with your workers who will be affected by this decision, either directly or through their health and safety representative. Their experience may help you identify hazards and choose practical and effective control measures. Regularly walking around the workplace, talking to your workers and observing how things are done will also help you identify hazards. Conducting a survey of your workers can provide valuable information about work-related health issues such as workplace bullying, stress, as well as muscular aches and pains that can signal potential hazards. Workers and their health and safety representatives may need access to information such as technical guidance about workplace hazards and risks (plant, equipment and substances). Information should not be withheld just because it is technical or may be difficult to understand. The WHS Act requires that you allow any health and safety representative for a work group to have access to information you have relating to hazards (including associated risks) affecting workers in the work group and also any information about the health and safety of workers in the work group. This does not extend access to any personal or medical information concerning a worker without the worker’s consent. Further guidance on risk management is available in the How to manage work health and safety risks Code of Practice |  |

Identify the requirements in the Code of Practice with regards to consulting parties when developing procedures for communicating WHS policy and practice requirements.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety consultation, co-operation and co-ordination, Code of Practice 2011,S 2.4 | A procedure sets out the steps to be followed for work activities. You must consult with affected workers when developing procedures for: resolving work health and safety issues consulting with workers on work health and safety monitoring workers’ health and workplace conditions providing information and training. Procedures should be in writing to provide clarity and certainty at the workplace and assist in demonstrating compliance. They should clearly set out the role of health and safety representatives, and any other parties involved in the activity. The procedures should be easily accessible, for example by placing them on noticeboards and intranet sites. If issue resolution procedures are agreed to, the WHS Regulation includes minimum requirements including that these procedures are set out in writing and communicated to all workers to whom the procedure applies. |  |

Name three (3) parties who should consult to determine effective strategies for communicating WHS policy and practice.

| Answer | Health and Safety Representatives Designated work groups Health and safety committees |  |
| --- | --- | --- |

Name two (2) communication strategies that may be used in the building and construction industry.

| Answer | Issued site specific instructions and signage Verbal communications |  |
| --- | --- | --- |

Name two (2) education programs that may be implemented in the building and construction industry.

| Answer | General and site-specific induction training Other forms of specialist and targeted training |  |
| --- | --- | --- |

Fill in the blanks for the statement below:

consulted

training

The ‘How to manage work health and safety risks Code of Practice 2011’ identifies that control measures are more effective when ______________________ is provided and when workers are _____________________.

| Answer |  |  |
| --- | --- | --- |

Explain why it is important to review the effectiveness of communications and educational programs.

| Answer | After you have established appropriate education and training programs for staff, it is important to review them. Should communications and/or training have taken place, yet WHS hazards/risks are unaffected (i.e. not reduce in likelihood of occurrence), then they must be reviewed. |  |
| --- | --- | --- |

List three (3) methods for reviewing the effectiveness of communication and educational programs.

| Answer | Evaluating training programs for effectiveness Ensuring employees have an adequate understanding Conducting regular reviews of training needs |  |
| --- | --- | --- |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Fei XU |
| --- | --- |
| Participant signature |  |
| Date | 8/03/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

Fei XU Is capable of cconducting a workplace inspection and seeking expert advices when needed. He ccompleted an inspection report by himself and monitored and reviewed control measures, updating related records with any new findings. He also established communication strategies and educational programs in accordance with statutory requirements and best practice. Fei can imple agreed control measures in conjunction with relevant workplace personnels His English is good and he can effectively communicate by using verbal languages to team mate who are from different culture backgrounds

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to conduct and report on a WHS risk analysis, including workplace inspection. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - A copy of a Site Specific Safety Plan or Construction Safety Plan, including: - Copies of Risk Assessments completed on a project - Copies of Safety Work Method Statements used on a project - A copy a workplace safety inspection checklist that has been used on a project The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | FEI XU |  |  |
| Name of observer: | ROBIN |  |  |
| Role of observer (Confirm suitability to sign) | Project Manager |  |  |
| Email address of observer | BRAVOUGG@GMAIL.COM |  |  |
| Signature of observer: |  |  |  |
| Date: | 8/03/2017 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Conduct a workplace inspection to evaluate construction site safety for the range of occupations in the workplace according to WHS legislation and company policies, including: - Identifying construction hazards and potential risk areas - Identifying and prioritising hazards - Identifying and prioritising risks |  |  |  |
| - Seek expert advice as appropriate, including: - Advice from workplace personnel regarding the application of building codes and standards - Consulting with relevant parties regarding compliance with statutory requirements (including SWMS and site safety plans) - Consulting with appropriate personnel to determine effective strategies for communicating WHS policy. |  |  |  |
| - Complete an inspection report in accordance with best practice and statutory obligations: including: - Recommendations from findings - Required approaches to remediation |  |  |  |
| - Implement agreed control measures in conjunction with relevant workplace personnel, implementing negotiation and conflict resolution skills, as necessary. |  |  |  |
| - Monitor and review control measures, updating records with findings |  |  |  |
| - Establish communication strategies and educational programs specific to the building and construction industry in accordance with statutory requirements and best practice. |  |  |  |
| - Review the effectiveness of communication and educational programs, updating records with findings. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication |  |  |  |
| The Participant’s performance was:  Satisfactory Not Satisfactory |  |  |  |

observer Comments

Fei XU Is capable of cconducting a workplace inspection and seeking expert advices when needed. He ccompleted an inspection report by himself and monitored and reviewed control measures, updating related records with any new findings. He also established communication strategies and educational programs in accordance with statutory requirements and best practice. Fei can imple agreed control measures in conjunction with relevant workplace personnels His English is good and he can effectively communicate by using verbal languages to team mate who are from different culture backgrounds..