CPCCBC4006B

Select, procure and store construction materials for low rise projects

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Everett Putney |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | Click here to enter a date. |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 32 |  | Yes | No |

Assessor Comments

- This assessment reflects on what is required to supervise the systems through which materials are typically selected, acquired and stored on site for projects described by the Building Code of Australia (BCA) as low rise building or construction work (low rise' licensing classification with reference to Class 1 and 10 construction and Class 2 to 9 with a gross floor area not exceeding 2000M², not including Type A or Type B construction). It ensures the delivery to the site of materials that meet contract specifications and service requirements for low rise projects.

Who is responsible for supervision on a building construction site?

| Answer | Builder |  |
| --- | --- | --- |

Who is then responsible for supervision when a Builder has a Construction Manager on site?

| Answer | Construction Manager and Builder |  |
| --- | --- | --- |

Who is responsible to supervise and coordinate tradespeople working on site?

| Answer | Site Foreman |  |
| --- | --- | --- |

Identify ten (10) materials commonly used for low rise buildings in your region (with slab on ground, prefabricated timber wall frames and roof trusses, brick veneer construction, plasterboard linings and texture coated external finish over brickwork along with metal or concrete tiled roof). 

For each material, identify the following:

- Its name

- A supplier for this material

- Properties of the material

| Answer | # | Material name | Supplier | Properties |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | concrete | HyTec | Structural |  |
|  | 2 | Linea cladding | Hudson/Mitre 10 | External light weight |  |
|  | 3 | Matrix cladding | Hudson/Mitre 10 | External light weight |  |
|  | 4 | Exotec cladding | Hudson/Mitre 10 | External light weight |  |
|  | 5 | Glass/glazing | Axis Glass | Windows/balustrades |  |
|  | 6 | blockwork | Adbri | Structural, feature walls |  |
|  | 7 | Fibre Cement | Boral | Internal linings |  |
|  | 8 | Structural timber | Hudson/Mitre 10 | structural |  |
|  | 9 | Steel frames | Rondo | Light gauge steel, non structural |  |
|  | 10 | Render | GMF | External wall covering |  |

For each of the materials in Q4. identify the following:

- Quality of the material (i.e. safety, required fire resistance rating, suitability to the application, durability, serviceability, cost effectiveness and compliance with Australian Standards)

- Compatibility and non-compatibility of the material (i.e. what it would/would not be suitable for)

| Answer | # | Quality | Compatibility |  |
| --- | --- | --- | --- | --- |
|  | 1 | Fire rated, suitable for slabs, insitu walls, cost effective and best option for the above due to its strength. | Not suitable for partition walls, standalone roofs without structural intent for example or anywhere with excessive tensile strength unless structurally engineered to do so with effective reinforcement layouts |  |
|  | 2 | Useful in most build types however has differing fire resistance ratings for each class. Very suitable for townhouse & medium rise builds for external light weight use | Not suitable for internal use unless in a feature. Not necessarily cost effective to do so. |  |
|  | 3 | Useful in most build types however has differing fire resistance ratings for each class. Very suitable for townhouse, medium & high rise builds for external light weight use | Not suitable for internal use unless in a feature. Not necessarily cost effective to do so. |  |
|  | 4 | Useful in most build types however has differing fire resistance ratings for each class. Very suitable for townhouse, medium & high rise builds for external light weight use | Not suitable for internal use unless in a feature. Not necessarily cost effective to do so. |  |
|  | 5 | Suitable for balustrading, external windows, shower screens and robe doors depending on the finish. No fire rating however can be made with energy efficient ratings and stronger wind ratings | Not suitable for anything other than mentioned to the left |  |
|  | 6 | Can be fire rated (15.801 blocks). Mostly used for external heavy weight constructions or internal fire rated parti walls. Durable provided structurally engineered with appropriate block fill and steel) | Can be used in lieu of light weight construction however can be more expensive and provide a ‘dull’ building face |  |
|  | 7 | Can be fire rated to a certain point. Mostly rated in use as a whole system i.e. stud, insulation, 10mm plasterboard etc. | Suitable for mostly internal & external wall linings as well as wet areas and tile underlays. Soffits can sometimes be lined with external FC |  |
|  | 8 | Mostly suitable for internal frames, trusses, girders, pole plates etc. non-fire rated yet termite rated to an extent | Non suitable for areas heavily exposed to weather i.e. not covered, lined, painted etc. |  |
|  | 9 | Suitable for internal framing/infill framing. Fire rated. Can be more expensive than timber yet has a stronger acoustic rating. Never needs termite treatment | Suitable for all structural timber uses |  |
|  | 10 | Mostly to block/Brick walls to lift the appearance. Sometimes prone to cracking | Suitable for external feature coverings. |  |

For each of the materials in Q4. identify the following:

- Environmental impacts

- Tolerances for installing and assembling these materials in regard to the nature of the work being performed and the requirements of Housing/Residential industry and the Australian standards

| Answer | # | Environmental impact | Tolerances |  |
| --- | --- | --- | --- | --- |
|  | 1 | Minimal, disposal if not used can incur costs. Can be crushed and reused in other construction materials such as crusher dust | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |
|  | 2 | Noting most cladding types are a mix of recycled materials and glue, they can be degraded and pressed to created new material. Having said that cladding can create heavy loads of offcuts depending on the architectural requirement | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |
|  | 3 | Noting most cladding types are a mix of recycled materials and glue, they can be degraded and pressed to created new material. Having said that cladding can create heavy loads of offcuts depending on the architectural requirement | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |
|  | 4 | Noting most cladding types are a mix of recycled materials and glue, they can be degraded and pressed to created new material. Having said that cladding can create heavy loads of offcuts depending on the architectural requirement | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |
|  | 5 | Noting glass is expensive to buy and install most window schedules and balustrading shop drawings are required before fabrication. Therefore little waste is expected on any project. | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |
|  | 6 | Blocks if used and calculated correctly should create little wastage and affect to the environment. Most companies will provide a credit for unused materials | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |
|  | 7 | FC can be re-used noting the main substrates is sand and cement products. Having said that most waste will never see a recycling plant | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |
|  | 8 | Little impact as a substrate noting it’s offcuts can be re-used until depleted. Treatment of such timbers can affect the environment noting most have an arsenic base | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |
|  | 9 | Nill impact noting it can be 100% recycled | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |
|  | 10 | Render is mostly sand yet can harden in large clumps making it straight to the tip. Unlikely to be recycled | Require an example/scenario indicating ‘nature of work’ otherwise requirements can be 10 pages long’ |  |

For this task, please refer to the brief below. You are required to develop a preliminary quote and construction timeframe with schedule.

You have been approached as a local builder by Ian and Grace Fleming to build their home on their 1 hectare of level accessible land at 106 Blue Heeler Drive, Greenbank, Queensland.

The home is a slab on ground, four bedrooms and is of standard brick veneer construction with plasterboard internal lining and metal roof over 600mm eaves and two car garage with attached and roofed alfresco deck off the lounge and kitchen areas (see floor plan supplied below).

Levelling for the house pad requires a cut of 350mm to nothing (west to east) from the garage side of the building with north south reasonably level across the floor slab pad. The deck is facing in a northerly direction.

The Clients have provided you with a contour survey and the geotechnical report which suggests stable substrate soils with the possibility of small to medium boulder size rock within a metre underground. They also advise you, with photographs, that several large eucalypt trees will need to be removed for the house slab location. They want these trees chipped and all bark to be stockpiled on the site for future landscaping.

There is a 7 metre wide double gate access and 100 metres of cracker dust and gravel road into the property and ending literally at the future location of the garage doors. The block is fully fenced with timber posts and chain wire approximately 2 metres high. There is a concrete causeway rated to 120T at the entry gate and crossing over the roadside water table. There is bitumen road access all the way to the property from all access routes with nearest major community hubs being Springfield, and Greenbank/Browns Plains/Parkinson.

In preparation for developing your preliminary quote you need to create a schedule for this build describing estimates for time and labour. You are using sub-contract labour only except for the plumber and electrician who will supply both labour and materials). You have your own team of 3 carpenters with one apprentice. You are to schedule your supply chain for ordering and delivery of materials, including hiring additional plant and equipment for this build.

The list below is not definitive and you can add and subtract as determined:

| Base Stage | Framing Stage | Enclosed Stage | Fixing Stage |
| --- | --- | --- | --- |
| - Site located and confirmed - Site signs installed - Safety signage installed - Traffic Management Plan established - Safety Plan instigated - Land clearing determined and actioned - Temporary sanitary units established - Temporary power supply installed - Sedimentary controls installed - Waste skip installed - Profiling/Layout for foundations - Excavation - Footings established - Base brickwork installed - Reinforcement installed - Rough in (Electrician & Plumber) - Other (Security/VacuMaid/etc.) - Slab on ground established | - Erect load bearing walls - Erect non-load bearing walls - Install sheet, and strap bracing, tie downs etc. - Roofing timbers installed (Trusses) - Other? | - External brickwork established and laid (excluding sills) - Fascia and gutters installed - Roof fixed (without soffit lining, final scribing and screwing off) - External windows fixed - External doors fitted with locks - Garage doors fitted or if not then internal door from garage to house fitted with lock | - Internal linings (stopping setting and finishing) - Roof (final scribing and fully screwed off) - Soffits lined and brick sills installed - Architraves (http://www.woman.com.au/julie-rainbow-clarity-road/) - Cornices - Skirting - Doors to rooms - Baths - Shower trays - Wet area waterproofing - Wet area tiling - Built-in shelves fitted and fixed - Built-in cabinets fitted and fixed - Built-in cupboards fitted and fixed |

| Answer | Base Stage Surveyor (grids, slabs, structural posts) – 2 visits $3,500.00 Site and safety signs $200.00 Traffic management plan inc submission to council $200.00 Bulk earthworks/clearing – 20 hours and 10 trucks $15,000.00 Temp site shed & toilet 20 weeks $3,000.00 Silt fences $350.00 Skip bins for entirety of projects (4m bins x 5) $1,100.00 Supply Concrete for slab – 35m3 @ 155 per m3 $5,425.00 Place concrete, profiles, edge board and steel $22,000.00 Framing Stage Supply of frames & trusses $6,200.00 Supply of builders paper, tie downs and fixings $2,420.00 Supply of structural beams where required $1,500.00 Installation of all the above using 3 trades @ $45.00 per hour and 1 apprentice @ $28.00 per hour. Estimate 8 days to complete framing and trusses $10,432.00 Electrican Temp power – 2 boards $1,500.00 LV reticulation & trenching – 1 week with machine $10,000.00 Rough in – 3 weeks with 2 tradespersons $19,200.00 Fit off - 3 weeks with 2 tradespersons $19,200.00 Supply fixtures - $5,000.00 Plumber Connection of temp shed and toilet $500.00 Site water and sewer connection + meter $1,500.00 Underslabs – 2 days with 2 tradespersons $2,240.00 Rough in – 3 weeks with 2 tradespersons $16,800.00 Fit off - 3 weeks with 2 tradespersons $16,800.00 Supply of PC items $4,000.00 Enclosed Stage Brickwork supply and lay $22,000.00 Fascia and gutters installed $8,000.00 Supply and install colorbond roof and all flashings $21,200.00 External windows $6,000.00 Fixing Stage Internal Linings and cornices $22,000.00 Carpenter to install soffits & fixout – 8 days $10,432.00 Subtotal $216,289.00 Builders Margin $21,628.90 Council fees and insurances $3,200.00 Total Build Cost $241,117.90 |  |
| --- | --- | --- |

There are four types of residential structures nominated in the BCA. Two of these are residential structures with concrete skeleton and slabs and residential structures constructed from non-metallic materials. 

Name the other two.

| Answer | Residential Structures with steel and metallic column and member construction. Residential Structures with timber and other composite material construction. |  |
| --- | --- | --- |

Building materials can be generally categorised into two sources: Natural and Synthetic. Explain these two terms.

| Answer | Natural building materials are those that are not processed or minimally processed by industry such as lumber or glass Synthetic materials are made in industrial settings after much human manipulation such as plastics and petroleum based paints |  |
| --- | --- | --- |

Name at least five (5) naturally occurring materials that were and can still be used to create homes to suit local climate and weather conditions.

| Answer | Fabric, Rock, Clay, Brush. Ice |  |
| --- | --- | --- |

Choose any five of these building materials from Q10. and discuss the materials’ properties, and pros and cons for use and incorporation in a low rise residential construction.

| Answer | Rock -readily available -suitable protection -heavy to use -hard to keep warm -would mostly be used in a feature. For example a feature wall at an entrance Rammed Earth (compacted clay/heavy soils) -holds warmth in winter and keeps cool in summer - can be expensive to build requiring specialty machines -has been used in niche construction for structural walls. Not a common building material noting the cost to achieve its compaction requirements Fabric -can be created in a number of sizes, strengths, shapes etc. - relatively inexpensive -does not offer suitable protection -mostly used for ancillary structures such as gazebos, shade sails and the like Wood -multiple uses -Flexible under load -stronger under compressive force -susceptible to pests such as termites -mostly used for structural framing, trusses & girders Concrete -strong compressive strength -never achieves 100% strength i.e. keeps hardening - suitable for a multitude of construction types -low tensile strength -almost always used for slabs due to it’s compressive strength |  |
| --- | --- | --- |

| Answer |  |  |
| --- | --- | --- |

| Answer |  |  |
| --- | --- | --- |

| Answer |  |  |
| --- | --- | --- |

| Answer |  |  |
| --- | --- | --- |

| Answer |  |  |
| --- | --- | --- |

Every building material comes with an environmental cost of some sort. Discuss the principles that can help guide our choice of sustainable materials and construction systems.

| Answer | - Recyclable? Very recently we have put the question to a very large cladding manufacturer regarding their recycling program for FC. They were unable to demonstrate any such program yet due to our prompt have now allocated funds to research such a program - Durable and low maintenance. Materials requiring less replacement or upkeep with various chemicals - sustainable – materials with little impact on future building techniques etc |  |
| --- | --- | --- |

When using building systems what are the five steps we can follow for considering ways of reducing environmental impacts when erecting building structures?

| Answer | Avoid/minimise problems Select ideal strategy Energy and greenhouse Materials and waste Water management |  |
| --- | --- | --- |

You or the company you work for should have a Building Quality Standards Policy. Why would such a policy be needed and what would be useful source and reference materials in developing such a policy?

| Answer | To set the minimum standard the company is willing to accept under it’s own license. Handy to have to ensure the quality required to last the defects liability for all materials used on site QBCC, NCC and relevant Australian Standards are useful sources for such information for techniques of installation/minimal requirements of materials |  |
| --- | --- | --- |

What is the purpose of a building specification in conjunction with the plans and drawings for the building of a residential home or other building structure?

| Answer | The building specification sets out the project description and the requirements for materials and workmanship so you need to establish all building materials during the preliminary stages with your client |  |
| --- | --- | --- |

Explain the two terms Building Life Cycle (BLC) and Life Cycle Costing (LCC).

| Answer | Building life cycle refers to the view of a building over the course of its entire life, including: operational build, design, construction, demolition and waste treatment. Life Cycle Costing Many clients are now requesting long-term environmentally sustainable solutions which take energy and sustainable innovations, to a new level. |  |
| --- | --- | --- |

When would Life Cycle Costing (LCC) and sustainable solutions need to be considered?

| Answer | Designing, Building, Installing, Maintaining environmental innovations & Branding a service under LCC/sustainable solutions |  |
| --- | --- | --- |

What is a ‘Green Home’?

| Answer | A green home is a type of house designed to be environmentally friendly and sustainable, while also focusing on the efficient use of "energy, water, and building materials |  |
| --- | --- | --- |

When considering alternative building materials what must we take into consideration, other than the clients’ requirements?

| Answer | Material availability or lack of availability, Environmental issues, Life Cycle Assessment methodology |  |
| --- | --- | --- |

How do you take a delivery and deal with damages of materials and products delivered to site?

| Answer | confirm the correct quantity confirm the condition of the delivery. If extensive damages, reject the delivery |  |
| --- | --- | --- |

What is a critical factor to consider when coordinating the delivery of materials to site?

| Answer | Date/time of delivery Area of set down Traffic control Safety of personnel in the area Cost to hold on site for extended time Freight companies pay a small portion of damaged goods |  |
| --- | --- | --- |

Explain in detail how building materials are handled on a building site.

| Answer | Handling and storing materials involves diverse operations such as: - hoisting tons of steel with a crane - driving a truck loaded with concrete blocks - manually carrying bags and material - stacking drums, barrels, kegs, timber, or loose bricks |  |
| --- | --- | --- |

What is the main aim when storing materials on a construction site?

| Answer | Store them free from theft, weather and damage |  |
| --- | --- | --- |

What does it mean to store materials safely and carefully?

| Answer | Materials not delivered on schedule must be stored in a safe place, such as a lockable storage container located at an appropriate location on/off site. Try and store bulk materials on site, such as bricks and blocks on pallets placed nearby the place of use |  |
| --- | --- | --- |

What are some of the general points for storage of materials that office and worksite personnel should be aware of?

| Answer | - Site supervisors must provide adequate facilities for storage and handling of building materials - Construction site areas should be designated for material delivery and storage - Temporary storage area should be located away from vehicular traffic - Material delivery and storage areas should be located near the construction entrances - Where it is necessary, place erodible material on a hard surface - Don't place the material near a rubbish skip - Materials must not be stored/retained on a road reserve without first obtaining permission from Council - Material placed on the road reserve must not block traffic or cause a safety problem - Storage of reactive, ignitable, or flammable liquids must comply with the fire codes - Hazardous materials storage onsite should be minimized - Prevent, or eliminate pollutant discharge from material delivery and storage to stormwater system - During the rainy season, consider storing materials in a covered area - Don't store chemicals, drums, or bagged materials directly on the ground - clean up spills immediately - Material Safety Data Sheets (MSDS) should be supplied for all hazardous materials stored |  |
| --- | --- | --- |

How might you convey information to the workforce and visiting personnel when it comes to the use and installation of materials stored and/or delivered to site for immediate installation?

| Answer | Email, Tool box talks, Face to face Phone call |  |
| --- | --- | --- |

Paying attention to natural phenomena affecting design/build considerations is important. A region and site location will present unique biological, geographical, geological, and meteorological conditions which may require special materials and methods. 

Briefly explain who you can go to if you need further advice in this regard.

| Answer | Safety Data Sheets of materials if required Subcontractors/suppliers of particular materials trained in their use/installation |  |
| --- | --- | --- |

What can you do (on-site and in the office) to improve reliable documentation and records of deliveries from suppliers, material, plant and equipment acceptance at site from delivery and installations by your own tradespeople and sub-contractors?

| Answer | Keep accurate registers of all materials delivered and due to be delivered to site including relevant contacts, emergency details etc. I personally tend to do this for a cost management point of view yet works well to keep tabs on the overall delivery of all materials |  |
| --- | --- | --- |

What considerations come into play when selecting building materials? When would it be appropriate to consider alternative materials or building practices?

| Answer | - What material property (e.g. strength, durability, density, combustibility etc.),is required for construction type - Does the material and construction technique provide safety against natural hazards (e.g. fire, heavy rain, cyclones, earthquake etc.)? - Is the material produced locally, or is it partially or entirely imported? - What are the costs of the material (can it be produced at lower costs on the building site)? - Where has it been produced (transport costs!)? - Is the material made with recycled content? - Is the material suitable for the climate condition - Has the material a lifespan equivalent to the projected life of the building |  |
| --- | --- | --- |

Explain the difference between ferrous and non-ferrous metals. What is the principal reason why in cosmetic applications non-ferrous metals would be preferred over ferrous metals in building and construction?

| Answer | Non-Ferrous Metals do not contain Iron, are not magnetic and are usually more resistant to corrosion than ferrous metals. Some examples of Non-Ferrous Metals are: Aluminium & Aluminium Alloys |  |
| --- | --- | --- |

Supervision of your project requires a consistent, organised approach to each phase of construction. What would this entail?

| Answer | An evolving construction program that changes to reflect each stage of the build and takes into account delays etc |  |
| --- | --- | --- |

Please read the information below in responding to Error! Reference source not found.

Given the variety of circumstances in residential construction, it becomes very difficult to create a master checklist covering all circumstances. Checklists by their very nature need to be comprehensive enough to provide sufficient information upon which to act or react and serve merely be regarded as a guide and should NOT be used as a means to troubleshoot your project.

The most important aspect of the checklist is to understand the level of detail you'll encounter should you decide to act as the builder or when using the services of a construction manager or general contractor to assist in project management duties.

You are to produce a sample quality control checklist. 

Keep in mind that whilst your checklist needs to be reasonably comprehensive, it will not become a substitute for good observation and critical thinking about quality control for your project and the product and material specifications applicable to it. 

Your checklist should cover everything about the build from begin to end.

As an example could have headings like:

- PRECONSTRUCTION – including  site access, temporary facilities, storage and protection, cleaning and debris

- SITEWORKS – including  demolition layout, site clearing, excavation, back fill, grading, retain walls, asphalt paving, concrete paving, brick paving, public utilities, storm water controls, foundation drainage, trees plants groundcover

- FOUNDATION – including slabs, damp proofing, including set outs, footings including white ant system protection, walls, slabs, damp proofing and wall and crawl spaces

- FRAMING – including floor, wall, roof, roof sheeting and flashing etc.

| Answer |  |  |
| --- | --- | --- |