CPCCBC4014A

Prepare simple building sketches and drawings

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4014A Prepare simple building sketches and drawings cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Christine Muter |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4014A

Prepare simple building sketches and drawings

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Christine Muter |
| --- | --- |
| Participant signature |  |
| Date | 20/09/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 23 |  | Satisfactory | Not Yet Satisfactory |

ASSESSOR Comments

- Identify three (3) two-dimensional drawings/sketches that may be used to clarify or communicate ideas to clients or other parties. For each drawing/sketch, identify four (4) key features that may be shown.

| Answer | Drawing/sketch type | Key features |  |
| --- | --- | --- | --- |
|  | Concept Sketches | These offer a quick way to get thoughts onto paper and to note any special or critical areas in the planning stage whether the proposed building will run east /west, north/ south, if it will be a single storey or two storey construction with a general concept design of the internal spaces, ie. Bedrooms, bathroom, livings areas etc. It will also identify underground power, the sewage connection, any existing roads, pathways with the lot and street name |  |
|  | Sectional Views | This is a view of the structure after it has been cut vertically in an area of the structure to show the details of structural parts and how they are required to be built and assembled, for example the heights from the ground to the floor, the height between the floor and the ceiling and the requirements for the roof construction Sections views are also used on a hidden part of an object, this could be in regards to the type of building or structural re enforcing that is being used as part of the construction |  |
|  | Land Boundaries, contours and footprint of Buildings | These show the boundaries of the site, any adjoining boundaries of surrounding/adjoining properties. It will also identify the various contour levels on the site and the position of the proposed structure on the site. |  |

- Name two (2) advantages of a three-dimensional sketch/drawing.

| Answer | They provide a more realistic overview of a space within the drawing and they provide the ability to review the space/ area from several different angles instead of one singular view |  |
| --- | --- | --- |

- Name one (1) individual you might contact to establish the scope and standard of the job being undertaken.

| Answer | The builder employed to carry out the works |  |
| --- | --- | --- |

- You need to inspect the relevant area to take measurements for your drawings/sketches. Identify two (2) safety requirements you would need to observe on site.

| Answer | Ensure the correct tools are checked for serviceability(any calibration tools etc) tag, report and record any found with issues, and take any relevant checklists in order to take the measurements accurately. Carry out a site risk assessment prior to attending to identify any potential hazards that may be encountered whilst on site to ensue risks are minimised. |  |
| --- | --- | --- |

- Identify two (2) types of tools/equipment you would use when inspecting the area to take measurements.

| Answer | 1.Measuring wheel for accurately measuring the boundary of the site and 2. A Gradient or digital level to identify angels and degrees as a percentage |  |
| --- | --- | --- |

- Identify two (2) types of tools/equipment you might need when completing sketches/drawings.

| Answer | 1.Writing an sketching equipment such as clipboard, pencils, eraser, ruler 2.Clipboard and Paper i |  |
| --- | --- | --- |

- What action should be taken if any of the tools/equipment identified in Q5. and Q6. are found to be defective? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Report, Record |  |
|  |  | Stop, Report, Record |  |
|  |  | Stop, Tag, Report, Record |  |
|  |  | Stop, Fix, Report, Record |  |

- True or False (Tick your response): It is best practice to double check measurements before recording them.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Explain the purpose of the scale on sketches/drawings.

| Answer | A scale is a drawing that depicts an object with accurate sizes reduced or enlarged by a certain amount |  |
| --- | --- | --- |

- You have measured a wall that is 2.4m high. Using a scale of 1:100, what size will this be drawn? Provide your answer in mm. Show your workings.

| Answer | Answer is 24mm 2400( 2.4m in mm is 2400) divide by 100 ( the scale amount) |  |
| --- | --- | --- |

- Explain the purpose of a key or legend on sketches/drawings.

| Answer | They key or legend helps identify specific details within the drawing, it identified the symbols used in the plan to identify these and their meaning |  |
| --- | --- | --- |

- Which of the following are true about drawing contours? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Equally spaced contour lines indicate uniform slope |  |
|  |  | Closely spaced contour lines indicate flat surface |  |
|  |  | Closely spaced contour lines indicate steep ground |  |
|  |  | Concentric closed contours with decreasing values toward the centre indicate a pond |  |
|  |  | Concentric closed contours with decreasing values toward the centre indicate a hill |  |

- Identify the Volume and Parts of the NCC that relate to metal sheet roof cladding for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume 2 Part 3.5.1 | Volume 1 Section B |  |

- Which Australian Standard is referenced in the NCC with regards to the requirements for the design and installation of metal sheet roof cladding?

| Answer | Australian Standard AS 1562.1:2018 |  |
| --- | --- | --- |

- Sketch the following standard design symbols.

| Answer | Item | Sketch |  |
| --- | --- | --- | --- |
|  | Compass (North) |  |  |
|  | Sink with one bowl |  |  |
|  | Stairs |  |  |
|  | Toilet |  |  |

- Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | BK | Brick |  |
|  | CBD / CPD | Cupboard |  |
|  | DP | Downpipe |  |
|  | INSUL | Insulation |  |
|  | KIT | Kitchen |  |
|  | MSB | Main Switch Board |  |
|  | PBD | Plasterboard |  |
|  | RWT | Rain Water Tank |  |
|  | SHR | Shower |  |
|  | TEL | Telephone |  |
|  | WC | Water Closet / toilet |  |

- True or False (Tick your response): The dashed/chain lines on the section view shown below represent the portions that have been in contact with the cutting plane.

Source: https://www.mcvts.net/cms/lib/NJ01911694/Centricity/Domain/131/Section_Views.pdf

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Explain the use of hatching lines for sectional drawings.

| Answer | Hatching lines represent / show the material type that is in contact with the cutting plane on a sectional view of a diagram . |  |
| --- | --- | --- |

- True or False (Tick your response): In an offset sectional drawing, the cutting plane does not travel in a straight line.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Which of the following information should be included in the title block for a sketch/drawing? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Client details |  |
|  |  | Contact details for person who prepared drawings |  |
|  |  | Date drawing was completed |  |
|  |  | Drawing number |  |
|  |  | Drawing title |  |
|  |  | Job number |  |
|  |  | Project name and address |  |
|  |  | Revisions / Amendments (if applicable) |  |
|  |  | Scale |  |

- True or False (Tick your response): Amendments usually have to be signed or initialled by both the builder and the client to show they both agree.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Explain why it is good practice to have a single repository for sketches/drawings.

| Answer | Keeping the sketched in a single central location ensures that there is one place / area so that the drawings can be easily accessible to ensure that the drawings being used are the most current ones for the project which may have been changed with variations etc. |  |
| --- | --- | --- |

- Name the Regulation and identify the section that outlines the requirement for structure designers to produce a safety report. 
Briefly describe the requirements.

| Answer | Section of Act | Requirements |  |
| --- | --- | --- | --- |
|  | Section 295 of the Work Health and Safety Regulation 2011 (QLD) | Designers have a duty to comply with Section 22 of the Work Health and Safety Act 2011 and under Section 295 of the Work Health and Safety Regulation 2011 requires a designer of a structure or part of a structure that is to be constructed to give the PCBU who commissioned the design a written report. This report must specify the hazards relating to the design of the structure that, so far as the designer is reasonably aware: - Create a risk to the health or safety of persons who are to carry out any construction work on the structure or part - Are associated only with the particular design and not with other designs of the same type of structure. |  |

CPCCBC4014A

Prepare simple building sketches and drawings

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name |  |
| --- | --- |
| Participant signature |  |
| Date | Click here to enter a date. |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4014A

Prepare simple building sketches and drawings

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to produce sketches and drawings for a small work project (Note: The participant is not required to demonstrate complex drafting skills). Sketches and drawings may be simplified versions taken from architectural drawings, designed to capture design concepts or options. They may be used for estimating purposes and to show measurements and other requirements for building and construction works. Standard drawing conventions and features must be implemented (including direction, scale, key, contours, symbols and abbreviations). The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - Building sketches from projects the candidate has worked on The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer, digital camera, pen and paper. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Christine Muter |  |  |
| Name of observer: | John Gammon |  |  |
| Role of observer (Confirm suitability to sign) | Construction Manager |  |  |
| Email address of observer | john.gammon@tbs.com.au |  |  |
| Signature of observer: |  |  |  |
| Date: | 20/09/2021 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Identify and record types of drawings required and key features, in compliance with the cope and standard of the job. |  | As per described in comments | As per described in comments |
| - Identify and follow WHS requirements on site and statutory/authority requirements. |  | As per described in comments | As per described in comments |
| - Gather and check tools and equipment required for inspection, measurement and producing drawings. |  | As per described in comments | As per described in comments |
| - Inspect relevant area, taking and recording required measurements. |  | As per described in comments | As per described in comments |
| - Create simple 2D and 3D sketches and drawings, using standard drawing conventions and incorporating relevant codes and standards. |  | As per described in comments | As per described in comments |
| - Create sectional drawings of simple structural elements, using standard drawing conventions. |  | As per described in comments | As per described in comments |
| - Record essential information on the drawings with symbols and abbreviations, according to standard drawing conventions. |  | As per described in comments | As per described in comments |
| - Label, date and process drawings according to organisational administration and quality procedures. |  | As per described in comments | As per described in comments |
| - Effectively communicate using the following methods: - Using language/concepts appropriate to cultural differences - Non-verbal communication |  | As per described in comments | As per described in comments |
| The participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments

Townsend Building Services Business model doesn’t include the drafting or reviewing of building plans in order to carry out the required rectifications cause by Insurable damages. Due to our Business model, being able to observe Christine’s understanding has been shown by obtaining sample drawings and identifying the key aspects of each. I have had a strong building background throughout my career and although Christine isn’t exposed to building plans as part of her employment, she has displayed an understanding of these. Christine is exposed to engineers drawings in her position and her interpretation and understanding of these is evident in her work, a few examples will be submitted with her assignment. .