CPCCBC4029B

Apply construction information to the sales process

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Ana Marchese |
| --- | --- |
| Participant signature |  |

| Assessor name | Brett Stoward |
| --- | --- |
| Assessor signature |  |

| Date | 14/12/2018 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 18 |  | Yes | No |

Assessor Comments

Outline the circumstances in which a building licence must be obtained before carrying out building work.

| Answer | In NSW when carry any building work valued over $5,000 When carry building work valued over $1,100 where it involves Hydraulic Services Design Building work of any value where it involves: - Drainage - Plumbing and Drainage - Gas Fitting - Termite Management – Chemical - Fire Protection - Completed Residential Building Inspection - Building Design – Low Rise, Medium Rise and Open - Site Classification |  |
| --- | --- | --- |

Describe the steps involved in organising electricity supply to site. Provide approximate timeframes for this process.

| Answer | Electrical contractor will be able to advise if supply is available. - Should the premise not have supply available, the electrical contractor can complete an enquiry on the client’s behalf. - The Distribution Network Service Provider (in NSW Ausgrid) will conduct the necessary network investigations and provide an indicative estimate of potential costs and timeframes for the client to make an informed decision within 20 business days If there is supply available, the electrical contractor will need to enter into a Network Connection Agreement with the Distribution Network Service Provider and advise of a Network Connection Offer within 10 business days After agreeing to the connection offer, the electrical contractor will prepare the premises for connection - The electrical contractor will submit an Electrical Work Request (EWR) to the Distribution Network Service Provider once the work is complete - A copy of the EWR will be sent to the client’s preferred electricity retailer Once all forms are received, the Distribution Network Service Provider will visit the premises to perform the requested work - The Distribution Network Service Provider must adhere to timeframes for new connections normally within 5 business days Timeframes are dependent on network availability, access requirements, traffic control requirements or any other impacts that are outside of the control of your Distribution Network Service Provider. |  |
| --- | --- | --- |

True or False (Tick your response): Staged approval is one type of approval that may be sought for the building approval stage.

| Answer | ☒ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☐ | False |  |  |

True or False (Tick your response): Building provision approvals are issued by the local government (e.g. Brisbane City Council).

| Answer | ☒ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☐ | False |  |  |

Describe the steps involved in organising water supply to site. Provide approximate timeframes for this process.

| Answer | - In NSW,, this application is submitted to Sydney Water for connection or disconnection of the water supply, sewerage network or alter network infrastructure. - The Water Approval Process works in conjunction with the Council development assessment process, and applications do not need to be submitted at the same time. Councils will require evidence that a connection is available. Standard connection for simple residential development take with 5 days, |  |
| --- | --- | --- |

Which section of the Building Code of Australia outlines energy efficiency performance standards?

| Answer | - Section J of the Building Code of Australia mandates that new and refurbished buildings to meet energy efficiency performance standards. |  |
| --- | --- | --- |

Describe the long-term benefits of sustainable building solutions. List one (1) environmental benefit and one (1) benefit for the client.

| Answer | Topics | Definition |  |
| --- | --- | --- | --- |
|  | Environmental benefit | Utilising local source of materials to reduce the impact in the environment (less energy/transport consumption. Use of recycling materials saves waterage and preserves existing resources. |  |
|  | Client benefit | Energy efficiency reduction in cost, better insulation, longer durability of materials therefore reducing replacement. |  |

Match the insurance types and definitions by writing the appropriate letter next to each number in the centre column.

| Answer | Insurances |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Builder’s All Risk | 1 -B | A | Covers personal injury or property damage. |  |
|  | 2 | Indemnity | 2 -C | B | Covers accidental risks of physical loss or damage during construction, third party liabilities and advance loss of profits. |  |
|  | 3 | Public Liability | 3 -A | C | Covers costs where a contractor has caused building errors or omissions. |  |
|  | 4 | Workers’ Compensation | 4 -D | D | Covers death, injury or illness of employees. |  |

True or False (Tick your response): Insurance should be taken out by the contractor prior to starting construction.

| Answer | ☒ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☐ | False |  |  |

Select and describe five (5) trades involved in the construction process.

| Answer | Trade | Description |  |
| --- | --- | --- | --- |
|  | Excavation | Prepares the site for building by removing all vegetation and excavating footings |  |
|  | Plumbing | Organises water supply; installs pipes and drains; installs roof and guttering; fits taps, hot water systems and toilets; ensures that downpipes are fitted and the sewer is connected |  |
|  | Electrician | Arranges electricity to be connected; places wiring for lighting and power cables; Fixes switches, installs power points, wires electrical systems. |  |
|  | Steel fixing | Installs reinforcing steel to improve the strength of concrete. |  |
|  | Concreting | Installs formwork and boxing; pours concrete (e.g. slabs). |  |

Order the following steps from 1 to 10 to identify an appropriate construction sequence.

| Answer | Step Number | Steps |  |
| --- | --- | --- | --- |
|  | 4 | Floor frame |  |
|  | 5 | Windows and external door frames |  |
|  | 7 | Wall insulation |  |
|  | 1 | Set out |  |
|  | 9 | Painting |  |
|  | 6 | Electrical wiring |  |
|  | 2 | Plumbing of waste pipes |  |
|  | 8 | Internal linings |  |
|  | 10 | Landscaping |  |
|  | 3 | Wall frames |  |

For each of the main stages of construction listed below, identify the main trades involved and an approximate duration of each stage for a standard.

| Answer | Stage | Description |  |  |
| --- | --- | --- | --- | --- |
|  | 1: Clearing of the site | Trades | - Depending on the nature of the land, there may be soil testing, site-levelling and clearing undertaken prior to actual construction - Excavation |  |
|  |  | Duration | - Approximate duration: Varies |  |
|  | 2: Slab or base stage | Trades | - Surveying - measuring out the design on the site, Steel fixing, Concreting - pouring the footings, plumbing - under slab drainage, (waterproofing) moisture barrier, bricklaying and special mesh for termite protection. |  |
|  |  | Duration | - Approximate duration: 1-2 weeks |  |
|  | 3: Frame stage | Trades | - This includes the installation of your internal and external support structure, along with walls (carpentry), conduit for electrical (electrical) and plumbing, the roof frame and sheeting, gutters and insulation. |  |
|  |  | Duration | - Approximate duration: 3-4 weeks |  |
|  | 4: Lockup stage | Trades | - You can literally start “locking up” the property because your windows, doors and remaining walls will be installed. It’s also safe for plumbers, electricians, cabinet makers and other tradesman to begin fitting out the property. |  |
|  |  | Duration | - Approximate duration: 4 weeks |  |
|  | 5: Fit-out or fixing stage | Trades | - Plumbing, electrical (including lights and power points), plastering, gas fitting, carpentry, tiling |  |
|  |  | Duration | - Approximate duration: 5-6 weeks |  |
|  | 6: Practical completion stage | Trades | - Painting and Landscaping |  |
|  |  | Duration | - Approximate duration: 7-8 weeks |  |

Identify five (5) factors that can affect the duration of each construction stage.

| Answer | 1. Project Size and location 2. Project function 3. Height of the building 4. Building complexity 5. Quality requirements |  |
| --- | --- | --- |

Describe how each of the following factors can affect construction times.

| Answer | Factor | Effect |  |
| --- | --- | --- | --- |
|  | Changed conditions | If the requirements of the job are altered, this will change the initially projected timeframes, especially where additional materials and labour must be organised. |  |
|  | Labour availability | The construction process sequence is very important when organising the required labour. For example, a plasterer should only be organised after all other trades have performed their tasks correctly – electricians and plumbers will not be able to access behind the walls as easily once the plasterer has put them up. |  |
|  | Material availability | Liaise with sub-contractors and suppliers to ensure materials are delivered when the sub-contractor is ready to complete their work. Materials that are rare or difficult to source should be ordered well in advance. |  |
|  | Weather | Weather will affect the ability of trades to complete their jobs on time. |  |

Describe the site establishment requirements for each of the points listed.

| Answer | Requirement | Description |  |
| --- | --- | --- | --- |
|  | Earthworks | Contact Dial before you Dig before undertaking any excavation to ensure that services will not be damaged during construction |  |
|  | Electricity | Disconnection of electricity may be required prior to construction and temporary utilities installed for the duration of the construction process |  |
|  | Neighbours | Provide advance notice to neighbours of intend start dates and site contact numbers |  |
|  | Waste disposal | Set up a designated area for rubbish bins and waste removal. Check if permits are required to place bins on the street. |  |
|  | Water connection | Book in a plumber to perform any required disconnections prior to any demolition taking place |  |

Identify the section of the Work Health and Safety Regulations that outlines the site establishment requirements listed.

| Answer | Regulation section | Requirement |  |
| --- | --- | --- | --- |
|  | In NSW Section 40 | The layout of the work area must allow for persons to enter, exit and move about without risk to health and safety |  |
|  | In NSW Section 41 | Adequate facilities must be provided for workers, including toilets, drinking water, washing facilities and eating facilities. |  |
|  | In NSW Section308 | A principal contractor for a construction project must install signs outside the work area that shows their name and telephone contact numbers and site office address. |  |

True or False (Tick your response): The owner is able to access the site at any time.

| Answer | ☐ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☒ | False |  |  |

Which of the following parties should the client contact to access the site? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | ☒ | Contractor |  |
|  | ☐ | Local Government |  |
|  | ☐ | Industry Regulator (e.g. Queensland Building and Construction Commission) |  |
|  | ☐ | Subcontractors |  |