CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Mohamed Elbagoury |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | Click here to enter a date. |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 87 |  | Yes | No |

Assessor Comments

Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

Which compass direction is always shown on plans and drawings?

| Answer | North |  |
| --- | --- | --- |

What information is provided on a location plan?

| Answer | Shows a site on a particular street |  |
| --- | --- | --- |

What detail does a site or block plan contain?

| Answer | Provides specific detail of the block on which a development is proposed |  |
| --- | --- | --- |

What do floor plans identify?

| Answer | It identify the detail of the proposed structure, such as rooms, Positions of fixtures, floor coverings |  |
| --- | --- | --- |

What is a landscape plan and what information might it provide?

| Answer | It might show details of plants, paths, garden layouts and retaining walls |  |
| --- | --- | --- |

What are elevation views? What do they provide to the plan reader?

| Answer | The elevation views shows the proposed structure from its sides (i.e. from north, south, east and west). Each side is want is called an elevation view. |  |
| --- | --- | --- |

What is a section?

| Answer | The section is a view of the house after it has been cut vertically at some point. Like floor plan it shows the size and thickness of structural members and relationships between spaces. |  |
| --- | --- | --- |

What are detailed drawings?

| Answer | The detailed drawing Illustrate a particular method of construction of finish |  |
| --- | --- | --- |

What are specifications?

| Answer | Specifications are details that support drawings and plans, providing details of the materials and techniques to be used, details of fixtures and fittings, and standards to be observed. |  |
| --- | --- | --- |

Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standards |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bull Nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete block |  |
|  | CF | Concrete floor |  |
|  | CG | Clear glass |  |
|  | CJ | Ceiling joist |  |
|  | CORR | corrugated |  |
|  | CWT | Cold water |  |
|  | DG | Double glazing |  |
|  | DH | Double hung |  |
|  | DPC | Damp proof course |  |
|  | DSB | Distribution switch board |  |
|  | DW | dishwasher |  |
|  | EJ | Expansion joint |  |
|  | FPBD | Fibrous plaster board |  |
|  | FFL | Finished floor level |  |
|  | FHR | Fire hose reel |  |
|  | GPO | General purpose outlet |  |
|  | HBD | hardboard |  |
|  | HTR | heater |  |
|  | HWD | hardwood |  |

Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Wall - Cavity brick |  |  |
|  | Wall - Brick veneer |  |  |
|  | Wall - Timber stud |  |  |
|  | Window - In cavity wall |  |  |
|  | Window - In single wall |  |  |
|  | Single leaves door opens 90° |  |  |
|  | Two leaves door each opens 90° |  |  |
|  | Pocket door |  |  |
|  | Surface exposed sliding door |  |  |
|  | Socket outlet (power) - general |  |  |

Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Hanged at side window elevation plan |  |  |
|  | Hanged on top window elevation plan |  |  |
|  | Sliding horizontal window elevation plan |  |  |
|  | Fixed window elevation pan |  |  |

What does a clouded area on a plan indicate?

| Answer | revision |  |
| --- | --- | --- |

An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

On what date was an amendment made to the window details?

| Answer | 5/10/2011 |  |
| --- | --- | --- |

True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What amendment occurred on 08/11/2011?

| Answer | The robe in bedroom 3 was deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

What is the Lot number of this site?

| Answer | 16 |  |
| --- | --- | --- |

What is the size of this lot?

| Answer | 3653msq |  |
| --- | --- | --- |

Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What size is the easement on the eastern boundary of the site?

| Answer | 5m |  |
| --- | --- | --- |

True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

What is the level at the northernmost point of the site?

| Answer | 1.2 |  |
| --- | --- | --- |

Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26.?

| Answer | Yes it is the same structure proposed in Q19 & Q20 The building is for H&M Smith, the drawing is an elevation view from the north for a 4 bed room residential building, of two stories, the ground floor is showing a family room and two verandas while the upper floor is showing bedroom 4 and 3 with a bathroom in-between. The drawing is having sectional details for the roof, ceiling, floor, veranda beam … etc |  |
| --- | --- | --- |

Who is the client?

| Answer | H&M Smith |  |
| --- | --- | --- |

Where is this project to be built?

| Answer | 16 Glen valley road, Richmond |  |
| --- | --- | --- |

What are the contact details for the draftsman?

| Answer | Ph.: 03 9514 2311 Mob: 04130113 113 18 Fairview rd. Richmond 3216 |  |
| --- | --- | --- |

What sheet number is this drawing?

| Answer | 5 |  |
| --- | --- | --- |

What timber is required for loading and non-load bearing walls?

| Answer | Timber weatherboard |  |
| --- | --- | --- |

What spacing is recommended for common studs?

| Answer | 600 max spacing |  |
| --- | --- | --- |

What type of roof truss is to be installed?

| Answer | Gang nail roof truss |  |
| --- | --- | --- |

What centres are the trusses positioned at?

| Answer | To the wall plates |  |
| --- | --- | --- |

What size are the veranda posts?

| Answer | 90*90 |  |
| --- | --- | --- |

What timber species are the veranda posts to come from?

| Answer | Dar Oregon |  |
| --- | --- | --- |

What pitch is the roof?

| Answer | 30° |  |
| --- | --- | --- |

What size are the noggings and at what spacing?

| Answer | 35*70 1350 max spacing |  |
| --- | --- | --- |

What size are the ceiling joists?

| Answer | Intermediate |  |
| --- | --- | --- |

What timber is used for roof battens and what spacing?

| Answer | Pine at 900 spacing |  |
| --- | --- | --- |

What thickness is the plasterboard sheeting?

| Answer | 10 mm |  |
| --- | --- | --- |

What type of flooring is to be installed?

| Answer | Plywood flooring |  |
| --- | --- | --- |

What is the roof cladding material?

| Answer | Corrugated metal roofing |  |
| --- | --- | --- |

Where will you find information about the footings?

| Answer | Engineers drawing |  |
| --- | --- | --- |

What do the letters ‘DAR’ refer when timber ordering?

| Answer | Dressed All Round, it has been machined on all sides leaving a smooth surface |  |
| --- | --- | --- |

Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | It refer to the type of wood used for the joist and the dimensions of it. And it means Laminated veneer lumber (LVL) |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

What is the Job Number?

| Answer | 00R-112 |  |
| --- | --- | --- |

What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

What does the drawing show?

| Answer | ELEVATION DRAWING FOR A HOUSE |  |
| --- | --- | --- |

Which view shows the front of the house?

| Answer | EAST ELEVATION |  |
| --- | --- | --- |

How many motor vehicles can be parked in the garage?

| Answer | 2 vehicles |  |
| --- | --- | --- |

What are the window and doors made from?

| Answer | East elevation - Timber entrance door, aluminium framed window, North elevation – aluminium framed sliding door and window South elevation – aluminium framed sliding window and timber door West elevation – aluminium framed window |  |
| --- | --- | --- |

What is the size of the eaves?

| Answer | 450 mm |  |
| --- | --- | --- |

What are the eaves lined with?

| Answer | Hardieflex |  |
| --- | --- | --- |

What type of roof structure is the roof built from?

| Answer | Timber framed roof trusses |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about roof structure in Q59. ?

| Answer | 2.1 |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | NCC volume Two |  |
| --- | --- | --- |

What is the roof cladding materials made from?

| Answer | Concrete roof tiles |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | 3.5.1.2 Roof and wall cladding |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | NCC volume Two |  |
| --- | --- | --- |

What is the roof pitch?

| Answer | 22.5 |  |
| --- | --- | --- |

Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | City Council |  |
| --- | --- | --- |

Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | Once the application is submitted and the fees is paid, the assessment process takes place. Each council has a development plan containing the planning policies that apply in that area. Applications for development plan consent and land division consent are assessed against the policies in the development plan. Applications for building consent are assessed against the Building Rules, including the Building Code of Australia and any relevant Minister's Specifications. This process takes from 4 weeks to 3 months |  |
| --- | --- | --- |

What is a prime cost (PC) item? Give an example.

| Answer | Prime cost (PC) is An amount of money included in a contract sum to purchase a specified item such as tiles, taps, doors or bathroom fittings Example: $500 door . |  |
| --- | --- | --- |

What is a provisional sum (PS) item? Give an example.

| Answer | A provisional sum is: An amount of money included in contract sum to cover work and/or materials, not detailed when entering the contract Example a building site may seem sandy and clean with 1inimal site works required, however, there may be large limestone rocks/tree trunks concealed below the surface which I emerge once site works commence, causing cost overrun. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify two (2) materials being used for the floors?

| Answer | Floor trusses Internal tile supply |  |
| --- | --- | --- |

True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

How much does the contracted plasterer cost?

| Answer | $12900 |  |
| --- | --- | --- |

How much do the kitchen appliances cost?

| Answer | $2700 |  |
| --- | --- | --- |

What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $275 |  |
| --- | --- | --- |

How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 sheets |  |
| --- | --- | --- |

What material has the highest total cost?

| Answer | The pine studs |  |
| --- | --- | --- |

How many metres are required of the material in Q80. ?

| Answer | All studs will give the total of 684 m |  |
| --- | --- | --- |

What beam is identified as being a better option than F17?

| Answer | E14 Beams |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Builder’s lift operator |  |
| --- | --- | --- |

If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $133.2 |  |
| --- | --- | --- |

Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar Bender and Fixer |  |
| --- | --- | --- |

If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $1285.8 |  |
| --- | --- | --- |

Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | In South Australia, the Development Act 1993 and associated Development Regulations 2008 provide the framework for development and building work. |  |
| --- | --- | --- |