RIICWM401E

Supervise civil works

ASSESSMENT SUMMARY

The assessment activities for RIICWM401E Supervise civil works cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Mathew pomerenke |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for RIICWM401E | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Theory Exam | Practical Assessment Completed Job Plan |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |

RIICWM401E

Supervise civil works

THEORY EXAM

| Participant name | Mathew pomerenke |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within two (2) hours, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

Which on-going risk management procedures and documentation do you access when supervising civil work and how do these link to legislative requirements?

| Answer | Some general principles of risk management include identifying potential hazards and risks , developing strategies to reduce or eliminate risks and documenting any accidents or near mises. It is important to ensure that all risk management procedures and documentation comply with relevant legislation The specific procedures and documentation that will be used will vary depending on the nature of the project and applicable legislation. However the general principles of risk management are the same |  |
| --- | --- | --- |

What important emergency response information must be shared with workers prior to work beginning?

| Answer | Emergency contact information, evacuation procedures, first aid and CPR training, hazard identification and control, emergency equipment |  |
| --- | --- | --- |

What are (1) the objectives of the Environmental Management Plan and (2) why you should read this as part of planning the job.

| Answer 1. 2. | Minimize environmental impacts, comply with environmental regulations, protect human health and the environment and promote sustainable development The environmental management plan provides guidance on how to minimize environmental impacts, potential environmental risks and hazards, helps to ensure that the project is carried out in an environmentally responsible manner. |  |
| --- | --- | --- |

What is (1) the ‘scope’ of work, (2) why is this important when developing a job plan and (3) where can you sources relevant information.

| Answer 1. 2. 3. | The range of tasks and responsibilities that are included in a particular job project It helps to ensure that everyone involved in the project is clear on what is expected of them Job descriptions, project proposals, industry standards and stakeholder input |  |
| --- | --- | --- |

In the general principles of surveying, what is the importance of ‘working from the whole to the part’ (1) when surveying and (2) how this is achieved?

| Answer 1. 2. | Importance Working from the whole to the part allows surveyors to grasp the overall layout and relationship between different elements of a site this helps in making informed decisions and avoiding errors, surveyors can establish control points and reference systems that ensure the accuracy of subsequent measurements and calculations. Understanding the over project scope allows for better planning of resources, equipment and timelines Adhering to this principle surveyors can ensure accuracy, efficiency and a comprehensive understanding of the project leading to successful outcome |  |
| --- | --- | --- |

Why can consulting the team enhance team practices? Include one (1) example.

| Answer | Consulting the team can enhance team practices because it allows for a diverse range of perspectives and ideas to be considered. This can lead to more creative solutions and better understanding of the task at hand. Example if a team is working on a project, consulting with each other can help identify potential problems or areas for improvement that may not have been considered otherwise. This can lead to a more successful outcome for a project |  |
| --- | --- | --- |

How do you communicate site safety requirements to ensure workers wear PPE suited to the site?

| Answer | Communicate clearly and effectively, provide training conduct site inspections, enforce compliance and provide feedback |  |
| --- | --- | --- |

Why is it important to coordinate tasks with other work supervisors and schedule team activities?

| Answer | Coordinating tasks with other work supervisors and scheduling team activities is important for improved communication, efficient resource allocation reduce conflicts, enhanced productivity and improved teamwork |  |
| --- | --- | --- |

What inclusions must be considered when preparing a job plan?

| Answer | Scope of work, timeline, resources, budget, safety plan, quality control, communication plan, risk assessment and contingency plan |  |
| --- | --- | --- |

What processes must you follow to acquire and confirm the acquisition of required resources at your workplace?

| Answer | Identify the need, request for resources , approval process, procurement, receipt and verification, documentation , training and support, regular review |  |
| --- | --- | --- |

What is discussed during the daily pre-start meeting?

| Answer | Daily pre-start meeting is a time for the team to come together and discuss the days work, identify any potential hazards and ensure everyone is on the same page. Some of the key topics that are typically discussed during a daily pre-start meeting include safety, work plan, communication, equipment, materials , weather and quality |  |
| --- | --- | --- |

What is the ‘rule of thumb’ basis for matching a machine to a job?

| Answer | Capable of preforming the task and has the capacity to handle the workload |  |
| --- | --- | --- |

Supervisors must have knowledge of factors affecting plant performance. What are four (4) factors that can affect plant performance and output and why must supervisors monitor these?

| Answer 1. 2. 3. 4. | Employee motivation Training and development Equipment and resources Work environment |  |
| --- | --- | --- |

What methods do you use to ensure team members adhere to safety requirements such as PPE during the work tasks?

| Answer | Pre-task briefing, demonstrations, regular inspections, feedback and communication enforcement |  |
| --- | --- | --- |

What is the supervisor’s role during site set out?

| Answer | The supervisor is responsible for ensuring the site is set up correctly, safely and efficiently This includes making sure that all necessary equipment is in place, site is properly marked out and that all safety procedures are being followed. |  |
| --- | --- | --- |

What are two (2) things you would tell the workers when explaining what the pegs mean during road set out? (30 words)

| Answer | The pegs indicate the Alignment of the road Level of the road The pegs are used to ensure that the road is built to the correct specifications. They are placed at regular intervals along the road and are used to guide the workers in building the road |  |
| --- | --- | --- |

What methods do you apply when monitoring civil works to ensure the required outcomes or met on a daily basis?

| Answer | Regular inspections, progress tracking, quality control, communication and coordination, documentation, risk management, performance evaluation. By implementing these methods, project managers can effectively monitor civil works and ensure that the required outcomes are met on daily basis. |  |
| --- | --- | --- |

Name and explain the purpose for three (3) of the different test results the supervisor must be able to read and understand at different hold or witness points of a project?

| Answer 1. 2. 3. | Soil testing – moisture, plasticity and permeability Concrete Slump testing – within guide lines of foundation requirements Concrete compression testing- to find out NPA of concrete |  |
| --- | --- | --- |

What is the purpose of road surface drainage and what are four (4) elements used to achieve this effectively?

| Answer Purpose Element 1. 2. 3. 4. | Road surface drainage deals with the drainage of stormwater runoff from the road surface and the surfaces adjacent to the road formation. Several elements can be used to intercept or capture this runoff and facilitate its safe discharge to an appropriate receiving location Kerb and channel Edge and median drainage Table drains and block Batter drains |  |
| --- | --- | --- |

What are the effects of inappropriate road drainage infrastructure?

| Answer | Inappropriate road drainage will make surface damage that can lead to soft spots and sub grade problems (pot wholes) |  |
| --- | --- | --- |

What are the three (3) main parts of roadway design and the importance of these features?

| Answer 1. 2. 3. | Alignment - it is important for safety, efficiency and aesthetics Cross- section – it is important for traffic capacity, safety and drainage Drainage – it is important for safety pavement preservation and environmental protection |  |
| --- | --- | --- |

How can you support sub-contractors to achieve their responsibilities?

| Answer | Provide clear and concise instructions, offer adequate training and resources , establish open communication channels, provide timely payment and foster a collaborative environment |  |
| --- | --- | --- |

How do supervisors determine human resources and sub-contractors needed for the job plan?

| Answer | Supervisors determine human resources and sub contractors needed for the job plan by analysing the job plan, estimating workforce needs and identifying sub contractor requirements, considering project timeline and budget, consulting with relevant stakeholders |  |
| --- | --- | --- |

How do you, as the supervisor, (1) determine and (2) share the communication systems to be used during site tasks with others? Include in your response (3) the communication systems generally used.

| Answer 1. 2. 3. | Two way radio Mobile phones Text messaging Determine the communication system used comes down to nature of work, size job site, location of job site |  |
| --- | --- | --- |

What methods do you apply when monitoring worker performance for safety and compliance?

| Answer | Performance reviews, safety audits, incident reporting, training and education, observation and feedback, data analysis and employee engagement. These methods can be used individually or in combination to effectively monitor worker performance for safety and compliance |  |
| --- | --- | --- |

How is quality assessed and confirmed?

| Answer | Visual inspection, testing, documentation , feedback |  |
| --- | --- | --- |

What are four (4) events that might occur on a normal working day that may significantly affect progress of civil works tasks and force the use of contingency plans.

| Answer 1. 2. 3. 4. | Bad weather Equipment failure Material shortages Labour strikes |  |
| --- | --- | --- |

What are four (4) contingency plans that may be in place to support a civil construction operational plan?

| Answer 1. 2. 3. 4. | Weather contingency plans Safety contingency plans Safety contigency plans Financial contingency plans |  |
| --- | --- | --- |

What is the relationship between various areas of civil work and how are these relationships relevant when developing job plans and work schedules?

| Answer | The relationships between different areas if civil work are crucial for effective project planning and execution. Understanding these relationships allows for efficient job plans and work schedules, leading to better project outcomes |  |
| --- | --- | --- |

Elaborate on four (4) different types of estimates used when developing job plans.

| Answer 1. 2. 3. 4. | Preliminary estimate – first estimate made based on limited information often used to determine if the job is feasible Definitive estimate – this is a detailed estimate based on more complete information it is used to determine the cost of the project Contingency estimate – this is an estimate of potential cost of unforeseen events it is used to ensure that a project has enough funds to cover unexcepted expenses Final estimate – this is a final estimate of the cost of the project. It is used to determine the actual cost of the project |  |
| --- | --- | --- |

What is the relationship between waste management and environmental protection?

| Answer | Waste management is essential for environmental protection by reducing pollution, conserving resources, and safe guarding human health |  |
| --- | --- | --- |

RIICWM401E

Supervise civil works

PRACTICAL ASSESSMENT

| Participant name | Mathew pomerenke |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel, unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 | Select Date. | Workplace Simulation |  |  |  |
| Task 2 (Occasion 1) | Select Date. | Workplace Simulation |  |  |  |
| Task 2 (Occasion 2) | Select Date. | Workplace Simulation |  |  |  |

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 2: OCCASION 1 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Details |
| 2.1B. | Specify task to be completed: |
| 2.1C. | Identify tool used to analyse risks and determine controls: |
| 2.1D. | PPE worn/applied (tick or specify): Hi-viz Helmet Gloves Safety boots Eye protection Ear protection Other (please specify): |
| 2.3G. | Specify adjustments made: |
| 2.4D. | Outline recommendations made: |

| ASSESSOR COMMENTS: PRACTICAL TASK 2: OCCASION 2 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Details |
| 2.1B. | Specify task to be completed: |
| 2.1C. | Identify tool used to analyse risks and determine controls: |
| 2.1D. | PPE worn/applied (tick or specify): Hi-viz Helmet Gloves Safety boots Eye protection Ear protection Other (please specify): |
| 2.3G. | Specify adjustments made: |
| 2.4D. | Outline recommendations made: |

RIICWM401E

Supervise civil works

Practical Task 1

Description of task: The participant must demonstrate undertaking the role of a civil construction supervisor and prepare a job plan which is to be carried out as part of Practical Task 2. The participant must include documentation demonstrating collating of information. If they do not have adequate existing documentation, they will be required to develop a new template for use or one from another job which illustrates the type of information they require. As part of this task, the participant is required to demonstrate the skills and knowledge to: - access, reading and correctly interpreting work documentation to ensure all work to be completed is compliant - identify and address potential risks, hazards and environmental issues, and implement control measures including selecting and wearing site appropriate PPE - consult with team members to confirm the task requirements - organise for the acquisition and availability of required resources - implement a job plan that meets task requirements Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Work team, work site, organisational policies, standard operating procedures and plans, safety processes, hazard and risk assessment documents relevant to the area of work, Project plans and specifications, EMP, TMP, Engineering survey, Testing schedule requirements, relevant SDS for any chemicals used as part of the work task being assessed.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |
| --- | --- |
| 1 Human resource requirements |  |
| A clear and accurate overall number of workers required to undertake the construction task. |  |
| Identify appropriate types of workers required for the task (e.g. skilled or unskilled labour, concreters, carpenters, road crews). |  |
| 2 Plant and machinery requirements |  |
| Identify appropriate types of plant and machinery required to complete the task, and justification their selection. |  |
| Identify the plant and machinery requiring a trained or licensed operator and any legislative document or note providing this information. |  |
| Identify how plant and equipment will be procured for the job and what procedures will be followed to undertake this (e.g. hire company, plant hired after requisition form submitted to the site engineer and accounts officer). |  |
| 3 Construction materials requirements |  |
| Interpret site plans and specifications to identify the types of construction materials to be used. |  |
| Apply accurate calculations of material quantities to be used during the task showing areas, volume or tonnage and wastage factors from plan requirements and engineer requirements. |  |
| Identify the total value of materials to be used. |  |
| Document contingency planning in place regarding ordering materials from alternate suppliers. |  |
| 4 Sub-contractor support requirements |  |
| Describe the requirement for all sub-contractor to undergo induction training prior to entering site. |  |
| Identify key personnel that sub-contractors will refer to. |  |
| Provide documentation from site including schedules or charts identifying: - The use of sub-contractors over a given period of time - Plans of drawing of work to be completed by sub-contractors - Sub-contractor induction records - Organisational chart indicating who sub-contractors will report to |  |
| 5 Waste disposal requirements |  |
| Identify relevant state authority acts, regulations, codes of practice and standards relating to waste removal (e.g. Environmental Protection Act). |  |
| Identify and explain how materials will be removed from site (e.g. Will they be taken to landfill, decontaminated). |  |
| Describe any specific organisational requirements for waste disposal.(as applicable). |  |
| 6 Coordination requirements |  |
| Identify and describe tasks that occur simultaneously on the job plan. |  |
| Accurately identify the time required to undertake the main construction steps. |  |
| Identify the persons required to monitor to different aspects of the project. |  |
| 7 Activity scheduling |  |
| - Clearly identify the overall length of time assigned to the project and significant milestones from production schedules, construction sequence drawings, Gannt charts or similar. |  |
| - Sequence steps effectively with an efficient use of time. |  |
| 8 Materials delivery scheduling |  |
| Clearly identify the key delivery dates for materials to be used. |  |
| Identify the locations for site deliveries. |  |
| Clearly explain the types of trucks, plant and machinery used during the materials delivery process. |  |
| 9 Risk assessment and management requirements |  |
| - Describe methods for prioritising risks onsite (e.g. using risks calculators, incident reports). |  |
| - Describe the processes involved with respect to deciding upon control measures. |  |
| 10 Occupational health and safety requirements |  |
| - Identify appropriate safety personnel for the site and their roles in the safety system. |  |
| - Provide examples of how hazards may be identified on site through inspections, conducting on going risk assessments, checking reports. |  |
| - Explain the reporting requirements for hazards and incidents on site. |  |
| - Identify the requirements for all workers on site to undergo site specific inductions. |  |
| - Explain procedures for identifying non-hazardous and hazardous materials. |  |
| - Explain procedures for handling substances/ materials onsite (e.g. SDS). |  |
| - Explain the site evacuation procedure including meeting points using the site evacuation plans. |  |
| - Identify and explain the location of firefighting and first aid equipment. |  |
| - Identify and explain the roles of emergency personnel on site (e.g. Fire warden, Chief fire warden). |  |
| 11 Quality management requirements, including testing scheduling requirements |  |
| - Identify the tolerances applicable to the civil construction task from plans and specifications. |  |
| - Determine from specifications and schedule testing milestones, the types of testing required. |  |
| - Identify the personnel required to monitor and confirm testing. |  |
| 12 Traffic management requirements |  |
| - Identify exclusion zones, signage and barriers required for the project from the traffic management plan. |  |
| - Document the personnel required to implement and monitor the traffic management plan. |  |
| 13 Environmental requirements |  |
| - Identify and document how noise, erosion, water quality, waste disposal, flora and fauna and cultural heritage artefacts/ sites were intended to be controlled on site. |  |
| 14 Task monitoring requirements |  |
| - Identify the persons required to monitor to different aspects of the project. |  |
| - Provide production charts, toolbox meeting schedule, briefing note, emails, charts diagrams or work instruction requirements for addressing issues with safety/performance. |  |
| 15 Task performance monitoring requirements |  |
| - Provide example documents including production charts, briefing note, emails, charts diagrams or work instruction indicating milestones or timeframes for the construction, organisation charts/diagrams or notes. |  |
| 16 Communication requirements |  |
| - Identify how regularly toolbox meetings occur on site. |  |
| - Explain the procedures for recording attendance and topics during toolbox meeting (agenda, sign on sheets). |  |
| - Identify communication technology which may be used onsite. |  |
| 17 Reporting requirements |  |
| - Document milestones and reporting requirements including reporting to who. |  |

RIICWM401E

Supervise civil works

Practical Task 2 (2X)

Description of task: The participant must demonstrate undertaking the role of a civil construction supervisor on two (2) occasions. Occasion 1 of this task will relate to the plan prepared in Practical Task 1. During this task, the participant the participant must display the skills and knowledge to effectively, safely and efficiently: - prepare to supervise civil works - prepare team members to execute task - facilitate a pre-start meeting to issue clear and timely instructions to team members and others involved - set out tasks as required for the effective completion of the civil works - while overseeing the execution of tasks: - monitor task performance for safety effectiveness and efficiency - initiate adjustments to civil works practice and job plan and ensure safe execution of work and achievement of required outcomes - oversee plant equipment and tools maintenance and confirm maintenance requirements are carried out and recorded according to manufacturer and site requirements - oversee work area clean up and approved waste disposal including recycling - report on the execution of tasks - complete and submit written and verbal reports - recommend required changes to conform task execution meets job requirements Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Work team, work site, organisational policies, standard operating procedures and plans, safety processes, hazard and risk assessment documents relevant to the area of work, Project plans and specifications, EMP, TMP, Engineering survey, Testing schedule requirements, relevant SDS for any chemicals used as part of the work task being assessed, all materials, tools, plant or equipment required to complete the task, required PPE and relevant safety equipment.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |  |
| --- | --- | --- |
|  | 1st Occasion | 2nd Occasion |
| 1 Prepare to supervise civil works |  |  |
| - Identify the location of the site using appropriate project plans. |  |  |
| - Determine the scope of the work task from plans and specifications. |  |  |
| - Determine and address potential risks, hazards and environmental issues and implement control measures. |  |  |
| - Select and wear PPE suited to site and tasks. |  |  |
| - Consult team members regarding resource requirements for task. |  |  |
| - Communicate with other supervisors/ team leaders to resolve coordination requirements. |  |  |
| 2 Prepare team to execute task |  |  |
| - Facilitate a pre-start meeting with team and others involved. |  |  |
| - Confirm resources required for task are acquired and available for task execution. |  |  |
| - Communicate clearly and concisely when sharing work instructions. |  |  |
| - Include sub-contractors in pre-start meeting and treat sub-contractors as part of the team. |  |  |
| - Discuss site safety, emergency and risk management procedures. |  |  |
| - Read project plans, specifications, site operational requirements such as those relating to location, or special types of construction. |  |  |
| - Explain any specific organisational, client and site requirements from site plans, specifications, geological, geotechnical data. |  |  |
| - Identify specific council, state or federal requirements for the site including those relating to finishes, quality, reporting and safety. |  |  |
| - Consider and explain any possible construction problems (or lack of), regarding the type of rock found onsite. |  |  |
| - Consider and explain any possible construction problems (or lack of), regarding the type of soil types found onsite. |  |  |
| - Identify and explain any possible above or sub-soil drainage required from relevant site plans. |  |  |
| - Clearly explain the meteorological data available for the site (rainfall, humidity, temperature and wind), and identify any issues in production (e.g. If a possibility of rain was evident), quality or safe work practices arising from the forecast. |  |  |
| - Identify key aspects of the contract for work and describe what personnel will be responsible for the work and how they will achieve this. |  |  |
| - Explain the log book, or scheduled servicing procedure for plant and machinery used on the site. |  |  |
| - Concisely explain how faulty or defective equipment will be reported and recorded onsite using example hazard identification forms. |  |  |
| - Interpret site plans or environmental plans to clearly indicate the location of site wash down facilities. |  |  |
| - Explain any environmental hazards posed from the use of plant and machinery including control measures which could be used to control these environmental hazards. |  |  |
| - Listen to and respond effectively to questions and queries. |  |  |
| - Ensure all workers understand the SWMS for the task and have signed their understanding. |  |  |
| - Ensure signs and barriers have been installed to protect work area as required. |  |  |
| - Set out task correctly from engineers’ survey pegs using plans and specifications. |  |  |
| 3 Oversee execution of tasks |  |  |
| - Watch workers undertake tasks, checking that tolerances are met as required. |  |  |
| - Monitor workers for safety concerns, making sure manual handling procedures are followed and work activities are conducted safely. |  |  |
| - Watch plant and equipment use to confirm machinery is used safely and within machine capabilities and limitations. |  |  |
| - Clearly communicates with workers regarding task execution using approved and understood communication methods. |  |  |
| - Write notes in diary if safety or compliance issues arise. |  |  |
| - Write notes of measurements made, material quality, deliveries and docket checks. |  |  |
| - Organise and make adjustments to civil works practice and job plan as required and confirm required outcomes are achieved. |  |  |
| - Confirm personnel allocated has checked that log books have been filled out and scheduled servicing has been undertaken on plant and equipment. |  |  |
| - Confirm that faulty or defective equipment is reported and recorded according to site requirements. |  |  |
| - Check that plant clean down is conducted in site wash down facilities. |  |  |
| - Oversee work area clearing and waste disposal to confirm site recycling and waste management is adhered to. |  |  |
| 4 Report on the execution of tasks |  |  |
| - Make verbal reports to relevant personnel regarding completion of stages, reaching milestones, testing required. |  |  |
| - Complete all diary entries clearly and legibly. |  |  |
| - Ensure all test results and checks are documented and stored according to site requirements. |  |  |
| - Make verbal or written recommendations regarding changes to task execution. |  |  |