| Task/ Activity | Description of Specific Hazard / Issue | Controls Identified | Responsible for implementation | How will the controls be monitored – what tools or processes will be used | Approval by | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Working at Heights | - Falls from heights - Falling Objects from Height - Failure of Scaffold and or Temporary works platforms | Physical Controls - Handrails - Designated Access points - W@H Personal protection systems - Levelled Ground - Work to be completed on ground where practicable - Guarding - Mechanical aids - Housekeeping and foundation prep - Platform Ladders - Exclusion Zones - Elevated Work Platforms - Scaffold - Doka System - Fall restraint equipment System Controls - RWP Workplace Health and Safety Management Plan - Working at Heights and Around Water Procedure - Harness Permit - Safe Work Method Statement - JHA - Emergency Preparedness - RWP Crisis and Emergency Plan - Rescue Plan - Temporary Work Design and Management Procedure - Hazardous Manual Tasks Procedure - Engineered Scaffold and temporary works drawings - Scaffold Management Procedure - Personal Protective Equipment Procedure - Temporary Works co Ordinator Training and Qualifications - Organisation online induction - RWP site induction - General Construction induction - RIIWHS204E - HRWL - Specialised Doka Training - Temporary Works Co Ordinator training and authorisation | Site Superintendents, Supervisors, Safety Advisors | - Lucidity Inspection schedule - Monthly Scaffold inspections - Doka inspections - Registers - Construction Meetings - Critical Risk reviews - Observations - HSR Meetings - Audits - SLT inspections - Temporary work registers | Superintendent/Engineer | Open - Ongoing |
| Task/ Activity | Description of Specific Hazard / Issue | Controls Identified | Responsible for implementation | How will the controls be monitored – what tools or processes will be used | Approval by | Status |
| Pedestrian access Traversing site | Mobile plant interaction | Physical Controls - Designated walkways - Barricading - Exclusion zones - Artificial lighting - Positive Communication - Reflective work wear - Foundation preparation - VMP System Controls - Working In and Around Plant Procedure - Managing Workplace Amenities Procedure - Personal Protective Equipment Procedure Training and Qualifications - Organisation online induction - RWP site induction - General Construction induction | Site Superintendents, Supervisors, Safety Advisors, all workers | - Work area inspections - HSR meetings - SLT Meetings - Safety Valuations - Hazard Reports - Construction meetings | Superintendent/Engineer | Open - Ongoing |