CPCCBC4026A Arrange building applications and approvals &

CPCCBC4031A Process client requirements

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Hamilton Hayden |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 15/01/2019 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 56 |  | Yes | No |

Identify four (4) details you would try to establish about the client’s design requirements.

| Answer | Client preferences for the project (eg: use of local materials) Client Quality Expectations (safety, design quality) Client Policies (energy or sustainability) Client Structural strategy (floor to height) |  |
| --- | --- | --- |

In what format should the client’s design requirements be recorded?

| Answer | Project Brief |  |
| --- | --- | --- |

Describe how explaining a product’s strengths and limitations can ensure the client’s design needs are met.

| Answer | The client can be assured they have selected a product that is well matched to their needs. |  |
| --- | --- | --- |

Identify two (2) types of technical/product knowledge you should provide to a client who wishes to maximise sustainability.

| Answer | Source of product; Availability in bulk; Lifespan of Product. |  |
| --- | --- | --- |

Which of the following may be considered a restrictive covenant? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Limit on building heights |  |
|  |  | Limit on fence heights |  |
|  |  | Limit on retaining wall heights |  |

Why should your client be notified about restrictive covenants?

| Answer | There may be implications for the client’s request. Eg: They want a 3-storey house but the local limits are 2-storeys. Restrictive covenants may require design change. If a covenant is breached, legal action may be taken against the party breaching it. |  |
| --- | --- | --- |

What document sets out what a client can build within the Council area?

| Answer | The Three Cities Plan. Sydney Council Documents. |  |
| --- | --- | --- |

- Outline the requirements of the Work Health and Safety Act 2011 with regards to the design of buildings.

| Answer | WHS Act states that there is a duty to ensure that the design does not pose risks to people using the building for its intended purpose. |  |
| --- | --- | --- |

True or False (Tick your response): The client requesting a design from an architect automatically holds copyright of the design.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify five (5) site features that would be identified when preparing a site plan.

| Answer | Site Locations, Access, General Layout, Surface Features, Existing Buildings. |  |
| --- | --- | --- |

Explain the role of a drilling rig in establishing site costs.

| Answer | Gather geology samples for identifying the strength of the foundations. |  |
| --- | --- | --- |

Identify three (3) site works activities that may be conducted by the builder.

| Answer | Activities intended to render an area suitable for building. Determining and setting out building or construction features. Earthmoving and establishing access/egress to and from the site. |  |
| --- | --- | --- |

Identify two (2) documents that a client would receive in relation to site investigations and costs.

| Answer | Ground Condition Report Preliminary Cost Report, Plans, Site photographs, Aerial Shots |  |
| --- | --- | --- |

Identify two (2) disclaimers you may need to give a client with regards to site costs.

| Answer | Pre-contract Conditions. Conditions for worse conditions encountered during excavation. Sloping sites which require stronger foundations. |  |
| --- | --- | --- |

Outline why it is important that the requirements (including variation requests) for sketch plans and drawings should be accurately communicated to drafting personnel.

| Answer | To ensure that the design intent is constructed accurately. To ensure time is not wasted and that architect and client agree on the work to be performed before detailed drawings are made. |  |
| --- | --- | --- |

- What might be prepared by the architect to quickly and simply explore initial ideas for designs?

| Answer | A hand sketch / Concept Drawing |  |
| --- | --- | --- |

- Name three (3) types of detailed plans that may be forwarded to the client for approval.

| Answer | Architectural Drawings Structural Engineer Drawings Mechanical and Electrical Drawings, Plans, Sections, Elevations, 3D Imagery |  |
| --- | --- | --- |

- Name three (3) costing details that should be able to be established from sketch plans.

| Answer | Costs relating to floor areas, likely forms of construction, façade treatments, parking, access |  |
| --- | --- | --- |

True or False (Tick your response): All documents should be sighted by the client, with details initialled.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What of the following are preliminary contracts that a builder could ask a client to sign prior to signing the main contract? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Estimate |  |
|  |  | Handover certificate |  |
|  |  | Quote |  |

True or False (Tick your response): Pre-contract agreements may influence the final contract development.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify one (1) standardised commercial building contract and explain the maximum value of the work it may be used for.

| Answer | Commercial Building Contracts – A lump sum contract to be used for the construction only of commercial building work with or without a Principal’s agent. |  |
| --- | --- | --- |

- Identify an appropriate standardised contract that would be appropriate for a house renovation exceeding $20,000.

| Answer | Level 2 – Renovation, Extension and Repair Contracts |  |
| --- | --- | --- |

Which of the following parties could preliminary contracts be submitted to for checking and recording? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contract association (e.g. Master Builders) |  |
|  |  | Parliament |  |
|  |  | Solicitor |  |

- List two (2) permits that may need to be organised when preparing contract documents.

| Answer | Building Approval, Permit for Electricity Supply, Permit for Rubbish Disposal. |  |
| --- | --- | --- |

- Outline the importance of preparing construction documents according to contract details.

| Answer | It will ensure that client requirements are met; e.g.: Materials orders are correct, subcontractors are organised accordingly. |  |
| --- | --- | --- |

For the purpose of development applications, describe how ‘development’ is defined in the legislation.

| Answer | Carrying out building work Carrying out plumbing or drainage work |  |
| --- | --- | --- |

At which of the following stages would a development approval be sought? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Town planning stage |  |
|  |  | Building approval stage |  |
|  |  | Construction stage |  |
|  |  | Final certification stage |  |

True or False (Tick your response): Building approval is separate to planning approval.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): Full approval is one type of approval that may be sought for the building approval stage.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

For each of the external specialists categories listed below, identify the information and documentation they may provide for an approval application.

| Answer | External specialist | Information and documentation provided |  |
| --- | --- | --- | --- |
|  | Building surveyors, quantity surveyors and site surveyors | Construction Cost Estimate, details about the site or building |  |
|  | Geotechnical and environmental specialists | Ground Conditions. |  |
|  | Structural, mechanical and electrical engineers | Structural Design |  |

Name two (2) documents that are required under legislation to support a development application.

| Answer | A planning report, Plans of the Development |  |
| --- | --- | --- |

What documents are required by Council to support a development application (in addition to those under Q32. )?

| Answer | Completed Development Services Lodgement Form Erosion Hazard Assessment Form Land Owner’s Consent |  |
| --- | --- | --- |

When arranging for building approval through a building certifier, you must give them scaled and detailed plans. Who should these plans be prepared by?

| Answer | An architect, or qualified draftsperson |  |
| --- | --- | --- |

Name three (3) approvals that must be lodged through the Council, as opposed to through a building certifier.

| Answer | Any town planning matters that are not accepted development Houses more than 9.5m in height Redevelopment or removal of heritage and character buildings |  |
| --- | --- | --- |

What should be considered when developing a plan for the process of lodging approval applications?

| Answer | Consider the scheduling required to obtain all of the required information. Client needs. |  |
| --- | --- | --- |

True or False (Tick your response): The plan in Q36. should be submitted for approval.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

The council assesses development under the application types listed below. Describe each assessment type and whether the public must be notified.

| Answer | Assessment type | Description |  |
| --- | --- | --- | --- |
|  | Code assessment | A code development application is assessed against the codes identified for the use within City Plan. This type of application can be assessed relatively quickly and does not require public notification. Where an accepted development, subject to requirements does not meet one or more of the acceptable outcomes of the relevant code, a code application is triggered. The application will only need to address the aspects of development that do not meet the accepted development requirements. |  |
|  | Impact assessment | An impact assessable development application is for development that must be assessed against all identified codes and City Plan as a whole to the extent relevant. This is to ensure that identified development impacts are addressed. These applications must be publicly notified to take into account the community's views |  |

Identify two (2) stakeholders that may be impacted by a planning application. Describe the impact on each stakeholder and strategies that could be adopted to maximise their support of the planning application.

| Answer |  | Stakeholder 1 | Stakeholder 2 |  |
| --- | --- | --- | --- | --- |
|  | Stakeholder type | Neighbour | Local Community |  |
|  | Impact of planning application on stakeholder | Ensure design intent does not impact the function of neighbouring design. | Ensure future works does not impact normal community function. |  |
|  | Strategies adopted to maximise stakeholder support | Notify | Notify, community consultation |  |

Identify the Council documentation that outlines whether your proposal will require an impact assessment.

| Answer | Part 5 of City Plan |  |
| --- | --- | --- |

Name three (3) projects that are always deemed ‘accepted development’.

| Answer | Building work to restore a building that has been accidentally damaged or destroyed. Temporary Buildings Sales Offices on a lot in a residential zone. |  |
| --- | --- | --- |

True or False (Tick your response): Projects that are ‘accepted development, subject to requirements’ do not need anything in writing from the Council to commence the activity, provided that a self-assessment has been carried out to ensure compliance with the relevant code.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Other than legislation relating to planning, name two (2) types of building legislation/requirements considered during the building approval process.

| Answer | Building Act, Building Code of Australia |  |
| --- | --- | --- |

During the building approval process, which of the following issues are considered under various codes and regulations? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building is soundly/safely designed and constructed |  |
|  |  | Degree of fire safety |  |
|  |  | Energy and water efficiency standards |  |
|  |  | Protection from pests |  |
|  |  | Sewerage and drainage |  |

Identify and describe two (2) industry standards that you may refer to when preparing a building application (e.g. to address Q44.).

| Answer | Standard number | Description |  |
| --- | --- | --- | --- |
|  | AS4505 | Garage Doors |  |
|  | AS1839 | Swimming Pools |  |

Identify and describe two (2) regulations that you may refer to when preparing a building application.

| Answer | Regulation name | Description |  |
| --- | --- | --- | --- |
|  | BD 004 | Masonry Structures |  |
|  | BD 001 | Steel Structures |  |

Describe the importance of using document control processes when submitting an application.

| Answer | To ensure that all documents are completed accurately and filed for future reference. The Current information is available. |  |
| --- | --- | --- |

How the currency of a plan determined?

| Answer | Check the revision or issue. |  |
| --- | --- | --- |

True or False (Tick your response): To ensure continuing progress of your application, you should confirm the application status at appropriate intervals.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify (and name, if applicable) whether there is an electronic portal used to check applications submitted to Council?

| Answer | PD Online for Brisbane City Council |  |
| --- | --- | --- |

Order the following stages from 1 to 6 to identify the application assessment process used by the Council.

| Answer | Stage Number | Stage |  |
| --- | --- | --- | --- |
|  | 6 | Appeals |  |
|  | 1 | Application |  |
|  | 5 | Decision and conditions |  |
|  | 3 | Information request |  |
|  | 4 | Public notification |  |
|  | 2 | Referrals |  |

True or False (Tick your response): If a planning application is incomplete at the time of lodgement, the Council will always issue a ‘not properly made’ action notice.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

The applicant has how many business days to comply with an action notice?

| Answer | 20 |  |
| --- | --- | --- |

Identify three (3) common application problems.

| Answer | Insufficient Information, Poor Quality Plans, Unacceptable Impacts |  |
| --- | --- | --- |

True or False (Tick your response): When considering an appeal, the applicant should seek private legal advice.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Minor amendments to an application should be negotiated with which of the following parties? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Approval authority |  |
|  |  | Client |  |
|  |  | Organisation |  |