CPCCBC4006B

Select, procure and store construction materials for low rise projects

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4006B Select, procure and store construction materials for low rise projects cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Lauren McKew |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4006B

Select, procure and store construction materials for low rise projects

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Lauren McKew |
| --- | --- |
| Participant signature |  |
| Date | 16/05/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 44 |  | Satisfactory | Not Yet Satisfactory |

ASSESSOR Comments

- Which of the following documents and/or people would you refer to when selecting materials for a building project? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Australian Standards |  |
|  |  | Contract |  |
|  |  | Drawings |  |
|  |  | Manufacturer |  |
|  |  | National Construction Code |  |
|  |  | Specifications |  |
|  |  | Tolerances |  |

- Identify the Volume and Parts of the NCC that relate to suitability of materials for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume Two, Part A2 | Volume One, Part A2 |  |

- Identify the assessment methods used to verify a performance solution.

| Answer | A performance solution must show that it is compliant with relevant performance requirements by one or a combination of the following:- Evidence of suitability which shows the use of a material, product, plumbing and drainage product, form of construction or design meets the relevant performance requirements, verification method as set out in the NCC, expert judgement or a comparison with deemed to satisfy provisions. |  |
| --- | --- | --- |

- Which of the following solutions for material selection often references the Australian Standards? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Deemed-to-Satisfy solution |  |
|  |  | Performance solution |  |

- Identify five (5) materials commonly used for low rise buildings in your region. For each material, identify the following:

- Its name

- Properties of the material

| Answer | # | Material name | Properties |  |
| --- | --- | --- | --- | --- |
|  | 1 | Concrete | - mixture of water, cement and aggregates - Cement and water harden and bind aggregate together to create a strong and durable material commonly used in construction |  |
|  | 2 | Engineered Timber Products | - Used for load-bearing and non-load-bearing construction - Can be made from conventional timber or a combination of wood products and binders to create enhanced timber materials for a wide range of construction applications |  |
|  | 3 | Glass | - made from mixtures of sand and silicates - very brittle and must be handled with care - used in construction for a variety of purposes including windows, doors, features, fencing, splashbacks, mirrors etc. |  |
|  | 4 | Structural Steel | - usually used as a structural component to support other elements in construction – ie. formwork for concrete - strong, flexible, and generally lasts a long time - Forms the backbone of many structures in construction |  |
|  | 5 | Plasterboard | - Usually used to construct ceilings and walls - Generally made from a layer of gypsum plaster sealed between thick paper - Different types of materials are used to make it depending on the application (ie. wet areas, acoustic requirements etc). |  |

- For each of the materials in Q5. identify the following:

- Quality of the material (i.e. durability, serviceability, cost effectiveness)

- Compatibility and non-compatibility of the material (i.e. what it would/would not be suitable for)

| Answer | # | Quality | Compatibility |  |
| --- | --- | --- | --- | --- |
|  | 1 | - Readily available - Available in many different strengths and classes to suit different construction requirements - Relatively inexpensive depending on mix design - Long life span | - Suitable for small structures right up to large, loadbearing, engineered structures (with the assistance of reinforcements within the structure) - Long lasting and suitable for multiple uses - Specific mix can be requested in drawings/specifications in order to deal with exposures/elements to prolong life of structure |  |
|  | 2 | - Widely available - Available in many different strengths for different applications - Can be made from many different type of trees or combinations of timbers to create a strong and durable product | - Suitable for many structures in construction due to the varied ranges available - Can be used as a structural element to a building due to its strength qualities, ability to flex and vertical load strength when compressed - If not treated appropriately can be exposed to rot/chemical reactions/insect destruction therefore timber type must be specified for the type of construction |  |
|  | 3 | - Available in many different strengths, colours and styles to suit multiple applications - Easily procured however transport/handling costs can be expensive due to fragile nature of glass - Lets light in to a structure and is critical for design and liveability of a building | - Suitable for windows, doors, mirrors, fencing, splashbacks, ceilings, features and many other applications of a building - Not usually suitable as a structural element of a structure unless specified and paired with strengthening material |  |
|  | 4 | - Available in a wide variety of strengths and classes and can be engineered to suit required applications - Man-made from iron melted down and combined with other substances to meet required properties depending on application | - Suitable for many types of construction including structural elements, coverings and features - Treated to withstand many elements for long-lasting durability |  |
|  | 5 | - Generally quick to install and inexpensive to purchase - Can be finished in a variety of pays including paint, tiles, render, wallpaper etc - Relatively low environmental impact and quick to manufacture | Suitable for a wide range of applications – mainly used for finishing framed surfaces – ie. walls and ceilings - Requirements can be specified for different applications – ie. acoustic requirements, wet areas, fire resistance - Not generally suitable as a structural element or when exposed to weather without being finished/sealed |  |

- Identify five (5) physical characteristics that may be considered when evaluating potential building materials.

| Answer | Five physical characteristics that are considered when evaluating potential building materials include: - Density and specific gravity - Strength - Electrical conductivity - Thermal conductivity and capacity - Moisture absorption - Acoustics |  |
| --- | --- | --- |

- Match the terms and definitions below by writing the appropriate letter next to each number in the centre column.

| Answer | Term |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Carbon footprint | 1 -A | A | The amount of energy required to manufacture a product |  |
|  | 2 | CO2e | 2 -E | B | Total GHD emissions produced during manufacture construction and use |  |
|  | 3 | Embodied energy | 3 -A | C | Greenhouse gas emissions |  |
|  | 4 | Embodied water | 4 -D | D | The amount of water required to manufacture a product |  |
|  | 5 | GHG | 5 -C | E | CO2equivalent tonnes emitted into the atmosphere |  |

- Which of the following materials would be considered as ‘low energy content’? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Bricks |  |
|  |  | By-products of other industrial processes |  |
|  |  | Cement |  |
|  |  | Glass |  |
|  |  | Metals |  |
|  |  | Plastics |  |
|  |  | Sand |  |
|  |  | Stone |  |
|  |  | Timber |  |

- Refer to the table below and identify the wall material type that has the smallest embodied energy and emission impact.

| Answer | Walls timber frame fibro clad |  |
| --- | --- | --- |

- Identify the Volume and Parts of the NCC that relate to energy efficiency of materials for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume Two - Part 2.6 | Volume One – Part J0 |  |

- Identify the relevant section of the Standards and Tolerances Guide and the associated Australian Standard that addresses residential slabs and footings.

| Answer | Standards and Tolerances Guide | Australian Standard |  |
| --- | --- | --- | --- |
|  | Section 3 | AS2780 |  |

- Identify the relevant section of the Standards and Tolerances Guide and the associated Australian Standard that addresses roof tiles.

| Answer | Standards and Tolerances Guide | Australian Standard |  |
| --- | --- | --- | --- |
|  | Section 7.4 | AS2049 & AS2050 |  |

- Identify the Volume and Parts of the NCC that relate to safety barriers for swimming pools for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume Two – Part 3.10.1.0 | Volume One – Part GP1.2 |  |

- In some states, barriers and fences for swimming pools are not entirely regulated by the NCC, as noted in the Ancillary Provisions and State and Territory Variations. Identify the applicable Act and Regulation that regulate safety barriers for swimming pools in your jurisdiction.

| Answer | Restriction of access to swimming pools in Queensland is regulated under the Building Act 1975 |  |
| --- | --- | --- |

- Identify the Australian Standard that relates to safety barriers for swimming pools.

| Answer | AS1926.1 and AS1926.2 |  |
| --- | --- | --- |

- True or False (Tick your response): According to the Standard in Q16. all gates must be fitted with a self-closing device that will return the gate to a closed position from any position without the use of manual force.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify the Volume and Parts of the NCC that relate to fire-resistance of building elements for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume Two – Schedule 5 | Volume One – Schedule 5 |  |

- Identify the Australian Standard that relates to fire-resistance tests for elements of construction.

| Answer | Fire-resistance tests of elements of building construction are described in AS1530.4. |  |
| --- | --- | --- |

- For all NCC Classes, where the ground or subfloor space is excessively damp or subject to frequent flooding, which durability class must the subfloor framing must be used where above ground?

| Answer | Where the subfloor space is above ground subfloor framing must be durability Class 1 or 2 timbers or H3 preservative treated timbers in accordance with AS1684.2, AS1684.3 or AS1684.4. |  |
| --- | --- | --- |

- Identify the Australian Standards that must be observed when meeting the requirements for Q20.

| Answer | AS1684 |  |
| --- | --- | --- |

- True or False (Tick your response): Where materials are not used as intended (or without considering NCC requirements), this can make a significant difference to the life or serviceability of the structure.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Explain why durability and serviceability impact on the cost effectiveness of chosen materials.

| Answer | Confirming the material is able to perform its intended purpose for a long period of time ensures that the material is cost effective and will not require replacement soon after installation. Selecting materials for their life-expectancy, ability to serve their purpose and suitability for the application ensures a long lasting and valuable product. Quality products which are durable should be selected to ensure that the material’s lifespan is equivalent to the projected life of the building or structure. |  |
| --- | --- | --- |

- Provide an example of short term degradation of materials and describe its effects on a building on structure.

| Answer | When exposed to outdoor elements such as weather or exposed to ingress of insects, some timbers suffer from short term degradation. Rot, crumbling, splitting and weathering are all symptoms of timber degradation. Ensuring the correct timber (ie. treated timber, naturally resistant timbers, hardwood etc) is selected for the type of construction will minimise the likelihood of short-term degradation and the negative effect this would have on a build. |  |
| --- | --- | --- |

- Provide an example of long term degradation of materials and describe its effects on a building on structure.

| Answer | Some metals can suffer from slower, longer term degradation meaning they will eventually need to be replaced. Metals can corrode when exposed to other elements such a oxygen or water. Ensuring a material which is likely to degrade slower than another is selected will help alleviate some issues that arise from metal degradation – ie. life expectancy will still be within reason and maintenance/replacement over time will limit the effect this cost has on the building. |  |
| --- | --- | --- |

- You have spoken to a supplier who has indicated that some of the client’s desired materials are unavailable. When considering alternative building materials what must be taken into consideration (i.e. other than the client’s requirements)?

| Answer | When considering an alternative material, its performance to meet or exceed the client’s desired material’s properties, its efficiency, its environmental sustainability and/or its cost effectiveness should be taken into consideration and discussed with the client prior to procuring the alternative items. |  |
| --- | --- | --- |

- Identify two (2) things that you should look for when receiving a delivery of materials.

| Answer | - Confirm the correct quantity has been supplied compared with what was ordered - Confirm the delivery is in good order and is not damaged |  |
| --- | --- | --- |

- If you encounter issues with delivery, what two (2) options do you have?

| Answer | - Accept the delivery with ‘conditions’ ensuring these are recorded - Reject the delivery in its entirety |  |
| --- | --- | --- |

- Which of the following should be assessed before shifting materials? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | What needs to be relocated (e.g. size, weight) |  |
|  |  | The relocation area, site layout and obstacles |  |
|  |  | Distance and time to relocate load |  |
|  |  | Whether a team is available and/or appropriate |  |
|  |  | Whether PPE must be worn (e.g. for hazardous materials) |  |

- What document should you refer to for guidance on how to handle and store hazardous chemicals?

| Answer | Material Safety Data Sheets will provide detailed information on the handling and storage of hazardous chemicals. |  |
| --- | --- | --- |

- Name three (3) ways you can reduce the risk of injury if a load is too heavy for one person to carry, push or pull.

| Answer | Reducing the risk of injury when transporting a load can be done by asking for assistance from other personnel, using mechanical aids or attempting to lighten the load by making multiple trips. |  |
| --- | --- | --- |

- In the diagram below a person is lifting a tool box.
Filling in the table, indicate how each body part should be used/placed.

F. Hands

E. Knees

D. Shoulders

C. Feet

B. Back

A. Neck

| Answer | Area of body (Letter) | How it should be placed |  |
| --- | --- | --- | --- |
|  | D | Level |  |
|  | A | Upright |  |
|  | F | Good grip |  |
|  | B | Natural curve |  |
|  | E | Bent |  |
|  | C | Flat, facing load |  |

- How should construction materials and components be handled? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Sorted to suit material type and size, separating hazardous material |  |
|  |  | Stacked or stockpiled for ease of identification and retrieval |  |
|  |  | Stored in access ways for ease of retrieval |  |
|  |  | Stored clear of access ways |  |
|  |  | Handled and positioned ready for installation |  |

- Identify the Australian Standard that relates to the storage and handling of LP gas.

| Answer | AS1596 relates to the storage and handling of LP gas. |  |
| --- | --- | --- |

- List one (1) testing method for each of the building materials listed below.

| Answer | Material name | Properties |  |
| --- | --- | --- | --- |
|  | Concrete | Concrete compressive strength is tested by taking cylinder samples of the concrete and then are tested in a hydraulic press for strength and durability. |  |
|  | Metals | Repeated loading is a test often used for different types of metals. It involves alternatively inflicting load and tension on the item over hundreds of times to test its ability to endure the pressure and compression. |  |
|  | Timber | Stress grading machines deflect the timber at several points along its length the deflections are then marked and recorded. The timber is then classified visually by its colour markings. |  |

- What Code gives detailed tables which state the required grade of timber for specific sizes and uses?

| Answer | Australian Timber Framing Code |  |
| --- | --- | --- |

- Name two (2) ways that defects may be seen in timber.

| Answer | Two types of defects which may be seen in timber include splits and evidence of insect damage. |  |
| --- | --- | --- |

- Identify two (2) tests that may be performed to determine the durability of stone.

| Answer | Durability of stone can be determined through acid immersion which is used to test how reactive the stone is to acidic conditions. Similarly, if water retention is an important factor in the durability of the stone in its application, saturation coefficients can be tested to determine suitability. |  |
| --- | --- | --- |

- Name two (2) ways that defects may be seen in stone.

| Answer | Defects in stone can be visually identified through the presence of cracks or discolouration where there should not be those imperfections or there are uniformity requirements for the application of the material. |  |
| --- | --- | --- |

- True or False (Tick your response): Under a contract, builders and other specialist tradespersons are the ones most likely to rectify or remedy problems that arise from incorrect installation, application or placement of materials.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Name two (2) established methods that may be used to preserve timber.

| Answer | Brushing or spraying timber with preservative treatments such as oils, chemicals and solvents are commonly used to protect timber from fungi and insect infestation. Alternatively or in conjunction with sprayed/painted treatments, metal barriers can be installed to deter insects from entering into the timber. |  |
| --- | --- | --- |

- Identify two (2) established methods that may be used to protect ferrous metals.

| Answer | Preserving iron and steel is commonly done by applying particular coatings such as galvanising or specialised paint. It can also be done by using cathodic protection which aims to control corrosion by making it the cathode of an electrochemical cell. |  |
| --- | --- | --- |

- Explain why the corrosion resistance of non-ferrous metals is superior to that of ordinary steel.

| Answer | Resistance to corrosion of non-ferrous materials such as aluminium, zinc and copper is superior to ordinary steel as they tend to produce a ceramic coating of oxide or salt on the surface which protects the metal from further corrosion. |  |
| --- | --- | --- |

- When should aluminium be coated with bituminous paint? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | At all times |  |
|  |  | When it is buried in soil |  |
|  |  | When it comes in contact with chemical materials |  |
|  |  | When it is immersed in water |  |
|  |  | When it is used on bitumen |  |

CPCCBC4006B

Select, procure and store construction materials for low rise projects

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Lauren McKew |
| --- | --- |
| Participant signature |  |
| Date | 19/05/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date |  |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4006B

Select, procure and store construction materials for low rise projects

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to supervise systems through which materials are selected, acquired and stored on site for the following projects: - Occasion 1: Construction classified as Class 1 and 10 construction - Occasion 2: Construction classified in Classes 2 to 9 with a gross floor area not exceeding 2000 square metres, not including Type A or Type B construction The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - Copy of PO sent to site team for materials. - Email correspondence sighting need for materials, delivery of materials and storage of materials (if relevant). The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer, Building Contract, Drawings, Specifications, Building materials (including timber, and ferrous and non-ferrous metal). |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Lauren McKew |  |  |
| Name of observer: | Jarryd Londino |  |  |
| Role of observer (Confirm suitability to sign) | Project Engineer |  |  |
| Email address of observer | jarryd@fgf.com.au |  |  |
| Signature of observer: |  |  |  |
| Date: | 19/05/2021 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| 1 Evaluate building materials |  |  |  |
| - Identify suitability of materials commonly used in the region for low rise buildings. |  |  |  |
| - Through research and testing, identify properties of different materials, their quality and compatibility and non-compatibility and maintain effective sampling and organisational record-keeping processes. |  |  |  |
| - Identify the environmental impacts of different materials. |  |  |  |
| - Identify the impact of allowable tolerances on the conversion of naturally occurring materials. |  |  |  |
| - Identify and check tolerances for installing and assembling materials with regard to the nature of the work being performed and the requirements of relevant construction industry Australian standards. |  |  |  |
| 2 Material selection |  |  |  |
| - Select materials structurally adequate and appropriate for the building system specified in the contract, drawings and specifications. |  |  |  |
| - Select materials after considering: - Safety - Required fire resistance rating - Suitability to the application - Durability - Serviceability and cost effectiveness - Compliance with NCC, legislation, regulations and Australian Standards |  |  |  |
| - Consider short and long-term degradation of materials in relation to the building’s proposed life cycle. |  |  |  |
| - Select and evaluate alternative materials, if specified materials were unavailable or unsuitable – justifying decisions to regulatory authorities, as necessary |  |  |  |
| - Finalise selection of materials in accordance with contractual requirements and in consultation with relevant professionals and the client, compiling results in a report. |  |  |  |
| - Effectively communicate using the following methods: - Language/concepts appropriate to cultural differences - Non-verbal communication |  |  |  |
| 3 Supervising materials on site |  |  |  |
| - Determine limitations and effects of transportation on material and components, taking action in potentially damaging circumstances. |  |  |  |
| - Ensure materials were correctly and safely handled on site using appropriate equipment and safe work practices. |  |  |  |
| - Communicate with manufacturers and suppliers to ensure materials are stored in accordance with specifications and Australian standards. |  |  |  |
| - Implement processes for inspecting all materials delivered on site for naturally occurring and/or manufactured defects before installation. |  |  |  |
| - Ensure personnel were aware of actions to be taken in the case of defects caused by incorrect installation, application or placement. |  |  |  |
| - Preserve and protect timber and ferrous and non-ferrous metal used in the construction process. |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments

As FGF Developments are a civil construction company we do not construct buildings, therefore for this assessment, Lauren and I have been over one of our commercial infrastructure projects and a residential subdivision project that we have recently worked on together to discuss and evaluate required material specifications. I believe Lauren capable of identifying required materials – usually governed by project specifications, local council requirements (FNQROC Regional Development Manual) and project drawings – for procurement purposes. She is effectively able to place orders and control orders including arranging deliveries and storage on site. Our ordering process requires placement of purchase orders which are usually accompanied by site specific delivery instructions regard safe and efficient deliveries to site. Lauren is also required to site delivery dockets signed off by our site foreman and reconcile invoices with regards to the original quote and purchase order. If we have had to alter a specification or suggest an alternate material for a project, Lauren has been able to assist in presenting our argument and gather required evidence in order for the change to be accepted.