Procedure for Accepting Deliveries to site

1. Pre-Delivery Preparation

- Designate a delivery zone: Mark a clear area on-site for drop-offs. Ensure it's accessible and safe.

- Assign a responsible person: Typically the site supervisor, builder, or homeowner should be present for all deliveries.

- Keep delivery schedule on hand: Maintain a printed or digital list of expected deliveries and contact numbers.

2. Accepting Deliveries

When a delivery arrives:

- Inspect Delivery Paperwork:

  - Check delivery docket or invoice against your order confirmation.

  - Verify delivery address and site contact name.

- Check the Following:

  - Correct quantity & type of materials

  - Condition of items (no damage or defects)

  - Brand or product specification matches order (important for Colorbond, insulation, etc.)

- Raise any issues immediately:

  - Note discrepancies or damage on the delivery docket before signing.

  - Take photos of damage or missing items.

  - Notify supplier ASAP for resolution.

3. Recording Deliveries

Maintain a Site Delivery Register or logbook:

- Date & Time of Delivery

- Supplier Name & Contact

- Materials Delivered

- Quantity Received

- Condition Upon Arrival

- Delivery Driver Name & Signature

- Receiver’s Name & Signature

- Notes (e.g., damage, short delivery)

You can do this on paper, or in digital form using tools like:

- Excel / Google Sheets

- Buildxact, AroFlo, or simPRO (for pros)

- Site Diary apps

4. Storage & Safety

- Store materials appropriately:

  - Off the ground, under cover if needed (especially for timber, insulation, or Colorbond sheets).

  - Secure against theft or weather.

- Label clearly for easy access and inventory control.

5. Communication & Follow-Up

- If delivery was incomplete or damaged:

  - Contact the supplier immediately.

  - Record all communications (who you spoke to, when, outcome).

  - Schedule re-delivery if needed.