CPCCBC4012B

Read and interpret plans and specifications

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4012B Read and interpret plans and specifications cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Callum Quinn |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name | Chris Abbott |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Callum Quinn |
| --- | --- |
| Participant signature |  |
| Date | 27/01/2022 |

| Assessor name | Chris Abbott |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 87 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

- Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

- Which compass direction is always shown on plans and drawings?

| Answer | North |  |
| --- | --- | --- |

- What information is provided on a location plan?

| Answer | Building Foot print, legal point of discharge, water and electrical feed, stormwater connection points. Site plans also show the a site on a particular street |  |
| --- | --- | --- |

- What detail does a site or block plan contain?

| Answer | Provides specific detail of the block on which a development is proposed |  |
| --- | --- | --- |

- What do floor plans identify?

| Answer | Shows detail of the proposed structure, such as rooms, positions of fixtures, floor coverings |  |
| --- | --- | --- |

- What is a landscape plan and what information might it provide?

| Answer | Shows details of plants, paths, garden layouts and retaining walls |  |
| --- | --- | --- |

- What are elevation views? What do they provide to the plan reader?

| Answer | Each side is what is called an elevation view. They provide a view of the proposed structure from its sides (i.e. from north, south, east and west). |  |
| --- | --- | --- |

- What is a section?

| Answer | In most cases elevation and plan views cannot show sufficient information to enable a builder or tradesman to see how various structural parts are to be built or assembled. The section is a view of the house after it has been cut vertically at some point. Like the floor plan it shows the size and thickness of structural members and relationships between spaces. |  |
| --- | --- | --- |

- What are detailed drawings?

| Answer | Illustrate a particular method of construction of finish. The details and dimensions are required for APPROVED FOR CONSTRUCTION drawings. |  |
| --- | --- | --- |

- What are specifications?

| Answer | Exact details in regards to the size, type and finish of materials and inclusion of items on the build. |  |
| --- | --- | --- |

- Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bull Nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Block |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Glass |  |
|  | CJ | Clear |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold Water Tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Prove Course |  |
|  | DSB | Distribution Switch Board |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion Joint |  |
|  | FPBD | Fibrous Plasterboard. |  |
|  | FFL | Finished Floor Level |  |
|  | FHR | Fire Hose Reel |  |
|  | GPO | General Purpose Outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

- Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Double Brick |  |  |
|  | Brick Veneer |  |  |
|  | Internal Wall |  |  |
|  | Window |  |  |
|  | Window With Sill |  |  |
|  | 90 degree opening door single leaf |  |  |
|  | 90 degree opening door double leaf |  |  |
|  | Cavity slider |  |  |
|  | Face mounted slider door |  |  |
|  | Single power point |  |  |

- Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Left hinged window |  |  |
|  | Top hinged window |  |  |
|  | Centre closing sliding door |  |  |
|  | Fixed glass panel |  |  |

- What does a clouded area on a plan indicate?

| Answer | A change has been made to this part of the plan |  |
| --- | --- | --- |

- An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

- On what date was an amendment made to the window details?

| Answer | 5/11/2011 |  |
| --- | --- | --- |

- True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What amendment occurred on 08/11/2011?

| Answer | Bed 3 wardrobe was deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

- What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

- What is the Lot number of this site?

| Answer | 16 |  |
| --- | --- | --- |

- What is the size of this lot?

| Answer | 3653m2 |  |
| --- | --- | --- |

- Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

- True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What size is the easement on the eastern boundary of the site?

| Answer | 5m |  |
| --- | --- | --- |

- True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

- What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

- Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

- Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26.?

| Answer | Two story home, Slab on grade, concrete verandas (pored separately) metal roof on scissor truss. 4 Bedrooms. Traditional slab type with edge and 1 internal beam poured with main slab. Timber subfloor and wall framing. |  |
| --- | --- | --- |

- Who is the client?

| Answer | H&M Smith |  |
| --- | --- | --- |

- Where is this project to be built?

| Answer | Lot 16 Glenvalley Road Richmond. |  |
| --- | --- | --- |

- What are the contact details for the draftsman?

| Answer | Joe Forman Ph 03 9514 23 11 Mob 0413113 113 18 fairview rd Richmond 3216 |  |
| --- | --- | --- |

- What sheet number is this drawing?

| Answer | 5 of 7 |  |
| --- | --- | --- |

- What timber is required for loading and non-load bearing walls?

| Answer | Loading F5 45x90 non loading F5 non load baring 35x90 |  |
| --- | --- | --- |

- What spacing is recommended for common studs?

| Answer | 600 |  |
| --- | --- | --- |

- What type of roof truss is to be installed?

| Answer | Scissor truss top chord 30 degrees and bottom chord 20 degrees every truss strapped to wall plate |  |
| --- | --- | --- |

- What centres are the trusses positioned at?

| Answer | 900 |  |
| --- | --- | --- |

- What size are the veranda posts?

| Answer | 90x90mm DAR |  |
| --- | --- | --- |

- What timber species are the veranda posts to come from?

| Answer | OREGON |  |
| --- | --- | --- |

- What pitch is the roof?

| Answer | 30 Degrees |  |
| --- | --- | --- |

- What size are the noggings and at what spacing?

| Answer | F5 35x70 1350 spacing |  |
| --- | --- | --- |

- What size are the ceiling joists?

| Answer | HK 240 joists at 450 spacing |  |
| --- | --- | --- |

- What timber is used for roof battens and what spacing?

| Answer | 35x70 900 spacing |  |
| --- | --- | --- |

- What thickness is the plasterboard sheeting?

| Answer | 10mm |  |
| --- | --- | --- |

- What type of flooring is to be installed?

| Answer | Slab on grade ground floor, 15mm ply flooring to timber sub floor |  |
| --- | --- | --- |

- What is the roof cladding material?

| Answer | Colourbond corrugated meal roofing |  |
| --- | --- | --- |

- Where will you find information about the footings?

| Answer | In the structural drawings. |  |
| --- | --- | --- |

- What do the letters ‘DAR’ refer when timber ordering?

| Answer | Dressed All Round |  |
| --- | --- | --- |

- Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | Laminated veneer lumber (LVL) is one of the most widely used engineered wood products for constructional applications. It is a composite product manufactured from multiple thin layers of veneer that are aligned with the length of the finished lumber. |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

- What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

- What is the Job Number?

| Answer | 00R-112 |  |
| --- | --- | --- |

- What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

- What does the drawing show?

| Answer | Elevation and sections plans for the residential dwelling drawn by Beth in Jan 2000 |  |
| --- | --- | --- |

- Which view shows the front of the house?

| Answer | East Elevation |  |
| --- | --- | --- |

- How many motor vehicles can be parked in the garage?

| Answer | Two Cars |  |
| --- | --- | --- |

- What are the window and doors made from?

| Answer | Aluminium windows, and doors except entry door and door on south elevation as these are timber |  |
| --- | --- | --- |

- What is the size of the eaves?

| Answer | 450mm |  |
| --- | --- | --- |

- What are the eaves lined with?

| Answer | Hardiflex |  |
| --- | --- | --- |

- What type of roof structure is the roof built from?

| Answer | Timber pitch roof truss(flat bottom) |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the requirements for the roof structure in Q59.?

| Answer | 3.4.3 Design of nailplated timber roof trusses: |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | AS 1720.5. |  |
| --- | --- | --- |

- What is the roof cladding materials made from?

| Answer | Concrete tiled roof over reflective sarking. |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Part 3.5 Roof cladding, gutters and downpipes and wall cladding |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | AS 2050—1995, Installation of roof tiles |  |
| --- | --- | --- |

- What is the roof pitch?

| Answer | 22.5 degree |  |
| --- | --- | --- |

- Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | Local council Sunshine Coast Council in our case. |  |
| --- | --- | --- |

- Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | Building approval is applied for with lodgement paperwork completed and possible fee’s applicable paid. Roughly 10 days after this council if happy with what is detailed in the plans will issue building approval. |  |
| --- | --- | --- |

- What is a prime cost (PC) item? Give an example.

| Answer | The cost of an unknown item included in a build. Appliances for example can often be specified and a prime cost of $1500 may be include within the build price. Should the actual cost once the selection is made be greater or less the build cost is then adjusted by this amount. |  |
| --- | --- | --- |

- What is a provisional sum (PS) item? Give an example.

| Answer | Provisional sums are for items that are not yet detailed or can’ be accurately estimated on. For get in some cases the QTY of earthworks can’t be calculated until the earth is actually uncovered and suitable founding material is exposed. In this case there may be for eg a prov sum of $200/m3 for export of unsuitable soils in a build for qty estimated as 50m3. This prov sum would be $10 000. After the earthworks are completed should the amount be greater and actually cost $5000 the client must pay this difference. Should the earthworks only need to remove 25m3 cube or half of the amount assumed then the build cost can be reduced by $5000. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

- True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify two (2) materials being used for the floors?

| Answer | Concrete & Timber |  |
| --- | --- | --- |

- True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- How much does the contracted plasterer cost?

| Answer | $12 900 |  |
| --- | --- | --- |

- How much do the kitchen appliances cost?

| Answer | $2 700 |  |
| --- | --- | --- |

- What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

- What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

- How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

- If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | 100*2.75 for the increase plus the original cost of $100 is $375 |  |
| --- | --- | --- |

- How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 |  |
| --- | --- | --- |

- What material has the highest total cost?

| Answer | Pine 90x45 mpg10 Long Lengths |  |
| --- | --- | --- |

- How many metres are required of the material in Q80. ?

| Answer | 80*3m = 240 Plus 25*3.3=82.5 82.5+240 = 322.50lm |  |
| --- | --- | --- |

- What beam is identified as being a better option than F17?

| Answer | E14 |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

- Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Ashphalter |  |
| --- | --- | --- |

- If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | 107.40*2 = $214.80 |  |
| --- | --- | --- |

- Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar Bender and Fixer |  |
| --- | --- | --- |

- If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | 214.30*6 = $1285.80 |  |
| --- | --- | --- |

- Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | Queensland Building and Construction Commission Act 1991 Queensland Building and Construction Commission Regulation 2018 |  |
| --- | --- | --- |

CPCCBC4012B

Read and interpret plans and specifications

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Callum Quinn |
| --- | --- |
| Participant signature |  |
| Date | 8/12/2021 |

| Assessor name | Chris Abbott |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4012B

Read and interpret plans and specifications

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to read and interpret plans and specifications in order to inform estimation, planning and supervisory activities. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - Any drawing with a cloud or mark-up you have inserted. - Copy of the correspondence regarding mark up and any site management/authorising engineer response. The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Drawings (including site plan), Specifications, Tender documentation, Telephone, Facsimile machine, Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Callum Quinn |  |  |
| Name of observer: | Damien Carroll |  |  |
| Role of observer (Confirm suitability to sign) | Direct Supervisor as senior estimator |  |  |
| Email address of observer | damien.carroll@murphybuilders.com.au |  |  |
| Signature of observer: |  |  |  |
| Date: | 8/12/2021 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| 1 Interpret drawings |  |  |  |
| - Identify purpose and advantage of different types of drawings. |  |  |  |
| - Identify different aspects of drawings. |  |  |  |
| - Identify, understand and apply: - Commonly used symbols and abbreviations - Common building and construction terms on drawings |  |  |  |
| - Identify building site from location drawings. |  |  |  |
| - Identify on the site plan: - True north - Building orientation - Key features in relation to building and other structures - Levels - Contours - Service entry points - Site features to be removed or retained |  |  |  |
| - Identify key features of plans, elevations and sections. |  |  |  |
| - Identify client requested variations to standard plans on drawings. |  |  |  |
| 2 Interpret specifications |  |  |  |
| - Identify and correctly apply Provisional Sum (PS) and Prime Cost (PC) values. |  |  |  |
| - Identify and confirm customer variations to standard specifications and communicate changes to organisational personnel. |  |  |  |
| - Apply and communicate correct interpretations of essential elements to estimation, planning and supervisory tasks (including labour hours/costs and material quantities/costs). |  |  |  |
| - Identify Building Codes or standards affecting the work to be undertaken (including references to Australian Standards and the National Construction Code). |  |  |  |
| - Identify key features of products included in the specification, including the design, purpose, aesthetics and cost relationships. |  |  |  |
| 3 Communicate with others |  |  |  |
| - Consult with industry professionals, clients, organisational personnel and appropriate local authorities. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Using technology (including telephone, facsimile, email) |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments