CPCCBC4012B

Read and interpret plans and specifications

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4012B Read and interpret plans and specifications cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Lauren Wilson |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Lauren Wilson |
| --- | --- |
| Participant signature |  |
| Date | 18/06/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 87 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

- Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

- Which compass direction is always shown on plans and drawings?

| Answer | North point |  |
| --- | --- | --- |

- What information is provided on a location plan?

| Answer | Location plan provides the site location on the particular street |  |
| --- | --- | --- |

- What detail does a site or block plan contain?

| Answer | Provides specific details of the site for the proposed development |  |
| --- | --- | --- |

- What do floor plans identify?

| Answer | Floor plans outline the detail of the proposed dwelling eg fixtures, fittings, floor coverings |  |
| --- | --- | --- |

- What is a landscape plan and what information might it provide?

| Answer | Landscape plan details the proposed layout of pathways, plants, gardens and retaining walls |  |
| --- | --- | --- |

- What are elevation views? What do they provide to the plan reader?

| Answer | Elevation views provide views of the proposed structure from all directions – north, south, east and west. |  |
| --- | --- | --- |

- What is a section?

| Answer | A section is a view of a house that has been cut vertically at some point. It shows the height from the ground to the floor and the height between the floor and the ceiling. It also outlines how the roof is to be constructed. |  |
| --- | --- | --- |

- What are detailed drawings?

| Answer | Detailed drawings are scale plans and dimensions, details and additional notes to demonstrate compliance with regulations and other statutory requirements |  |
| --- | --- | --- |

- What are specifications?

| Answer | Specification is a contractual document to be read in conjunction with the construction plans. This document provides a full description of the works, precise list of materials and finishes, full details of all fixtures to include – appliances, make and model, colours, grades, thickness etc |  |
| --- | --- | --- |

- Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed datum |  |
|  | B | Basin |  |
|  | BN | Bull nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete block |  |
|  | CF | Concrete floor |  |
|  | CG | Clear glass |  |
|  | CJ | Ceiling joist |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold Water Tank |  |
|  | DG | Double glazing |  |
|  | DH | Double hung |  |
|  | DPC | Damp proof course |  |
|  | DSB | Distribn. Switch board |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion joint |  |
|  | FPBD | Fibrous plaster board |  |
|  | FFL | Finished floor level |  |
|  | FHR | Fire hose reel |  |
|  | GPO | General power outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

- Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Brickwork on brickwork |  |  |
|  | Brick work and timber |  |  |
|  | Partition wall |  |  |
|  | Window |  |  |
|  | Window – not sure what kind – checked with project manager at work and he didn’t know either |  |  |
|  | Single leaves opening at 90 deg |  |  |
|  | Two leaves each opening at 90 deg |  |  |
|  | Cavity sliding door |  |  |
|  | Face fixed single door |  |  |
|  | Single power point |  |  |

- Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Hinged at side |  |  |
|  | Hinged at top |  |  |
|  | Sliding Horizontally |  |  |
|  | Fixed |  |  |

- What does a clouded area on a plan indicate?

| Answer | Amendment to plan |  |
| --- | --- | --- |

- An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

- On what date was an amendment made to the window details?

| Answer | 5/10/2011 |  |
| --- | --- | --- |

- True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What amendment occurred on 08/11/2011?

| Answer | Bed 3 robe deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

- What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

- What is the Lot number of this site?

| Answer | Lot 16 |  |
| --- | --- | --- |

- What is the size of this lot?

| Answer | 3653 m2 |  |
| --- | --- | --- |

- Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

- True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What size is the easement on the eastern boundary of the site?

| Answer | 5 m |  |
| --- | --- | --- |

- True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

- What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

- Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

- Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26.?

| Answer | Double storey dwelling Colourbond metal roofing at 30-degree pitch at highest point D.5 reflective foil over battens 35 x 70 FG pine battens at 900 spacing every batten strapped to trusses Scissor trusses at 900 spacing Top chord at 30 degree, bottom chord at 20 degree – every truss strapped to wall plate Intermediate ceiling joists at 450 spacing R 2.5 bulk insulation to ceiling 10 mm plasterboard to walls and ceiling WR board to wet area walls HJ 240 joists at 450 spacing 15 mm plywood flooring Timber weatherboards D.S reflective foil and R 1.5 bulk insulation to all external walls Starting joists 240 x 6.3 LVL continuous lengths Veranda beams 150 x 50 F7 Oregon Veranda rafters – 90 x 35 FS at 900 spacing |  |
| --- | --- | --- |

- Who is the client?

| Answer | H & M Smith |  |
| --- | --- | --- |

- Where is this project to be built?

| Answer | Lot 6 Glenvalley Road, Richmond 3216 |  |
| --- | --- | --- |

- What are the contact details for the draftsman?

| Answer | Joe Foreman Ph: 03 9514 2311 Mob: 0413 113 113 18 Fairview Road, Richmond, 3216 |  |
| --- | --- | --- |

- What sheet number is this drawing?

| Answer | 5 of 7 |  |
| --- | --- | --- |

- What timber is required for loading and non-load bearing walls?

| Answer | 45 x 90 loadbearing 35 x 90 non load bearing |  |
| --- | --- | --- |

- What spacing is recommended for common studs?

| Answer | 600 max spacing |  |
| --- | --- | --- |

- What type of roof truss is to be installed?

| Answer | Gang nail roof trusses at 900 max spacing |  |
| --- | --- | --- |

- What centres are the trusses positioned at?

| Answer | 900 spacing, top chord, 30 deg, bottom chord 20 deg |  |
| --- | --- | --- |

- What size are the veranda posts?

| Answer | 90 x 90 |  |
| --- | --- | --- |

- What timber species are the veranda posts to come from?

| Answer | DAR OREGON |  |
| --- | --- | --- |

- What pitch is the roof?

| Answer | Highest point - Pitch 30 deg Lowest point - 15 deg |  |
| --- | --- | --- |

- What size are the noggings and at what spacing?

| Answer | 35 x 70 1350 max spacing |  |
| --- | --- | --- |

- What size are the ceiling joists?

| Answer | HJ joists at 450 spacing |  |
| --- | --- | --- |

- What timber is used for roof battens and what spacing?

| Answer | 35 x 70 FG pine battens at 900 spacing every batten strapped to trusses |  |
| --- | --- | --- |

- What thickness is the plasterboard sheeting?

| Answer | 10 mm plasterboard to walls |  |
| --- | --- | --- |

- What type of flooring is to be installed?

| Answer | 15 mm plywood flooring |  |
| --- | --- | --- |

- What is the roof cladding material?

| Answer | Colourbond corrugated metal roofing |  |
| --- | --- | --- |

- Where will you find information about the footings?

| Answer | On the footing / slab plan – this must be approved by an engineer |  |
| --- | --- | --- |

- What do the letters ‘DAR’ refer when timber ordering?

| Answer | Dressed all round – timber is planed and all edges are smooth |  |
| --- | --- | --- |

- Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | Timber sizes - Laminated veneer lumber |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

- What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

- What is the Job Number?

| Answer | 00R-112 |  |
| --- | --- | --- |

- What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

- What does the drawing show?

| Answer | Elevations and sections |  |
| --- | --- | --- |

- Which view shows the front of the house?

| Answer | East elevation |  |
| --- | --- | --- |

- How many motor vehicles can be parked in the garage?

| Answer | 2 |  |
| --- | --- | --- |

- What are the window and doors made from?

| Answer | Aluminium |  |
| --- | --- | --- |

- What is the size of the eaves?

| Answer | 450 |  |
| --- | --- | --- |

- What are the eaves lined with?

| Answer | Hardiflex |  |
| --- | --- | --- |

- What type of roof structure is the roof built from?

| Answer | Timber frames roof trusses |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the requirements for the roof structure in Q59.?

| Answer | Volume 2 - Part 3.0 – Structural provisions |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | Australian Standard – AS 4440 – 2004 – Installation of Nail plated timber roof trusses |  |
| --- | --- | --- |

- What is the roof cladding materials made from?

| Answer | Concrete roof tiles |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Volume 2 – Part 3.5.1Roof Cladding – acceptable construction materials |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Australian Standard – AS 2050 – 1995 – Installation of Roof Tiles |  |
| --- | --- | --- |

- What is the roof pitch?

| Answer | 22.5degrees |  |
| --- | --- | --- |

- Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | Local council or Building Certifier |  |
| --- | --- | --- |

- Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | Complete and submit – pay fees IDAS form 1 & 2 Proof of land ownership – eg rates notice Sewer plan Engineered stamped plans Energy rating Wind rating Soil test Approval process approx 5 working days |  |
| --- | --- | --- |

- What is a prime cost (PC) item? Give an example.

| Answer | A prime cost is an amount allowed for in the contract for items such as tiles, floor coverings, plumbing fixtures, appliances etc Should the cost that was allowed for in the contract be more than what the client spends – the builder would then raise a variation for the cost difference. |  |
| --- | --- | --- |

- What is a provisional sum (PS) item? Give an example.

| Answer | Provisional sums are an allowance in the contract which are for items unknown at the time of contract. For example – earthworks – depending on the soil, ground, rocks, council requirements etc additional costs could occur for these unknown factors |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

- True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify two (2) materials being used for the floors?

| Answer | Carpet, vinyl and floating floors |  |
| --- | --- | --- |

- True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- How much does the contracted plasterer cost?

| Answer | $12 900 |  |
| --- | --- | --- |

- How much do the kitchen appliances cost?

| Answer | $2 700.00 |  |
| --- | --- | --- |

- What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

- What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

- How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 each |  |
| --- | --- | --- |

- If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $275.00 |  |
| --- | --- | --- |

- How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 |  |
| --- | --- | --- |

- What material has the highest total cost?

| Answer | Pine 90 x 45 mpg 10 ( Long Lengths) - $322.50 |  |
| --- | --- | --- |

- How many metres are required of the material in Q80. ?

| Answer | 322.50 m |  |
| --- | --- | --- |

- What beam is identified as being a better option than F17?

| Answer | Alternate option / better to use beams in leiu of F17 7/3.6, ½.1, 1/1.5, 1/1.2, 1/3.9 |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

- Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter – Road Construction |  |
| --- | --- | --- |

- If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $214.80 |  |
| --- | --- | --- |

- Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar Bender and Fixer |  |
| --- | --- | --- |

- If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $1285.80 |  |
| --- | --- | --- |

- Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | Queensland Building and Construction Commission - QBCC |  |
| --- | --- | --- |

CPCCBC4012B

Read and interpret plans and specifications

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Lauren Wilson |
| --- | --- |
| Participant signature |  |
| Date | 18/06/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4012B

Read and interpret plans and specifications

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to read and interpret plans and specifications in order to inform estimation, planning and supervisory activities. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - Any drawing with a cloud or mark-up you have inserted. - Copy of the correspondence regarding mark up and any site management/authorising engineer response. The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Drawings (including site plan), Specifications, Tender documentation, Telephone, Facsimile machine, Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: |  |  |  |
| Name of observer: |  |  |  |
| Role of observer (Confirm suitability to sign) |  |  |  |
| Email address of observer |  |  |  |
| Signature of observer: |  |  |  |
| Date: | Click here to enter a date. |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| 1 Interpret drawings |  |  |  |
| - Identify purpose and advantage of different types of drawings. |  |  |  |
| - Identify different aspects of drawings. |  |  |  |
| - Identify, understand and apply: - Commonly used symbols and abbreviations - Common building and construction terms on drawings |  |  |  |
| - Identify building site from location drawings. |  |  |  |
| - Identify on the site plan: - True north - Building orientation - Key features in relation to building and other structures - Levels - Contours - Service entry points - Site features to be removed or retained |  |  |  |
| - Identify key features of plans, elevations and sections. |  |  |  |
| - Identify client requested variations to standard plans on drawings. |  |  |  |
| 2 Interpret specifications |  |  |  |
| - Identify and correctly apply Provisional Sum (PS) and Prime Cost (PC) values. |  |  |  |
| - Identify and confirm customer variations to standard specifications and communicate changes to organisational personnel. |  |  |  |
| - Apply and communicate correct interpretations of essential elements to estimation, planning and supervisory tasks (including labour hours/costs and material quantities/costs). |  |  |  |
| - Identify Building Codes or standards affecting the work to be undertaken (including references to Australian Standards and the National Construction Code). |  |  |  |
| - Identify key features of products included in the specification, including the design, purpose, aesthetics and cost relationships. |  |  |  |
| 3 Communicate with others |  |  |  |
| - Consult with industry professionals, clients, organisational personnel and appropriate local authorities. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Using technology (including telephone, facsimile, email) |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments