| Safe Work Method Statement (SWMS) |  |  |  |  | A.Rolley and Sons 37 Ascot Way Little Mountain Qld, 4551 Phone: 07 5499 6811 Email: lee@arolleyandsons.com.au |  |
| --- | --- | --- | --- | --- | --- | --- |
| Project: |  | A.Rolley and Sons |  |  |  |  |
| SWMS No: |  |  | 02 | Work Activity: | Carpentry |  |  |  |

## All persons involved in the works must have the SWMS explained and COMMUNICATED to them prior to start of works via a toolbox Talk.

HIGH RISK CONSTRUCTION ACTIVITY!!   Work at Heights!! Operation of Power tools!! Use of hazardous chemicals / silica products!!

| # SWMS Details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Brief Description of Work Activity: |  | External Cladding, Soffits, Windows, Doors homes. |  |  |  |  |  |  |  |
| Location: | 32 Nothling Street, Moffat Beach, QLD, 4551 |  |  |  |  |  |  | Date: | 14/03/2022 |
| Employer/Contractor: | A.Rolley and Sons |  |  |  |  | ABN: | 59 027 944 938 | Date to be Reviewed: | 14/03/2023 |
| Personnel Responsible for Monitoring this Activity: |  |  |  |  | Supervisor Dan Rolley |  |  |  |  |
| Codes of Practice / Standards Consulted: These must be complied with. |  |  |  |  | Work Health Safety Act 2011, Work Health Safety Regulations 2011, AS/NZS1715-1716: Respiratory Devices; AS/NZS1270: Hearing Protection; Electrical Safety Act 2002, Electrical Safety Regulation 2013, Hazardous Manual Tasks COP 2011, How to Manage Work Health & Safety Risks COP 2011, Managing the risk of falls at workplaces COP 2018, AS/NZS 1336 Recommended practices for occupational eye protection AS/NZS 1337 Personnel Eye Protection, AS/NZS 4501 Occupational Protective Clothing, AS/NZS 3760 In Service Safety Inspection and Testing of Electrical Equipment, Work Health and Safety Consultation, Co – Operation and Co - Ordination COP 2011, Construction Work COP 2012, Managing Risks of hazardous Chemicals in a Workplace COP 2013 |  |  |  |  |
| Plant and Equipment Required for this Activity: |  |  |  |  | Power saw, hammer, Planner, nips, level, Ramset Gun, Nailing gun, Drill |  |  |  |  |
| Details of Maintenance Checks Required for this Activity: |  |  |  |  | Inspection of tools daily before use, Manufacturers servicing maintenance, 3 monthly test and tag of electrical equipment |  |  |  |  |
| Materials Used: |  |  |  |  | Fibro sheets, doors, Screws, Nails, Windows |  |  |  |  |
| Personal Protective Equipment (P.P.E.) required: |  |  |  |  | Enclosed Boots, Safety Glasses, shirt, A.Rolley and Sonss P.P.E. Requirements |  |  |  |  |
| SDS Required? (Yes / No) |  |  |  |  | Yes - As per Hazardous Substance Register |  |  |  |  |
| Personnel Qualifications Required for this Activity: Relevant WorkCover Ticket for task has been undertaken |  |  |  |  | Trade Certificate 3 in Carpentry, |  |  |  |  |
| Review and Corrective Actions: SWMS will be reviewed to ensure the work is being carried out to the statement, corrective action will be noted which are passed to the Supervisor to carry out remedial work for close out. If a failure is identified in the SWMS, works will stop, the SWMS will be amended and the changes communicated to the workforce. |  |  |  |  |  |  |  |  |  |  |  |
| Specific Training Required for this Activity: All personnel to have completed a Site Induction. Must be trained in this SWMS and have all relevant certification for this task. |  |  |  | Construction Industry Induction Blue / White Card, Knowledge of Safe Manual Handling Techniques |  |  |  |  |  |  |  |
| Personnel consulted on development of SWMS: Name:; Signature:; Name:; Signature:. Dan Rolley; Dan Rolley. A.Rolley and Sons Team. |
| # Approved By Senior Management |  |  |  |  |  |  |  |  |  |  |  |

The hierarchy of controls needs to be considered when identifying control measures. The highest level of control should be implemented wherever reasonably practicable.

| Name: | Dan Rolley | Signature: | Dan Rolley | Date: | 14/03/2022 |
| --- | --- | --- | --- | --- | --- |

| Hierarchy of Controls | Examples | Start with Highest Level of Control |  |
| --- | --- | --- | --- |
| Eliminate | Remove the hazard completely |  |  |
| Substitute | Substitute with a less hazardous material or plant |  |  |
| Engineer | Redesign the work process to be done differently |  |  |
| Isolate | Isolate the person from the hazard or hazard from person |  |  |
| Administration | Admin controls for e.g. Job rotation, supervision, Instruction |  |  |
| PPE | Using PPE (Least favored method of control) | Lowest Level of Control |  |

LIKELIHOOD	‘Likelihood that the stated consequence will occur’                                           EXPOSURE    ‘Consider the exposure (frequency) factor when determining the likelihood of the risk/hazard event occurring.’

| Almost certain | Almost inevitable outcome, expected to occur in most circumstances. |  | Hazard event occurs | Exposure factor |
| --- | --- | --- | --- | --- |
| Likely | Not a certainty but there is a good chance of occurrence. |  | Continuously | Many times daily |
| Possible | Could occur. |  | Frequently | Approximately once daily |
| Unlikely | Could occur but not expected. Would require multiple failures of systems/controls. |  | Occasionally | Once a week to once a month |
| Rare | Little chance of occurrence. Would require a combination of factors to result. |  | Infrequent | Once a month to once a year |

CONSEQUENCE	‘The outcome of an event expressed qualitatively, being a loss, injury, disadvantage or gain.’

|  | Health & Safety | Environment |
| --- | --- | --- |
| Catastrophic | Fatality or permanent disability (Class 1 incident) | High severity which has or may have permanent and/or irreversible effects |
| Major | Life threatening incident, Lost Time Injury or ongoing illness/health effects (Class 2 incident) | Medium severity which has or may have persistent but reversible effects |
| Moderate | Incident that requires medical treatment by a qualified medical practitioner (Class 2 incident) | Low severity which has short term and reversible effects |
| Minor | Incident that may require first aid treatment only (Class 3 incident) | Impact confined to area impacted by work operations |
| Insignificant | No injuries | Very low environmental impact, not noticeable |

| QUALITATIVE RISK ANALYSIS MATRIX: LEVEL OF RISK |  |  |  |  |  |  | RISK MATRIX |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CONSEQUENCE |  |  |  |  |  | Extreme | Extreme risk, immediate action required, works must not proceed at this level. |
| LIKELIHOOD | Insignificant 5 | Minor 4 | Moderate 3 | Major 2 | Catastrophic 1 |  | High | High risk, acceptable to proceed only with strict controls or a short duration |
| A - Almost Certain | H | H | E | E | E |  | Moderate | Moderate risk, acceptable to proceed with suitable controls |
| B - Likely | M | H | H | E | E |  | Low | Low risk, acceptable to proceed |
| C - Possible | L | M | H | E | E |  |  |  |
| D - Unlikely | L | L | M | H | E |  |  |  |
| E - Rare | L | L | M | H | H |  |  |  |

| ## L Acceptable M Acceptable with CONTROLS H ACCEPTABLE WITH STRICT CONTROL/SHORT DURATION E Unacceptable |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Procedure Break the job down into steps | Potential Safety and Environmental Hazards What can go wrong | Risk Rating |  |  | Control Measures | Residual Risk Rating |  |  | Person Responsible To ensure management method applied |  |
|  |  | L X | C = | R |  | L X | C = | R |  |  |
| Arrival on site / Induction and pre-site discussion and planning with Principal Contractor | Unauthorised site entry | B | 3 | H | Report to Principal Contractor site office/Supervisor upon entry to site. Assess site as per crossover and park in designated parking area. | D | 3 | M | Foreman Supervisor-Workers |  |
|  | Collisions with other equipment | A | 3 | E | Access site as per Traffic Management Plan. Ensure All warning devices and lights are functioning correctly and on. | D | 3 | M | Foreman Supervisor-Workers |  |
|  | Not site inducted Untrained and inexperienced persons | B | 4 | H | Personnel to complete site specific inductions before work commences. are also required to have completed company induction. Workers to have read/understood and signed into the company SWMS. | D | 4 | L | Foreman Supervisor-Workers |  |
|  | Non-performance to site rules and procedures | B | 4 | H | All persons to be conforming to P.P.E. requirements for Principal Contractor requirements for site. All accidents, incidents or hazards to be reported to Principle Contractor Site Safety Coordinator immediately. In the event of a reportable incident ensure that scene is made safe and scene not to be interfered with to enable investigation to be undertaken. Area to be appropriately excluded/signed to prevent unauthorized access. For all other accidents fill out Principle Contractor accident report form and conduct investigation all outcomes also to be reported to Principle Contractor. | D | 4 | L | Foreman Supervisor-Workers |  |
| Establish work area | Unauthorised entry | B | 3 | H | - Barriers, Signs and Spotters establishment of an exclusion zone if required - Only authorized persons allowed | D | 3 | M | Supervisor-Workers. |  |
|  | Slips Trips and Falls | B | 4 | H | - Ensure all housekeeping is - done and any objects that are not required are removed | D | 4 | L | Supervisor-Workers. |  |
| Install Cladding / Soffits Cutting cladding fibro cement sheeting Silica Product | Silica Dust inhalation | B | 3 | H | When cutting fibro cement sheeting cut in well ventilated area downwind of other workers. Workers must wear a minimum of Type M or L Dust Particulate Filter Respirator All respirators should be fit for purpose. Use M Class Vacuum and attachment to saw to collect silica dust. Use M Class Vacuum to collect excess dust when finished task. Dispose of dust in designated bin | D | 3 | M | Supervisor-Workers. |  |
|  | Electricity | B | 2 | E | All Electrical equipment is to have current 3 month test and tag All Electrical equipment is to be in good condition with no damage tapped up All Power supplies to be protected by RCD RCDs to have push button test prior to each shift and be tested for amperage and trip time every 3 months All electrical leads are to be placed as not to sustain damage and to be kept out of e.g. lead stands if required Any damaged or faulty equipment will be tagged out of service and removed from service Electrical equipment to be entered into electrical register | D | 2 | H | Supervisor-Workers. |  |
|  | Eye damage | A | 3 | E | Use of safety glasses on site which are worn task specific. | E | 3 | M | Supervisor-Workers. |  |
| Lift Board &Screw board into place | Musculo-Skeletal Injury | B | 3 | H | Lifting Aids such as trolleys to be used where possible. Where manual handling is required the following to be utilized; - Minimise the weight of loads where possible so excessive loads are not carried. - Use of good lifting techniques as follows; - A firm grip of the load - Keep load close to the body - Leg muscles to do the work when lifting keep back straight bend at the knees - Smooth lift avoiding twisting or jerking Weights that feel excessive or above a worker’s capacity not to be lifted | D | 3 | M | Supervisor-Workers. |  |
|  | Eye damage | A | 3 | E | Use of safety glasses on site which are worn task specific. | E | 3 | M | Supervisor-Workers. |  |
| Application of liquid nails with caulking gun | Worker not aware of control measures | A | 3 | E | Ensure all persons required have read the M.S.D.S. before use | D | 3 | E | Supervisor-Workers. |  |
|  | Swallowed | A | 3 | E | Rinse mouth with water. If swallowed, give a glass of water to drink. If vomiting occurs give further water. Seek medical advice. Worker to wash hands before eating drinking or smoking | D | 3 | E | Supervisor-Workers. |  |
|  | Eye | A | 3 | E | If in eyes, wash out immediately with water. In all cases of eye contamination, it is a sensible precaution to seek medical advice. Worker to wear Safety glasses while applying adhesive substance | D | 3 | E | Supervisor-Workers. |  |
|  | Skin | A | 3 | E | Wipe excess material from skin with a clean rag or paper towel (do NOT use solvent to clean skin). If skin contact occurs, remove contaminated clothing and wash skin with running water. If irritation occurs seek medical advice. Worker to wear Principle Contractors P.P.E. requirements for site. The use of caulking gun eliminates contact with skin | D | 3 | E | Supervisor-Workers. |  |
|  | Inhaled | A | 3 | E | Remove victim from area of exposure - avoid becoming a casualty. Seek medical advice if effects persist. If conditions unsuitably ventilated worker to wear P2 face mask | D | 3 | E | Supervisor-Workers. |  |
| Use of Circular Saw general cutting non silica products | Electricity | B | 2 | E | All electrical equipment is to have a current 3 month test and tag All electrical equipment ids to be in good condition with no damage taped up All generators are to have a permanently fixed RCD to have a push button test prior to each shift and be tested for amperage and trip Time every 3 months+ All Electrical leads are to be placed as to not sustain damage and to be kept out of water e.g. lead stands Any damaged or faulty equipment will be tagged out of service Electrical equipment to be entered into electrical register Ensure all guards are in position & in working order | D | 2 | H | Supervisor-Workers. |  |
|  | Projectiles from work activity | A | 3 | E | Worker must wear eye protection Eye protection must fit properly and be able to be used whilst using other items of PPE Eye protection must be fit for use | D | 3 | M | Supervisor-Workers. |  |
|  | Dust causing respiratory & environmental problems | A | 3 | E | A face mask to be worn if conditions are unsuitable Use in well ventilated area Monitor dust short breaks and or mechanical extraction if required. | D | 3 | M | Supervisor-Workers. |  |
|  | Loss of limbs | B | 2 | E | Keep fingers away from blade during use at all times Hold from handle only Ensure all guards are working & correctly positioned before use. Ensure workers are competent and know how to use drop saw & circular saw correctly and set up is on stable flat ground. For Stripping Timber Assess if it is necessary (Can I use a two different size timbers) Rest Timber on work bench or saw horse. Clamp timber or screw down and cut with circular saw away from Keep fingers away from blade during use at all times Hold from handle only Ensure correctly positioned before use | D | 2 | H | Supervisor-Workers. |  |
|  | Noise | A | 3 | E | Worker to wear P.P.E. for hearing muffs, ear plugs. | D | 3 | M | Supervisor-Workers. |  |
| Use of hand saw & nail off | Using paslode gas gun/use nail gun compressor | B | 3 | H | All personnel trained in the use of gas gun and nail gun, sign to be in place, calling when firing visual inspection of area, notify other trades of your intention to use these tools, wear PPE, safety glasses. | D | 3 | M | Supervisor-Workers. |  |
|  | Noise | B | 3 | H | Worker to wear P.P.E. for hearing muffs, ear plugs | D | 3 | M | Supervisor-Workers. |  |
|  | Projectiles from work activity | A | 3 | E | Worker must wear eye protection Eye protection must fit properly and be able to be used whilst using other items of PPE Eye protection must be fit for use | D | 3 | M | Supervisor-Workers. |  |
|  | Loss of limbs | B | 2 | E | Rest Timber on work bench or saw horse. Clamp timber or screw down and cut with hand saw, Keep fingers away from blade during use at all times Hold from handle only Ensure correctly positioned before use | D | 2 | H | Supervisor-Workers. |  |
|  | Noise | A | 3 | E | Worker to wear P.P.E. for hearing muffs, ear plugs compliant with AS/NZS 1270 | D | 3 | M | Supervisor-Workers. |  |
|  | Minor cuts and abrasions | B | 4 | H | Use of hand tools where possible instead of hands. | D | 4 | L | Supervisor-Workers. |  |
| Installation of Doors Manually handle with one-person door frame into to position | Musculo-Skeletal Injury | B | 3 | H | Lifting Aids such as trolleys to be used where possible. Where manual handling is required the following to be utilized; - Minimise the weight of loads where possible so excessive loads are not carried. - Use of good lifting techniques as follows; - A firm grip of the load - Keep load close to the body - Leg muscles to do the work when lifting keep back straight bend at the knees - Smooth lift avoiding twisting or jerking Weights that feel excessive or above a worker’s capacity not to be lifted | D | 3 | M | Supervisor-Workers. |  |
| Wedge door frame into correct position and screw off using a battery operated drill | Projectiles from work activity | A | 3 | E | Worker must wear eye protection Eye protection must fit properly and be able to be used whilst using other items of PPE Eye protection must be fit for use Sunglasses may not be eye protection | D | 3 | M | Supervisor-Workers. |  |
|  | Eye damage | A | 3 | E | Use of safety glasses on site which are worn task specific. | E | 3 | M | Supervisor-Workers. |  |
| Measure and Mark Out Positioning of hinges on door frame and mark out hinges. | Unexperienced workers | B | 4 | H | - All Staff to be a qualified Carpenters unless they are an apprentice in which case they will supervised at all times by a qualified tradesman | D | 4 | L | Supervisor-Workers. |  |
| Plane door to size using an electric Planer to match doorway | Electricity | B | 2 | E | All electrical equipment is to have a current 3 month test and tag All electrical equipment ids to be in good condition with no damage taped up All generators are to have a permanently fixed RCD to have a push button test prior to each shift and be tested for amperage and trip Time every 3 months All Electrical leads are to be placed as to not sustain damage and to be kept out of water e.g. lead stands Any damaged or faulty equipment will be tagged out of service Electrical equipment to be entered into electrical register Use Battery operated drills where possible | D | 2 | H | Supervisor-Workers. |  |
|  | Worker not using the equipment properly | B | 4 | H | Ensure worker is competent to use equipment provided Ensure equipment is only use as it is designed to be used Ensure no guards are removed Ensure not safety mechanisms are disabled during use Ensure material can be moved easily in cutting area | D | 4 | L | Supervisor-Workers. |  |
|  | Projectiles from work activity | B | 3 | H | Worker must wear eye protection Eye protection must fit properly and be able to be used whilst using other items of PPE Eye protection must be fit for use Sunglasses may not be eye protection | D | 3 | M | Supervisor-Workers. |  |
|  | Eye damage | A | 3 | E | Use of safety glasses on site which are worn task specific. | E | 3 | M | Supervisor-Workers. |  |
|  | Loss of limbs | B | 2 | E | Keep fingers away from blade during use Hold from handles only | E | 2 | H | Supervisor-Workers. |  |
| Route rebate for hinges in the door Using a router | Projectiles from work activity | B | 3 | H | Worker must wear eye protection Eye protection must fit properly and be able to be used whilst using other items of PPE Eye protection must be fit for use Sunglasses may not be eye protection | D | 3 | M | Supervisor-Workers. |  |
|  | Eye damage | A | 3 | E | Use of safety glasses on site which are worn task specific. | E | 3 | M | Supervisor-Workers. |  |
|  | Loss of limbs | B | 2 | E | Keep fingers away from blade during use Hold from handles only | E | 2 | H | Supervisor-Workers. |  |
| Screw hinges onto door | Projectiles from work activity | B | 3 | H | Worker must wear eye protection Eye protection must fit properly and be able to be used whilst using other items of PPE Eye protection must be fit for use Sunglasses may not be eye protection | D | 3 | M | Supervisor-Workers. |  |
|  | Dust causing respiratory & environmental problems | B | 3 | H | A P2 face mask to be worn if dust is prevalent Monitor dust short breaks and or mechanical extraction if required Monitor dust in relation to surrounding public | E | 3 | M | Supervisor-Workers. |  |
|  | Eye damage | A | 3 | E | Use of safety glasses on site which are worn task specific. | E | 3 | M | Supervisor-Workers. |  |
| Lift door into place with one person chock underneath | Musculo-Skeletal Injury | B | 3 | H | Lifting Aids such as trolleys to be used where possible. Where manual handling is required the following to be utilized; - Minimise the weight of loads where possible so excessive loads are not carried. - Use of good lifting techniques as follows; - A firm grip of the load - Keep load close to the body - Leg muscles to do the work when lifting keep back straight bend at the knees - Smooth lift avoiding twisting or jerking Weights that feel excessive or above a worker’s capacity not to be lifted | D | 3 | M | Supervisor-Workers. |  |
|  | Projectiles from work activity | B | 3 | H | Worker must wear eye protection Eye protection must fit properly and be able to be used whilst using other items of PPE Eye protection must be fit for use Sunglasses may not be eye protection | D | 3 | M | Supervisor-Workers. |  |
| Mark out door knob and lock hole location Cut out | Eye damage | A | 3 | E | Use of safety glasses on site which are worn task specific. | E | 3 | M | Supervisor-Workers. |  |
|  | Loss of limbs | B | 2 | E | Keep fingers away from blade during use Hold from handles only | E | 2 | H | Supervisor-Workers. |  |
| Place door lock and door knob and screw off | Eye damage | A | 3 | E | Use of safety glasses on site which are worn task specific. | E | 3 | M | Supervisor-Workers. |  |
| Installation of Windows Frames Use of team lifts | Falls from heights | B | 2 | E | All works above 2 metres must be fall protected hand rails on a work platform. | D | 2 | H | Supervisor-Workers. |  |
|  | Falling objects | A | 3 | E | No personnel to work directly under erection area, only personnel involved in erection to be in works zone. | D | 3 | M | Supervisor-Workers. |  |
|  | Slips, trips and falls | B | 4 | H | Clear area of trip hazards; do not work in mud or in muddy conditions. Remove and dry any water that may be lying about. Clear area of debris. | D | 4 | L | Supervisor-Workers. |  |
|  | Windows falling out of opening | B | 2 | E | Do not attempt works individually, always work in pairs one person external, one internal one person to have contact with window until the window is secured and in position. | D | 2 | H | Supervisor-Workers. |  |
| Fixing windows to structure | Using paslode gas gun/use nail gun compressor | A | 3 | E | All personnel trained in the use of gas gun and nail gun, sign to be in place, calling when firing visual inspection of area, notify other trades of your intention to use these tools, wear PPE, safety glasses. | D | 3 | M | Supervisor-Workers. |  |
|  | Projectiles from work activity | A | 3 | E | Worker must wear eye protection Eye protection must fit properly and be able to be used whilst using other items of PPE. Eye protection must be fit for use. Sunglasses may not be eye protection. | D | 3 | M | Supervisor-Workers. |  |
|  | Noise | A | 3 | E | Worker to wear P.P.E. for hearing muffs, ear plugs compliant with AS/NZS 1270. | D | 3 | M | Supervisor-Workers. |  |
| Work at Heights Method 1. Fixed Scaffold | Falls from heights Collapsing scaffold Death and/or serious injury | B | 2 | E | Ensure scaffold safety tag is in place and in date prior to accessing. Fixed Scaffolding to be erected by those with a High-Risk Work Scaffold License The scaffold should be built as per the manufacturer’s specification. Competent persons to consult with site management to ensure that the scaffold is safe to use before starting work. When practical use mechanical means of lifting heavy loads to the workplace | D | 2 | H | Supervisor-Workers. |  |
|  | Incorrect access point | B | 2 | E | Look for tag / certificate of service Confirm with Principle Contractor that it is safe, completed and ready for use prior to starting task, Worker to inspect before use if unsure or doesn’t look safe stop works immediately! Look for possible out of service tags if incomplete, (awareness of other trades possibly having illegally altering or left it incomplete format) or if it looks defective notify Principle Contractor Supervisor. When entering / exiting Guard rail or scaffold do so via designated entry point only with ladder secured and using 3 points of contact whilst on extension ladder, - Ladder must extend 1 metre past assess point and be secured either top or bottom or both. DO NOT Climb through Hand Rails!!! | D | 2 | H | Supervisor-Workers. |  |
|  | Incorrect Scaffold | A | 3 | E | Worker to look and confirm for handover certificate or tag of service to be displayed by installer prior to use. Worker must inspect scaffold before use if not safe do not use notify supervisor and get fixed before use. | D | 3 | M | Supervisor-Workers. |  |
| Work at Heights Method 2. Using Ladders General | Incorrect use. Falls, defective equipment. | A | 3 | E | - No work to be carried out above a 2m fall height without fall protection. - Do not work near any opening, stairwell or barrier. Inspect Ladder before use. | D | 3 | M | Supervisor-Workers. |  |
|  | Ladder has faults that could lead to falls. | A | 3 | E | - An approved, industrial-rated (120kg) ladder is to be used to gain reach. - Above 2 metres a suitable work platform to be used with edge protection. - Ensure the platform ladder complies with AS1892 marked industrial and have - a SWL of 120kg. | D | 3 | M | Supervisor-Workers. |  |
|  | Falls when ascending and descending. | A | 3 | E | - Maintain three points of contact when ascending & descending | D | 3 | M | Supervisor-Workers. |  |
| Work at Heights Method 3. Working off Mobile Scaffold | Falling objects | A | 3 | E | No persons are to be underneath scaffold while workers on it. No unauthorized persons allowed in area. Kickboards are to be correctly fastened at all times or exclusion zone implemented in place when workers on it. | D | 3 | M | Supervisor-Workers. |  |
|  | Falls | A | 3 | E | Stay within confines of scaffold working platform Experienced Subcontractors only Use scaffold as per manufactures design and specification only Assess only as per designated assess points Do not ride on scaffolding, get off before re-positioning scaffold. Lock off castors before starting work. Work off full decks at all times. Keep hatch closed when working on platforms. Use Ladders to access the work platform, do not climb on the scaffold frame. | D | 4 | L | Supervisor-Workers. |  |
|  | Falling objects | A | 3 | E | No persons are to be underneath scaffold while worker is on it No unauthorised persons allowed in area | D | 4 | L | Supervisor-Workers. |  |
|  | Collapse | B | 2 | E | Ensure it is built by competent persons only up to 4 metres platform working height For Scaffold over 4 metre platform working height only licenced Scaffolders to construct and handover certificate to be issued. Ensure it is built on firm sold ground Ensure it is built to manufactures specifications and instructions | D | 2 | H | Supervisor-Workers. |  |
| Work at Heights Method 4. Use of Extension Ladder | Unknown hazards | A | 3 | E | Assess installation site. Check: Proximity to overhead electric lines Type of roof (pitched, tile, corrugated iron, presence of sky lights etc) Load bearing capacity of roof Height of work area Type of height access equipment required Number of persons expected on roof/ceiling space Power supply location Location of existing services (pipes, cables, conduits) Use ladder stop to secure ladder into position. | D | 3 | M | Supervisor-Workers. |  |
|  | Personal Injury: | A | 3 | E | Secure ladder. Persons to be instructed on use of ladder Identifiable as being of commercial quality. 120 kg or more and complies with AS/NZS 1892 Inspected before use and periodically thereafter. Removed from use immediately if damaged. Have non slip feet and be secured at the top. Elevated at an appropriate angle. (Horizontal distance from base to vertical plane of support should be one-quarter the ladder height.) Extend to at least one metre above the stepping off point. Positioned away from power lines. All persons to be aware when ladder is in use so over load is not reached and industrial commercially rated. Ensure the ladder is in an easily accessible area | D | 3 | M | Supervisor-Workers. |  |
| Work at Heights Method 5. Working off Trestles and Planks | Falls from height | B | 3 | H | Work within confines of platform Planks and Trestles will not be used with a working deck above 2m a plank clamp with be applied, minimum two planks wide. Ensure trestles are set up on firm and stable ground Ensure trestle comply with AS/NZS 1892 Ensure Trestles are not damaged and perfect working condition | D | 3 | M | Supervisor-Workers. |  |
|  | Collapse | A | 3 | E | Ensure trestles are rated 150kg suitable for planks to be used. If trestle does not have correct manufacturers markings on it do not use it. Ensure that trestles are set up correctly and stable on firm solid ground as per manufactures specifications. If necessary, use shovel to level! Ensure that plank has at 300mm lap onto trestle | D | 3 | M | Supervisor-Workers |  |
| Working on roof Note Edge protection to be in place prior to accessing roof top. | Incorrect access point | B | 2 | E | Look for tag / certificate of service Confirm with Principle Contractor that it is safe, completed and ready for use prior to starting task, Worker to inspect before use if unsure or doesn’t look safe stop works immediately! Look for possible out of service tags if incomplete, (awareness of other trades possibly having illegally altering or left it incomplete format) or if it looks defective notify Principle Contractor Supervisor. When entering / exiting Guard rail or scaffold do so via designated entry point only with ladder secured and using 3 points of contact whilst on extension ladder, - Ladder must extend 1 metre past assess point and be secured either top or bottom or both. DO NOT Climb through Handrails!!! | D | 2 | H | Supervisor-Workers. |  |
| Before Starting Tasks consider Asbestos Sheeting possibility | Possibility of cutting through asbestos sheeting | B | 2 | E | Before starting tasks worker and Supervisor are to identify any possible asbestos sheets, ask Site Supervisor for asbestos register and check for location, if not on register and unsure stop task and get area tested. If in doubt Do Not Proceed with task! | D | 2 | H | Supervisor - Workers |  |
|  | Possibility of cutting through asbestos sheeting cont! | B | 2 | E | Company policy is for all asbestos to be only removed by qualified professionals as per code of practice and Australian Standards. In the event a wall, soffit or pit is suspected of asbestos work to stop immediately, then Supervisor to be informed and an external contractor qualified to removed asbestos will be engaged. | D | 2 | H | Supervisor - Workers |  |
| Use of hazardous substances general | Explosion / Contamination | B | 2 | H | No hazardous to be stored on site removed off site daily should storage be required prior consultation with Principal Contractor required. Ensure that S.D.S. are kept on site and available. Ensure where possible where possible, hazardous substances are replaced with non-hazardous substances. Ensure that hazardous substances are only used in ventilated areas. Appropriate P.P.E. to worn in accordance with the SWMS. .If an incident occurs ensure a contamination report is completed. Ensure that any identified areas of contamination from the report are addressed prior to the commencement of works. | D | 2 | H | Supervisor-Workers. |  |
| Environment Issues Closing Down of site at end of day Housekeeping | Rubbish or discarded materials | B | 4 | H | Ensure all work areas are maintained in a tidy and safe manner. Dispose of all rubbish in the industrial bins. All surplus timber etc. To be stacked clear of work areas or placed in the industrial bin. | D | 4 | L | Supervisor-Workers. |  |
| SWMS Management & Compliance | Failure in SWMS Injury to person Damage to equipment / property |  |  |  | If the SWMS has identified failures the following steps will take place; - Stop works and identify the failure. - Investigate improvements - Discuss / consult - Update SWMS - Review, amend SWMS, communicate to staff via toolbox |  |  |  | Supervisor-Workers. |  |

Confidentiality and Privilege Notice:

This Safe Work Method Statement is intended only to be used only by the addressee. It is confidential and may contain legal privileged information. If you are not the client of Suncoast Safety Pty Ltd any use, distribution disclosure or copying of this Safe Work Method Statement or any attachment is strictly prohibited. Confidentiality and legal privilege attached to this Safe Work Method Statement are not waived or lost by reason of the mistaken delivery to you. If you have received this Safe Work Method Statement in error, please delete it and notify us immediately by telephone or e-mail.

This document is copy righted 2020 by Suncoast Safety (07) 54932961 all rights reserved.

| # Reviews |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Review No: | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| Name & Initials | Dan Rolley D.R. |  |  |  |  |  |  |  |  |
| Date: | 22.07.2020 |  |  |  |  |  |  |  |  |

Review using the tool SWMS Review Checklist and guidance from Knowledge Resource using the Prepare, Review and Accept SWMS

If poisoning occurs, contact Poisons Information Centre on 13 11 26.

| ENVIRONMENTAL ISSUES |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Task | Activity that could harm the Environment and create an Environmental Risk | Risk Rating |  |  | Environmental Control Measures | Residual Risk Rating |  |  | Person Responsible |
|  |  | P | C | T |  | P | C | T |  |
| Housekeeping | Rubbish or discarded materials | B | 4 | H | Ensure all work areas are maintained in a tidy and safe manner. - Dispose of all rubbish in the industrial bins. - All surplus timber etc to be stacked clear of work areas or placed in the industrial bin | E | 4 | L | Supervisor-Foreman. Workers |
| Contaminate the environment or create a trip hazard | Worker slips or trips and sustains personal damage | B | 3 | H | Environmental Representative or Foreman is to be informed immediately. | E | 3 | M | Supervisor-Foreman. Workers |

Disclaimer:

This Suncoast Safety document has been carefully prepared to reflect acceptable WHS practices and applicable laws. However, it is intended to be generic and may not be suitable for your particular work. You should make your own assessment of its applicability and suitability for your particular work. It is your responsibility to ensure that you use safe work practices at all time. Suncoast Safety accepts no responsibility for any injury, loss, or damage suffered arising from or in purported reliance on this document. As WHS laws and requirements may vary from State to State and Territory, this document has no application to work outside Queensland.

All Rights reserved Suncoast Safety 2020 (07) 54932961

TOOLBOX

Date: ........................................................................................................................................

Project/Location:....................................................................................................................

Supervisor/Presenter:............................................................................................................

Topics Discussed:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

| Corrective Action | Actions By | Action Complete |  |
| --- | --- | --- | --- |
|  |  | Sign Off | Date |

Attendees

| Name | Signature |  | Name |  |  | Signature |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HAZARDOUS SUBSTANCE REGISTER |  |  |  |  |  |  |  |  |
| This Hazardous Substance Register will record all hazardous substances and chemicals that are located on site. It will be placed in the front of a folder that contains the safety data sheets (SDS) for each substance. The folder will be located in the site office and maintained by the site supervisor or their representative. |  |  |  |  |  |  |  |  |
| Product name/Chemical name |  | Supplier |  | SDS Attached | SDS Issue Date |  | Hazardous Substance Yes / No |  |
| No More Gaps |  | Selleys Australia |  | Yes | March 2017 |  | No |  |
| Polyfilla Expanding Foam |  | Selleys Australia |  | Yes | March 2016 |  | Yes |  |
| Selleys Aquadhere |  | Selleys Australia |  | Yes | April 2016 |  | No |  |
| Sikaflex Pro |  | Sika Australia |  | Yes | April 2018 |  | Yes |  |
| Liquid Nails |  | Selleys Australia |  | Yes | August 2019 |  | Yes |  |
| Mineral Turps |  | Recohem Inc |  | Yes | May 2016 |  | Yes |  |
| Bostik Kitchen & Bathroom |  | Bostik Australia |  | Yes | October 2016 |  | No |  |
| Ultra Set |  | Bostik Australia |  | Yes | May 2016 |  | Yes |  |
| SDS must be less than five years from date of issue. |  |  |  |  |  |  |  |  |

| # signoff |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| We the undersigned, confirm that the SWMS nominated above has been explained and its contents are clearly understood, accepted. We the undersigned have had the opportunity to consult and discuss the SWMS to be able to make any changes necessary to the document as required. We the undersigned clearly understand the controls outlined in this SWMS must be applied as documented if a failure is identified in the SWMS, works will stop immediately, the SWMS will be amended and the changes communicated to the workforce via a Toolbox Talk. All workers involved in the works must sign off below thus signifying the acceptance of this SWMS prior starting work. |  |  |  |  |  |  |
| Name |  | Qualification Required for this Activity | Signature | Date | Time | Employer |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |
|  |  |  |  |  |  | A.Rolley and Sons |