CPCCBC4012B

Read and interpret plans and specifications

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4012B Read and interpret plans and specifications cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Lauren McKew |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Lauren McKew |
| --- | --- |
| Participant signature |  |
| Date | 21/01/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 87 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

- Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

- Which compass direction is always shown on plans and drawings?

| Answer | North. |  |
| --- | --- | --- |

- What information is provided on a location plan?

| Answer | A location plan sets out the exact location of the site for construction. Information on the location plan includes the Lot Number/s, deposit plan numbers, North Point and Scale to identify exactly where the site is within a location/development area. |  |
| --- | --- | --- |

- What detail does a site or block plan contain?

| Answer | A site or block plan contains the information on the block where the proposed building/structure is to be located. It will include a datum which identifies a reference height to other levels on the site. It will also depict where on the site the building is situated and its orientation to the North point. |  |
| --- | --- | --- |

- What do floor plans identify?

| Answer | Floor plans identify the details of the proposed building or structure. Floor plans show details such as the rooms, their relative sizes, where fixtures are located, floor covering information and general set out of the internal rooms/areas of the proposed building. |  |
| --- | --- | --- |

- What is a landscape plan and what information might it provide?

| Answer | Landscaping plans identify the specific details of exterior items such as details of plants to be installed/planted, paths, garden areas and layouts, retaining walls and other landscaping features to be included as part of the project. |  |
| --- | --- | --- |

- What are elevation views? What do they provide to the plan reader?

| Answer | Elevation views of the proposed building/structure provide further detail on the exterior elements of the project with views from the side of the building shown (ie. east, west, north and south. Items such as building heights, levels, finishes, doors, windows and roof types are often depicted on elevation views. |  |
| --- | --- | --- |

- What is a section?

| Answer | As section view is a drawings detail which appears as if the building or structure has been cut in half at some point of the build. It shows the internal features and requirements in further detail than what an elevation plan shows. Items such as wall thickness, wall material, structural requirements, wall finishes, wall heights and levels, proposed roof construction and the relationship between different spaces in the building. |  |
| --- | --- | --- |

- What are detailed drawings?

| Answer | Detailed drawings depict particular information on a required service or finish relevant to the build. For example, plumbing details, electrical details, wiring details, heating/cooling details, structural details etc. are all specific areas of the build which require exact and detailed information to ensure the build is constructed in accordance with relevant codes and standards. It also assists the relevant trades in preparing their quote for the works and ensuring their scope of works is detailed as required. |  |
| --- | --- | --- |

- What are specifications?

| Answer | Specifications provide information on all key details relevant to the project. Specifications generally include a full description of the scope of the works, details specific products (including types, brands, models, thickness, colours etc required), materials and finishes to be used, details of all fixtures and fittings to be installed and any other important information relevant to the desired outcome of the project. Specifications are read in conjunction with the plans as they provide specific details on what is to be used throughout the project and what the expected result of the project is. |  |
| --- | --- | --- |

- Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bull Nose |  |
|  | BTH | Bathtub |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Block |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Glass |  |
|  | CJ | Ceiling Joint |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold Water Tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Proof Course |  |
|  | DSB | Distribution Switch Board |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion Joint |  |
|  | FPBD | Fibrous Plaster Board |  |
|  | FFL | Finished Floor Level |  |
|  | FHR | Fire Hose Reell |  |
|  | GPO | General Purpose Outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

- Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Wall – Cavity Double Brick |  |  |
|  | Wall – Brick Veneer |  |  |
|  | Wall – Timber Stud |  |  |
|  | Window in Cavity Wall |  |  |
|  | Window in Single Wall |  |  |
|  | Single opening door 90deg. |  |  |
|  | Double door, two leaves. 90deg opening each |  |  |
|  | Cavity sliding door |  |  |
|  | Exposed sliding door/ external door |  |  |
|  | Power socket outlet - general |  |  |

- Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Hinged at side |  |  |
|  | Hinged at top |  |  |
|  | Sliding horizontally |  |  |
|  | Fixed |  |  |

- What does a clouded area on a plan indicate?

| Answer | That a change or amendment has been made to the drawing and the pricing/specifications etc. should be checked to see if the change results in the requirement for a variation to be charged or a tradesperson/subcontractor to be notified. |  |
| --- | --- | --- |

- An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

- On what date was an amendment made to the window details?

| Answer | 5 October 2011 |  |
| --- | --- | --- |

- True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What amendment occurred on 08/11/2011?

| Answer | The robe in bedroom 3 was removed. |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

- What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

- What is the Lot number of this site?

| Answer | Lot 16 |  |
| --- | --- | --- |

- What is the size of this lot?

| Answer | 3,653m2 |  |
| --- | --- | --- |

- Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

- True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What size is the easement on the eastern boundary of the site?

| Answer | 5 meters |  |
| --- | --- | --- |

- True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

- What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

- Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

- Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26.?

| Answer | 2 storey brick home to be constructed as per sectional plans and in accordance with specifications. Scissor trusses to be utilised for roof construction and finished with corrugated colourbond. The building is the same structure proposed in questions 19 – 26 however the sectional detail provides further information about the construction of the residence. |  |
| --- | --- | --- |

- Who is the client?

| Answer | H & M Smith |  |
| --- | --- | --- |

- Where is this project to be built?

| Answer | Lot 16 Glenvalley Road, Richmond, Victoria, 3216 |  |
| --- | --- | --- |

- What are the contact details for the draftsman?

| Answer | Joe Foreman Ph: 0395142311 Mob: 0413 113 113 Address: 18 Fairview Road, Richmond, Victoria, 3216 |  |
| --- | --- | --- |

- What sheet number is this drawing?

| Answer | 5 of 7 |  |
| --- | --- | --- |

- What timber is required for loading and non-load bearing walls?

| Answer | F5 Grade Timber |  |
| --- | --- | --- |

- What spacing is recommended for common studs?

| Answer | Maximum of 600mm |  |
| --- | --- | --- |

- What type of roof truss is to be installed?

| Answer | Scissor Truss |  |
| --- | --- | --- |

- What centres are the trusses positioned at?

| Answer | 900mm Spacing |  |
| --- | --- | --- |

- What size are the veranda posts?

| Answer | 90mm x 90mm |  |
| --- | --- | --- |

- What timber species are the veranda posts to come from?

| Answer | Dar Oregon |  |
| --- | --- | --- |

- What pitch is the roof?

| Answer | 30 degree pitch |  |
| --- | --- | --- |

- What size are the noggings and at what spacing?

| Answer | Noggins at 35mm x 70mm with maximum 1350mm spacing. |  |
| --- | --- | --- |

- What size are the ceiling joists?

| Answer | HJ 240mm Joists |  |
| --- | --- | --- |

- What timber is used for roof battens and what spacing?

| Answer | F5 Timber, 35mm x 70mm at maximum 900mm spacing. |  |
| --- | --- | --- |

- What thickness is the plasterboard sheeting?

| Answer | 10mm Plasterboard |  |
| --- | --- | --- |

- What type of flooring is to be installed?

| Answer | 15mm Plywood flooring. |  |
| --- | --- | --- |

- What is the roof cladding material?

| Answer | Corrugated Colourbond roofing to be used (colour not specified) |  |
| --- | --- | --- |

- Where will you find information about the footings?

| Answer | The foundation plans, usually with the sectional drawings will show information on the required sizes and depths of the footings. |  |
| --- | --- | --- |

- What do the letters ‘DAR’ refer when timber ordering?

| Answer | DAR refers to timber being Dressed All Round which means the timber has been machined on all sides to leave a smooth surface. |  |
| --- | --- | --- |

- Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | It means that a 240mm high x 63mm wide joists are to be used to support level 1 of the build. LVL refers to Laminated Veneer Lumber which is a high strength engineered wood product primarily used for structural purposes in construction. |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

- What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

- What is the Job Number?

| Answer | 00R-112 |  |
| --- | --- | --- |

- What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

- What does the drawing show?

| Answer | The drawing is showing the elevations and a sectional drawing of the proposed building to provide information on the external treatments and structural elements of the build. |  |
| --- | --- | --- |

- Which view shows the front of the house?

| Answer | East Elevation |  |
| --- | --- | --- |

- How many motor vehicles can be parked in the garage?

| Answer | 2 |  |
| --- | --- | --- |

- What are the window and doors made from?

| Answer | Aluminium |  |
| --- | --- | --- |

- What is the size of the eaves?

| Answer | 450mm |  |
| --- | --- | --- |

- What are the eaves lined with?

| Answer | Hardiflex |  |
| --- | --- | --- |

- What type of roof structure is the roof built from?

| Answer | Timber framed roof truss structure. |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the requirements for the roof structure in Q59.?

| Answer | Volume 2 of the NCC, Part 3.4.3 – Timber Framing provides more detail regarding requirements for the roof structure for this build. |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | Australian Standard 1684 will provide further detail regarding roof structure depending on whether or not the proposed building is in a cyclonic or non-cyclonic region. |  |
| --- | --- | --- |

- What is the roof cladding materials made from?

| Answer | Concrete roof tiles. |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Volume 2 of the NCC, Part 3.5.2 – Roof Tiles and Shingles provides more detail regarding requirements for the roof cladding for this build |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Australian Standard 2050 will provide details regarding the roof cladding material requirements. |  |
| --- | --- | --- |

- What is the roof pitch?

| Answer | 22.5 degree pitch. |  |
| --- | --- | --- |

- Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | Local Council for a Building Approval. |  |
| --- | --- | --- |

- Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | A building certifier is required to certify the building plans for approval and lodge them with Cairns Regional Council. Documents which must be lodged with Council as well as the application fee include the building application (Planning Act Form 2), Notice of engagement (Form 56), decision notice (Form 6), Final inspection certificate, Copy of plans, drawings and specifications and other stamped approved document and any other documents relevant to the building works (ie. fire safety reports, encumbrances or estates etc.). Applications are usually approved within one week provided all documentation is provided and the build meets requirements of Council and relevant standards/codes. |  |
| --- | --- | --- |

- What is a prime cost (PC) item? Give an example.

| Answer | A prime cost or PC item is a sum included in a contract for purchasing of specific items – usually fixtures, such as appliances, sanitaryware, tiles or doors. It is an agreed amount included when the contract is entered into before the specific item has been selected for the build. The estimate for the PC item must be at least the lowest amount that the item could reasonably cost. An example of a PC inclusion could be bathroom tapware allowance which are required to be selected after the contract is signed but before construction can start. |  |
| --- | --- | --- |

- What is a provisional sum (PS) item? Give an example.

| Answer | A provisional sum or PS item is a sum that is included in a contract to cover works and/or materials which are not specifically detailed at the time of entering into the contract. Estimates to the best of the builder’s ability are required to be provided as part of the provisional sum however sometimes the amount may be exceeded due to unforeseen circumstances. An example of a provisional sum item could be removing existing vegetation from the site prior to works commencing ie. clearing and grubbing of the site. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

- True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify two (2) materials being used for the floors?

| Answer | Carpet and vinyl flooring. |  |
| --- | --- | --- |

- True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- How much does the contracted plasterer cost?

| Answer | $12,900 |  |
| --- | --- | --- |

- How much do the kitchen appliances cost?

| Answer | $2,700 |  |
| --- | --- | --- |

- What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

- What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

- How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

- If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $275.00 |  |
| --- | --- | --- |

- How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 |  |
| --- | --- | --- |

- What material has the highest total cost?

| Answer | Pine 90 x 45 mgp10 (long lengths) for studs over 2.7m long. |  |
| --- | --- | --- |

- How many metres are required of the material in Q80. ?

| Answer | 322.5m |  |
| --- | --- | --- |

- What beam is identified as being a better option than F17?

| Answer | E14 beams |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

- Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter |  |
| --- | --- | --- |

- If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $214.80 |  |
| --- | --- | --- |

- Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar Bender and Fixer |  |
| --- | --- | --- |

- If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $1,285.80 |  |
| --- | --- | --- |

- Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | Australian Standards, National Construction Code, Build Act 1975 (Qld) and the Building Regulation 2006 (Qld) as well as the requirements as per the Cairns Regional Council and the Planning Act 2006 should all be referenced when reading plans and building contracts. |  |
| --- | --- | --- |

CPCCBC4012B

Read and interpret plans and specifications

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Lauren McKew |
| --- | --- |
| Participant signature |  |
| Date | 20/01/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4012B

Read and interpret plans and specifications

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to read and interpret plans and specifications in order to inform estimation, planning and supervisory activities. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - Any drawing with a cloud or mark-up you have inserted. - Copy of the correspondence regarding mark up and any site management/authorising engineer response. The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Drawings (including site plan), Specifications, Tender documentation, Telephone, Facsimile machine, Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Lauren McKew |  |  |
| Name of observer: | Jarryd Londino |  |  |
| Role of observer (Confirm suitability to sign) | Project Manager |  |  |
| Email address of observer | jarryd@fgf.com.au |  |  |
| Signature of observer: |  |  |  |
| Date: | 20/01/2021 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| 1 Interpret drawings |  |  |  |
| - Identify purpose and advantage of different types of drawings. |  |  |  |
| - Identify different aspects of drawings. |  |  |  |
| - Identify, understand and apply: - Commonly used symbols and abbreviations - Common building and construction terms on drawings |  |  |  |
| - Identify building site from location drawings. |  |  |  |
| - Identify on the site plan: - True north - Building orientation - Key features in relation to building and other structures - Levels - Contours - Service entry points - Site features to be removed or retained |  |  |  |
| - Identify key features of plans, elevations and sections. |  |  |  |
| - Identify client requested variations to standard plans on drawings. |  |  |  |
| 2 Interpret specifications |  |  |  |
| - Identify and correctly apply Provisional Sum (PS) and Prime Cost (PC) values. |  |  |  |
| - Identify and confirm customer variations to standard specifications and communicate changes to organisational personnel. |  |  |  |
| - Apply and communicate correct interpretations of essential elements to estimation, planning and supervisory tasks (including labour hours/costs and material quantities/costs). |  |  |  |
| - Identify Building Codes or standards affecting the work to be undertaken (including references to Australian Standards and the National Construction Code). |  |  |  |
| - Identify key features of products included in the specification, including the design, purpose, aesthetics and cost relationships. |  |  |  |
| 3 Communicate with others |  |  |  |
| - Consult with industry professionals, clients, organisational personnel and appropriate local authorities. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Using technology (including telephone, facsimile, email) |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments

Lauren is competent in reading and interpreting plans and specifications. As part of her role in supporting myself and our other project managers and estimating team, interpreting drawings is a key aspect of her day-to-day requirements. Lauren is confidently able to do take-offs for procurement purposes from interpreting various drawings and is able to effectively communicate proposed changes and subsequent variations. Lauren is also required to undertake extensive document control and preparations of drawing registers and transmittals for all projects and any changes that come through on drawings from clients, engineers and consultants. This requires ability to recognise changes between revisions and accurately record updates as they are received. I believe Lauren has an excellent understanding of reading and interpreting drawings and is able to effectively perform this aspect of her job.