BSBOHS201A

Participate in OHS processes

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Linda Hudakova |
| --- | --- |
| Participant signature |  |

| Assessor name | Brett Stoward |
| --- | --- |
| Assessor signature |  |

| Date | 20/11/2018 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 24 |  | Yes | No |

Assessor Comments

Match the terms and definitions by writing the appropriate letter next to each number in the centre column.

| Answer | Terms |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | PRA | 1 - F | A | Outline the requirements of the work to be undertaken. |  |
|  | 2 | JSA | 2 - E | B | Outline what to do in the event of fires and other accidents |  |
|  | 3 | Plans and specifications | 3 - A | C | Contains important information about hazardous materials or chemicals. |  |
|  | 4 | Manufacturer specifications | 4 - D | D | Outlines how tools and equipment should be used (e.g. lubricant levels, maintenance requirements). |  |
|  | 5 | SDS | 5 - C | E | Identifies hazards and potential hazards that may be encountered while working. |  |
|  | 6 | Emergency procedures | 6 - B | F | A personal assessment of risk |  |

For each of the hazards identified below, describe the injuries/damage they may cause (i.e. to fellow workers, other people and/or animals) and suggest a suitable control measure.

| Answer | Hazard | Injuries or damage | Control measure |  |
| --- | --- | --- | --- | --- |
|  | Hazardous chemical | Respiratory damage, skin rashes, chemical burns | PPE-gloves |  |
|  | Noise | Hearing damage, headaches, fatigue | PPE-Hearing protection |  |
|  | Manual handling | Strains, sprains | Team lift Using lifting equipment |  |

Who should you speak to if you find problems (e.g. hazards, accidents and emergencies)?

| Answer | - supervisor, site/project manager or Health & Safety Representative, using the report form supplied by your organisation |  |
| --- | --- | --- |

What should you do if you discover a fault with plant, tools or equipment?

| Answer | - Stop using the equipment - Tag it out of service - Report it to your supervisor - Record the details of the fault |  |
| --- | --- | --- |

Which legislation relates to safety in the workplace? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | ☐ | Occupational Health and Safety Act 2015 |  |
|  | ☒ | Work Health and Safety Act 2011 |  |
|  | ☐ | Safety Act 1900 |  |
|  | ☐ | Safety and Health Act 2011 |  |

True or False (Tick your response): Employers are required by legislation to implement safety measures which are reasonably practicable.

| Answer | ☒ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☐ | False |  |  |

True or False (Tick your response): Workers are required by legislation to follow reasonable employer directives (e.g. to undertake first aid and/or fire safety training).

| Answer | ☒ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☐ | False |  |  |

The following symbols appear under the heading ‘Required PPE’ on an SDS. What do the symbols represent?

| Answer | Symbol | Meaning |  |
| --- | --- | --- | --- |
|  |  | Overalls |  |
|  |  | Globes required |  |
|  |  | Goggles required |  |
|  |  | Respirator required |  |

Describe the meaning and use of the signs displayed below.

| Answer | Sign | Meaning |  |
| --- | --- | --- | --- |
|  |  | Location of the first aid kit |  |
|  |  | Warns off unauthorised persons from entering area |  |
|  |  | Location of fire fighting facilities |  |

Refer to the Risk Matrix below when responding to Q10. to Q12.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

Which rating is designated for the highest risk and which rating is used for the lowest risk?

| Answer | Highest Risk: E-Extreme Lowest Risk: L-Low |  |
| --- | --- | --- |

What is the risk rating for the following circumstances: An event which is possible and would require low-level hospital treatment.

| Answer | Medium |  |
| --- | --- | --- |

What are the factors that would lead to a risk rating of E: Extreme?

| Answer | The situation is very likely and could cause death or permanent disability, serious injury |  |
| --- | --- | --- |

When should safety checks be undertaken on machinery and equipment? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | ☐ | Before operation |  |
|  | ☐ | After operation |  |
|  | ☒ | Both before and after operation |  |

Order the elements of the hierarchy of control, from 1 to 6 (i.e. most effective to least effective).

| Answer | Label (1 – 6) | Element |  |
| --- | --- | --- | --- |
|  | 3 | Isolate the hazard |  |
|  | 6 | Provide and use Personal Protective Equipment (PPE) |  |
|  | 1 | Eliminate the hazard |  |
|  | 4 | Control the hazard by engineering means |  |
|  | 5 | Control the hazard by administrative means |  |
|  | 2 | Substitute the hazard |  |

Why is elimination the best method of hazard control and PPE considered the least effective?

| Answer | Elimination is the best method as there is no possibility of the hazard becoming a risk. PPE is the least effective as there is still a possibility that the hazard will occur, with the PPE only possibility reducing the consequence slightly |  |
| --- | --- | --- |

If you were to replace a flammable solvent with a water-based solvent, what category of control measure would that be under the hierarchy of control?

| Answer | Substitution |  |
| --- | --- | --- |

In the diagram below a person is lifting a tool box.
Filling in the table, indicate how each body part should be used/placed.

F. Hands

E. Knees

D. Shoulders

C. Feet

B. Back

A. Neck

| Answer | Area of body (Letter) | How it should be placed |  |
| --- | --- | --- | --- |
|  | D | Level |  |
|  | A | Upright |  |
|  | F | Good grip |  |
|  | B | Natural curve |  |
|  | E | Bent |  |
|  | C | Flat, facing load |  |

In a medical emergency, what is the first step in the DRSABCD action plan that you need to follow?

| Answer | Danger-you need to ensure that the area is safe |  |
| --- | --- | --- |

In the event of a fire emergency, what should you do? 
Fill in the table below to explain the acronym R.A.C.E.

| Answer | Letter | Meaning |  |
| --- | --- | --- | --- |
|  | R | Remove people from immediate danger |  |
|  | A | Alert people of emergency |  |
|  | C | Confine fire and smoke |  |
|  | E | Evacuate to the assembly area |  |

What should you do as part of the clean up process? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | ☐ | Store all plant, tools and equipment without cleaning |  |
|  | ☒ | Maintain plant, tools and equipment |  |
|  | ☒ | Dispose of materials that cannot be reused or recycled |  |
|  | ☒ | Report faults in logbook and/or site records |  |
|  | ☒ | Clean all plant, tools and equipment before storage |  |

Read the scenario below and complete the Hazard Report Form.

SCENARIO

Today at 10:45am a delivery truck reversed into a car park (located at 54 Standish Street, Salisbury) to deliver gardening materials. There was nobody in the car park to guide the driver and he backed into a number of drums of diesel located to the left of the photograph (below).  One drum was pierced and its contents erupted from the container.

The marked area on the photo represents the diesel spill, measuring 3m x 4m.

HAZARD REPORT FORM

| GENERAL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date | 20/11/2018 |  |  |  |  |  |
| Submitted by | Linda Hudakova |  |  |  |  |  |
| HAZARD DETAILS |  |  |  |  |  |  |
| Location | 54 Standish Street, Salisbury |  |  |  |  |  |
| Description of issue | Delivery truck collided with diesel drums when reversing into car park Diesel spill bitumen surface approximately 3m x 4m |  |  |  |  |  |
| Risk class | ☐ Extreme | ☒ High | ☐ Medium |  |  | ☐ Low |
| CONTROL MEASURES |  |  |  |  |  |  |
| Corrective action required | Lock entry to car park Erect barricades in car park Ensure no plant or vehicles are started to prevent fire ignition source. |  |  |  |  |  |
| Person to action | Supervisor |  |  | When | ☒ Immediately ☐ Within 24 hours ☐ Within 7 days |  |

| Answer |  |  |
| --- | --- | --- |

Who would you submit a hazard report form to?

| Answer | PCBU, Supervisor, Health and Safety Representative |  |
| --- | --- | --- |

Who would approve proposed risk controls and organise the required resources for implementation of the risk control?

| Answer | PCBU, Supervisor, Health and Safety Representative |  |
| --- | --- | --- |

Identify one (1) way you can assess if risk controls have been effective.

| Answer | Reviews, consultation, feedback/safety date |  |
| --- | --- | --- |