CPCSUS4002

Use building science principles to construct energy efficient buildings

ASSESSMENT SUMMARY

The assessment activities for CPCSUS4002 Use building science principles to construct energy efficient buildings cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Dawei Yang |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. | Satisfactory | Not Satisfactory |
| Workbook | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for CPCSUS4002 | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Theory Exam Workbook Practical Assessment | Residential Construction Energy Efficiency report Commercial Construction Energy Efficiency report |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

CPCSUS4002

Use building science principles to construct energy efficient buildings

THEORY EXAM

| Participant name | Dawei Yang |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within two (2) hours, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

Define systems theory.

| Answer | Systems theory is the study of the nature of complex systems where one can investigate small parts of a system that work together to produce benefits for a whole system where they all belong. |  |
| --- | --- | --- |

Below are elements of an energy-efficient building. Basing on systems theory, explain how each element (part of a system) contributes to the overall energy efficiency of a building (whole system).

| Answer | Element | Explanation |  |
| --- | --- | --- | --- |
|  | Thermal envelope | The thermal envelope is everything about the building that helps to protect the living area from the outside atmosphere, which will keep the living space well-insulated. |  |
|  | Ventilation | Ventilation functions to distribute and aid the flow of air inside a room or building, providing good quality air. |  |
|  | Cooling system | Cooling system (e.g. air conditioning) functions to provide cooling comfort inside a room or building. |  |

Clarify health risks and considerations associated with the following building types and indoor activities that must be considered.

| Answer | Type of building | Indoor activity | Risks | Health and safety considerations |  |
| --- | --- | --- | --- | --- | --- |
|  | House | Cooking | Development of health problems due to prolonged exposures to harmful gases while cooking | Kitchens must be installed with windows to aid natural air circulation |  |
|  | House | Washing | Chemicla with water made the earth pollution | Must pull over into drainage pipe |  |
|  | Factory | Roof tile manufacturing | Keep the inside as an indoor environment | Roof window or rollover must be installed on the roof |  |
|  | School | Laboratory experiment | Development of respiratory and neurological problems due to inhalation of poisonous substances | Laboratories must have windows (in addition to special ventilating equipment) to allow contaminated air to dissipate and clean air to replenish. |  |

Explain how failing to meet requirements for minimisation of heat gain in summer can affect the building occupants’ health.

| Answer | Heat-related illnesses include dehydration, heat cramps, heat exhaustion, heat stroke and worsening of existing medical conditions. It is important to be aware of the signs and symptoms of heat-related illness, to recognise and treat affected people quickly. |  |
| --- | --- | --- |

Discuss three (3) challenges and considerations to be made, related to beachfront buildings.

| Answer | Humidity High salt exposure High temperature |  |
| --- | --- | --- |

Explain how each of the following weather conditions impact a building.

| Answer | Weather condition | Explanation |  |
| --- | --- | --- | --- |
|  | Cyclonic activity | Cyclonic activity, which is characterised by high winds, can cause cladding materials (i.e. wall and roof cladding) to be blown away. When cyclones are accompanied by heavy rain for a long duration, building materials can be damaged due to flooding. |  |
|  | Extreme heat | Extreme heat causes wood to expand, which is the reason why door and window frames often get stuck, especially during the summer. |  |
|  | Risk of fire | Risk of fire can also dramatically decrease the life cycle of a building, especially components made of steel. Fire greatly reduces the overall structural capacity of steel. |  |
|  | Heavy rain | Heavy rains may produce rainwater puddles around the building that tend to seep into the ground and towards structures found below the ground level. This may cause cracks in basement walls where water can intrude and cause dampness, attracting mould and mildew and termites. |  |
|  | Flooding | Flooding causes building materials to come in contact with large quantities of contaminated water. This leads to corrosion of steel elements and decay in wood. |  |
|  | Storm weather | Storm weather has more or less similar impacts with cyclonic activity, heavy rain, and flooding. High winds can cause structural roof materials to be blown off, and the heavy rain that comes with the storm causes flooding, which damages steel and wood materials used in the building envelope. |  |

Describe techniques or methods that are applied to improve the durability of concrete, particularly in coastal areas.

| Answer | Addition of admixtures in concrete mixes |  |
| --- | --- | --- |

Discuss the purpose of building using cavity brick masonry techniques and the benefits in relation to energy efficiency.

| Answer | This refers to a technique that involves laying two separate layers of brick (i.e. one exterior leaf and one interior leaf) with a small cavity or space between them. This improves the overall resistance of the brick wall against moisture or dampness, as opposed to only having one layer of brick (which is susceptible to damage against rainwater due to its porous nature). |  |
| --- | --- | --- |

Discuss how the Greenhouse and Energy Minimum Standards (GEMS) Act affects which products are used during energy efficient construction.

| Answer | Specifications under the GEMS Act, which include the requirements for Minimum Energy Efficiency Standards (MEPS) and Energy Rating Labelling (ERL), are set out in a further statutory instrument called a GEMS Determination. A GEMS Determination sets out the requirements which products need to comply with in order to be ‘legally registered, supplied, or offered for supply in Australia.’ |  |
| --- | --- | --- |

Describe the ‘Minimum Energy Performance Standards’ (MEPS).

| Answer | The Minimum Energy Performance Standards (MEPS) is a standard used in Australia to promote energy efficiency of products. It specifies the minimum level of energy performance that appliances, lighting, and electrical equipment must meet in order to be sold and used. |  |
| --- | --- | --- |

Specify what each of the following Australian Standards address relevant to energy efficiency of buildings.

| Answer | Weather condition | Explanation |  |
| --- | --- | --- | --- |
|  | AS/NZS ISO 9972:2015 | Thermal performance of buildings - Determination of air permeability of buildings - Fan pressurization method |  |
|  | AS/NZS 2904:1995 | Damp-proof courses and flashings |  |
|  | AS 4654.1-2012 | Waterproofing membranes for external above-ground use, Part 1: Materials |  |
|  | AS 2047-2014 | Windows and external glazed doors in buildings |  |
|  | AS/NZS 4859.1:2018 | Thermal insulation materials for buildings, Part 1: General criteria and technical provisions |  |
|  | AS/NZS 5141:2018 | Residential heating and cooling systems - Minimum applications and requirements for energy efficiency, performance and comfort criteria |  |

What is ANSI/ASHRAE 55-2020 - Thermal Environmental Conditions for Human Occupancy?

| Answer | Created by the American Society of Heating, Refrigerating and Air-conditioning Engineers (ASHRAE) and approved by the American National Standards Institute (ANSI), this standard contains provisions regarding conditions for environmental thermal acceptability (i.e. evaluation of thermal environments). |  |
| --- | --- | --- |

Discuss theory of heat flow and how heat transfers between objects.

| Answer | heat will continue to flow (from a warm object or space to a cooler object or space) until both objects or spaces reach equilibrium, which means that both their temperatures are equal. |  |
| --- | --- | --- |

Listed below are different types of central heating systems.

- Compare each system by indicating how much their running cost (either low, medium, or high) and their greenhouse gas emissions, and answer the questions that follow.

| Answer | Energy Efficient System | Running Cost | Greenhouse Gas Emissions |  |
| --- | --- | --- | --- | --- |
|  | High efficiency ducted natural gas | low | low |  |
|  | Hydronic zoned natural gas or heat pump | low | low |  |
|  | Ducted reverse cycle or heat pump | Medium | Medium; low with renewable electricity sourc |  |
|  | In-slab high off-peak electric | Medium to high | Very high; low with green/renewable electricity source |  |

Discuss the factors that may affect variations in HVAC energy use between different types of buildings.

| Answer | The size and layout of the building are fundamental factors that influence HVAC system design. |  |
| --- | --- | --- |

Explain the difference between exhaust ventilation systems and supply ventilation systems.

| Answer | Exhaust ventilation system exhausts air from the building, effectively depressurising it and causing outside air to be pulled in through intentional, passive vents due to the change in pressure supply ventilation system pressurises a structure by forcing outside air into the building using a fan. |  |
| --- | --- | --- |

Why should you review mechanical ventilation systems and equipment to aid air flows prior to purchase and installation?

| Answer | Review the mechanical ventilation system and equipment of the HVAC systems you researched in Subchapter 2.3. Consider its type, how it can help air flow in a structure, and the operational costs associated with it. A system can be considered appropriate according to several factors, including the type of building to be constructed, the size of the building or room, appropriateness to the climate where a structure is built, and availability of the system |  |
| --- | --- | --- |

Read the installation guide below to respond to Q18. & Q19.

Describe how this product should be applied to a stud wall and around a corner.

| Answer | When wrapping around corners, 150mm should extend around that corner. • When applying a new roll horizontally it should overlap the previous roll by 150mm. • Timber frame: Affix using galvanised staples/metal fixings every 150mm, where possible all overlaps and end joints should line up with a wall stud. |  |
| --- | --- | --- |

Explain why exterior cladding should be installed without delay following the installation of the wall wrap.

| Answer | : Wall wrap is a secondary sarking material and is not designed to withstand prolonged direct exposure to the elements - accordingly, the exterior cladding should be installed without delay. Product exposed to harsh weather conditions for more than 6 weeks for walls should be inspected for damage prior to installation of the exterior cladding and damaged product should be repaired or replaced to comply with the product warranty |  |
| --- | --- | --- |

Explain passive solar heating and why this must be considered when designing the position for a building.

| Answer | north-facing glass areas exposed to the sun, which traps solar radiation and provides heating. |  |
| --- | --- | --- |

What is embodied energy?

| Answer | embodied energy is the energy built into products or materials. It is the energy used by all the processes associated with the building material or product, including natural resource extraction, production, transport, and distribution |  |
| --- | --- | --- |

Explain why consideration of embodied energy is made when selecting construction materials and products for energy-efficient buildings.

| Answer | In the construction of energy-efficient buildings, one of the primary concerns is greenhouse gas emissions, which is the by-product of energy inputs. The role of embodied energy is to give you a general idea of the amount of greenhouse gas emission produced as a by-product of processing that material or product. A material with a high embodied energy means that a considerable amount of greenhouse gas had been produced for a single unit of the material. Conversely, a material with low embodied energy means only a small amount of greenhouse gas had been produced as a by-product. Thus, in choosing construction products or materials, you should choose those with low embodied energy as they are considered more sustainable, more energy-efficient, and eco-friendly. |  |
| --- | --- | --- |

Discuss energy efficient treatment that can be applied to combat insulating material gaps around old style halogen downlights.

| Answer | Led down light |  |
| --- | --- | --- |

Discuss effective energy efficient treatment that can be used for the following situations.

| Answer | Situation | Effective energy efficient treatment |  |
| --- | --- | --- | --- |
|  | Gaps under and around doors | Close up the gaps by adding weatherstrips. |  |
|  | Openings for floor or ceiling ducts used to transmit heating or cooling | These openings should be duct sealed. Duct sealing should be done using silicone caulking or a special cement (mastic) with an embedded fabric. |  |
|  | Wall, floor or ceiling cracks | Caulk and seal possible air leakage points where plumbing, ducting, or electrical wiring comes through cracks. |  |
|  | Window seals | Use caulk to seal cracks less than a quarter-inch in width on the non-movable parts of windows, primarily around the frame and where the trim meets the wall. |  |
|  | Thermal mass | Avoid using material with high thermal mass |  |

Discuss the benefit of using a Nationwide House Energy Rating Scheme (NatHERS) assessor for predicting the energy efficiency of a building before building.

| Answer | NatHERS is a star rating system that rates a home’s energy efficiency based on the design. It uses a software program (based on scientific research) that models the house and estimates the energy needed for heating and cooling, factoring in both energy efficiency and comfort of building occupants. |  |
| --- | --- | --- |

Discuss the digital tools builders use to communicate with clients, suppliers, architects, and others.

| Answer | 1. ‘Chat’ tools 2. Digital design tools 3. Project management software 4. Virtual Reality |  |
| --- | --- | --- |

CPCSUS4002

Use building science principles to construct energy efficient buildings

WORKBOOK

| Participant name | Dawei Yang |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written activities as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - The timeframe for these tasks will be established by your Assessor or Training Plan, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

NOTE

At times, you will be provided with a specific business context to which you must respond, otherwise it is anticipated that you will use a workplace of your choosing (your own, or hypothetical) to provide the context to the task.

At all times, the responses in your assessment must reference Australian (Queensland or Federal) documentation, including legislation, regulations, codes of practice, standards and guidance publications.

Access and review Section J of the National Construction Code (NCC) Volume 1 and Part 2.6 of the NCC Volume 2 and answer the following questions. - What is the NCC’s objective regarding Energy Efficiency? - List all the performance requirements found under P2.6.1.

- What is the NCC’s objective regarding Energy Efficiency?

.The Objective is to reduce greenhouse gas emissions.

- List all the performance requirements found under P2.6.1.

the function and use of the building; and the internal environment; and the geographic location of the building; and the effects of nearby permanent features such as topography, structures and buildings; and solar radiation being— utilised for heating; and controlled to minimise energy for cooling; and the sealing of the building envelope against air leakage; and the utilisation of air movement to assist cooling; and water resources available; and pertinent water management measures of the responsible water authority.

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Use the internet to research building science methods on the evaluation of energy efficiencies and respond to the questions below: - One (1) local (Australian) - The Evaluation of the 5-Star Energy Efficiency Standard for Residential Buildings - One (1) international - Energy performance of buildings: The evaluation of design and construction measures concerning building energy efficiency in Iran

- Local research on building science method

- Research Title

a)The Evaluation of the 5-Star Energy Efficiency Standard for Residential Buildings

- Author/publisher

Published by the Department of Industry

- Date research is published

2016

- Summary of research

In 2006, the Building Code of Australia set a new residential building energy efficiency standard of 5 stars, as rated by software tools accredited under the Nationwide House Energy Rating Scheme (NatHERS). The Regulation Impact Statement (RIS) on the 5-star standard analysed its likely impact on the energy efficiency of new houses relative to the previous standard finding that the new standard would reduce heating and cooling energy costs, as well as greenhouse gas emissions. In 2012, to assess whether the new standard was achieving its goals, the Australian Government asked CSIRO to: find out whether the 5-star standards have actually reduced heating and cooling energy use of houses compared with those built to the earlier 3.5 to 4-star standard determine the actual benefits and costs of meeting the 5-star standard. To undertake this task, CSIRO studied 414 houses in the principal centres of population of three BCA climate zones over a winter and summer period.

- URL/Source

https://www.dcceew.gov.au/energy/publications/evaluation-5-star-energy-efficiency-standard-residential-buildings

- - Continued

- International research on building science method

- Research Title

Energy performance of buildings: The evaluation of design and construction measures concerning building energy efficiency in Iran

- Author/publisher

School of Civil Engineering, College of Engineering, University of Tehran, 16 Azar Ave. , Tehran, P.O. Box 11155-4563, Iran

- Date research is published

Received 11 September 2013, Revised 8 February 2014, Accepted 12 February 2014, Available online 26 February 2014.

- Summary of research

Passive solar energy is the most applicable renewable energy system in buildings in Iran. • Design and construction measures concerning building energy efficiency are identified. • Twenty three design and construction measures are identified and categorized into 12 groups. • Twelve groups of measures are evaluated and classified into three levels of importance. • The most important measures are related to materials and isolation

- URL/Source

https://www.sciencedirect.com/science/article/abs/pii/S0378778814001492

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Research two (2) manufacturers of energy-efficient materials and respond to the following: - Identify the name of the manufacturer - Identify the website links of the manufacturer you provided - Provide two (2) examples of their products: - energy-efficient material - energy-efficient system

Manufacturer 1

- Name of manufacturer

Boral

- Website / link

https://www.boral.com.au/envisia?gad_source=1&gclid=Cj0KCQiA5rGuBhCnARIsAN11vgQ8q5edQk61S415NsmkRUAsDIDfBrQFRIbXhQjZY-3b8A5lAQtRs5waAuugEALw_wcB

- Products

- energy-efficient material Lower Carbon Concrete

- energy-efficient system cement and chemical manufacturing reduces fossil fuel use in industry

Manufacturer 2

- Name of manufacturer

GE 7.5W Heritage Dimmable LED Pilot ES Globe in Cool White

- Website / link

https://www.beaconlighting.com.au/ge-7-5w-heritage-dimmable-led-pilot-es-globe-in-cool-white

- Products

- energy-efficient material Light

- energy-efficient system Less power consume

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Research the work health and safety requirements and guidelines related to installing ceiling insulation in your state or territory and respond to the following: - List your State/Territory - WHS regulator researched - Risks related to installing ceiling insulation - Guidelines to follow during insulation installation - Guidelines to follow after insulation installation.

- List your State/Territory

QLD

- WHS regulator researched

the Work Health and Safety Act 2011 the Work Health and Safety Regulation 2011 codes of practice.

- Risks related to installing ceiling insulation

serious electric shocks or burns caused by contact with defective electrical cables - e.g. perished or rodent-damaged cable insulation exposed terminals or conductors of electrical equipment in the ceiling space - e.g. behind light fittings, fans, etc. electrical cables damaged while installing insulation materials - e.g. by cutting, piercing, nailing or stapling into electrical cables, and foil insulation which has become energised through contact between the foil and a source of electricity. fire resulting from combustible insulation material installed near recessed lights (in particular halogen downlights) and accessories without suitable clearances/barriers in place the safe operating temperature of electrical cables may be affected when surrounded by thermal insulation material - advice regarding such de-rating of electrical cables may be obtained from a person with specialist expertise such as a licensed electrical contractor insulation containing synthetic mineral fibres (SMFs) - rockwool, glasswool, other fibres or dust can irritate the skin, eyes and upper respiratory tract hazardous substances - e.g. asbestos, pesticides, chemicals or lead heat and humidity - these and other factors can cause heat stress disease or infection from vermin or insects in the ceiling cavity or in untreated or damp insulation combustion of non fire-retardant insulation accessing and traversing the roof cavity, tripping over debris, material and tools working at heights - e.g. falling through ceilings, falling through roof penetrations, falling from roofs inadequate lighting, and hazardous manual tasks - e.g. handling awkward bulky bats and heavy foil rolls in cramped spaces, use of hand tools when cutting, or removing roofing materials.

- Guidelines to follow during insulation installation

identify and mark the position of all electrical cables in the work area and ensure that all insulation fixing points are well clear of electrical cables and equipment make sure you do not damage any electrical cables or electrical equipment while trimming, cutting or fixing insulation if installing electrically conductive ceiling insulation, and there are one or more non-capable circuits located in the ceiling structure of the building, ensure that: you are aware of how any non-capable circuit has been marked by the licensed electrical contractor; and any non-capable circuit is not covered anywhere by the electrically conductive ceiling insulation; and any non-capable circuit is kept at least 25mm away from the electrically conductive ceiling insulation material do not place insulation material over recessed fans, lights and associated equipment such as transformers (in particular halogen downlights) maintain minimum required clearances for insulation materials around electrical equipment (e.g. downlights) and fit fire-resistant barriers where required as specified in clause 4.5.2.3 of the Wiring Rules (AS/NZS 3000:2007) Default clearances are 50 mm from incandescent and 200 mm from halogen downlights including 50 mm clearance for any associated transformer - unless the downlight is designed for the application or is installed within a suitable fire-resistant enclosure. follow the safe work procedures for removing and installing insulation - any safe work procedure must include measures to ensure the electrical safety of all persons and property, both during and after the installation keep in constant contact with someone wear appropriate, well maintained and correctly-fitted PPE when working with dusty insulation such as fibreglass or when working in dusty ceilings spaces, including: a half-face (class P1 or P2) disposable particulate respirator, in accordance with AS/NZS 1715 a head-covering and goggles, to avoid eye irritation long-sleeved, loose-fitting clothing and gloves, to minimise skin contact wear appropriate footwear follow the developed measures to prevent or minimise the risk of injury from hazardous manual tasks handle the insulation with care and minimise the release of fibres or dust step on ceiling joists or other beams - not the ceiling material, and keep your work areas clean and clear of fibres and dust, by regularly using an industrial vacuum cleaner fitted with high-efficiency particulate filters, and place waste in plastic bags capable of containing dust.

- Guidelines to follow after insulation installation.

Immediately after you have installed the insulation: if you suspect that electrical equipment such as cables, light fittings, etc have been damaged, or the ceiling insulation could be energised by contact with electricity, do NOT turn the power on - liaise with the owner/occupier and engage a licensed electrical contractor to assess/control the electrical risks dispose of debris and waste appropriately, and wash your hands, face, neck and hair, with soap and water.

| Satisfactory Outcome |  |  |
| --- | --- | --- |

CPCSUS4002

Use building science principles to construct energy efficient buildings

PRACTICAL ASSESSMENT

| Participant name | Dawei Yang |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel, unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 (Occasion 1) | Select Date. | Workplace Simulation |  |  |  |
| Task 1 (Occasion 2) | Select Date. | Workplace Simulation |  |  |  |

ASSESSOR COMMENTS: GENERAL

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 1- Residential construction project |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Provide detail |
| 1.1E. 1.2A. | Specify how research on a range of energy efficient design and construction methods was conducted: |
| 1.2C. 1.2D. | Specify how levels of energy efficiency, ventilation and indoor air quality were determined: |

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 2 Commercial construction project |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Details |
| 1.1E. 1.2A. | Specify how research on a range of energy efficient design and construction methods was conducted: |
| 1.2C. 1.2D. | Specify how levels of energy efficiency, ventilation and indoor air quality were determined: |

CPCSUS4002

Use building science principles to construct energy efficient buildings

Practical Task 1 (2X)

Description of task: The participant must demonstrate the skills and knowledge required to incorporate building science principles into the construction of energy efficient buildings by making recommendations in the form of a report for the use of energy efficient materials, products and construction methods for one (1) residential construction project (Occasion 1) and one commercial construction project (Occasion 2) (template available) in the State/Territory of place of work, including: - Determine the building envelope expectations. - Determine the building use and energy efficiency expectations. - Identify risks associated with occupants’ indoor activities and health and safety considerations. - Determine environmental and climatic conditions that impact the resilience of building materials. - Determine construction methods to be applied that factor in durability of materials. - Consult relevant legislation, standards, regulations and codes to identify minimum energy efficient standards to be met. - Assess energy efficiency construction. - Research and consult relevant domestic and international codes, standards and examples of best practice that meet or exceed current energy efficient standards and apply building science principles. - Review embodied energy of the specified products or materials to evaluate the energy efficiency. - Research heating, ventilation and air conditioning (HVAC) systems to identify levels of energy efficiency, ventilation and indoor air quality. - Review mechanical ventilation systems and equipment to aid air flows for appropriateness and operational costs. - Research and identify durability and appropriateness of moisture and vapour barriers to gain high levels of energy efficiency. - Research and report on energy efficient construction methods and systems. - Decide on method of construction. - Incorporate building science principles for energy efficient materials and products used in the building envelope. - Identify site location and building position to maximise energy efficiency relating to environmental factors, climatic conditions and use of building. - Identify and assess energy efficiency of thermal energy, heating and cooling, air and moisture flows throughout the building. - Develop and document strategies to minimise unwanted air leakages. - Evaluate and select mechanical ventilation systems and equipment for appropriateness to the site and operational costs. - Evaluate and select materials for interior and exterior barriers for energy efficiency, durability and appropriateness for building site climate. These tasks may be simulated or real. The Participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. The Observer is to provide detailed comments on the tasks completed by the Participant (on the Observer Comments, overleaf). The Assessor is to verify the Observer’s comments, noting their findings in the Assessor Comments (on the back of the Practical Assessment cover page) and provide the assessment outcome.

Resources required: Construction project (1 residential and 1 commercial) Plans, drawings, access to internet, access to PC, report template (workplace or supplied). policies, procedures and other quality documentation, Communication equipment.

| Observer details |  |
| --- | --- |
| Name of observer: | Steve Zhu |
| Role of observer (Confirm suitability to sign) | Builder |
| Email address of observer | Steve.zhu.au@hotmail.com |
| Signature of observer: |  |
| Date: | 17/01/2024 |

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |  |
| --- | --- | --- |
|  | 1st Occasion | 2nd Occasion |
| 1 Determine the building envelope expectations. |  |  |
| Determine the building use and energy efficiency expectations. | þ |  |
| Identify risks associated with occupants’ indoor activities and health and safety considerations. | þ |  |
| Determine environmental and climatic conditions that impact the resilience of building materials. | þ |  |
| Determine construction methods to be applied that factor in durability of materials. | þ |  |
| Consult relevant legislation, standards, regulations and codes to identify minimum energy efficient standards to be met. | þ |  |
| 2 Assess energy efficiency construction |  |  |
| Research and consult relevant domestic and international codes, standards and examples of best practice that meet or exceed current energy efficient standards and apply building science principles. | þ |  |
| Review embodied energy of the specified products or materials to evaluate the energy efficiency. | þ |  |
| Research heating, ventilation and air conditioning (HVAC) systems to identify levels of energy efficiency, ventilation and indoor air quality. | þ |  |
| Review mechanical ventilation systems and equipment to aid air flows for appropriateness and operational costs. | þ |  |
| Research and identify durability and appropriateness of moisture and vapour barriers to gain high levels of energy efficiency. | þ |  |
| Research and report on energy efficient construction methods and systems. | þ |  |
| 3 Decide on method of construction. |  |  |
| Incorporate building science principles for energy efficient materials and products used in the building envelope. | þ |  |
| Identify site location and building position to maximise energy efficiency relating to environmental factors, climatic conditions and use of building. | þ |  |
| Identify and assess energy efficiency of thermal energy, heating and cooling, air and moisture flows throughout the building. | þ |  |
| Develop and document strategies to minimise unwanted air leakages. | þ |  |
| Evaluate and select mechanical ventilation systems and equipment for appropriateness to the site and operational costs. | þ |  |
| Evaluate and select materials for interior and exterior barriers for energy efficiency, durability and appropriateness for building site climate. | þ |  |
| Use digital tools and devices to communicate effectively with others | þ |  |

Observer comments: