CPCCBC4012B

Read and interpret plans and specifications

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4012B Read and interpret plans and specifications cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Christine Muter |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Christine Muter |
| --- | --- |
| Participant signature |  |
| Date | 20/08/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 87 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

- Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

- Which compass direction is always shown on plans and drawings?

| Answer | North |  |
| --- | --- | --- |

- What information is provided on a location plan?

| Answer | Scale of the drawing – usually 1:5000 Lot Numbers Plan numbers North Point |  |
| --- | --- | --- |

- What detail does a site or block plan contain?

| Answer | Reference height to other levels – A datum The building location in the site and its north point Key Features can also include; The outline of the boundaries on the site Street frontage Any trees/vegetation Contours and slopes of the site Easements, driveways, drainage Soil and surface water drains and pipe sizes Utilities locations such as sewer, was electricity and gas Details of any retaining walls or paved areas |  |
| --- | --- | --- |

- What do floor plans identify?

| Answer | Floor plans identify the details of the proposed structure such at where the different internal rooms will be ie, bedrooms, bathrooms, kitchen, any staircases and external areas such a patio or balcony. It details the position of fixings such as toilets, baths, sinks, doors, windows and floor coverings. |  |
| --- | --- | --- |

- What is a landscape plan and what information might it provide?

| Answer | Landscape plans outline garden layouts, paths, retaining walls and plant location. |  |
| --- | --- | --- |

- What are elevation views? What do they provide to the plan reader?

| Answer | Elevation views allows the plan reader to see the proposed structure from its sides ; north, south, east and west |  |
| --- | --- | --- |

- What is a section?

| Answer | A section of a plan is like a “slice” of the proposed structure. This assists the builders to determine how the various sections are to be constructed. It shows the size and thickness of its structure such as the height from the ground to the floor level, the height between the floor and the ceilings and how the roof is to be constructed |  |
| --- | --- | --- |

- What are detailed drawings?

| Answer | Detailed plans illustrate a particular method of construction of finish |  |
| --- | --- | --- |

- What are specifications?

| Answer | Specifications are the details that support the drawings and plans by providing the details of the materials to be used, the fixtures and fittings and standards to be observed. |  |
| --- | --- | --- |

- Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed datum |  |
|  | B | Basin |  |
|  | BN | Bull Nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Bock |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Glass |  |
|  | CJ | Ceiling Joist |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold Water Tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Proof Course |  |
|  | DSB | Distribution Switch Board |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion Joint |  |
|  | FPBD | Fibrous plaster board |  |
|  | FFL | Finished floor level |  |
|  | FHR | Fire hose reel |  |
|  | GPO | General purpose outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

- Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Wall – Cavity Brick |  |  |
|  | Wall – Brick Veneer |  |  |
|  | Wall - timber stud |  |  |
|  | Window with frame |  |  |
|  | Window – in single wall |  |  |
|  | Door – single leave opening 90deg |  |  |
|  | Doors two leaves each opening 90deg |  |  |
|  | Door – sliding into pocket (cavity sliding door) |  |  |
|  | Sliding door – exposed on wall |  |  |
|  | General power socket outlet |  |  |

- Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Casement Window hinged at side |  |  |
|  | Awning Window hinged at top |  |  |
|  | Window-sliding horizontally |  |  |
|  | Window –fixed |  |  |

- What does a clouded area on a plan indicate?

| Answer | A client requested Variation |  |
| --- | --- | --- |

- An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

- On what date was an amendment made to the window details?

| Answer | 05/10/2011 |  |
| --- | --- | --- |

- True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What amendment occurred on 08/11/2011?

| Answer | Bedroom 3 robe was deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

- What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

- What is the Lot number of this site?

| Answer | Lot 16 |  |
| --- | --- | --- |

- What is the size of this lot?

| Answer | 3653m2 |  |
| --- | --- | --- |

- Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

- True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What size is the easement on the eastern boundary of the site?

| Answer | 5 metres |  |
| --- | --- | --- |

- True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

- What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

- Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

- Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26.?

| Answer | Yes, this is a sectional drawing of the proposed dwelling as the client details, site address and drawing dates and drawing number are the same as the drawing in the aerial site plan. |  |
| --- | --- | --- |

- Who is the client?

| Answer | H&M Smith |  |
| --- | --- | --- |

- Where is this project to be built?

| Answer | Lot 16, Glenvalley Road, Richmond, 3216 |  |
| --- | --- | --- |

- What are the contact details for the draftsman?

| Answer | Joe Foreman Ph: 03 9514 2311 Mobile: 0143 133 133 18 Fairview Road, Richmond, 3216 |  |
| --- | --- | --- |

- What sheet number is this drawing?

| Answer | 5 of 7 |  |
| --- | --- | --- |

- What timber is required for loading and non-load bearing walls?

| Answer | HJ 240 Joists |  |
| --- | --- | --- |

- What spacing is recommended for common studs?

| Answer | Max 600mm spacing |  |
| --- | --- | --- |

- What type of roof truss is to be installed?

| Answer | Gangnail Scissor trusses at 900mm spacing. |  |
| --- | --- | --- |

- What centres are the trusses positioned at?

| Answer | Top chord 30 deg bottom chord 20deg with every truss strapped to a wall plate |  |
| --- | --- | --- |

- What size are the veranda posts?

| Answer | 90 x 90 |  |
| --- | --- | --- |

- What timber species are the veranda posts to come from?

| Answer | DAR Oregon |  |
| --- | --- | --- |

- What pitch is the roof?

| Answer | 30 degrees |  |
| --- | --- | --- |

- What size are the noggings and at what spacing?

| Answer | 35 x 70 at max spacing of 1350mm |  |
| --- | --- | --- |

- What size are the ceiling joists?

| Answer | Intermediate at 450 mm spacing |  |
| --- | --- | --- |

- What timber is used for roof battens and what spacing?

| Answer | 35 x 70 FG Pine Battens |  |
| --- | --- | --- |

- What thickness is the plasterboard sheeting?

| Answer | 10 mm |  |
| --- | --- | --- |

- What type of flooring is to be installed?

| Answer | 15 mm Plywood Flooring |  |
| --- | --- | --- |

- What is the roof cladding material?

| Answer | Corrugated Colorbond roofing |  |
| --- | --- | --- |

- Where will you find information about the footings?

| Answer | By referencing the Engineers Drawings |  |
| --- | --- | --- |

- What do the letters ‘DAR’ refer when timber ordering?

| Answer | DAR means “dressed all round”, which means that it has been planed smooth on all surfaces. This is a machined finish with a finely bevelled appearance on the surface of the timber. |  |
| --- | --- | --- |

- Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | The 240 x63 LVL noted at the verandah is the horizontal joist for the Verandah. Laminated veneer lumber (LVL) is an engineered wood product that uses multiple layers of thin wood assembled with adhesives. It is typically used for headers, beams, rimboard, and edge-forming material. |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

- What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

- What is the Job Number?

| Answer | OOR-112 |  |
| --- | --- | --- |

- What is the scale of the drawing?

| Answer | 1:1000 |  |
| --- | --- | --- |

- What does the drawing show?

| Answer | North, South, East, West Elevations and a section of the proposed dwelling |  |
| --- | --- | --- |

- Which view shows the front of the house?

| Answer | Eastern Elevation |  |
| --- | --- | --- |

- How many motor vehicles can be parked in the garage?

| Answer | 2 |  |
| --- | --- | --- |

- What are the window and doors made from?

| Answer | Aluminium framed windows and timber doors |  |
| --- | --- | --- |

- What is the size of the eaves?

| Answer | 450mm |  |
| --- | --- | --- |

- What are the eaves lined with?

| Answer | Hardiflex |  |
| --- | --- | --- |

- What type of roof structure is the roof built from?

| Answer | Timber roof trusses |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the requirements for the roof structure in Q59.?

| Answer | NCC 2019 VOLUME 2 Part 3.5 |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | Australian Standard 1562.1-2018 has been revised and most of the new changes relates to material quality and fastening techniques |  |
| --- | --- | --- |

- What is the roof cladding materials made from?

| Answer | Concrete roof tiles |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Part 3.5.2 |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | AS 2050—2002 |  |
| --- | --- | --- |

- What is the roof pitch?

| Answer | 22.5 degrees |  |
| --- | --- | --- |

- Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | The Planning Act 2016 and the Building Act 1975 require a private certifier to lodge building work documentation with the relevant Local Council for approval |  |
| --- | --- | --- |

- Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | There are 6 stages of approval for the Brisbane City Council, the approval process takes up to approx 15-20 days for a standard application. The stages are: 1 Application – the application is reviewed then allocated to and assessment manager 2. Referrals - the relevant approvals and permits are sought by the applicant 3. Information request - At this stage the Council may request additional information if necessary 4. Public Notification – Some applications for development will need to be announced publically 5 Decision and Conditions – Council reviews all the documentation and makes a decision to either refuse or approve the Application 6. Appeals Process – Once the Councils decisions is made on the application, the applicant may appeal this decision as per the Planning Act 2016 |  |
| --- | --- | --- |

- What is a prime cost (PC) item? Give an example.

| Answer | Prime cost is an amount of money that is included in the sum of a contract to purchase items such as doors, tiles, fittings for bathrooms. This is an agreed amount estimated at the contract stage with items then being selected at a later stage. This allowance also needs to be reasonably estimated by the builder and they are estimated at or above the lowest amount that these items may cost |  |
| --- | --- | --- |

- What is a provisional sum (PS) item? Give an example.

| Answer | A provisional sum is an amount of money that is included in the contract sum. This allows for the costs of work and or materials that are not detailed when entering the contract, these are typically for site works including excavation etc. Builders need to ensure that these are reasonable however unforseen issues may arise and therefore the provisional sum allowed may exceed and these additional costs are then added. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

- True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify two (2) materials being used for the floors?

| Answer | Carpet and Floating floors |  |
| --- | --- | --- |

- True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- How much does the contracted plasterer cost?

| Answer | $12,900 |  |
| --- | --- | --- |

- How much do the kitchen appliances cost?

| Answer | $2700 |  |
| --- | --- | --- |

- What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

- What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

- How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

- If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $275 |  |
| --- | --- | --- |

- How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 |  |
| --- | --- | --- |

- What material has the highest total cost?

| Answer | Pine 90 x45 mgp10 ( long lengths) |  |
| --- | --- | --- |

- How many metres are required of the material in Q80. ?

| Answer | 322.5m |  |
| --- | --- | --- |

- What beam is identified as being a better option than F17?

| Answer | E14 |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

- Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter |  |
| --- | --- | --- |

- If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $214.80 |  |
| --- | --- | --- |

- Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar Bender and Fixer |  |
| --- | --- | --- |

- If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $1258.80 |  |
| --- | --- | --- |

- Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | National Construction Code - NCC Volume One Class 2 – 9: commercial, multi-residential, public and industrial buildings and structures - NCC Volume Two Class 1: residential buildings i.e. houses Class 10: non-habitable buildings and structures i.e. garages, sheds, carports, etc. - NCC Volume Three Plumbing and drainage |  |
| --- | --- | --- |

CPCCBC4012B

Read and interpret plans and specifications

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Christine Muter |
| --- | --- |
| Participant signature |  |
| Date | 18/08/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4012B

Read and interpret plans and specifications

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to read and interpret plans and specifications in order to inform estimation, planning and supervisory activities. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - Any drawing with a cloud or mark-up you have inserted. - Copy of the correspondence regarding mark up and any site management/authorising engineer response. The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Drawings (including site plan), Specifications, Tender documentation, Telephone, Facsimile machine, Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Christine Muter |  |  |
| Name of observer: | John Gammon |  |  |
| Role of observer (Confirm suitability to sign) | Construction Manager |  |  |
| Email address of observer | john.gammon@tbs.com.au |  |  |
| Signature of observer: |  |  |  |
| Date: | 20/08/2021 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| 1 Interpret drawings |  |  |  |
| - Identify purpose and advantage of different types of drawings. |  |  |  |
| - Identify different aspects of drawings. |  |  |  |
| - Identify, understand and apply: - Commonly used symbols and abbreviations - Common building and construction terms on drawings |  |  |  |
| - Identify building site from location drawings. |  |  |  |
| - Identify on the site plan: - True north - Building orientation - Key features in relation to building and other structures - Levels - Contours - Service entry points - Site features to be removed or retained |  |  |  |
| - Identify key features of plans, elevations and sections. |  |  |  |
| - Identify client requested variations to standard plans on drawings. |  |  |  |
| 2 Interpret specifications |  |  |  |
| - Identify and correctly apply Provisional Sum (PS) and Prime Cost (PC) values. |  |  |  |
| - Identify and confirm customer variations to standard specifications and communicate changes to organisational personnel. |  |  |  |
| - Apply and communicate correct interpretations of essential elements to estimation, planning and supervisory tasks (including labour hours/costs and material quantities/costs). |  |  |  |
| - Identify Building Codes or standards affecting the work to be undertaken (including references to Australian Standards and the National Construction Code). |  |  |  |
| - Identify key features of products included in the specification, including the design, purpose, aesthetics and cost relationships. |  |  |  |
| 3 Communicate with others |  |  |  |
| - Consult with industry professionals, clients, organisational personnel and appropriate local authorities. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Using technology (including telephone, facsimile, email) |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments

As Townsend Building Services is a company that facilitates the rectification and repairs to property as a result of claimable events such as natural disasters, loss of water, accidental damages etc, it is rare that we are required to follow or draft a set of plans to follow in order to facilitate the completion of the repairs. Although we don’t have any current major loss claims that require the drafting of plans for Christine t be able to review within the workplace, Christine has presented me with the plans that were supplied as part of the training module and has reviewed additional examples available on the internet and has been either able to identify the various elements of the site and proposed buildings on the plans, or has been able to readily source this information to identify them. Christine displays a strong understanding in the difference between PS and PC items as these form part of our contracts and scope of works when preparing these documents and how they impact the final costs of the contract. Christine’s role requires the monitoring of each stage for Building approval and the certifications required at the completion of works to enable the required Form 21 under Legislation, she demonstrates competency throughout these processes.