# JDL CONSTRUCTION PL PTY LTD

# ABN: 38 162 454 346

# Work Health and Safety (WHS) Management Plan

| Client name | Address | Contact number | Position | WHS responsibilities |
| --- | --- | --- | --- | --- |
| Transport and Main Roads | Shop A, 429 Old Cleveland Road, Cooporoo | 30668516 | Builder | John Lewis |

| Description of project: | Ceiling Repairs, aircon repairs |
| --- | --- |
| Location of project: | Shop A 429 Old Cleveland road, Cooporoo |
| Start and finish dates: | 21st September 2016 to 24th September 2016 |

Emergency Evacuation Plan

Foot path to the front of shop on Old Cleveland Road

- Contents

Contents	1

1	Project information	3

1.1	Management and review	3

1.2	Principal contractor details	3

1.3	Details of persons at workplace with WHS responsibilities	3

2	Roles and responsibilities	4

2.1	Principal contractor	4

2.2	Contractors	4

2.3	Workers	5

3	General WHS information	6

3.1	Legislation	6

3.2	Codes of Practice and other guidance	6

3.3	WHS policy	7

3.4	Insurances	8

4	Risk management	9

4.1	Identifying hazards and managing risks	9

4.2	Hierarchy of control	9

5	High risk construction work	11

5.1	High risk construction work	11

5.2	Licences for high risk work	11

5.3	Asbestos	12

6	Emergency and incident response	13

6.1	Emergency preparedness	13

Emergency procedure	13

Emergency meeting point	13

Emergency contact list for the site	13

6.2	Incident procedure	13

6.3	Notifiable incidents	13

6.4	First aid	14

7	Induction and training	16

7.1	Worker induction	16

7.2	Worker training	16

8	Consultation and communication	17

8.1	Consultation	17

8.2	Communication	17

8.3	Disciplinary procedures	17

9	Site safety procedures	18

9.1	Site rules	18

9.2	Site amenities	18

9.3	Site security	18

9.4	Site signage	18

9.5	Personal protective equipment	18

9.6	Managing construction hazards specified in the Regulations	19

Falls from heights	19

Falling objects	19

Demolition work	20

Excavation work/trenching	20

Work near overhead or underground essential services	20

Electrical	21

Plant	22

Scaffolds	22

9.7	Managing other construction hazards	23

Ladder safety	23

Manual handling	23

Slips, trips and falls	23

Hand operated and power tool use	23

Sun safety	24

10	Safe Work Method Statements (SWMS)	25

Project information

Management and review

This WHS Management Plan has been developed to outline our approach to managing work health and safety (WHS).

We must make this plan available to all workers and contractors on this project and ensure they have the opportunity to read, understand, clarify and ask questions

We must keep a copy of the WHS Management Plan readily available for the duration of the project

We must review the plan regularly throughout this project and make any revisions known to those working on the project

Principal contractor details

| Business name: | JDL Construction PL Pty Ltd |
| --- | --- |
| Address: | 63 Amelia Street Nundah QLD 4012 |
| Contact person: | John Lewis |
| Work phone: | 0403 548 540 |
| Mobile phone: |  |
| Fax: |  |
| Email: | info@jdlconstruction.com.au |
| ABN: | 38 162 454 346 |
| Contract licence number: | QBCC Lic 1235961 |
| Principal contractor signature: |  |

Details of persons at workplace with WHS responsibilities

| Name | Position | WHS responsibilities |
| --- | --- | --- |
| John Lewis | Builder | Builder |
| Tom Wheeler | Builder |  |
| Craig Chambers | Carpenter |  |
| Kieran Lewis | Apprentice |  |
| Scott Whelan | Apprentice |  |

Roles and responsibilities

Principal contractor

The principal contractor of this project is responsible for:

preparing, updating and implementing this WHS Management Plan, including all associated procedures

identifying and observing all legal WHS requirements

ensuring that all works are conducted in a manner without risk to workers

planning to do all work safely

participating in the planning and design stages of trade activities

identifying WHS training required for an activity

ensuring workers undertake identified WHS training

communicating and consulting with workers

investigating hazard reports and ensuring that corrective actions are undertaken

assisting in rehabilitation and return to work initiatives

dispute resolution

Contractors

Contractors who are engaged for this project are responsible for:

fulfilling the duties of PCBU for their own operations

identifying all high risk construction work associated with their activities and ensuring safe work method statements are developed and implemented

complying with the duties as listed under ’Workers’ (see 2.3)

following all safety policies and procedures and site rules

complying with this WHS Management Plan

complying with any direction given to them by the principal contactor

undertaking site-specific induction before starting work and signing off that they have completed this induction

ensuring the workers they engage undertake site specific

ensuring they have the correct tools and equipment and these are in a serviceable condition for the task

Workers

All workers on this project (including those employed by contractors) are responsible for:

taking reasonable care of their own health and safety

taking reasonable care that their conduct does not adversely affect others

complying with instruction, so far as they are reasonably able

cooperating with reasonable notified policies or procedures

General WHS information

Legislation

| Relevant legislation | Tick if applicable |
| --- | --- |
| Queensland Work Health and Safety Act 2011 |  |
| Queensland Work Health and Safety Regulations 2011 |  |
| AS3012:2010 – Electrical installations – construction and demolition sites |  |

Codes of Practice and other guidance

| Relevant Codes of Practice | Tick if applicable |
| --- | --- |
| Confined spaces |  |
| Construction work |  |
| Cranes |  |
| Demolition work |  |
| Excavation work |  |
| Housing construction work |  |
| How to safely manage and control asbestos in the workplace |  |
| How to safely remove asbestos |  |
| Managing electrical risks at the workplace |  |
| Managing noise and preventing hearing loss at work |  |
| Managing risks of plant in the workplace |  |
| Managing the risks of falls in the workplace |  |
| Managing the work environment and facilities |  |
| Preventing falls in housing construction |  |
| Scaffolding |  |
| Work health and safety consultation, cooperation and coordination |  |
| Working in the vicinity of overhead and underground electrical lines |  |

WHS policy

Insurances

| Insurance type | Company | Policy number | Expiry date |
| --- | --- | --- | --- |
| Public Liability Insurance | QBE | 02A933551BPK | 08/04/2016 |
| Worker Compensation Insurance | Work Cover | WAB130416485 | 30/06/2016 |

Risk management

Identifying hazards and managing risks

We will systematically identify hazards and assess risks before the project starts by using the hierarchy of control (see 4.2) in conjunction with:

developing Safe Work Method Statements (SWMS) to control risks associated with high risk construction work

using a risk management form to control general construction risks where necessary

We will also identify risks:

before we buy or re-order any chemicals

when introducing a new task

when new information is received about tasks, procedures, equipment or chemicals.

All hazards that are identified throughout the project must be reported immediately to the principal contractor.

We will inform our workers of our risk management procedures and ensure they are trained in risk management (see 7).

Hierarchy of control

We will control all risks we identify by applying the Hierarchy of Controls as follows:

Eliminate

Substitute

Isolate

Engineering controls

Administrative controls

Personal Protective Equipment.

Where possible, we will implement risk controls that are high in the order and will implement multiple controls where necessary.

High risk construction work

High risk construction work

We have identified the following high-risk activities for this project. A Safe Work Method Statement (SWMS) has been developed for each of the high-risk construction work activities. We will also develop SWMSs for any additional high-risk work that is introduced or identified during the project.

| High risk construction work activity | Safe Work Method Statement developed and attached Yes/No |
| --- | --- |
| General Earthworks | No |
| Work Near or On a Road | No |
| Manual Handling | Yes |
| Excavation & Trenching | No |
| Working Near Service (Overhead & Underground) | No |
| Electrical Equipment | Yes |
| Use of Quick Cut and Power Saw | No |
| Formwork & Concrete Placement | No |
| Asbestos Removal | No |
| Demolition | No |
| Working at 2m or more fall height | Yes |

We will collect and file completed SWMS in Section 10, which forms part of this WHS Management Plan.

We will review the SWMS where:

- there is a need to change the method of carrying out of the high risk construction work

- a risk has been identified that is not included and managed within a SWMS.

Licences for high risk work

We require workers to be licenced to undertake high-risk work. Our register of licence holders is below:

| Licence holder name | Type of licence | Expiry date |
| --- | --- | --- |
| N/A |  |  |

Asbestos

The person conduction business or undertaking will ensure:

all workers understand our procedures for asbestos and follow the correct removal processes

all workers are trained and use the appropriate personal protective equipment

only licenced asbestos removalists are used to remove asbestos where the quantity to be remove exceeds the 10 square metre limit or is friable

the correct signage and controls are in place before any removal of asbestos commences

the asbestos is wrapped and disposed of correctly.

Emergency and incident response

Emergency preparedness

To ensure we are prepared for an emergency we:

Show all workers and subcontractors the emergency point as part of their induction (this is included in our induction checklist)

Display emergency procedures in the site office or other visible location

Check and mark fire extinguishers at the beginning of the project and six-monthly after that

Emergency procedure

In the event of a fire or similar emergency evacuation:

Stop work immediately and vacate the workplace

Assist anyone in the workplace who may not be familiar with the evacuation procedures

Call emergency services on 000 or on 112 from a mobile phone. Other emergency numbers are on display in the site office (if applicable)

Notify the principal contractor

Assemble in the nominated assembly points until you receive further instructions from the principal contractor or emergency services personnel

Emergency meeting point

Our emergency meeting point is at JDL Construction Job Truck.

Emergency contact list for the site

Our emergency contact list is provided overleaf.

Incident procedure

If an incident occurs at the workplace the procedure is:

Immediately notify the principal contractor

do not interfere with the scene of the incident

Depending on the nature and severity of the injury, the principal contractor will notify Workplace Standards (see 6.3).

The person conduction business or undertaking may record details of the incident and will ensure any remedial action is taken.

Notifiable incidents

We will report the following incidents to Workplace Standards:

A fatality

An incident requiring hospitalisation

A dangerous incident, which could have resulted in someone being killed, or suffering a serious bodily injury (see 6.3 of Part A for definition).

In the event of such an occurrence:

Notify the principal contractor who must notify Workplace Standards by the quickest means possible. The number for Workplace Standards is 1300 366 322 – this number is on the emergency contact list

Fax an Incident Notification Form to Workplace Standards as soon as possible following the incident (must be within 48 hours)

Do not disturb the site until given clearance by the principal contractor who will take advice from Workplace Standards

The principal contractor will confirm the reporting requirements required by Workplace Standards and Tasmania Police

The principal contractor shall only give permission to disturb the site when notified by Workplace Standards that a formal investigation is not required

If a formal investigation is required, the principal contractor will secure the site

First aid

We will supply adequate first aid equipment, which will be available JDL Construction Job Truck

If anyone becomes aware that an item of first aid is out of stock or out of date, they are to notify the principal contractor immediately

In the event of a person being injured, trained first aid personnel should:

Stabilise the person and administer first aid

Phone an ambulance (depending on the extent of the injuries)

If emergency services are called, notify the principal contractor immediately. In all other circumstances notify the principal contractor as soon as practicable.

EMERGENCY CONTACT NUMBERS

AMBULANCE POLICE FIRE SERVICE 000 or 112 (mobile) (BOTH NUMBERS ARE ACCESSIBLE WHILE MOBILE KEY PADS ARE LOCKED)

EMERGENCY CENTRE Name: Coorporoo Medical Center Address:, 358 Old Cleveland road, Coorporoo Phone: tel: 38472377 Operating hours: 8am – 6pm

LOCAL INFORMATION Police Station: 131 444 Poisons Information Centre: 13 11 26 Telstra: Local Council: (07) 3403 8888 Electrical Emergency: Dial before you dig: 1100 Gas Emergency: 1800 808 526 Water Emergency: 13 23 64 Workplace Standards: 1300 366 322 Professional Association: Union:

INTERNAL INFORMATION Principal contractor: JDL Construction PL Pty Ltd Contact details: 0403 548 540 Site supervisor: Contact details:

Induction and training

Worker induction

The person conduction business or undertaking will work with other contractors to ensure a site specific induction is provided for all workers before starting work.

This induction must outline:

The expectations outlined in this WHS Management Plan, including all policies and procedures

The emergency meeting point

The site rules

The facilities

Any site specific hazards

High risk construction work activities

Worker training

The person conduction business or undertaking will:

Ensure workers are trained and competent for the work to be carried out

Ensure workers are trained to deal with any risks associated with the work and understand the control measures in place

Ensure all workers have had relevant white card training (or other appropriate training from another jurisdiction)

Ensure on-site training and supervision is provided

Organise external training for specific tasks where required

Seek high risk licences for all high risk work and maintain a register of licences

Communicate with other contractors to ensure their workers are appropriately trained and competent.

Consultation and communication

Consultation

We will consult with all workers and contractors on WHS issues for this project:

At toolbox meetings where anyone can raise issues for discussion

Informally during the planning of activities or the development of Safe Work Method Statements

When changes to workplace arrangements could affect the health and safety of workers

During investigations into any incident to establish details of the incident or to formulate corrective action to prevent the incident re-occurring

We will also consult with contractors and suppliers on WHS issues associated with any products or services provided for the contract:

During the negotiation phase before agreeing on the work requirements

Before starting any contractor operations

When any changes to workplace arrangements occur that could affect the health and safety of the contractors or affect their work procedures

Communication

We will ensure our workers and other contractors are aware of WHS requirements by providing them with this WHS Management Plan before starting work on the project. Contractors are expected to make their workers aware of all WHS requirements.

We will communicate relevant WHS information to everyone involved in this project by:

Induction

Pre-work meetings

Toolbox meetings

Incident reports and outcomes

Distributing safety alerts or guidance material about industry specific hazards/incidents

Disciplinary procedures

If anyone does not comply with the requirements of this Plan, the following will apply:

First violation: verbal warning (and advise contractor if it involves their worker/s)

Second violation: written notification (and advise contractor if it involves their worker/s)

Third violation: complete removal/suspension from the project.

For a serious breach of safety, workers can be immediately dismissed or removed from the site without notice.

Site safety procedures

Site rules

- All workers to wear High-Vis Clothing as Required

- All workers to use correct PPE for tasks including, ear protection and eye protection

- All workers to wear steel cap boots on site

Site amenities

Toilets and drinking water will be provided on site.

All workers are to have good hygiene standards and clean up after themselves.

Site security

The person conducting business or undertaking, will so far as reasonably practicable, secure the site by:

Keeping the building secure during the project

Erecting a fence to prevent unauthorised access

Locking gates to the site outside normal hours of operation

Workers and contractors are expected to keep the site secure, for example by closing or locking gates.

Site signage

At a minimum, we must display the following signs on the entrance to the site:

The person conducting business or undertaking’s name, contact details and after-hours telephone number

The location of the site office.

All signage will be clearly visible from outside the workplace.

Personal protective equipment

We will provide the personal protective equipment (PPE) to workers at the workplace, unless the PPE has been provided by another contractor.

The person providing the PPE must ensure that the PPE is:

Suitable for the nature of the work and any hazard associated with the work

A suitable size and fit and reasonably comfortable for the worker who is to use or wear it

Maintained, repaired or replaced so that it continues to minimise risk to the worker who uses it, including by:

Ensuring it is clean and hygienic

Ensuring it is in good working order

Ensuring it is used or worn by the worker, so far as is reasonably practicable.

The person supplying the PPE must also:

Provide workers with information, training and instruction in the proper use, wearing, storage and maintenance of PPE

Ensure that any other person at the workplace (such as home owners, clients or inspectors) is appropriately provided with PPE to wear as required.

Workers must:

Follow all instructions to wear and use PPE

Take reasonable care of PPE

Managing construction hazards specified in the Regulations

Falls from heights

We will manage the risks associated with falls from heights by:

Ensuring that where practicable, any work involving the risk of a fall is undertaken on the ground or on a solid construction (such as an elevated work platform)

Where this is not practicable, providing a fall prevention device such as secure fencing, edge protection, working platforms and/or covers

Where this is not practicable, providing a work positioning system such as plant or a structure (other than a temporary work platform) that enables a person to be positioned and safely supported

Where this is not practicable, providing a fall arrest system such as a safety harness system. Workers will be trained in emergency procedures for fall arrest systems

When undertaking work involving the risk of a fall from height, workers must:

Follow all instructions

Work with a buddy when using a ladder

Only use approved work platforms

#### Falling objects

Where practical, we will provide adequate protection against the risk of falling objects through the use of control measures such as barrier screen, toe-boards and by storing and stacking materials safely.

Where this is not possible, a risk assessment must be undertaken and appropriate control measures implemented to manage the risk of injuries from falling objects.

Demolition work

We expect to undertake demolition work for this project. We will submit a demolition work notification form to Workplace Standards to meet our requirement to advise Workplace Standards at least five days before the project starts.

Excavation work/trenching

Anyone undertaking excavation work must not start work unless they have:

Found out about any underground services that may be affected by their works, before starting work

Implemented control measures to avoid direct or inadvertent contact with underground services

Pot-hole dug (by hand) to expose existing services before any mechanical excavation near the services

Any issues must be reported to the principal contractor.

Safe Work Method Statements (SWMS) are included in this WHS plan for trenches of at least 1.5 metres. Workers must be familiar with and implement the control measures in the SWMS.

Work near overhead or underground essential services

We will ensure, where reasonably practical, that that no-one comes within an unsafe distance of an overhead or underground power line.

If maintaining a safe distance is not reasonably practical, we will:

Assess the risk associated with the proposed work

Implement control measures consistent with the risk assessment

Contact and consult with the local essential service provided.

##### For work near overhead power lines up to and including 132,000kV:

work is not permitted within 3 metres of overhead power lines

the principal contractor (or contractor in charge of the work) must have written authority from the electrical supply authority to work within the “no go” (exclusion) zone

if using plant or equipment within 3 to 6.4 metres of overhead power lines ensure you have a safety observer.

##### For work near overhead power lines of greater than 133kV:

work is not permitted within 8 metres of overhead power lines

the principal contractor (or contractor in charge of the work) must have written authority from the electrical supply authority to work within the “no go” (exclusion) zone

if using plant or equipment within 8 to 10 metres of overhead power lines ensure you have a safety observer.

##### For excavation work near underground essential services:

take all reasonable steps to obtain current underground essential services information before directing or allowing the excavation work to start

provide this information to any person engaged to carry out the excavation work

consider this information when carrying out, directing, or allowing the carrying out of the excavation work

ensure this information is available for inspection.

Electrical

Power supplied to the site must only come from:

an electricity distributers main

an existing switchboard permanently installed at the premises

a compliant low voltage generator

a compliant inverter.

Switchboards and distribution boards used on site must:

be of robust construction and materials capable of withstanding damage from the weather and other environmental and site influences (IP23 minimum rating)

be securely attached to a post, pole, wall or other structure unless it is of a stable freestanding design able to withstand external forces likely to be present

incorporate suitable support and protection for flexible cords and cables and prevent mechanical strain to the cable connections inside the board

protect all live parts at all times

be individually distinguished by numbers, letters or a combination of both (where multiple boards are present).

Flexible cords used on construction sites must be rated heavy duty.

To avoid confusion with individual earthing conductors, green sheathed flexible power cords must not be used on site.

Flexible cords must be either protected by a suitable enclosure or barrier (flexible or rigid conduit) or located where they are not subjected to mechanical damage, damage by liquids or high temperature (elevated on stands or hung from nonconductive support brackets).

We will ensure our cords do not exceed the maximum length as stated in Table 1 of AS3012 below:

Rated current		          Conductor size	                Maximum length in metres

| 10amp | 1.5mm 2.5mm 4.0mm | 35 60 100 |
| --- | --- | --- |
| 15/16 amp | 1.5m 2.5m 4.0mm | 25 40 65 |
| 20 amp | 2.5mm 4.0m 6.0mm | 30 50 75 |

We will maintain an in-service inspection and test regime for all portable electrical leads, tools and earth leakage devices.

We will ensure that after the equipment has been inspected and tested, it will be fitted with a durable, non-reusable, non-metallic tag. The tag will include the name of the person or company who performed the test and the test and re-test date.

Records of all inspections, tests, repairs and faults related to all electrical equipment will be recorded in a testing and tagging register.

RCDs and portable equipment must be inspected, tested and tagged every 3 months.

Workers must conduct an RCD push button test after connection to a socket and before connection to equipment at least once a day.

Workers must report any damaged electrical equipment to the principal contractor. It will be removed from service and either repaired or replaced and subsequently inspected and tested as required.

New electrical equipment must be recorded in the register and subjected to the in-service testing regime within the first 3 months of service.

Plant

To ensure all plant used complies with the requirements of the WHS Regulations:

only use plant for the purpose for which it was designed

use all health and safety features and warning devices on plant

follow all information, training and instruction provided

guarding must be permanently fixed and is not permitted to be removed

no person other than the operator may ride on the plant unless the person is provided with a level of protection that is equivalent to that provided to the operator

We will ensure that:

all plant is regularly maintained, inspected and tested by a relevant competent person

the plant has a warning device that will warn persons who may be at risk from the movement of the plant

all plant that lifts or suspends loads is specifically designed to lift or suspend that load.

Scaffolds

We will ensure:

that the scaffold is erected by a competent person (having regard for high risk licence for above 4 metres)

that before we use the scaffold, the competent person has advised (in writing) that it is safe

that scaffolding is inspected by a competent person:

before use of the scaffold is resumed after an incident occurs that may reasonably be expected to affect the stability of the scaffold

before use of the scaffold is resumed after repairs

at least every 30 days.

that, if an inspection indicates that any scaffold or its supporting structure creates a risk to health or safety:

any necessary repairs, alterations and additions will be made or carried out

the scaffold and its supporting structure will be inspected again by a competent person before use of the scaffold is resumed.

Workers must:

not use incomplete scaffolding

report any scaffolding issues to the principal contractor

comply with the directions of any tags attached to the scaffold

We will prevent unauthorised access to the scaffold by:

removing ladders where there is no site fencing

Managing other construction hazards

Ladder safety

We will manage hazards associated with ladders by:

using ladders according to the manufacturer’s instructions

only allowing one person at a time on a ladder

performing all work from a ladder while facing the ladder

not setting up ladders on scaffolds or elevated work platforms to gain extra height

Manual handling

We will manage hazards associated with manual handling by:

ensuring all users follow good manual handling practices

assessing risk assessments

providing mechanical lifting aids where applicable

Slips, trips and falls

We will manage hazards associated with slips, trips and falls by:

using a slips, trips and falls checklist as required

checking for hazards that could cause someone to slip, trip or fall by doing a visual check

ensuring workers keep the site tidy as part of the written site rules

Hand operated and power tool use

We will manage hazards of hand operated and power tool use by:

regularly checking all tools to ensure they are in a safe working order

recording all electrical tools in a tag and testing register

testing and tagging electrical tools every 3 months

communicating any issues identified with power tools to workers through a toolbox meeting.

Before using power tools, workers must ensure:

electrical connections are secure

electricity supply is through an RCD

safety guards are in position

the machine is switched off before activating the electricity supply

appropriate PPE is used as required by manufacturer’s guidelines or as guided by the principal contractor

Workers must report any issues with power tools to the principal contractor. Unsafe tools will be tagged and removed from service

Sun safety

All persons on site should:

wear adequate clothing (eg hats) and other protection methods (eg sunscreen) to protect themselves from the effects of working while exposed to UV rays.

manage working in the sun to avoid dehydration and heat stress related illnesses

Safe Work Method Statements (SWMS)

This section of our plan includes our completed Safe Work Method Statements for all high-risk construction work.

List of Safe Work Method Statements as follows:

JDLC-SWMS-01 General Earthworks

JDLC-SWMS-02 Work on or near a road

JDLC-SWMS-03 Manual Handling

JDLC-SWMS-04 Excavation and Trenching

JDLC-SWMS-05 Working near services (overhead and underground)

JDLC-SWMS-06 Electrical Equipment

JDLC-SWMS-07 Use of Quick Cut Saw, Demo Saw, and Power Saw

JDLC-SWMS-08 Formwork and Concrete Placement

JDLC-SWMS-09 Asbestos Removal

JDLC-SWMS-10 Demolition

JDLC-SWMS-11 Working at 2m or more fall height