RIICWM401E

Supervise civil works

ASSESSMENT SUMMARY

The assessment activities for RIICWM401E Supervise civil works cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Benjamin Wixon |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for RIICWM401E | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Theory Exam | Practical Assessment Completed Job Plan |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |

RIICWM401E

Supervise civil works

THEORY EXAM

| Participant name | Benjamin Wixon |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within two (2) hours, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

- Which on-going risk management procedures and documentation do you access when supervising civil work and how do these link to legislative requirements?

| Answer | - SWMS – WHS act , WHS regs - JSA – WHS regs - EMP – Enviro protection Act - SDS – Haz chems regulation - TCP – MUTCD – AS 1742 - Emergency procedures – WHS regs |  |
| --- | --- | --- |

- What important emergency response information must be shared with workers prior to work beginning?

| Answer | • Site-specific emergency procedures (evacuation routes, muster points, first aid contacts) • Emergency contact details for supervisors and first aiders • UHF radio protocols and correct frequency for emergency use • Location of emergency equipment (fire extinguishers, spill kits, first aid kits) • Site access and muster point locations • Hazardous materials present and where to access Safety Data Sheets (SDS) • Incident reporting procedures (how to report accidents, near misses, hazards) |  |
| --- | --- | --- |

- What are (1) the objectives of the Environmental Management Plan and (2) why you should read this as part of planning the job.

| Answer 1. 2. | The EMP outlines the environmental management policies and procedures for the site. Its main objectives are to ensure compliance with environmental legislation and to manage risks related to waterway protection, noise, vibration, dust, lighting, waste disposal, and hazardous chemicals. Reading the EMP during planning helps confirm that environmental protection requirements have been met. It ensures that site activities won’t breach legal obligations and allows you to identify any environmental risks earl |  |
| --- | --- | --- |

- What is (1) the ‘scope’ of work, (2) why is this important when developing a job plan and (3) where can you sources relevant information.

| Answer 1. 2. 3. | The scope refers to the parts or components of a project that a person is responsible for at their level. For example, a leading hand’s scope differs from a foreman’s or a supervisor’s. It defines the extent of responsibility and the tasks to be managed within the overall project. Understanding the scope is essential because it clarifies what needs to be done, who is responsible, and how resources should be allocated. It helps ensure accurate planning, sequencing, compliance, and coordination of tasks. Relevant scope information can be sourced from a range of project materials and communications. These include project plans, technical drawings, and specifications that outline the physical and procedural requirements of the job. Pre-start meetings and supervisor briefings provide verbal clarification and updates, while site inspection notes offer real-time observations that may affect planning. Organisational policies and procedures also help define boundaries, responsibilities, and compliance requirements. |  |
| --- | --- | --- |

- In the general principles of surveying, what is the importance of ‘working from the whole to the part’ (1) when surveying and (2) how this is achieved?

| Answer 1. 2. | This principle ensures errors are localized and controlled, rather than magnified across the entire survey. By starting with a broad, high-precision framework and then filling in details, surveyors avoid distortions that can occur when small errors in local measurements accumulate. Surveyors begin by establishing primary control points across the entire site using high-precision instruments. These points form the backbone of the survey. From there, they create secondary and tertiary control networks with decreasing levels of precision to fill in gaps and guide detailed measurements. |  |
| --- | --- | --- |

- Why can consulting the team enhance team practices? Include one (1) example.

| Answer | Consulting the team enhances team practices by fostering shared ownership, practical input, and stronger communication. When team members are involved in planning and problem-solving, they’re more likely to raise concerns early, suggest workable alternatives, and stay engaged with the outcome. During a few days of unexpected rain delays, the team was consulted to reassess the work sequence. Instead of pausing operations entirely, they proposed shifting resources stockpiling materials close to the work front to minimise the delay when back up and running/ |  |
| --- | --- | --- |

- How do you communicate site safety requirements to ensure workers wear PPE suited to the site?

| Answer | - Site inductions outlining mandatory PPE based on site-specific hazards - Toolbox talks reinforcing PPE requirements and addressing changes - Clear signage at entry points and high-risk zones - PPE details included in SWMS and task-specific work instructions - Supervisor checks and peer reminders to maintain compliance - Training and fit testing for specialised PPE where require |  |
| --- | --- | --- |

- Why is it important to coordinate tasks with other work supervisors and schedule team activities?

| Answer | Coordinating tasks with other work supervisors and scheduling team activities is vital to keeping a civil site efficient and safe. It ensures crews—whether handling earthworks, structures, or traffic—work in sync rather than clashing or causing delays. Without coordination, even well-planned tasks can fall apart due to access issues, resource bottlenecks, or safety risks. |  |
| --- | --- | --- |

- What inclusions must be considered when preparing a job plan?

| Answer | - Scope of work - Resources required - Sequence of activities - Timeframes - Safety requirements - Environmental considerations - Quality requirements - Coordination needs - Communication methods - Contingencies |  |
| --- | --- | --- |

- What processes must you follow to acquire and confirm the acquisition of required resources at your workplace?

| Answer | - Identify resource needs - Check existing stock or availability - Submit requisitions or purchase orders - Confirm approvals - Coordinate delivery and access - Inspect and verify on arrival |  |
| --- | --- | --- |

- What is discussed during the daily pre-start meeting?

| Answer | Our daily prestart meetings usually cover relevant safety messages from the previous shift, feedback about what went well, what didnt go well and how it can be improved. Then it goes into detail about the more specific works being undertaken for the day with any possible interaction issues with other crews. Then finishes off with a note on what deliveries we are expecting for the day and stretches. |  |
| --- | --- | --- |

- What is the ‘rule of thumb’ basis for matching a machine to a job?

| Answer | Use the smallest, most efficient machine that can safely and effectively complete the task. |  |
| --- | --- | --- |

- Supervisors must have knowledge of factors affecting plant performance. What are four (4) factors that can affect plant performance and output and why must supervisors monitor these?

| Answer 1. 2. 3. 4. | • Operator skill and experience • Ground conditions and terrain • Machine suitability and maintenance status • Weather and environmental impacts Supervisors monitor these factors to keep machines running safely and efficiently. It helps prevent breakdowns, delays, and safety risks, while ensuring the right machine is used under the right conditions. |  |
| --- | --- | --- |

- What methods do you use to ensure team members adhere to safety requirements such as PPE during the work tasks?

| Answer | - Site inductions outlining PPE requirements - Toolbox talks reinforcing task-specific PPE use - Clear PPE signage - Supervisor checks and visual monitoring during shifts - Peer accountability among crew members - Immediate correction of non-compliance - Positive reinforcement for consistent safe behaviour |  |
| --- | --- | --- |

- What is the supervisor’s role during site set out?

| Answer | During site set out, the supervisor’s role is to verify survey data, confirm alignment with approved plans, and coordinate with surveyors and other crews to ensure access and timing. They check that the area is safe and ready, communicate set-out details to workers, and monitor progress to resolve any issues. Accurate set-out ensures construction starts in the correct location, with the right dimensions and orientation. |  |
| --- | --- | --- |

- What are two (2) things you would tell the workers when explaining what the pegs mean during road set out? (30 words)

| Answer | I would tell workers that white pegs mark the road centreline for alignment, and yellow offset pegs show reference points used to measure positions without disturbing the main set-out marks. |  |
| --- | --- | --- |

- What methods do you apply when monitoring civil works to ensure the required outcomes or met on a daily basis?

| Answer | • Daily pre-start checks • Visual inspections of work areas and plant • Cross-checking against drawings and specifications • Tracking productivity against targets • Monitoring safety controls • Communicating with crews and supervisors • Documenting progress • Adjusting plans as needed |  |
| --- | --- | --- |

- Name and explain the purpose for three (3) of the different test results the supervisor must be able to read and understand at different hold or witness points of a project?

| Answer 1. 2. 3. | Compaction Test Results Purpose: To confirm that soil or backfill has been compacted to the required density, ensuring stability and preventing future settlement. This is often a hold point before placing concrete or asphalt. Concrete Test Results Purpose: To verify that concrete meets design specifications for workability and strength. Slump tests check consistency during placement, while compressive strength tests confirm structural integrity before formwork removal or load application. Asphalt Test Results Purpose: To ensure the asphalt mix meets quality standards for durability, flexibility, and compaction. These results are reviewed before sealing or opening the road to traffic. |  |
| --- | --- | --- |

- What is the purpose of road surface drainage and what are four (4) elements used to achieve this effectively?

| Answer Purpose Element 1. 2. 3. 4. | The main purpose of road surface drainage is to remove water efficiently from the pavement and surrounding areas. - Crossfall - Longitudinal slope - Kerbs and gutters - Catch basins and drainage inlets |  |
| --- | --- | --- |

- What are the effects of inappropriate road drainage infrastructure?

| Answer | - Water pooling and flooding on the road surface - Reduced pavement lifespan due to moisture infiltration - Formation of potholes, cracks, and rutting - Stripping of bitumen from aggregates in asphalt - Erosion of embankments and cuttings - Mud pumping and corrugation in flexible pavements - Environmental damage from polluted runoff |  |
| --- | --- | --- |

- What are the three (3) main parts of roadway design and the importance of these features?

| Answer 1. 2. 3. | - Cross Section Design Includes lanes, shoulders, medians, kerbs, and drainage. It defines how the road handles traffic, runoff, and safety barriers. A well-designed cross section ensures safe vehicle movement and effective water shedding. - Horizontal Alignment Refers to the road’s path in plan view — straight sections (tangents) and curves. It affects visibility, driver comfort, and safety. Proper alignment reduces crash risk and improves traffic flow. - Vertical Alignment Covers grades and vertical curves — the road’s slope and elevation changes. It influences drainage, fuel efficiency, and braking performance. Good vertical alignment ensures safe climbs, descents, and sight distances. |  |
| --- | --- | --- |

- How can you support sub-contractors to achieve their responsibilities?

| Answer | • Ensure they understand and comply with site safety procedures and rules • Communicate work standards, quality expectations, and scheduling requirements clearly • Coordinate their activities with other crews to avoid clashes and delays • Approve any work variations in writing before they begin • Remove any subcontractor personnel who are negligent, incompetent, or pose safety risks • Check that supplied machinery is safe and in good working order |  |
| --- | --- | --- |

- How do supervisors determine human resources and sub-contractors needed for the job plan?

| Answer | - Review job documents including plans, specifications, and scope of works - Break the job into major activities using a Work Breakdown Structure - Identify labour, plant, equipment, and material needs for each activity - Use a capability matrix to match workers to tasks based on skill level - Confirm availability and approval of staff and subcontractors - Check licences, tickets, and insurance coverage - Factor in risks like leave, weather, and supply delays - Consult with project managers or engineers to finalise resource allocation |  |
| --- | --- | --- |

- How do you, as the supervisor, (1) determine and (2) share the communication systems to be used during site tasks with others? Include in your response (3) the communication systems generally used.

| Answer 1. 2. 3. | Determine communication systems: • Review site procedures and task complexity • Consider crew size, risk level, and WHS needs • Identify plant, traffic, and subcontractor comms • Ensure systems suit site conditions and resources Share communication systems with others: • Cover systems in pre-starts and toolbox talks • Include in SWMS and site inductions • Provide equipment and confirm correct use • Reinforce through signage and supervisor direction Communication systems generally used: • UHF radios on set frequencies • Toolbox talks and face-to-face briefings • Mobile phones for urgent contact • SWMS, diaries, and JSEA forms • Hand signals and spotters for plant • Email or digital tools for updates |  |
| --- | --- | --- |

- What methods do you apply when monitoring worker performance for safety and compliance?

| Answer | - Conduct routine pre-start and toolbox meetings to reinforce safety expectations - Observe task performance directly to check safe work practices and compliance - Compare actual work against job plans and SWMS requirements - review JHAs |  |
| --- | --- | --- |

- How is quality assessed and confirmed?

| Answer | - Review specifications, drawings, and job documents - Follow manufacturer guidelines and technical standards - Conduct inspections and tests at hold/witness points - Monitor work against plans, SWMS, and quality benchmarks - Record outcomes in site diaries, reports, and NCRs - Consult with engineers or supervisors to resolve issues |  |
| --- | --- | --- |

- What are four (4) events that might occur on a normal working day that may significantly affect progress of civil works tasks and force the use of contingency plans.

| Answer 1. 2. 3. 4. | Unexpected weather conditions (e.g. heavy rain, lightning, heat) Late or incorrect delivery of materials or equipment Plant or machinery breakdowns Traffic management changes or stakeholder disruption |  |
| --- | --- | --- |

- What are four (4) contingency plans that may be in place to support a civil construction operational plan?

| Answer 1. 2. 3. 4. | - Alternative material sourcing plans - Labour coverage strategies - Equipment breakdown response procedures - Weather impact mitigation measures |  |
| --- | --- | --- |

- What is the relationship between various areas of civil work and how are these relationships relevant when developing job plans and work schedules?

| Answer | Various areas of civil work—such as earthworks, drainage, structures, pavements, and services—are interdependent. Each task often relies on the completion and quality of the previous one. For example, drainage must be installed before backfilling, and subgrade must be compacted before placing pavement layers. Resources like labour, plant, and materials are often shared across these tasks, so delays or changes in one area can affect others. These relationships are critical when developing job plans and work schedules because they help determine task sequencing, resource allocation, and coordination between crews. Understanding these dependencies also supports risk management and helps avoid clashes, downtime, or rework. |  |
| --- | --- | --- |

- Elaborate on four (4) different types of estimates used when developing job plans.

| Answer 1. 2. 3. 4. | Labour Estimate - Determines the number and types of workers needed to complete the job within the specified timeframe - Helps plan crew composition, shift coverage, and skill requirements - Material Estimate - Calculates the types and quantities of materials required for the job - Supports procurement planning, delivery scheduling, and cost control - Time Estimate - Predicts the duration of each task and the overall project timeline - Informs sequencing, critical path analysis, and deadline management - Plant and Equipment Estimate - Identifies the type and amount of machinery needed for each operation - Assists with logistics, hire planning, and ensuring equipment capability matches task demands |  |
| --- | --- | --- |

- What is the relationship between waste management and environmental protection?

| Answer | Waste management and environmental protection are closely linked in civil construction, as the way waste is handled directly impacts land, water, and air quality. Poor practices—like improper disposal, unmanaged runoff, or failure to contain hazardous materials, can lead to pollution and long-term environmental damage. Effective waste management, including recycling, reuse, and responsible disposal, supports sustainability and helps conserve natural resources. These practices are not just good policy—they’re legally required under acts like the Environmental Protection Act and the Waste Reduction and Recycling Act. On site, environmental management plans (EMPs) include waste handling procedures to ensure compliance and minimise environmental impact. |  |
| --- | --- | --- |

RIICWM401E

Supervise civil works

PRACTICAL ASSESSMENT

| Participant name | Benjamin Wixon |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel, unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 | Select Date. | Workplace Simulation |  |  |  |
| Task 2 (Occasion 1) | Select Date. | Workplace Simulation |  |  |  |
| Task 2 (Occasion 2) | Select Date. | Workplace Simulation |  |  |  |

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 2: OCCASION 1 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Details |
| 2.1B. | Specify task to be completed: |
| 2.1C. | Identify tool used to analyse risks and determine controls: |
| 2.1D. | PPE worn/applied (tick or specify): Hi-viz Helmet Gloves Safety boots Eye protection Ear protection Other (please specify): |
| 2.3G. | Specify adjustments made: |
| 2.4D. | Outline recommendations made: |

| ASSESSOR COMMENTS: PRACTICAL TASK 2: OCCASION 2 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Details |
| 2.1B. | Specify task to be completed: |
| 2.1C. | Identify tool used to analyse risks and determine controls: |
| 2.1D. | PPE worn/applied (tick or specify): Hi-viz Helmet Gloves Safety boots Eye protection Ear protection Other (please specify): |
| 2.3G. | Specify adjustments made: |
| 2.4D. | Outline recommendations made: |

RIICWM401E

Supervise civil works

Practical Task 1

Description of task: The participant must demonstrate undertaking the role of a civil construction supervisor and prepare a job plan which is to be carried out as part of Practical Task 2. The participant must include documentation demonstrating collating of information. If they do not have adequate existing documentation, they will be required to develop a new template for use or one from another job which illustrates the type of information they require. As part of this task, the participant is required to demonstrate the skills and knowledge to: - access, reading and correctly interpreting work documentation to ensure all work to be completed is compliant - identify and address potential risks, hazards and environmental issues, and implement control measures including selecting and wearing site appropriate PPE - consult with team members to confirm the task requirements - organise for the acquisition and availability of required resources - implement a job plan that meets task requirements Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Work team, work site, organisational policies, standard operating procedures and plans, safety processes, hazard and risk assessment documents relevant to the area of work, Project plans and specifications, EMP, TMP, Engineering survey, Testing schedule requirements, relevant SDS for any chemicals used as part of the work task being assessed.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |
| --- | --- |
| 1 Human resource requirements |  |
| - A clear and accurate overall number of workers required to undertake the construction task. |  |
| - Identify appropriate types of workers required for the task (e.g. skilled or unskilled labour, concreters, carpenters, road crews). |  |
| 2 Plant and machinery requirements |  |
| - Identify appropriate types of plant and machinery required to complete the task, and justification their selection. |  |
| - Identify the plant and machinery requiring a trained or licensed operator and any legislative document or note providing this information. |  |
| - Identify how plant and equipment will be procured for the job and what procedures will be followed to undertake this (e.g. hire company, plant hired after requisition form submitted to the site engineer and accounts officer). |  |
| 3 Construction materials requirements |  |
| - Interpret site plans and specifications to identify the types of construction materials to be used. |  |
| - Apply accurate calculations of material quantities to be used during the task showing areas, volume or tonnage and wastage factors from plan requirements and engineer requirements. |  |
| - Identify the total value of materials to be used. |  |
| - Document contingency planning in place regarding ordering materials from alternate suppliers. |  |
| 4 Sub-contractor support requirements |  |
| - Describe the requirement for all sub-contractor to undergo induction training prior to entering site. |  |
| - Identify key personnel that sub-contractors will refer to. |  |
| - Provide documentation from site including schedules or charts identifying: - The use of sub-contractors over a given period of time - Plans of drawing of work to be completed by sub-contractors - Sub-contractor induction records - Organisational chart indicating who sub-contractors will report to |  |
| 5 Waste disposal requirements |  |
| - Identify relevant state authority acts, regulations, codes of practice and standards relating to waste removal (e.g. Environmental Protection Act). |  |
| - Identify and explain how materials will be removed from site (e.g. Will they be taken to landfill, decontaminated). |  |
| - Describe any specific organisational requirements for waste disposal.(as applicable). |  |
| 6 Coordination requirements |  |
| - Identify and describe tasks that occur simultaneously on the job plan. |  |
| - Accurately identify the time required to undertake the main construction steps. |  |
| - Identify the persons required to monitor to different aspects of the project. |  |
| 7 Activity scheduling |  |
| - Clearly identify the overall length of time assigned to the project and significant milestones from production schedules, construction sequence drawings, Gannt charts or similar. |  |
| - Sequence steps effectively with an efficient use of time. |  |
| 8 Materials delivery scheduling |  |
| - Clearly identify the key delivery dates for materials to be used. |  |
| - Identify the locations for site deliveries. |  |
| - Clearly explain the types of trucks, plant and machinery used during the materials delivery process. |  |
| 9 Risk assessment and management requirements |  |
| - Describe methods for prioritising risks onsite (e.g. using risks calculators, incident reports). |  |
| - Describe the processes involved with respect to deciding upon control measures. |  |
| 10 Occupational health and safety requirements |  |
| - Identify appropriate safety personnel for the site and their roles in the safety system. |  |
| - Provide examples of how hazards may be identified on site through inspections, conducting on going risk assessments, checking reports. |  |
| - Explain the reporting requirements for hazards and incidents on site. |  |
| - Identify the requirements for all workers on site to undergo site specific inductions. |  |
| - Explain procedures for identifying non-hazardous and hazardous materials. |  |
| - Explain procedures for handling substances/ materials onsite (e.g. SDS). |  |
| - Explain the site evacuation procedure including meeting points using the site evacuation plans. |  |
| - Identify and explain the location of firefighting and first aid equipment. |  |
| - Identify and explain the roles of emergency personnel on site (e.g. Fire warden, Chief fire warden). |  |
| 11 Quality management requirements, including testing scheduling requirements |  |
| - Identify the tolerances applicable to the civil construction task from plans and specifications. |  |
| - Determine from specifications and schedule testing milestones, the types of testing required. |  |
| - Identify the personnel required to monitor and confirm testing. |  |
| 12 Traffic management requirements |  |
| - Identify exclusion zones, signage and barriers required for the project from the traffic management plan. |  |
| - Document the personnel required to implement and monitor the traffic management plan. |  |
| 13 Environmental requirements |  |
| - Identify and document how noise, erosion, water quality, waste disposal, flora and fauna and cultural heritage artefacts/ sites were intended to be controlled on site. |  |
| 14 Task monitoring requirements |  |
| - Identify the persons required to monitor to different aspects of the project. |  |
| - Provide production charts, toolbox meeting schedule, briefing note, emails, charts diagrams or work instruction requirements for addressing issues with safety/performance. |  |
| 15 Task performance monitoring requirements |  |
| - Provide example documents including production charts, briefing note, emails, charts diagrams or work instruction indicating milestones or timeframes for the construction, organisation charts/diagrams or notes. |  |
| 16 Communication requirements |  |
| - Identify how regularly toolbox meetings occur on site. |  |
| - Explain the procedures for recording attendance and topics during toolbox meeting (agenda, sign on sheets). |  |
| - Identify communication technology which may be used onsite. |  |
| 17 Reporting requirements |  |
| - Document milestones and reporting requirements including reporting to who. |  |

RIICWM401E

Supervise civil works

Practical Task 2 (2X)

Description of task: The participant must demonstrate undertaking the role of a civil construction supervisor on two (2) occasions. Occasion 1 of this task will relate to the plan prepared in Practical Task 1. During this task, the participant the participant must display the skills and knowledge to effectively, safely and efficiently: - prepare to supervise civil works - prepare team members to execute task - facilitate a pre-start meeting to issue clear and timely instructions to team members and others involved - set out tasks as required for the effective completion of the civil works - while overseeing the execution of tasks: - monitor task performance for safety effectiveness and efficiency - initiate adjustments to civil works practice and job plan and ensure safe execution of work and achievement of required outcomes - oversee plant equipment and tools maintenance and confirm maintenance requirements are carried out and recorded according to manufacturer and site requirements - oversee work area clean up and approved waste disposal including recycling - report on the execution of tasks - complete and submit written and verbal reports - recommend required changes to conform task execution meets job requirements Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Work team, work site, organisational policies, standard operating procedures and plans, safety processes, hazard and risk assessment documents relevant to the area of work, Project plans and specifications, EMP, TMP, Engineering survey, Testing schedule requirements, relevant SDS for any chemicals used as part of the work task being assessed, all materials, tools, plant or equipment required to complete the task, required PPE and relevant safety equipment.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |  |
| --- | --- | --- |
|  | 1st Occasion | 2nd Occasion |
| 1 Prepare to supervise civil works |  |  |
| - Identify the location of the site using appropriate project plans. |  |  |
| - Determine the scope of the work task from plans and specifications. |  |  |
| - Determine and address potential risks, hazards and environmental issues and implement control measures. |  |  |
| - Select and wear PPE suited to site and tasks. |  |  |
| - Consult team members regarding resource requirements for task. |  |  |
| - Communicate with other supervisors/ team leaders to resolve coordination requirements. |  |  |
| 2 Prepare team to execute task |  |  |
| - Facilitate a pre-start meeting with team and others involved. |  |  |
| - Confirm resources required for task are acquired and available for task execution. |  |  |
| - Communicate clearly and concisely when sharing work instructions. |  |  |
| - Include sub-contractors in pre-start meeting and treat sub-contractors as part of the team. |  |  |
| - Discuss site safety, emergency and risk management procedures. |  |  |
| - Read project plans, specifications, site operational requirements such as those relating to location, or special types of construction. |  |  |
| - Explain any specific organisational, client and site requirements from site plans, specifications, geological, geotechnical data. |  |  |
| - Identify specific council, state or federal requirements for the site including those relating to finishes, quality, reporting and safety. |  |  |
| - Consider and explain any possible construction problems (or lack of), regarding the type of rock found onsite. |  |  |
| - Consider and explain any possible construction problems (or lack of), regarding the type of soil types found onsite. |  |  |
| - Identify and explain any possible above or sub-soil drainage required from relevant site plans. |  |  |
| - Clearly explain the meteorological data available for the site (rainfall, humidity, temperature and wind), and identify any issues in production (e.g. If a possibility of rain was evident), quality or safe work practices arising from the forecast. |  |  |
| - Identify key aspects of the contract for work and describe what personnel will be responsible for the work and how they will achieve this. |  |  |
| - Explain the log book, or scheduled servicing procedure for plant and machinery used on the site. |  |  |
| - Concisely explain how faulty or defective equipment will be reported and recorded onsite using example hazard identification forms. |  |  |
| - Interpret site plans or environmental plans to clearly indicate the location of site wash down facilities. |  |  |
| - Explain any environmental hazards posed from the use of plant and machinery including control measures which could be used to control these environmental hazards. |  |  |
| - Listen to and respond effectively to questions and queries. |  |  |
| - Ensure all workers understand the SWMS for the task and have signed their understanding. |  |  |
| - Ensure signs and barriers have been installed to protect work area as required. |  |  |
| - Set out task correctly from engineers’ survey pegs using plans and specifications. |  |  |
| 3 Oversee execution of tasks |  |  |
| - Watch workers undertake tasks, checking that tolerances are met as required. |  |  |
| - Monitor workers for safety concerns, making sure manual handling procedures are followed and work activities are conducted safely. |  |  |
| - Watch plant and equipment use to confirm machinery is used safely and within machine capabilities and limitations. |  |  |
| - Clearly communicates with workers regarding task execution using approved and understood communication methods. |  |  |
| - Write notes in diary if safety or compliance issues arise. |  |  |
| - Write notes of measurements made, material quality, deliveries and docket checks. |  |  |
| - Organise and make adjustments to civil works practice and job plan as required and confirm required outcomes are achieved. |  |  |
| - Confirm personnel allocated has checked that log books have been filled out and scheduled servicing has been undertaken on plant and equipment. |  |  |
| - Confirm that faulty or defective equipment is reported and recorded according to site requirements. |  |  |
| - Check that plant clean down is conducted in site wash down facilities. |  |  |
| - Oversee work area clearing and waste disposal to confirm site recycling and waste management is adhered to. |  |  |
| 4 Report on the execution of tasks |  |  |
| - Make verbal reports to relevant personnel regarding completion of stages, reaching milestones, testing required. |  |  |
| - Complete all diary entries clearly and legibly. |  |  |
| - Ensure all test results and checks are documented and stored according to site requirements. |  |  |
| - Make verbal or written recommendations regarding changes to task execution. |  |  |