CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4002A Manage occupational health and safety in the building and construction workplace cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name |  |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Patrick Bellew |
| --- | --- |
| Participant signature |  |
| Date | 15/12/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 28 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

For each of the common building and construction jobs listed below, identify one (1) hazard specific to that job.

| Answer | Job | Risks |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Power tools not tested and tagged |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Un qualified workers high risk tickets for all high risk works |  |
|  | Crane use | Out of date lifting equipment, needs up to date register |  |
|  | Demolition | Uncontrolled asbestos removal, must be done by licenced operator |  |
|  | Formwork | Formwork not braced; no rails around catch decks. No access until signed. of by installer |  |
|  | Scaffolding use | Incomplete scaffold. No entry until scaffold permit signed off |  |
|  | Work at heights | Falling objects, Exclusion zones to be set up before works with spotter |  |
|  | Welding | Welding flash, Welding burns. All PPE to be worn and exclusion zones in place |  |

Using the risk table below, determine the level of risk for each of the hazards identified in Q1. Identify how each risk level was determined, by identifying the likelihood and consequences of each hazard.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

| Answer | Job | Level of risk |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | 3-B Could cause electric shock. |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | 3-B Persons to be qualified for high risk works |  |
|  | Crane use | 3-A Exclusion zones at all times, Falling objects |  |
|  | Demolition | 2-A Qualified removalist at all times, Dust suppression and all swms to be adhered to |  |
|  | Formwork | 3-A Incomplete Form work is not safe to work of, hand rails required for all catch decks |  |
|  | Scaffolding use | 3-A Incomplete scaffold is dangerous, No access until signed of |  |
|  | Work at heights | 3-A Falling objects onto workers below, Exclusion zones at all times |  |
|  | Welding | 3-E Not wearing PPE can cause eye burns or welding flash. PPE and exclusion zones at all times as per swms |  |

Identify an appropriate control measure for each of the hazards identified in Q1.

| Answer | Job | Control measure |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Test and tagging done every 3 months and logged on register in office. Service manuals to be logged as well |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | All worker to have appropriate tickets and qualifications for high risk works, All to sign onto swms |  |
|  | Crane use | All lifting gear to be inspected daily buy Dogman/rigger. Lift register to be supplied before works commence |  |
|  | Demolition | All demolition works to be carried out as to legislation. Dust control, Exclusion zones and PPE at all times. Licenced operators to only remove material to approved dumping area |  |
|  | Formwork | Exclusion zones to be set up, Barriers and signs to stop any unwanted person entering area before it is deemed safe and signed of |  |
|  | Scaffolding use | Scaffold to be closed to every one until it is signed of and tagged at bottom of scaffold declaring it is safe. |  |
|  | Work at heights | Exclusion zones must be in place to protect persons from falling objects |  |
|  | Welding | Correct welding helmet, Shield screens for person welding to keep flashes in the work area only |  |

True or False (Tick your response): Risk control options can come from both internal sources (e.g. policies and procedures) and external sources (e.g. codes and standards).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Name two (2) workplace personnel you may speak to for safety advice.

| Answer | Safety officer on site Supervisor and or workers on site |  |
| --- | --- | --- |

Referring to the WHS Regulations, explain how the following documents contribute towards identifying hazards and risks in the workplace.

| Answer | Document | Description |  |
| --- | --- | --- | --- |
|  | Safe Work Method Statements | A document that sets out high risk construction work activities to be carried out at a workplace, The hazards from these activities and measure’s to be in place to control risk |  |
|  | Site Safety Plan / WHS Management Plan | A document to outline how a general contractor will manage the risk on a specific site, It will have a specific plan with it. |  |

Identify two (2) advisory standards you might refer to safely undertake building and construction work and briefly summarise their requirements.

| Answer | Standard name | Requirements |  |
| --- | --- | --- | --- |
|  | As2047 | Windows and external glazed doors in buildings. This covers the installation of windows and doors |  |
|  | AS2159 | Piling works. Design and installation guideline’s Covers all piling works for building and or bridge works |  |

Building and construction industry contracts require building legislation and the National Construction Code to be observed. Refer to these documents when responding to Q8. to Q10.

Identify the requirements in the National Construction Code Volume 1 with regards to air pressurisation for fire-isolated exits. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | E2.2A | AS1668.1 |  |

Identify the requirements in the National Construction Code Volume 2 with regards to smoke alarms. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | 3.7.2.2 | AS3786 AS1670 AS3786 |  |

Name the building legislation (Act and Regulations) that apply within your jurisdiction.

| Answer | NCC 2019 Queensland building and construction commission reg. 2003. QBCC act 2014 |  |
| --- | --- | --- |

Which of the following should be considered when developing a risk control plan? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | How and when controls will be implemented |  |
|  |  | Evaluation processes for implemented controls |  |
|  |  | Consultation with individuals and parties |  |
|  |  | Responsibilities for implementation |  |

True or False (Tick your response): If control measures do not adequately control risks, they must be revised.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following are appropriate techniques, tools and processes for identifying hazards and risks? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Workplace inspections |  |
|  |  | Compensation claim information |  |
|  |  | Safe Work Method Statements |  |
|  |  | Information issued by WHS regulators |  |

Which of the following can be used to assist with workplace inspections? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Hazard and risk checklists |  |
|  |  | Job Safety Analysis (JSA) |  |
|  |  | Manifests and registers |  |
|  |  | Surveys |  |
|  |  | Workplace walk-through |  |

Which of the following are limitations of using a generic hazard/risk assessment checklist? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Specifics of the workplace are not considered |  |
|  |  | Not all checklist points will be applicable |  |
|  |  | Such checklists/processes must be completed on a computer |  |
|  |  | You always have to purchase these checklists |  |

Identify the applicable WHS Act in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Act name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Qld WHS Act 2011 S26A | Manage hazards and risks arising from the work carried out as part of the conduct of the pcbu in a way that is different to the code but provides a standard of health and safety that is equivalent to higher than the standard required under the code |  |

Identify the applicable WHS Regulation in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Regulation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Qld WHS regulation 2011 part 3.1 S34 | Duty to identify Hazrads |  |

Identify a relevant policy in your organisation with regards to identifying hazards and risks and briefly describe its requirements.

| Answer | Policy name | Requirements |  |
| --- | --- | --- | --- |
|  | QA 814 Fatigue management policy | To ensure adequate rest periods between shifts. Extended period of hours under supervision |  |

Identify the statutory requirements in your jurisdiction with regards to the completion of inspection reports.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | WHS 2011 qld | S187 Plant supplier inspection S204 Installation or commissioning S321 Replacement of cards, Induction |  |

Which of the following individuals and parties may require information about workplace hazards and proposed risk controls? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Health and Safety Committees |  |
|  |  | PCBU |  |
|  |  | All members of the public |  |
|  |  | Workers |  |

Identify the requirements in the Code of Practice with regards to consulting parties when managing risks.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety consultation co-operation and co-ordination code of practice 2011 | 2.1 Managing risks |  |

Identify the requirements in the Code of Practice with regards to consulting parties when developing procedures for communicating WHS policy and practice requirements.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Code of practice Developing procedures S2.4 | Resolving WHS safety issues Consult with workers on health and safety Monitoring workers health and workplace conditions Providing information and training |  |

Name three (3) parties who should consult to determine effective strategies for communicating WHS policy and practice.

| Answer | Safety professional’s Employees and contractors Managers and supervisors |  |
| --- | --- | --- |

Name two (2) communication strategies that may be used in the building and construction industry.

| Answer | Verbal communication Written communications, including emails and memos |  |
| --- | --- | --- |

Name two (2) education programs that may be implemented in the building and construction industry.

| Answer | General and site-specific induction training Other forms of specialist and target training |  |
| --- | --- | --- |

Fill in the blanks for the statement below:

Consulted

Training

The ‘How to manage work health and safety risks Code of Practice 2011’ identifies that control measures are more effective when ______________________ is provided and when workers are _____________________.

| Answer |  |  |
| --- | --- | --- |

Explain why it is important to review the effectiveness of communications and educational programs.

| Answer | If hazards and risks are not affected they must be reviewed |  |
| --- | --- | --- |

List three (3) methods for reviewing the effectiveness of communication and educational programs.

| Answer | Ensuring employees have adequate understanding Conducting post training surveys Conduct regular reviews of training needs |  |
| --- | --- | --- |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Patrick Bellew |
| --- | --- |
| Participant signature |  |
| Date | 15/12/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to conduct and report on a WHS risk analysis, including workplace inspection. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - A copy of a Site Specific Safety Plan or Construction Safety Plan, including: - Copies of Risk Assessments completed on a project - Copies of Safety Work Method Statements used on a project - A copy a workplace safety inspection checklist that has been used on a project The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: |  |  |  |
| Name of observer: |  |  |  |
| Role of observer (Confirm suitability to sign) |  |  |  |
| Email address of observer |  |  |  |
| Signature of observer: |  |  |  |
| Date: | Click here to enter a date. |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Conduct a workplace inspection to evaluate construction site safety for the range of occupations in the workplace according to WHS legislation and company policies, including: - Identifying construction hazards and potential risk areas - Identifying and prioritising hazards - Identifying and prioritising risks |  |  |  |
| - Seek expert advice as appropriate, including: - Advice from workplace personnel regarding the application of building codes and standards - Consulting with relevant parties regarding compliance with statutory requirements (including SWMS and site safety plans) - Consulting with appropriate personnel to determine effective strategies for communicating WHS policy. |  |  |  |
| - Complete an inspection report in accordance with best practice and statutory obligations: including: - Recommendations from findings - Required approaches to remediation |  |  |  |
| - Implement agreed control measures in conjunction with relevant workplace personnel, implementing negotiation and conflict resolution skills, as necessary. |  |  |  |
| - Monitor and review control measures, updating records with findings |  |  |  |
| - Establish communication strategies and educational programs specific to the building and construction industry in accordance with statutory requirements and best practice. |  |  |  |
| - Review the effectiveness of communication and educational programs, updating records with findings. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments