CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4002A Manage occupational health and safety in the building and construction workplace cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Tess Veldink |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Tess Veldink |
| --- | --- |
| Participant signature |  |
| Date | 30/03/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 28 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

For each of the common building and construction jobs listed below, identify one (1) hazard specific to that job.

| Answer | Job | Risks |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Electrocution |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Blinding |  |
|  | Crane use | Crushing |  |
|  | Demolition | Crushing |  |
|  | Formwork | Cuts |  |
|  | Scaffolding use | Fall from heights |  |
|  | Work at heights | Falling |  |
|  | Welding | Burns |  |

Using the risk table below, determine the level of risk for each of the hazards identified in Q1. Identify how each risk level was determined, by identifying the likelihood and consequences of each hazard.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

| Answer | Job | Level of risk |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | C2H |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | C3M |  |
|  | Crane use | A3H |  |
|  | Demolition | A3H |  |
|  | Formwork | E2M |  |
|  | Scaffolding use | A2E |  |
|  | Work at heights | A2E |  |
|  | Welding | D2M |  |

Identify an appropriate control measure for each of the hazards identified in Q1.

| Answer | Job | Control measure |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Ensure test and tag is in date, inspect the power tool before use |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Ensure to be wearing the correct PPE such as laser beam glasses |  |
|  | Crane use | Ensure to check and inspect all crane equipment before use and ensure there is a dogman |  |
|  | Demolition | Ensure site is inspected by an engineer |  |
|  | Formwork | Ensure to be wearing the correct PPE such as gloves and glasses |  |
|  | Scaffolding use | Ensure scaffold is built correctly with toe bars and mid-rails |  |
|  | Work at heights | Ensure all work at heights is done 2m from a live edge unless edge protection or PPE is provided |  |
|  | Welding | Ensure to wear the correct PPE including gloves, long sleeves and long pants, welding helmet |  |

True or False (Tick your response): Risk control options can come from both internal sources (e.g. policies and procedures) and external sources (e.g. codes and standards).

| Answer | þ | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Name two (2) workplace personnel you may speak to for safety advice.

| Answer | Key personnel – involved in WHS decision making WHS specialists – experts in the specific WHS areas |  |
| --- | --- | --- |

Referring to the WHS Regulations, explain how the following documents contribute towards identifying hazards and risks in the workplace.

| Answer | Document | Description |  |
| --- | --- | --- | --- |
|  | Safe Work Method Statements | The primary purpose of a SWMS is to help supervisors, workers and any other persons at the workplace to understand the requirements that have been established to carry out the high risk construction work in a safe and healthy manner. The SWMS: sets out the work activities in logical sequences, identifies hazards, describes control measures. The aim of a SWMS is to: describe the activity or task to be undertaken, identify the resources, manpower and skills associated with the task, assess and select control measures (as appropriate), systematically plan the activity so it can be completed efficiently and effectively. |  |
|  | Site Safety Plan / WHS Management Plan | A work health and safety (WHS) management plan can assist principal contractors to manage their workplace health and safety obligations. The principal contractor for a construction project must prepare a written WHS management plan for the workplace before work on the project commences. The plan must state: - the names, positions and health and safety responsibilities of all persons at the workplace whose positions or roles involve specific health and safety responsibilities in connection with the project - the arrangements in place, between any persons conducting a business or undertaking at the workplace where the construction project is being undertaken, for consultation, cooperation and the coordination of activities in relation to compliance with their duties - the arrangements in place for managing any work health and safety incidents that occur - any site-specific health and safety rules, and the arrangements for ensuring that all persons at the workplace are informed of these rules - the arrangements for the collection and any assessment, monitoring and review of safe work method statements at the workplace. |  |

Identify two (2) advisory standards you might refer to safely undertake building and construction work and briefly summarise their requirements.

| Answer | Standard name | Requirements |  |
| --- | --- | --- | --- |
|  | NCC volume 1 | NCC Volume One primarily applies to Class 2 to 9 (multi-residential, commercial, industrial and public) buildings and structures. It contains clearly identified, non-mandatory explanatory information within the text to assist with interpretation of the requirements. |  |
|  | NCC volume 2 | NCC Volume Two primarily applies to Class 1 (residential) and 10 (non-habitable) buildings and structures. It contains clearly identified, non-mandatory explanatory information within the text to assist with interpretation of the requirements. |  |

Building and construction industry contracts require building legislation and the National Construction Code to be observed. Refer to these documents when responding to Q8. to Q10.

Identify the requirements in the National Construction Code Volume 1 with regards to air pressurisation for fire-isolated exits. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | Service & equipment. Smoke detection & alarm systems – Spec E2.2a | Smoke detectors required to activate air pressurisation systems for fire-isolated exits and zone pressurisation systems must— (i) be installed in accordance with AS 1670.1; and (ii) have additional smoke detectors installed adjacent to each bank of lift landing doors set back horizontally from the door openings by a distance of not more than 3 m. (b) Smoke detectors required to activate— (i) automatic shutdown of air-handling systems in accordance with Table E2.2b; or (ii) a smoke exhaust system in accordance with Specification E2.2b, must— (iii) be spaced— (A) not more than 20 m apart and not more than 10 m from any wall, bulkhead or smoke curtain; and (B) in enclosed malls and walkways in a Class 6 building not more than 15 m apart and not more than 7.5 m from any wall, bulkhead or curtain; and have a sensitivity— (A) in accordance with AS 1670.1 in areas other than a multi-storey walkway and mall in a Class 6 building; and (B) not exceeding 0.5% smoke obscuration per metre with compensation for external airborne contamination as necessary, in a multi-storey walkway and mall in a Class 6 building. (c) Smoke detectors provided to activate a smoke control system must— (i) (A) form part of a building fire or smoke detection system complying with AS 1670.1; or (B) be a separate dedicated system incorporating control and indicating equipment complying with AS 1670.1; and (ii) activate a building occupant warning system complying with Clause 7, except that smoke detectors provided solely to initiate automatic shutdown of air-handling systems in accordance with (b)(i) need not activate a building occupant warning system. |  |

Identify the requirements in the National Construction Code Volume 2 with regards to smoke alarms. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | Fire Safety Part 3.7.5 Smoke alarms and evacuation lighting – 3.7.5.2 Smoke alarm requirements | Smoke alarms must— (a) be located in— (i) Class 1a buildings in accordance with 3.7.5.3 and 3.7.5.5; and (ii) Class 1b buildings in accordance with 3.7.5.4 and 3.7.5.5. (b) comply with AS 3786, except that in a Class 10a private garage where the use of the area is likely to result in smoke alarms causing spurious signals, any other alarm deemed suitable in accordance with AS 1670.1 may be installed provided that smoke alarms complying with AS 3786 are installed elsewhere in the Class 1 building; and (c) be powered from the consumer mains source where a consumer mains source is supplied to the building; and (d) be interconnected where there is more than one alarm. |  |

Name the building legislation (Act and Regulations) that apply within your jurisdiction.

| Answer | Queensland Building and Construction Commission Act 1991 (Qld) (QBCC Act) |  |
| --- | --- | --- |

Which of the following should be considered when developing a risk control plan? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | How and when controls will be implemented |  |
|  | þ | Evaluation processes for implemented controls |  |
|  | þ | Consultation with individuals and parties |  |
|  | þ | Responsibilities for implementation |  |

True or False (Tick your response): If control measures do not adequately control risks, they must be revised.

| Answer | þ | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following are appropriate techniques, tools and processes for identifying hazards and risks? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | Workplace inspections |  |
|  | þ | Compensation claim information |  |
|  | ¨ | Safe Work Method Statements |  |
|  | þ | Information issued by WHS regulators |  |

Which of the following can be used to assist with workplace inspections? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | Hazard and risk checklists |  |
|  | þ | Job Safety Analysis (JSA) |  |
|  | þ | Manifests and registers |  |
|  | þ | Surveys |  |
|  | þ | Workplace walk-through |  |

Which of the following are limitations of using a generic hazard/risk assessment checklist? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | Specifics of the workplace are not considered |  |
|  | ¨ | Not all checklist points will be applicable |  |
|  |  | Such checklists/processes must be completed on a computer |  |
|  |  | You always have to purchase these checklists |  |

Identify the applicable WHS Act in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Act name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Guide to the Work Health and Safety Act 2011 QLD – Duty of officers section 27 | It is the duty of an officer of a PCBU to exercise due diligence to ensure the PCBU complies with its health and safety duties and obligations. An officer may be charged with an offence under the WHS Act independently of any breach of duty by the PCBU. Due diligence includes personally taking reasonable steps to: - acquire and keep current information on work health and safety matters - understand the nature and operations of the business or undertaking and associated hazards and risks - ensure the PCBU has, and uses, appropriate resources and processes to eliminate or minimise risks to health and safety - ensure the PCBU has appropriate processes to receive and consider information about incidents, hazards and risks, and to respond in a timely manner - ensure the PCBU has, and implements, processes for complying Guide to the Work Health and Safety Act 2011 11 with their duties and obligations (e.g. reports notifiable incidents, consults with workers, complies with notices, provides training and instruction and ensures HSRs receive training entitlements). |  |

Identify the applicable WHS Regulation in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Regulation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety regulation 2011 – Chapter 3 general risk and workplace management | Duty to identify hazards A duty holder, in managing risks to health and safety, must identify reasonably foreseeable hazards that could give rise to risks to health and safety. Managing risks to health and safety A duty holder, in managing risks to health and safety, must: (a) eliminate risks to health and safety so far as is reasonably practicable; and (b) if it is not reasonably practicable to eliminate risks to health and safety — minimise those risks so far as is reasonably practicable. Hierarchy of control measures (1) This regulation applies if it is not reasonably practicable for a duty holder to eliminate risks to health and safety. (2) A duty holder, in minimising risks to health and safety, must implement risk control measures in accordance with this regulation. (3) The duty holder must minimise risks, so far as is reasonably practicable, by doing one or more of the following: (a) substituting (wholly or partly) the hazard giving rise to the risk with something that gives rise to a lesser risk; (b) isolating the hazard from any person exposed to it; (c) implementing engineering controls. (4) If a risk then remains, the duty holder must minimise the remaining risk, so far as is reasonably practicable, by implementing administrative controls. (5) If a risk then remains, the duty holder must minimise the remaining risk, so far as is reasonably practicable, by ensuring the provision and use of suitable personal protective equipment. |  |

Identify a relevant policy in your organisation with regards to identifying hazards and risks and briefly describe its requirements.

| Answer | Policy name | Requirements |  |
| --- | --- | --- | --- |
|  | Usher Group - Workplace Heath & Safety Policy | Employees: You play an important role in creating and maintaining a safe working environment. As such, you have an obligation and a responsibility to: > conduct yourself in a manner that is appropriate, always exhibiting safe and healthy work practices; > be responsible for your own health and safety, ensuring you do not conduct yourself in a manner that may compromise the health and safety of others; > declare whether you suffer from a medical condition that may impact your ability to perform in your role; > undertake fitness for work assessments if inherent requirements of the job are unable to be performed; > follow all safe work method statements, safe travel practices, procedures, instructions and rules; > comply with legislation and law relating to workplace health and safety and follow all safe work practices, procedures, instructions and rules; > encourage others to work in a safe and healthy manner; and > uphold and familiarise yourself with the duties that apply to you within the Act. |  |

Identify the statutory requirements in your jurisdiction with regards to the completion of inspection reports.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Heath & Safety Act 2011 - 103E Assessment Reports | The work health and safety officer must— prepare a report (an assessment report) for the assessment carried out under section 103D(1); and if the assessment identifies any risks to health and safety arising from the work carried out as part of the conduct of the business or undertaking—include in the report recommendations about managing the risks. The work health and safety officer must, within 30 days after the assessment has been carried out, give a copy of the assessment report to— - (a) if the work health and safety officer is not the person conducting the business or undertaking—the person conducting the business or undertaking; and - (b) if there is a health and safety committee at the business or undertaking—the committee. |  |

Which of the following individuals and parties may require information about workplace hazards and proposed risk controls? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | Health and Safety Committees |  |
|  | þ | PCBU |  |
|  | ¨ | All members of the public |  |
|  | þ | Workers |  |

Identify the requirements in the Code of Practice with regards to consulting parties when managing risks.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work heath & safety consultation cooperation and coordination – WHS Act section 46 | The WHS Act requires a PCBU to consult, cooperate and coordinate activities with all other persons who have a work health or safety duty in relation to the same matter, so far as is reasonably practicable. In these situations, each duty holder should exchange information to find out who is doing what and work together in a cooperative and coordinated way so risks are eliminated or minimised so far as is reasonably practicable. Examples of where a PCBU will have a health and safety duty include where: - the PCBU engages workers to carry out work - the PCBU directs or influences workers in carrying out work - other persons may be put at risk from work carried out in their business or undertaking - the PCBU manages or controls a workplace or the fixtures, fittings or plant at a workplace - the PBCU’s business or undertaking involves designing, manufacturing, importing or supplying plant, substances or structures for use at a workplace - the PBCU’s business or undertaking involves installing, constructing or commissioning plant or structures at a workplace. |  |

Identify the requirements in the Code of Practice with regards to consulting parties when developing procedures for communicating WHS policy and practice requirements.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work heath & safety consultation cooperation and coordination – WHS Act section 49 – Developing work heath and safety procedure | A procedure sets out the steps to be followed for work activities. You must consult with affected workers when developing procedures for: - consulting with workers on work health and safety - resolving work health and safety issues - monitoring workers’ health and workplace conditions - providing information and training for workers. Procedures should be in writing to provide clarity and certainty at the workplace and assist in demonstrating compliance. They should clearly set out the role of health and safety representatives, both legislative and workplace specific, and other parties involved in the activity. The procedures should be easily accessible, for example by placing them on noticeboards and intranet sites. |  |

Name three (3) parties who should consult to determine effective strategies for communicating WHS policy and practice.

| Answer | Management, Health & Safety committees and designated work groups |  |
| --- | --- | --- |

Name two (2) communication strategies that may be used in the building and construction industry.

| Answer | - Issued site specific instructions and signage - Verbal communications |  |
| --- | --- | --- |

Name two (2) education programs that may be implemented in the building and construction industry.

| Answer | - General and site-specific induction training - Other forms of specialist and targeted training |  |
| --- | --- | --- |

Fill in the blanks for the statement below:

consulted

training

The ‘How to manage work health and safety risks Code of Practice 2011’ identifies that control measures are more effective when ______________________ is provided and when workers are _____________________.

| Answer |  |  |
| --- | --- | --- |

Explain why it is important to review the effectiveness of communications and educational programs.

| Answer | It may reduce the frequency and severity of workplace injury and disease, as employees gain knowledge and skills to perform their duties safely. |  |
| --- | --- | --- |

List three (3) methods for reviewing the effectiveness of communication and educational programs.

| Answer | - Reviewing training materials for accuracy of content, currency and suitability - Conducting post-training surveys 3.Conducting regular reviews of training needs |  |
| --- | --- | --- |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Tess Veldink |
| --- | --- |
| Participant signature |  |
| Date | 30/03/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to conduct and report on a WHS risk analysis, including workplace inspection. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - A copy of a Site Specific Safety Plan or Construction Safety Plan, including: - Copies of Risk Assessments completed on a project - Copies of Safety Work Method Statements used on a project - A copy a workplace safety inspection checklist that has been used on a project The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Tess Veldink |  |  |
| Name of observer: | Amanda Pennycook |  |  |
| Role of observer (Confirm suitability to sign) | Safety Manager |  |  |
| Email address of observer | Tess.v@ushergroup.com |  |  |
| Signature of observer: |  |  |  |
| Date: | 13/04/2021 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Conduct a workplace inspection to evaluate construction site safety for the range of occupations in the workplace according to WHS legislation and company policies, including: - Identifying construction hazards and potential risk areas - Identifying and prioritising hazards - Identifying and prioritising risks |  | þ | þ |
| - Seek expert advice as appropriate, including: - Advice from workplace personnel regarding the application of building codes and standards - Consulting with relevant parties regarding compliance with statutory requirements (including SWMS and site safety plans) - Consulting with appropriate personnel to determine effective strategies for communicating WHS policy. |  | þ | þ |
| - Complete an inspection report in accordance with best practice and statutory obligations: including: - Recommendations from findings - Required approaches to remediation |  | þ | þ |
| - Implement agreed control measures in conjunction with relevant workplace personnel, implementing negotiation and conflict resolution skills, as necessary. |  | þ | þ |
| - Monitor and review control measures, updating records with findings |  | þ | þ |
| - Establish communication strategies and educational programs specific to the building and construction industry in accordance with statutory requirements and best practice. |  | þ | þ |
| - Review the effectiveness of communication and educational programs, updating records with findings. |  | þ | þ |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication |  | þ | þ |
| The Participant’s performance was: þ Satisfactory Not Satisfactory |  |  |  |

observer Comments

This subject was about Health and Safety in the building and Construction Workplace. All the criteria's were discussed and two pieces of evidence was submitted for support each topic as occasion 1 and occasion 2. The evidence submitted included 2 x SWMS, Workplace health and safety policy from the Usher Group, 2 x internal audits, 2 x Site Management Plans and 2 x Hazard Identification Report Forms. All evidence was from Queensland Projects – Norfolk, Queen Street Village, Herston Quarter & 443 Queen Street. Workplace inspection The first thing discussed was the Usher Group policy this is to ensure that the team understood the requirements from the policy. A workplace inspection report was explained and the 3 main points of the report were mentioned. 2 x internal audits were referenced when discussing the 3 main point further. Seek Expert Advice The importance of getting expert advice when reporting risk and hazards was highlighted. Along with 2 x SWMS & 2 x Site Safety plans which comply with the statutory requirements. Risk management process communication and strategy was discussed to ensure it implemented as per the WHS legislation. Inspection Report The use of an inspection report was highlighted at the start and then was related back to the Usher Group Hazard Identification Report Form – 2 x form was submitted for evidence. These reports were discussed in detail. Agreed Control Measures The risk control process was touched on and explained that the process isn’t always practicable. Interim control measures were listed and related to the hazard identification reports, as this is something that can be implemented due to the nature of the reports. Monitor and Review Control Measures The monitoring of the implementation plan, the importance of tracking the control measures and revising the plan, in relation to the WHS act was discussed. Communication Strategies The benefits of communicating important information to employees was touched on and a section from the code of practise 2011 backed up the information given. Effectiveness of Control Measures Measuring the effectiveness of the control measure was lastly discussed, this can be done through multiple ways as listed in the practical document. Effective communication Effective communication was used through out using non-verbal communication such as email and using the correct terminology when discussing theory and concepts