CPCCBC4006

Select, procure and store construction materials for building and construction projects

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4006 Select, procure and store construction materials for building and construction projects cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Michael Trott |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. | Satisfactory | Not Satisfactory |
| Workbook | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for CPCCBC4006 | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Theory Exam Workbook | Practical Assessment Materials List - HO1 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

CPCCBC4006

Select, procure and store construction materials for building and construction projects

THEORY EXAM

| Participant name | Michael Trott |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within two (2) hours, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

Discuss how you ensure you have located the correct plans and specifications when determining materials needed for a building job.

| Answer | Cross-reference the drawings with the construction specifications to ensure consistency. The specifications provide detailed information about the materials, quality standards, and construction methods |  |
| --- | --- | --- |

Clarify what codes and standards you need to read to confirm the materials specified are suitable for the proposed construction project.

| Answer | You need to read and be aware of the NCC codes as this is the primary code that provides technical guidelines for the design and construction of buildings and other structures in Australia. |  |
| --- | --- | --- |

How do you assess the suitability of properties of specified materials for the building project in the region?

| Answer | Consider the compatibility of materials with each other. Certain materials may react negatively when in contact with others, leading to deterioration or structural issues Evaluate the local climate and environmental conditions. Factors such as temperature extremes, humidity, seismic activity, and exposure to saltwater can influence material durability and performance. Quality, compatibility and compliance |  |
| --- | --- | --- |

Provide a brief explanation of the listed NCC compliance assessment methods used to determine compliance using a Performance Solution pathway.

| Answer | Assessment method | Description |  |
| --- | --- | --- | --- |
|  | Evidence of suitability | This pathway involves providing evidence of suitability to demonstrate that a proposed solution meets the required performance standards |  |
|  | Verification methods | Verification methods are employed to assess and confirm the compliance of the performance solution. |  |
|  | Comparison with DTS Provisions | This method involves comparing the proposed performance solution directly with the relevant Deemed to Satisfy provisions in the NCC. The goal is to demonstrate that the proposed solution either meets or exceeds the performance levels set by the DTS provisions. |  |
|  | Expert Judgement | This method involves engaging a qualified expert or professional to assess whether the proposed performance solution complies with the relevant performance requirements of the NCC. The expert uses their judgment and expertise to verify compliance |  |

What information do the specifications give you regarding materials to be used?

| Answer | . They help ensure that the materials used in the project meet the necessary standards, codes, and performance criteria. They also give you a guide to what the materials properties are and special requirements for materials. |  |
| --- | --- | --- |

Which legislation details the requirements relevant for building work in the state/territory where you are based?

| Answer | Queensland – Building act 1975 |  |
| --- | --- | --- |

How can you ascertain whether materials are compatible with one another?

| Answer | By consulting standards Australia as this provides the needed standards on preparing and evaluating material testing in order to check compatibility. You can search for the specific material you intend to test to find relevant Australian Standards on them |  |
| --- | --- | --- |

What are structural insulated panels (SIPs)?

| Answer | It is a building system for residential construction that is energy efficient, cost-effective, and not to mention, impressively strong they are made up of two oriented strand boards sandwiching an insulating foam core. |  |
| --- | --- | --- |

Why are SIPs a good sustainable alternative to traditional timber framed construction? Give two (2) reasons.

| Answer | - They help reduce constructional waste - Better control for indoor environmental conditions |  |
| --- | --- | --- |

What makes a SIP building sustainable?

| Answer | SIP are sustainable because they are highly energy efficient |  |
| --- | --- | --- |

Mud bricks are an alternative material product used in building and construction. Briefly explain what makes it sustainable.

| Answer | Due to its production requiring and utilising very little energy |  |
| --- | --- | --- |

What makes straw bales a sustainable building product?

| Answer | With low embodied energy, the life cycle of straw bale is energy-efficient, with its bailing process and transport the only processes that expend so much energy |  |
| --- | --- | --- |

Why is hemp an environmentally sustainable material used in building and construction?

| Answer | Because its very versatile and also has great resistance against fire and cracking under movement, making it ideal for structures and buildings in areas that are prone to earthquakes and other seismic activities. |  |
| --- | --- | --- |

Discuss two (2) different environmental effects that timber can succumb to.

| Answer | - Decay and rot - Insect infestation |  |
| --- | --- | --- |

Discuss methods/products that can be used to protect or preserve the timber from the environmental effects mentioned in the previous answer.

| Answer | Proper storage away from such things as moisture and using timber that has been pressure treated. |  |
| --- | --- | --- |

Discuss two (2) different environmental effects that steel can succumb to.

| Answer | - Corrosion - Rusting |  |
| --- | --- | --- |

Discuss methods/products that can be used to protect or preserve the steel from the environmental effects mentioned in the previous answer.

| Answer | 1- Galvanising 2-Store in areas that are covered and out of sunlight |  |
| --- | --- | --- |

Provide three (3) differences between ferrous and non-ferrous metals.

| Answer | - Ferrous metals are magnetic whereas non-ferrous metals are non magnetic - Ferrous metals contain iron whereas non-ferrous metals don’t - Ferrous metals rust whereas non-ferrous metals don’t |  |
| --- | --- | --- |

Discuss the key environmental impacts of construction materials.

| Answer | Construction materials can generate large amounts of waste, including offcuts, packaging materials, and demolition debris. Improper waste management can lead to landfilling, contributing to environmental degradation and potential leaching of hazardous substances |  |
| --- | --- | --- |

Discuss how the use of natural materials such as natural stone or timber surfaces such as bench tops is affected by tolerance standards.

| Answer | Tolerance standards dictate the acceptable variations in dimensions for natural materials. In the case of benchtops, measurements are crucial to ensure proper fit and functionality. Not allowing specified dimensions may result in gaps, uneven surfaces, or difficulties in installation. |  |
| --- | --- | --- |

Discuss when a joint in a benchtop is considered defective in your state or territory, regardless of whether the benchtop is timber, laminate or natural stone.

| Answer | Within the first 12 months from completion of the work, bench tops of timber, laminate, natural stone or similar materials are defective if they have joints that are not uniform, close-fitted, aligned and in the same plane. These requirements also apply to vertical surfaces of similar material and finish. Joints are defective if they are not sealed or flush-filled with a suitable flexible sealant of matching colour |  |
| --- | --- | --- |

Discuss what structural adequacy means when referring to construction materials.

| Answer | Structural adequacy means the ability of the building components and materials to withstand or resist vertical load forces exerted by the building to these materials, in addition to the materials own weight. |  |
| --- | --- | --- |

Clarify some determinations you must make to ensure a construction material is deemed safe.

| Answer | 1- Clarify type of building as this will dictate type of material 2-Whether the material is sustainable 3- Whether they are adaptable to the climate of the region where it will be used |  |
| --- | --- | --- |

Discuss why you might suggest refurbishing a building using fire-resistant building materials where possible.

| Answer | As this will help the fire not to grow throughout the structure and may bring buildings up to certain compliance codes. They can also hely with the structural integrity of the building |  |
| --- | --- | --- |

Complete the table below by identifying the type of fire-resisting construction (i.e. Type A, Type B, or Type C) given the following class of building and number of storeys each class of building has.

| Answer | Rise in Storeys | Class of building: 2,3,9 | Class of building: 5,6,7,8 |  |
| --- | --- | --- | --- | --- |
|  | 4 or more | A | A |  |
|  | 3 | A | B |  |
|  | 2 | B | C |  |
|  | 1 | C | C |  |

Describe how you would monitor alternative forms of construction to ensure compliance.

| Answer | These alternative materials are to be evaluated in the same manner that manufactured materials are evaluated, with a little wiggle room to accommodate certain variations from the expected and indicated standards they have to meet. They are basically held to the same standards as normal materials |  |
| --- | --- | --- |

Discuss what criteria you must meet when selecting the building materials.

| Answer | Structural requirements Durability and longevity Cost effectiveness Sustainability Energy efficiency Code compliance Fire resistance and safety Accessibility and availability Climate consideration Manufacturer credibility Cost |  |
| --- | --- | --- |

Discuss which professionals you might consult regarding materials to be used for the construction project.

| Answer | Manufactures as they possess deep knowledge of their products. Architects and designers as you can then meet their requirements for the design of the build. Cost consultants as they will help choose materials to suit budget. A civil or structural engineer as they can advise best materials to use to meet certain requirements. Contractors will also give great knowledge on best types of materials to use as they are the ones that ultimately install the material. |  |
| --- | --- | --- |

Describe what is meant by procuring materials for the construction project.

| Answer | This is the process that of acquiring and securing the necessary materials required to construct a building or structure. |  |
| --- | --- | --- |

Clarify some of the limitations and effects of transportation on materials and components, and what you should check for on delivery to the site.

| Answer | Traffic conditions – This is normally subject to various road conditions which can delay delivery. Weather – Can slow down and even halt delivery of materials to site. Quantity – You can only fit a certain amount of material on a delivery so may need to break up into a number of deliveries . which will slow the progress. Costs - Economic changes for the materials itself, fuel price, maintenance of transport vehicles and other relevant factors can influence the total cost of delivery. Another limitation is that construction projects operate only with a limited and predetermined budget. You should check for key things like if the material delivery matches the order and general condition of the material and for compliance. |  |
| --- | --- | --- |

What action can you take to ensure materials are transported and delivered as required.

| Answer | Check with suppliers that they are fully stocked with the materials you need in advance and create a resource schedule. Be up to date with current events which could delay delivery of materials to site. |  |
| --- | --- | --- |

Discuss what must be considered when allocating storage areas for delivered materials.

| Answer | Physical characteristics – consider shape weight and size so they adequately fit into designated areas. Location and access – can they be accessed easily by workers and is it near to the delivery site which helps. Safety – Are they stored correctly and away from workers where they could cause harm. Project plan – When and what type of material will be turning up at designated times and dates |  |
| --- | --- | --- |

Discuss methods that are applied on building construction sites to ensure the safe handling of materials and construction components.

| Answer | Material should be properly secured. Keep flammable and combustible materials in fire resistant containers. Determine and publish safe loads designated floors can take. Maintain clear passage ways and corridors. Construct ramps and walkways between areas of different levels to stop spills and accidents. |  |
| --- | --- | --- |

Describe processes that must be undertaken when materials and components are delivered to the site.

| Answer | Receive and inspection – Check quantity and quality. Documentation and record keeping. Material handling and unloading – safely unload materials Storage and organisation Protection from elements Ensure |  |
| --- | --- | --- |

Describe what a non-conformance report is used for.

| Answer | This is a report to describe the item or items that are to be reported as not meeting the standards or sub-par. Measures on how to prevent issues and come up with potential resolutions can also be provided in a non-conformance report. This can then be used for future training for you or future suppliers to stop previous mistakes from repeating. |  |
| --- | --- | --- |

Provide a brief explanation of the operational and functional features of the following materials handling equipment.

| Answer | Materials handling equipment | Operational and functional features |  |
| --- | --- | --- | --- |
|  | Trucks | Manual or power-lifted rear bed to carry and transport materials horizontally Trailers to transport materials safely from external elements (theft, weather conditions, dirt and debris) |  |
|  | Cranes | Stabilisers (for mobile cranes) or support frames (for static cranes) to ensure structural strength of cranes Cable and hook/hoist to be able to carry and transport materials from one place to another |  |
|  | Pallet lifter | Material handling tool designed to lift and move pallet. It is widely used for efficient transportation of goods. |  |
|  | Wheelbarrow | A wheelbarrow is a simple yet versatile tool used for transporting and carrying materials, especially in construction, gardening, and landscaping. Its design includes various operational and functional features to facilitate efficient and easy use. They are normally single or dual wheels with a tray and a handle They have a load capacity and a hydraulic lift with a release valve with easy handles and controls |  |

Discuss the digital technology and applications you use to access, extract, integrate and compile information relevant to the building project.

| Answer | Construction management software like buildexact and service mate. Mobile apps and the internet. |  |
| --- | --- | --- |

CPCCBC4006

Select, procure and store construction materials for building and construction projects

WORKBOOK

| Participant name | Michael Trott |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written activities as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - The timeframe for these tasks will be established by your Assessor or Training Plan, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

NOTE

At times, you will be provided with a specific business context to which you must respond, otherwise it is anticipated that you will use a workplace of your choosing (your own, or hypothetical) to provide the context to the task.

At all times, the responses in your assessment must reference Australian (Queensland or Federal) documentation, including legislation, regulations, codes of practice, standards and guidance publications.

Research each of the following materials and respond to the following: - Identify one (1) property of each building material. - Briefly explain the application of the building material. - Identify one (1) limitation of the building material.

Cement

- Property of the material.

When mixed with water it exhibits both adhesive and cohesive properties that makes it suitable for binding durable solid construction materials

- Application of the material.

Cement is used to make mortar, a paste that binds bricks, stones, and other masonry units together. Mortar provides strength and durability to the overall structure of buildings

- Limitation of the material.

Cement has low tensile strength, low toughness and long curing time, which is why materials with cement may develop cracks.

Ceramics

- Property of the material.

Ceramics are glassy materials that are produced from a mixture of minerals and water

- Application of the material.

Can be used as cutting tools

- Limitation of the material.

Immensely brittle

Concrete

- Property of the material.

Concrete is known for its compressive strength, which is its ability to withstand loads or pressure.

- Application of the material.

Used for foundations, walls, floors, and roofs, providing structural stability and support.

- Limitation of the material.

Prone to cracking

Glass

- Property of the material.

Glass is made from sand and silicates

- Application of the material.

The function of glass varies from being used for windows, to doors and skylights. More modern designs in the industry now use glass for flooring and wall structures

- Limitation of the material.

Very expensive and brittle which means it needs to be handled with care

Plaster

- Property of the material.

Created building material mainly from clay, limestone or cement. Initially, it comes as a form of a powder and is mixed in water when to be used

- Application of the material.

Plaster is used for walls and ceilings since it has weather resistant properties (water, dust resistant) and it is highly adhesive

- Limitation of the material.

Installation is challenging and expensive

Portland cement mortar

- Property of the material.

Portland cement mortar is a mixture of Portland cement, sand, and water.

- Application of the material.

Used in brick, block and stone masonry construction

- Limitation of the material.

Its limitation is that it is to be mixed with sand and water only

Lime mortar

- Property of the material.

It is composed of lime, sand, and water and is known for its unique properties.

- Application of the material.

Lime mortar is commonly used for pointing or repointing the joints between bricks or stones in masonry structures.

- Limitation of the material.

Hardens slowly and is less adhesive than Portland cement

Paint

- Property of the material.

Paint is primarily known for its ability to add colour to surfaces. The colour can be customized to meet aesthetic preferences or specific design requirements.

- Application of the material.

Paint is also used for aesthetic purposes to cover walls in whatever colour the client chooses

- Limitation of the material.

Gets dirty easily

Epoxy

- Property of the material.

Epoxy resin is a material used as an adhesive ingredient, as well as for plastics, paints, coatings, primers and sealers.

- Application of the material.

It can be used as an electrical insulator or a structural sealant, and it is usually corrosion resistant.

- Limitation of the material.

Emits toxic fumes that can be a health hazard.

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Research each of the following building products and respond to the following: - Identify properties of each building material. - Briefly explain the application of the building product. - Identify limitations of the product.

Sawn timber

- Properties

Wood or lumber that have been cut different dimensions, usually rectangular or wedged in shape.

- Application of the product.

Used for structural support in building

- Limitations.

low thermal conductivity and poor sound insulation.

‘Massive’ or cross laminated timber (CLT)

- Properties

Structural building material composed of layers of wood panels glued together at right angles.

- Application of the product.

Used as floor slabs, beams, columns and shear/loading bearing walls.

- Limitations.

Costly and has poor sound insulation

Steel framing

- Properties

Steel is known for its high strength, providing structural integrity and the ability to support heavy loads. This strength-to-weight ratio is advantageous for constructing tall buildings and large structures.

- Application of the product

Is common among high-rise, warehouse and industrial buildings.

- Limitations

Requires other kinds of material to help support it

Asphalt shingles- roofing

- Properties

This roofing consists of asphalt, miner granules, felt and fibreglass pressed together and made into a rectangular shape with three tabs

- Application of the product

Used most commonly in residential roofing

- Limitation of the material

Has a short lifespan

Plasterboard

- Properties

A panel made of calcium sulphate dihydrate

- Application of the product

Used to make interior ceilings, partitions, and walls.

- Limitations

Low resistance against water

Structural steel

- Properties

Structural steel is a widely used construction material known for its strength, durability, and versatility.

- Application of the product

Used for structural beams

- Limitation of the material

Experiences a high expansion rate in changing temperatures.

| Satisfactory Outcome |  |  |
| --- | --- | --- |

For the following activity, name the state/territory that you wish to examine.

| State / Territory | e.g. Queensland |
| --- | --- |

Identify current work health and safety legislation and related documentation such as codes of practice that will be relevant to your chosen workplace, and where such documentation can be found: - Name and location of Legislation. - Name and location of Regulation. - Information relating to the hierarchy of controls. - Information related to hazardous manual tasks. - Information related to mechanical material handling. - Information regarding licensing and competency for operating mechanical material handling equipment.

- Name and location of Legislation (Commonwealth or State)

Legislation: Work Health and Safety Act 2011 (QLD) and Work Health and Safety Regulation 2011 (QLD)

- Name and location of Regulation

How to Safely Remove Asbestos Code of Practice 2021."

- Information relating to the Hierarchy of controls

Elimination Substitution Isolation Engineering controls Administrative controls PPE

- Information related to hazardous manual tasks

WHS Regulation Part 8.10 Division 1 Can remove: • up to 10m2 of non-friable asbestos or ACM • ACD that is: - associated with the removal of less than 10m2 of non-friable asbestos or ACM - not associated with the removal of friable or non-friable asbestos and is only a minor contamination.

- Information related to mechanical material handling

Work health and safety Queensland Guide to machinery and equipment safety

- Information regarding licensing and competency for operating mechanical material handling equipment.

WHS Regulation section 458 – Duty to ensure asbestos remover is licensed

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Access the NCC via the internet to locate the requirements applicable to a building development in the Brisbane area, containing 6 sole-occupancy units, two (2) on each level, each being a separate dwelling. This building includes one (1) basement level designated for car parking. The proposed building structure requires reinforced concrete floors and columns. Document your findings or where in the NCC the information can be located.

Building Description

Classification

Class 2 building Part A6 building classifications

Rise in Storeys

4

Number of Storeys

3

Type of construction

A – Table C1.1

Climate Zone and description

Zone 2 – warm humid summer, mild winter

Fire resistance and stability

Locate fire resistance information

CP1 Structural stability during a fire CP2 Spread of fire specific to class 2 C1.1 fire resistant construction 2.3 lintels 2.8 Carparks in Class 2 and 3 buildings

Compartmentation and fire separation

Part C3 compartmentation and fire separation C3D8 C3D9 C3D10-B S5C9 CARPARKS IN CLASS 2 AND 3 BUILDINGS S5C20 C3D15

Fire hazard properties of materials, assemblies, fixtures and linings

Specification 5 fire resisting construction S5C14 S5C16 S5C17 S5C19 S5C20 S5C21 S5C22 S5C23

Material requirements

Reinforced concrete requirements relative to Australian Standards

Part 3.2.3 Concrete and reinforcing Concrete must be manufactured to comply with AS 3600; 3.2.3.2 Steel reinforcement

Glazing requirements specifically for windows, sliding doors with frames and balconies

Part 3.6 Glazing

| Satisfactory Outcome |  |  |
| --- | --- | --- |

CPCCBC4006

Select, procure and store construction materials for building and construction projects

PRACTICAL ASSESSMENT

| Participant name | Michael Trott |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel, unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 | Select Date. | Workplace Simulation |  |  |  |

ASSESSOR COMMENTS: GENERAL

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 1 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Provide detail |
| 1.1A. | Specify what the plans and specifications read relate to: Technical specifications |
| 1.1A. | Specify codes and construction requirements reviewed relevant to the site location and type of building reviewed: NCC C1.28 |
| 1.1E. 1.1F. | Specify how the environmental impact of the materials was evaluated: Transportation impact and storage on site |
| 1.1G. 1.1H. | Specify how impact of allowable tolerances on the conversion of naturally occurring materials was determined: Risk assessment |
| 1.2H. 1.2I. | Specify how expert advice from manufacturers, architects, designers and engineers was achieved: Wall was engineered to ensure 60 minute fire rating |
| 1.2J. 1.3A. 1.3B. 1.3C. 1.3D. 1.3E. 1.3I. | Specify action taken when non-compliant or damaged materials or components were delivered: Contacted supplier and returned damaged items, then arranged new material |
| 1.3F. | Specify how on site storage location for materials was determined: Area was cleared and ensured no moisture was present,, then material was covered and protected by temporary fencing |
| 1.3G. 1.3H. | Specify workplace safety requirement instructed to others when storing and accessing stored materials and components: Micheal instructed technicians to boards in 1 at a time with 2 tecs on each board then lay them flat on the bracing on the ground |
| 1.3J. | Specify records maintained: All boards were checked and counted as they came off truck then ticked of material list |

CPCCBC4006

Select, procure and store construction materials for building and construction projects

| Practical Task 1 |  |
| --- | --- |
| Description of task: The participant must demonstrate the skills and knowledge to select, procure and supervise the delivery and on-site storage of materials for one construction project for a residential or commercial building up to a maximum of two storeys including: - Identify and evaluate building materials. - Locate and read current and relevant plans, specifications, codes and construction requirements. - Research properties of specified materials. (HO1 template) - Assess the suitability of materials for the building project in the region. (HO1 template) - Evaluate materials based on their quality, compatibility and compliance. (HO1 template) - Analyse reports, manufacturer specifications and other reference material regarding material technologies, sustainability and environmental efficiency. (HO1 template) - Determine the environmental impacts of materials. - Determine impact of allowable tolerances on the conversion of naturally occurring materials. - Establish material assembly and installation tolerances to meet requirements of relevant construction industry standards. - Select and procure building material. - Select materials that are structurally appropriate for the building system specified in the contract. (HO1 template) - Select materials for their safety, fire resistance rating, suitability, durability, serviceability and cost effectiveness. (HO1 template) - Consider short and long-term degradation of materials in relation to the building's proposed life cycle. (HO1 template) - Evaluate and consider alternative materials if specified materials are unavailable or unsuitable. (HO1 template) - Finalise selection of materials for use. (HO1 template) - Ensure that materials are selected in accordance with contractual requirements and in consultation with relevant professionals and the client. - Procure and order materials following workplace purchasing requirements. - Seek expert advice from manufacturers, architects, designers and engineers about selecting and procuring alternative materials. - Collect evidence of suitability from suppliers or manufacturers. - Ensure non-compliant materials are not used. - Supervise on-site delivery and storage of materials. - Determine limitations and effects of transportation on materials and components. - Take action to limit potentially damaging circumstances related to transport and storage. - Check delivered materials for compliance with specifications, quantity, quality, defects, or damage. - Check delivered materials for damage and ensure they correspond with delivery order. - Test and measure products against relevant industry standards - Allocate on-site storage space and safe and effective storage of materials and components. - Instruct relevant persons of workplace safety requirements when accepting and storing delivered materials and components. - Supervise safe handling of materials and ensure the security and protection of materials meets manufacturer specifications. - Inform relevant persons of actions relating to quality and inconsistencies of delivered building materials. - Maintain records of deliveries, damaged and inferior materials and variations to specified materials. These tasks may be simulated or real. The Participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. The Observer is to provide detailed comments on the tasks completed by the Participant (on the Observer Comments, overleaf). The Assessor is to verify the Observer’s comments, noting their findings in the Assessor Comments (on the back of the Practical Assessment cover page) and provide the assessment outcome. |  |
| Resources required: A Building and construction project; access to stakeholders; Workplace policies and procedures; Quality documents; access to relevant government legislation, the NCC and other relevant codes and standards; Construction drawings, site plans and specifications; Manufacturer reports on materials; Digital devices, applications and software. |  |
| Observer details |  |
| Name of observer: | Bruce Davie |
| Role of observer (Confirm suitability to sign) | General Manager |
| Email address of observer | bdavie@core4service.com |
| Signature of observer: |  |
| Date: | 26/02/2024 |

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |
| --- | --- |
| 1 Identify and evaluate building materials |  |
| Locate and read current and relevant plans, specifications, codes and construction requirements. |  |
| Research properties of specified materials. |  |
| Assess the suitability of materials for the building project in the region. |  |
| Evaluate materials based on their quality, compatibility and compliance. |  |
| Analyse reports, manufacturer specifications and other reference material regarding material technologies, sustainability and environmental efficiency. |  |
| Determine the environmental impacts of materials. |  |
| Determine impact of allowable tolerances on the conversion of naturally occurring materials. |  |
| Establish material assembly and installation tolerances to meet requirements of relevant construction industry standards. |  |
| 2 Select and procure building material |  |
| Select materials that are structurally appropriate for the building system specified in the contract. |  |
| Select materials for their safety, fire resistance rating, suitability, durability, serviceability and cost effectiveness. |  |
| Consider short and long-term degradation of materials in relation to the building's proposed life cycle. |  |
| Evaluate and consider alternative materials if specified materials are unavailable or unsuitable. |  |
| Finalise selection of materials for use. |  |
| Ensure that materials are selected in accordance with contractual requirements and in consultation with relevant professionals and the client. |  |
| Procure and order materials following workplace purchasing requirements. |  |
| Seek expert advice from manufacturers, architects, designers and engineers about selecting and procuring alternative materials. |  |
| Collect evidence of suitability from suppliers or manufacturers. |  |
| Ensure non-compliant materials are not used. |  |
| 3 Supervise on-site delivery and storage of materials |  |
| Determine limitations and effects of transportation on materials and components. |  |
| Take action to limit potentially damaging circumstances related to transport and storage. |  |
| Check delivered materials for compliance with specifications, quantity, quality, defects, or damage. |  |
| Check delivered materials for damage and ensure they correspond with delivery order. |  |
| Test and measure products against relevant industry standards |  |
| Allocate on-site storage space and safe and effective storage of materials and components. |  |
| Instruct relevant persons of workplace safety requirements when accepting and storing delivered materials and components. |  |
| Supervise safe handling of materials and ensure the security and protection of materials meets manufacturer specifications. |  |
| Inform relevant persons of actions relating to quality and inconsistencies of delivered building materials. |  |
| Maintain records of deliveries, damaged and inferior materials and variations to specified materials. |  |

Observer comments: