RIICPL401D

Apply the principles for the installation of underground service using open excavation

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Jordan Booth |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | Click here to enter a date. |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 20 |  | Yes | No |

Assessor Comments

- List two (2) types of statutory compliance that is relevant to installing underground service using open excavation.

| Answer | Workplace Health & Safety Act 2011 Electrical Safety Act 2002 |  |
| --- | --- | --- |

- Where, in your workplace, would you locate procedures relevant to the installation of underground service using open excavation?

| Answer | Refer to the companies Business procedures, ie Safe Work Procedure, Standard operating Procedure (SOP) or Safe Work Method Statement (SWMS) |  |
| --- | --- | --- |

- Identify two (2) methods used to identify and locate other related underground services and utilities.

| Answer | Contact Dial Before You Dig (DBYD) Use DYBD Accredited contractor to undertake Ground Penetrating Radar (GPR) |  |
| --- | --- | --- |

- Identify two (2) engineering survey principles that relate to the installation of underground service using open excavation.

| Answer | Providing a framework (Survey Control) Planning – Define a job specification |  |
| --- | --- | --- |

- In the space below, identify and explain how one (1) environmental management principle impacts on the task of installing underground service using open excavation.

| Answer | Principle | Explanation |  |
| --- | --- | --- | --- |
|  | During Installation | Installation of services may require clearing of an area that will have an impact on the native flora and fauna. Trench spoil could create sediment & erosion control issues. |  |

- Describe three (3) processes that are involved in developing a ‘work plan’.

| Answer | Planning Process | Description |  |
| --- | --- | --- | --- |
|  | Familiarisation | Review all relevant project documentation including project drawings, plans specification, resources and constraints to familirase with the requirements of the project |  |
|  | Prepare Work Breakdown Structure (WBS) | List all of the key tasks /steps required to complete the project |  |
|  | Allocate resources, time and cost | Allocate resources, time and costs to each of the items identified in the WBS |  |

- Describe two (2) estimating principles that are relevant to you in planning for the installation of underground service using open excavation.

| Answer | Estimation Principle | Description |  |
| --- | --- | --- | --- |
|  | Scope Definition | Gain clear understanding of the scope of works requirements and site constraints ie Quantities of conduits, depth of trench, interaction with existing assets |  |
|  | Execution strategy | Determine how the project will be completed. What resources will be required, ie conduit materials, service location, bedding material, plant and labour |  |

- Identify three (3) principles of leadership and how they apply to you in your role as a Supervisor in the installation of underground service using open excavation.

| Answer | Leadership Principle | Application in the workplace |  |
| --- | --- | --- | --- |
|  | Communicate safety & project related information | Conduct prestarts and toolbox talks onsite to convey key safety information – Location of assets, review permits and SWMS etc |  |
|  | Manage risk | Ensure SWMS and excavation permits are prepared and in place prior to works being undertaken |  |
|  | Embed safety practices | Lead by example on site and set high expectations |  |

- List three (3) key factors that influence the selection of operational techniques when planning to install underground service using open excavation.

| Answer | Site constraints including available access and working space Size of the asset to be installed and depth of trench Ground conditions and stability |  |
| --- | --- | --- |

- List three (3) key factors that influence your selection of resources when planning to install underground service using open excavation.

| Answer | The size, depth and length of the installation of the service Ground conditions and stability Site constraints including available access and working space |  |
| --- | --- | --- |

- Explain the principle of scheduling tasks and activities.

| Answer | Estimate the time required for each task. Determine the time required for project completion Establish the time intervals in which each activity must start and finish Identifying the activities crucial to a timely project completion (critical path) |  |
| --- | --- | --- |

- In your workplace, what are two (2) key things you must consider when scheduling the delivery of materials to site.

| Answer | Ensure lead time is considered to ensure materials are ordered early enough that they arrive on site when required. Not ordering materials too early if there is no room on-site to store them close to the work area |  |
| --- | --- | --- |

- In the generalised list below, number each task in order of sequence for the installation of underground service using open excavation.

| Answer | Order | Tasks |  |
| --- | --- | --- | --- |
|  | 1 | Plan the work |  |
|  | 8 | Backfill the trench |  |
|  | 5 | Prepare and maintain batters |  |
|  | 3 | Prepare the work environment |  |
|  | 7 | Enclose with bedding materials |  |
|  | 4 | Excavate the trench |  |
|  | 10 | Update service plans and drawings |  |
|  | 2 | Check for any underground or overhead services |  |
|  | 6 | Lay service piping & install services |  |
|  | 9 | Clean up work area |  |

- List three (3) key aspects relevant to managing open excavation underground service installation.

| Answer | Treatment of the trench with appropriate controls to make safe, ie battering, benching, shoring etc Ensure so far as reasonably practicable that the works area is secure from unauthorised / inadvertent access ie fencing / delineation Ensure appropriate resources are available to complete the works in a safe and timely manner |  |
| --- | --- | --- |

- List five (5) daily tasks you conduct (as the Project Manager / Lead), when managing client and site operational requirements, in relation to the installation of underground service using open excavation.

| Answer | Conduct site prestart meeting Induct new personnel Update and sign on to Excavation Permit Supervise the works and complete site inspection report Complete daily supervisors report Communicate with the workforce Plan umpcoming works and ensure appropriate resources are available onsite |  |
| --- | --- | --- |

- From the list below, which areas of civil works are related to the installation of underground service using open excavation? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Environmental conservation |  |
|  |  | Infrastructure development |  |
|  |  | Bridge construction |  |
|  |  | Commercial development |  |
|  |  | Rail construction |  |
|  |  | Lifting and cranage |  |
|  |  | Road construction/maintenance |  |
|  |  | Pipeline construction |  |
|  |  | Ship building |  |
|  |  | Industrial development |  |
|  |  | Systems development |  |
|  |  | Road transport |  |

- Identify two (2) places where you can locate information that relates to plant and equipment capabilities and operation.

| Answer | The OEM Manual Manufacturers representatives / trainers Organisation’s records management system |  |
| --- | --- | --- |

- Describe the process for maintaining plant, equipment, and tools.

| Answer | Plant equipment and tools should be maintained in accordance with the OEM requirements, specification and service schedule as well as some high risk plant requiring compliance checks at intervals by Engineers and regulators. This may include: Daily prestart checks Regular inspections Maintenance and service checks |  |
| --- | --- | --- |

- In your own words, explain three (3) terms that are commonly used in the installation of underground service using open excavation.

| Answer | Benching – Stepping the installation at 1:1 ratio for batter stability, typically where trench is deeper than 1.5m Battering – Battering the side of an excavation, typically at 45 degrees for trench stability, typically where trench is deeper than 1.5m Shoring: Using a structural device to retain the side walls of a trench |  |
| --- | --- | --- |

- Calculate the following:

- What is the total volume of the shape below?

1.56m x 1.85 x 3.725 =10.75m3

- What is the surface area of the shape below?

2(1.56x1.8) + 2(3.72*1.8) + 2(3.72*1.56) =30.6144m2

- In the shape below, calculate the grade of the dotted line using the grade ratio of 1:10. What is the total grade amount?

Grade = rise over run =1560/3725 = 0.4763588 = 1:2.387821

Not sure why it says the grade ration is 1:10?

Depth 1850mm

Length 3725mm

Width 1560mm

Grade 1:10

- You need an excavator for 3 days and have been asked to make sure there is enough diesel to last the whole 3 days.  The excavator uses 15 litres per hour and will be operating for approximately 6 hours per day.  The company buys its diesel at cost ($1.10 per Litre) and it has a policy that requires all orders include a contingency of 10%.

  - What is the total consumption of diesel for the 3 days?

3x6x15 = 270L

  - How much diesel will you need to order?

Allow for contingency 10% = 270*1.1 = 297L

  - What is the total cost of your order?

$297 * $1.10 = $326.70 + GST if applicable