Fire Resistance Checklist

This checklist ensures compliance with fire resistance requirements for buildings as per the National Construction Code (NCC) and relevant Australian Standards.

Building Details

Building Class: Class 2
Type of Construction: B
Building Location: Simulation Brisbane
Building Use: Multi-residential apartments

Checklist Items

1. Fire Resistance Levels (FRLs)

- Walls:

  - External walls meet the required FRL of: //___ (e.g., structural adequacy, integrity, insulation).

  - Fire separation between compartments meets FRL requirements.

  - Non-combustible materials used where required (e.g., external walls in certain construction types).

- Floors and Ceiling Systems:

  - Floors meet the required FRL of: //___ as per NCC.

  - Ceiling membranes provide adequate fire resistance.

- Columns and Beams:

  - Structural elements meet the required FRL of: //___.

  - Reinforced concrete or steel members are protected against fire.

2. Passive Fire Protection

- Fire Separation:

  - Fire-resistant barriers installed between sole-occupancy units.

  - Vertical fire separation provided between floors.

- Fire Doors:

  - Fire-rated doors installed with the required FRL.

  - Doors self-closing and compliant with AS 1905.1.

- Service Penetrations:

- Openings for services (e.g., pipes, ducts) sealed with fire-resistant materials.

  - Compliance with AS 4072.1 for service penetration protection.

- Glazing:

  - Fire-resistant glazing installed where required.

- Compliance with AS 1288 for windows and AS 1530.4 for fire testing.

3. Active Fire Protection

- Fire Detection and Alarm Systems:

  - Smoke alarms and detection systems installed as per NCC and AS 1670.1.

  - Systems functional and tested regularly.

- Sprinkler Systems:

  - Installed and compliant with NCC requirements for active fire suppression.

  - Compliance with AS 2118 for sprinkler systems.

- Fire Extinguishers and Hose Reels:

  - Fire extinguishers installed as per NCC and AS 2444.

  - Hose reels installed where required (Class 5–9 buildings).

4. Compartmentation

- Building Compartments:

  - Compartments sized and separated as per NCC Part C2 requirements.

  - Compliance with maximum floor area and volume limits for fire compartments.

- Egress Pathways:

  - Fire-isolated stairways, corridors, and ramps installed with required FRLs.

  - Exit doors and signage installed as per NCC.

5. Materials and Testing

- Non-Combustible Materials:

  - Materials used in external and internal fire-rated elements comply with NCC and AS 1530.1.

- Evidence of Suitability:

  - Certificates of compliance or test reports provided for fire-rated materials.

- Products meet NCC specifications and relevant Australian Standards.

6. Maintenance and Inspection

- Active fire systems (alarms, sprinklers) tested and maintained as per AS 1851.

- Passive systems (fire doors, barriers, seals) inspected for integrity.

- Records of fire system maintenance kept on-site.

Comments/Notes

No major non-compliances identified.

Fire separation and active systems meet NCC requirements.

Checklist Completed By

Name: Abbie Heaslip
Date: 8/7/25
Signature: