CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4002A Manage occupational health and safety in the building and construction workplace cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Harley arrowsmith |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Harley arrowsmith |
| --- | --- |
| Participant signature |  |
| Date | 20/04/2022 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 28 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

For each of the common building and construction jobs listed below, identify one (1) hazard specific to that job.

| Answer | Job | Risks |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Lacerations if using equipment such as power saw |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Sever injury/death |  |
|  | Crane use | Falling objects |  |
|  | Demolition | Dust inhalation/falling objects |  |
|  | Formwork | Fall from heights/lacerations |  |
|  | Scaffolding use | Falling objects/fall from heights |  |
|  | Work at heights | Falling from heights |  |
|  | Welding | Burn/eye damage |  |

Using the risk table below, determine the level of risk for each of the hazards identified in Q1. Identify how each risk level was determined, by identifying the likelihood and consequences of each hazard.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

| Answer | Job | Level of risk |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | H Possible serious injury |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | H Possible death or permanent disability |  |
|  | Crane use | H Unlikely death or permanent disability |  |
|  | Demolition | H Possibly serious injury |  |
|  | Formwork | H Possibly serious injury |  |
|  | Scaffolding use | H Possibly serious injury |  |
|  | Work at heights | H Possible serious injury |  |
|  | Welding | H possibly serious injury |  |

Identify an appropriate control measure for each of the hazards identified in Q1.

| Answer | Job | Control measure |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Ensure guards are fitted and all PPE is worn. |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Ensure all safety checks of equipment has been carried out and equipment is used to manufactures specifications. PPE is worn and work areas are clear |  |
|  | Crane use | Safety checks of machinery and chains, set up correctly and exclusion zones are in place. |  |
|  | Demolition | Dust control measures in place such as vacuums or the use of water to control dust. Exclusion zones are in place. PPE |  |
|  | Formwork | Exclusion zones, edge protection, PPE, all equipment tested and tagged. |  |
|  | Scaffolding use | Edge protection in place, exclusion zones in place, only qualified scaffolders to construct and dismantle scaffold. |  |
|  | Work at heights | Edge protection in place, harnesses if required, PPE |  |
|  | Welding | All equipment checked prior to use, correct PPE to be worn. Shielding up |  |

True or False (Tick your response): Risk control options can come from both internal sources (e.g. policies and procedures) and external sources (e.g. codes and standards).

| Answer | þ | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Name two (2) workplace personnel you may speak to for safety advice.

| Answer | HSR and supervisor |  |
| --- | --- | --- |

Referring to the WHS Regulations, explain how the following documents contribute towards identifying hazards and risks in the workplace.

| Answer | Document | Description |  |
| --- | --- | --- | --- |
|  | Safe Work Method Statements | It must identify the work to be conducted that is high risk, state hazards relating to that, describe measures to be implemented to control risk and describe how the control measures are to be implemented, monitored and reviewed |  |
|  | Site Safety Plan / WHS Management Plan | The site safety plan must include the arrangements in place for managing any work health and safety incidents that occur the arrangements for collection and any assessment, monitoring and review of safe work method statements at the workplace |  |

Identify two (2) advisory standards you might refer to safely undertake building and construction work and briefly summarise their requirements.

| Answer | Standard name | Requirements |  |
| --- | --- | --- | --- |
|  | Working at heights standard | A SWMS is required where work carried out at heights greater than 2 metres. Fall prevention is mandatory. |  |
|  | Confined spaces standard | This standard requires that adequate steps be taken to eliminate or control hazards it also requires that all persons involved in the entry of a confined space be trained and instructed on the nature of the hazards and the precautions to be followed |  |

Building and construction industry contracts require building legislation and the National Construction Code to be observed. Refer to these documents when responding to Q8. to Q10.

Identify the requirements in the National Construction Code Volume 1 with regards to air pressurisation for fire-isolated exits. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | E2.2a smoke detection for smoke control systems | Smoke detectors required to activate air pressurisation systems for fire isolated exits and zone control systems must be installed in accordance with AS1670.1 Have additional smoke detectors installed adjacent to each bank of lift landing doors set back horizontally from the door openings by a distance of not more than 3m |  |

Identify the requirements in the National Construction Code Volume 2 with regards to smoke alarms. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | 3.7.5.2 smoke alarm requirements | Smoke alarms must— (a) be located in— (i) Class 1a buildings in accordance with 3.7.5.3 and 3.7.5.5; and (ii) Class 1b buildings in accordance with 3.7.5.4 and 3.7.5.5. (b) comply with AS 3786, except that in a Class 10a private garagewhere the use of the area is likely to result in smoke alarms causing spurious signals, any other alarm deemed suitable in accordance with AS 1670.1 may be installed provided that smoke alarms complying with AS 3786 are installed elsewhere in the Class 1 building; and (c) be powered from the consumer mains source where a consumer mains source is supplied to the building; and (d) be interconnected where there is more than one alarm. |  |

Name the building legislation (Act and Regulations) that apply within your jurisdiction.

| Answer | Building act 1975 QLD Building regulation 2021 QLD |  |
| --- | --- | --- |

Which of the following should be considered when developing a risk control plan? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | How and when controls will be implemented |  |
|  | þ | Evaluation processes for implemented controls |  |
|  | þ | Consultation with individuals and parties |  |
|  | þ | Responsibilities for implementation |  |

True or False (Tick your response): If control measures do not adequately control risks, they must be revised.

| Answer | þ | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following are appropriate techniques, tools and processes for identifying hazards and risks? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | Workplace inspections |  |
|  |  | Compensation claim information |  |
|  | þ | Safe Work Method Statements |  |
|  | þ | Information issued by WHS regulators |  |

Which of the following can be used to assist with workplace inspections? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | Hazard and risk checklists |  |
|  | þ | Job Safety Analysis (JSA) |  |
|  |  | Manifests and registers |  |
|  |  | Surveys |  |
|  | þ | Workplace walk-through |  |

Which of the following are limitations of using a generic hazard/risk assessment checklist? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | Specifics of the workplace are not considered |  |
|  | þ | Not all checklist points will be applicable |  |
|  |  | Such checklists/processes must be completed on a computer |  |
|  |  | You always have to purchase these checklists |  |

Identify the applicable WHS Act in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Act name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety act 2011 part 2 health and safety duties 26A | A PCBU must comply with the code or manage hazards and risks carried out in a way that is different to the code but still provides a standard of health and safety equivalent to or higher than the standard required under the code |  |

Identify the applicable WHS Regulation in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Regulation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety regulation 2011 Chapter 6 construction work part 6.3 division 1 Part 3.1 | A PCBU must manage risks associate with carrying out construction work in accordance with Part 3.1 A requirement not to exceed an exposure standard A duty to implement a specific control measure A duty to assess risk |  |

Identify a relevant policy in your organisation with regards to identifying hazards and risks and briefly describe its requirements.

| Answer | Policy name | Requirements |  |
| --- | --- | --- | --- |
|  | Hazard identification and control policy | Identify the Hazard Assess the risk Implement risk controls Monitor and review |  |

Identify the statutory requirements in your jurisdiction with regards to the completion of inspection reports.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health safety act 2011 103E | Prepare a report for the assessment carried out under section 103D(1) and if the assessment identifies and risks to health and safety arising from the work carried out include in the report recommendations about managing risks. The work health and safety officer must withing 30 days after the assessment has been carried out give a copy of the assessment report to the person conducting business or undertaking (if the WHSO is not the PCBU) or the committee (if the WHSO is not the PCBU) |  |

Which of the following individuals and parties may require information about workplace hazards and proposed risk controls? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | þ | Health and Safety Committees |  |
|  | þ | PCBU |  |
|  |  | All members of the public |  |
|  | þ | Workers |  |

Identify the requirements in the Code of Practice with regards to consulting parties when managing risks.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | How to manage work health and safety risks code of practice 2021 WHS Act section 47 consult workers WHS act section 46 Duty to consult with other duty holders | A PCBU must consult with workers who are or are likely to be directly affected by a health and safety matter. Consultation with workers and their health and safety representatives is required at each step of the risk management process The WHS act requires that a PCBU to consult, cooperate and coordinate with all other persons who have a work health or safety duty in relation to the same matter, so far is reasonably practical. |  |

Identify the requirements in the Code of Practice with regards to consulting parties when developing procedures for communicating WHS policy and practice requirements.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and Safety consultation cooperation and coordination code of practice 3.2 WHS Act | Workers must be given reasonable opportunity to express their views, raise WHS issues and contribute to health and safety decisions If there are HSR’s for the workplace you must include them in the discussions with or without the direct involvement of workers. |  |

Name three (3) parties who should consult to determine effective strategies for communicating WHS policy and practice.

| Answer | PCBU, Subcontractor, Health and safety representative |  |
| --- | --- | --- |

Name two (2) communication strategies that may be used in the building and construction industry.

| Answer | Meetings, Email |  |
| --- | --- | --- |

Name two (2) education programs that may be implemented in the building and construction industry.

| Answer | General and site-specific induction training Other forms of specialist and targeted training |  |
| --- | --- | --- |

Fill in the blanks for the statement below:

consulted

training

The ‘How to manage work health and safety risks Code of Practice 2011’ identifies that control measures are more effective when ______________________ is provided and when workers are _____________________.

| Answer |  |  |
| --- | --- | --- |

Explain why it is important to review the effectiveness of communications and educational programs.

| Answer | It is important to review the effectiveness of communications and educational programs to ensure that the likelihood of WHS hazards and risks are minimised, if they are unaffected some altercations may need to be put in place to ensure the programs are effective |  |
| --- | --- | --- |

List three (3) methods for reviewing the effectiveness of communication and educational programs.

| Answer | - Evaluating training programs for effectiveness - For example: reviewing incident reports/trends - Ensuring employees have an adequate understanding - Reviewing training materials for accuracy of content, currency and suitability - Conducting post-training surveys - Conducting regular reviews of training needs |  |
| --- | --- | --- |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Harley Arrowsmith |
| --- | --- |
| Participant signature |  |
| Date | 26/04/2022 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to conduct and report on a WHS risk analysis, including workplace inspection. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - A copy of a Site Specific Safety Plan or Construction Safety Plan, including: - Copies of Risk Assessments completed on a project - Copies of Safety Work Method Statements used on a project - A copy a workplace safety inspection checklist that has been used on a project The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Harley Arrowsmith |  |  |
| Name of observer: |  |  |  |
| Role of observer (Confirm suitability to sign) |  |  |  |
| Email address of observer |  |  |  |
| Signature of observer: |  |  |  |
| Date: | 26/04/2022 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Conduct a workplace inspection to evaluate construction site safety for the range of occupations in the workplace according to WHS legislation and company policies, including: - Identifying construction hazards and potential risk areas - Identifying and prioritising hazards - Identifying and prioritising risks |  |  |  |
| - Seek expert advice as appropriate, including: - Advice from workplace personnel regarding the application of building codes and standards - Consulting with relevant parties regarding compliance with statutory requirements (including SWMS and site safety plans) - Consulting with appropriate personnel to determine effective strategies for communicating WHS policy. |  |  |  |
| - Complete an inspection report in accordance with best practice and statutory obligations: including: - Recommendations from findings - Required approaches to remediation |  |  |  |
| - Implement agreed control measures in conjunction with relevant workplace personnel, implementing negotiation and conflict resolution skills, as necessary. |  |  |  |
| - Monitor and review control measures, updating records with findings |  |  |  |
| - Establish communication strategies and educational programs specific to the building and construction industry in accordance with statutory requirements and best practice. |  |  |  |
| - Review the effectiveness of communication and educational programs, updating records with findings. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments