RIIMPO402D

Apply the principles of earthworks construction

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Brian Miller |
| --- | --- |
| Participant signature |  |

| Assessor name | Brett Stoward |
| --- | --- |
| Assessor signature |  |

| Date | 29/05/2019 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 45 |  | Yes | No |

Assessor Comments

- Compliance documentation and reporting:

- Give one (1) example of each of the following types of compliance documentation relating to earthworks construction:

- Legislation.

- Organisational requirements.

- Client requirements.

- Site operational requirements.

- What are two (2) government agencies or statutory authorities you may have to report to?

| Answer | - Legislation - Occupational Health & Safety Organisational requirements - Workplace relations Client requirements - Client standards and expectations Site operational requirements - Emergency response and evacuation procedures - Workcover, EPA |  |
| --- | --- | --- |

- Administrative documentation and project quality management:

- List three (3) types of administrative documentation that you may have to complete.

- What are three (3) factors of project quality management?

| Answer | - Daily Activities, Ordering and re-ordering, Pay and other worker entitlements - Material Standard, Work standards, Client standards and expectation |  |
| --- | --- | --- |

- What is the aim of OHS/WHS legislation?

| Answer | The aim of the legislation is to ensure the safety of everyone working on or visiting the site |  |
| --- | --- | --- |

- List three (3) requirements that may be covered by the environmental management plan for your worksite?

| Answer | - Dust management. - Waste/clean-up management. Water quality protection |  |
| --- | --- | --- |

- What are the three (3) plans that outline the requirements and procedures for controlling traffic and people around and through the work zone?

| Answer | Traffic Management plan, Vehicle Management plan, Site Isolation plan. |  |
| --- | --- | --- |

- Why is it important to consult and communicate with others during the risk management process?

| Answer | Identifying risks and hazards and coming up with ways of controlling them includes talking to the people with knowledge of the situation, or who are directly affected by any action you may take. |  |
| --- | --- | --- |

- What should you do if you don’t understand the meaning of a particular term, phrase or word used on your worksite?

| Answer | seek further clarification at team meetings or speak directly with your manager. |  |
| --- | --- | --- |

- Geotechnical, geological, survey and engineering data:

- List three (3) ways that geotechnical data is used in earthworks.

- List three (3) uses of soil or geological data.

- What is survey and engineering information used for?

| Answer | - Identify soil and rock types, check conditions and materials on and under the ground surface, Determine their physical/ mechanical and chemical properties, - Defining material types, identifying wet and dry areas, Locating water tables - Survey and Engineering information is used to define work areas and zones. |  |
| --- | --- | --- |

- Hydrological and meteorological data:

- Why is hydrological data important?

- List three (3) factors that may be included in meteorological data.

| Answer | - Hydrological data is important as it mainly concerns surface water & ground water on the site, and it is critical to the success of earthworks operations - Rainfall, Humidity, Rainfall |  |
| --- | --- | --- |

- Project plans and specifications:

- What are three (3) things provided in project plans and drawings?

- What is the purpose of project and task specifications?

| Answer | - Location of the site and earthworks in relation to the surrounding area. - The position of structures, roads, access areas. - Layout of drainage lines - Project task & specifications list the types & characteristics of particular earth works tasks to be carried out. |  |
| --- | --- | --- |

- Drainage and dewatering:

- List two (2) examples of drainage and dewatering requirements.

- Why must any drainage and dewatering issues be managed promptly and effectively once you have identified them?

| Answer | Any form of erosion or sediment control, Pumping out of sumps or pits The most important part about draining and dewatering a construction site is that no dirty or contaminated water or pollution must leave the site. If this happens, severe penalties can apply that can be imposed by federal, state and local government authorities. |  |
| --- | --- | --- |

- Why is it important that earthworks construction activities are scheduled within a job plan?

| Answer | As most earthworks’ projects have strict timelines to be met it is important to have a job plan or program of work. In your role as supervisor or technical specialist you need to ensure the availability of a job plan that makes the most efficient use of the resources on site and meets all task requirements |  |
| --- | --- | --- |

- Soil density and testing:

- Why do you need to test the relationship between soil density and moisture when scheduling activities?

- What does ‘plasticity index’ mean?

- Why is it important for materials to have the right balance of particle sizes?

| Answer | - To gain adequate compaction it is necessary to have the density and moisture in balance. The plasticity index (PI) determines how the materials can be shaped and will relate to the clay content within the material. - It is important to have the right balance of particle sizes to ensure the correct construction of layers. |  |
| --- | --- | --- |

- Earthwork calculations:

- How do you calculate the following in earthworks construction:

- Grade or slope.

- Volume.

- Area.

- What is percentage or cross fall?

- How do you work out the rates of resource consumption?

| Answer | Grade = Rise / Run, Volume = Length x Width x Depth, Area =Width x Depth - This is generally the slope placed on earthworks to ensure water can drain off the work area. For example, a 3% cross fall would equate to a 3-metre rise across a 100-metre area - Daily production figures will often be used as a guideline to work out consumption rates. |  |
| --- | --- | --- |

- Sequencing of civil works task:

- How does an understanding of the relationship between the different areas of civil works help in planning work?

- What is the sequence of tasks for drainage works?

| Answer | - As a supervisor you may be involved in planning which civil construction work activities will be completed and in what order. You therefore need to understand the relationship between the different areas of civil works and the sequences of the tasks involved. - Forming drain lines, forming out the work area, that is building the frame to hold the drain system, laying pipes, box drains or culverts, Backfilling, Landscaping as appropriate. |  |
| --- | --- | --- |

- Job planning and administration:

- List four (4) requirements that should be included when drafting a job plan.

- What are three (3) examples of administration procedures and requirements?

| Answer | - Human Resources, Plant and Machinery, OHS/WHS, Risk Management - Procurement, Supply Agreements, Accounts Procedures |  |
| --- | --- | --- |

- Interpret requirements and provide estimates:

- Why is it important to correctly interpret the requirements of the project contract?

- What are two (2) factors you may have to provide estimates for, and how do you go about it?

| Answer | - It is important to correctly interpret the requirements of the project contract to ensure that all earthworks plan, client requirements and quality standards are known, understood and achieved - You may be required to estimate the costs and/or time factors for a job. Basically, to estimate the cost of completing a task you add the cost of production to the cost of materials. |  |
| --- | --- | --- |

- List four (4) types of resources that will need to be available and ready for earthworks construction tasks.

| Answer | Labour, Plant and Tools, Construction Materials, Sub- contractor services |  |
| --- | --- | --- |

- How can you be sure that you have followed the relevant resource requirements and procedures?

| Answer | You will need to confirm that you have accounted for these by checking the project and task documentation |  |
| --- | --- | --- |

- How do you confirm that plant and equipment is suitable for the task?

| Answer | This will mean checking the intended application or use of each item is in line with its capabilities and limitations. |  |
| --- | --- | --- |

- Provide instructions and supervise team members:

- List three (3) ways you can provide clear and timely instructions to the work team.

- List three (3) team members and others who may be involved in the earthworks activities you are supervising.

| Answer | - Email. - Instant messaging. - Message boards. - Engineering staff. - Administrative staff. - Other people necessary to the operations. |  |
| --- | --- | --- |

- List two (2) factors that may be detailed in your worksite communication requirements and procedures?

| Answer | Documentation requirements, e.g. verbal or written reports. Protocols for using communications equipment. |  |
| --- | --- | --- |

- Prepare messages and instructions:

- Why is it important to prepare for and conduct briefings, toolbox and site meetings?

- What should you keep in mind when preparing and delivering short messages or instructions?

| Answer | - As a supervisor you need to ensure that thorough preparation goes into these briefings and meetings so that appropriate and relevant information is conveyed to the team - When preparing messages or instructions, make sure they are appropriate and not confusing for the receiver. It helps to remember the target audience you’re aiming your message at |  |
| --- | --- | --- |

- Why is it important for a leader/supervisor to develop and maintain workplace relationships?

| Answer | It promotes a good work environment and encourages effective interactions between team members, supervisors, management, and any additional people that may be involved with the work activities. |  |
| --- | --- | --- |

- What factors will determine the leadership style and techniques you adopt?

| Answer | The leadership techniques you adopt will depend on the nature and dynamics of the group and the experience level of the people concerned. |  |
| --- | --- | --- |

- Supervisor techniques:

- What are four (4) things you need to be able to do as a supervisor?

- List the five (5) techniques commonly used in mentoring.

| Answer | - Communicate. - Organise. - Plan. - Manage people and resources - Accompanying, Sowing, Catalysing, Showing, Harvesting |  |
| --- | --- | --- |

- What do set out requirements and procedures include?

| Answer | While set out principles are the same across the civil construction industry, procedures and requirements for task set out will be determined by the type of earthworks being carried out and the specifications you are working under. Procedures will be established at the beginning of the project, but you may have to adjust or modify them according to the work location or geographical features of the site |  |
| --- | --- | --- |

- What are construction offsets and how are they established?

| Answer | Offsets refer to how far from a control line an activity is to take place, or an area is to be cleared in establishing construction offsets, quite often survey markers and level pegs are used to define the work area. |  |
| --- | --- | --- |

- List three (3) factors of road geometry.

| Answer | Grade or slope Curves and curve radius Crests. |  |
| --- | --- | --- |

- What are levelling devices used for in the setting out process?

| Answer | Determining drainage grades. Determining gradients, batters and benches. Measuring offsets. Helping with road geometry. |  |
| --- | --- | --- |

- What are the four (4) aspects of project and task monitoring that you must carry out?

| Answer | Ongoing risk assessment and management. Engineering survey. Sampling and testing. General supervision, observation and recording. |  |
| --- | --- | --- |

- Engineering principles and interpret test results:

- Why should earthworks operators be aware of the engineering survey principles for the job?

- Why do you need to interpret material properties and compaction test results?

| Answer | - It is important that all earthworks operators are aware of the relevant engineering survey principles that apply. Remember that not all work will be set out by surveyors, and in some cases it will be the supervisor or the operators of plant and machinery who will have to get compliance with specified tolerances for the earthworks - In the initial stages of the job, testing should be undertaken so that the properties are known and understood to allow for additional treatments when necessary. The testing results will indicate material variances throughout the works. Test results are also used to ensure levels of compaction are achieved based on specifications. |  |
| --- | --- | --- |

- Monitor earthworks tasks:

- List four (4) criteria that may relate to performance of the tasks you are monitoring.

- What are three (3) methods you might use for monitoring earthworks tasks?

| Answer | - Ongoing risk assessment and management. - Engineering survey. - Sampling and testing. - . Field checklists. - Inspection and test plans. - Site inspections that may be daily, weekly, or based on a work area. |  |
| --- | --- | --- |

- Why should routine inspections of earthworks operations be carried out?

| Answer | Consistent monitoring and inspection allow you to ensure that all tasks are carried out safely and efficiently and that the required outcomes are achieved. |  |
| --- | --- | --- |

- List four (4) methods and operational techniques involved in earthworks construction practice.

| Answer | Site preparation. Extraction. Load and haulage. Placement. |  |
| --- | --- | --- |

- How will you find out the operational techniques for the earthworks job you are working on?

| Answer | The operational techniques for the earthworks construction project you are working on will be outlined in the relevant procedures for the work activity. |  |
| --- | --- | --- |

- In ground surface treatment, what is proof rolling?

| Answer | A plant item such as a roller, loaded truck, or other nominated item may be used to see if there is any movement in the ground underneath. Movement under the wheels could indicate unsuitable or insufficiently compacted material in lower layers |  |
| --- | --- | --- |

- Give two (2) examples of how you should inform operators and other personnel/workers in writing and verbally that changes to the job plan or relevant earthworks construction practice have been initiated?

| Answer | Written communication – plans, sketches and drawings, work instructions or other documentation. Oral communication – briefings, toolbox meetings, verbal instructions, discussions with operators, team meetings, or relevant conversations based on the activity. |  |
| --- | --- | --- |

- What are four (4) maintenance activities that may be carried out by earthworks construction operators?

| Answer | Fitting and removing attachments. Minor repairs, e.g. replacing ripping teeth or boots. Helping the mechanics with maintenance and repairs. Recording and reporting of damage, defects or faults |  |
| --- | --- | --- |

- How can you ensure that your team carries out maintenance activities in line with site procedures and the manufacturer’s specifications?

| Answer | You may need to create a schedule to ensure that regular times are allocated to your team for cleaning, checking, repairing and carrying out other maintenance tasks. |  |
| --- | --- | --- |

- List four (4) reasons why you should follow organisational and/or site requirements and procedures for preparing and presenting reports.

| Answer | Activity planning, Ordering and tracing of materials, Budgeting , Legal requirements for Reporting |  |
| --- | --- | --- |

- What are three (3) guidelines you should keep in mind when preparing reports?

| Answer | Keep it clear and concise – use a style of language that is simple enough to be read quickly but technical enough to meet the needs of the audience. Pay attention to your grammar and make sure the information is clear. Proofread it – errors in your report reduce your credibility as well as that of your organisation. |  |
| --- | --- | --- |

- What are logbooks and diaries used for?

| Answer | Logs, logbooks and diaries – used to track daily and weekly activities. Logbooks are often used to verify the number of hours an operator has worked on a machine, training that has been completed or other task-related information. |  |
| --- | --- | --- |

- List four (4) things that you need to do when recommending changes or improvements to the safety, effectiveness or efficiency of tasks.

| Answer | Summarise the situation. Clarify why you are providing advice. Outline all the considerations. Explain the risks or hazards |  |
| --- | --- | --- |

- How do you develop individual workers or the team as a whole?

| Answer | Communication is essential to a successful relationship between team members and supervisors. Giving feedback to individuals and the team is an important part of leadership Communicate. Organise. Plan. Manage people and resources. Liaise with both management and your work team |  |
| --- | --- | --- |