CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | GENEVIEVE INCLAN |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 27/02/2019 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 87 |  | Yes | No |

Assessor Comments

Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

Which compass direction is always shown on plans and drawings?

| Answer | North |  |
| --- | --- | --- |

What information is provided on a location plan?

| Answer | A location plan details the: - Lot number/s - Deposit plan numbers - North point *Scale (usually 1:5000) |  |
| --- | --- | --- |

What detail does a site or block plan contain?

| Answer | A Site or Block Plan usually gives the following information: - Outline of site boundaries showing location of proposed building and existing dwellings - Street frontage names - Major geological and topographical features (trees and vegetation) - Datum points, contours and slopes - Driveways, stormwater drainage, easements and right of carriage - Soil and surface water drains, pipe sizes - Service runs from the house to mains - utility services location (sewer, water, gas, electricity) - Details of paving and retaining walls |  |
| --- | --- | --- |

What do floor plans identify?

| Answer | The key features to be identified and record in a floor plans may include: - Ceiling heights and variations - Doors - Light fittings and power supplies - Services - Wall penetrations - Walls |  |
| --- | --- | --- |

What is a landscape plan and what information might it provide?

| Answer | Landscape plan is one of the types of plans which shows details of plants, paths, garden layouts and retaining walls |  |
| --- | --- | --- |

What are elevation views? What do they provide to the plan reader?

| Answer | Elevation views View of the proposed structure from its sides (i.e. from north, south, east and west). Each side is what is called an elevation view. |  |
| --- | --- | --- |

What is a section?

| Answer | A section is a vertical view (slice) through the building. The direction of section lines or arrows is usually shown on the plan to help interpret the sectional view. |  |
| --- | --- | --- |

What are detailed drawings?

| Answer | Detailed drawings or plans is one of the types of plans that illustrate a particular method of construction of finish. |  |
| --- | --- | --- |

What are specifications?

| Answer | Specifications set out the minimum standards or codes required for all structural components and provide details of building materials and fittings, such as: - Windows, frame material and type of glass - Doors, size, material and the swing on the door - Finishes/colours to walls, floors and ceilings - Door types and models of locks, latches, hinges, push plates and pull handles |  |
| --- | --- | --- |

Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bull Nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Block |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Glass |  |
|  | CJ | Ceiling Joist |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold Water Tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Proof Course |  |
|  | DSB | Distribution Switch Board |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion Joint |  |
|  | FPBD | Fibrous Plaster Board |  |
|  | FFL | Finished Floor Level |  |
|  | FHR | Fire Hose Reel |  |
|  | GPO | General Purpose Outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Cavity brick |  |  |
|  | Brick veneer |  |  |
|  | Timber stud |  |  |
|  | Windows in Cavity wall |  |  |
|  | Windows in single wall |  |  |
|  | Single Leaves opening 90° |  |  |
|  | Double Leaves each opening 90° |  |  |
|  | Door sliding into a pocket |  |  |
|  | Doors sliding, exposed on wall |  |  |
|  | Power point single |  |  |

Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Hinged as side |  |  |
|  | Hinged at top |  |  |
|  | Sliding horizontally |  |  |
|  | Fixed |  |  |

What does a clouded area on a plan indicate?

| Answer | On plans, revisions or amendments are represented by a clouded area |  |
| --- | --- | --- |

An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

On what date was an amendment made to the window details?

| Answer | 5/10/2011 |  |
| --- | --- | --- |

True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What amendment occurred on 08/11/2011?

| Answer | Bed 3 robe deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

What is the Lot number of this site?

| Answer | Lot 16 |  |
| --- | --- | --- |

What is the size of this lot?

| Answer | 3653m2 |  |
| --- | --- | --- |

Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What size is the easement on the eastern boundary of the site?

| Answer | 5m easement |  |
| --- | --- | --- |

True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26. ?

| Answer | The building is a proposed residential house with 2 levels. The sectional drawing above gives detail with regards to the building materials to be used on the roof, walls, flooring and ceilings. Yes, it is the same structure proposed in Q19 to Q26. |  |
| --- | --- | --- |

Who is the client?

| Answer | H&M Smith |  |
| --- | --- | --- |

Where is this project to be built?

| Answer | Lot 16 Glenvalley Road, Richmond |  |
| --- | --- | --- |

What are the contact details for the draftsman?

| Answer | Joe Foreman Ph: 03 9514 2311 Mob: 0413 113 113 18 Fairview Road, Richmond 3216 |  |
| --- | --- | --- |

What sheet number is this drawing?

| Answer | Sheet 5 of 7 |  |
| --- | --- | --- |

What timber is required for loading and non-load bearing walls?

| Answer | F5 timber, 45x90 to load bearing & 35x90 to non load bearing |  |
| --- | --- | --- |

What spacing is recommended for common studs?

| Answer | 600 max spacing |  |
| --- | --- | --- |

What type of roof truss is to be installed?

| Answer | Gang nail roof trusses |  |
| --- | --- | --- |

What centres are the trusses positioned at?

| Answer | 900 max spacing |  |
| --- | --- | --- |

What size are the veranda posts?

| Answer | 90x90 |  |
| --- | --- | --- |

What timber species are the veranda posts to come from?

| Answer | DAR Oregon |  |
| --- | --- | --- |

What pitch is the roof?

| Answer | 30° pitch |  |
| --- | --- | --- |

What size are the noggings and at what spacing?

| Answer | 35x70 Noggings at 1350 max spacing |  |
| --- | --- | --- |

What size are the ceiling joists?

| Answer | 240 joists |  |
| --- | --- | --- |

What timber is used for roof battens and what spacing?

| Answer | 35x70 FG pine battens at 900 spacing every batten strapped to trusses |  |
| --- | --- | --- |

What thickness is the plasterboard sheeting?

| Answer | 10mm thick |  |
| --- | --- | --- |

What type of flooring is to be installed?

| Answer | 15mm Plywood flooring |  |
| --- | --- | --- |

What is the roof cladding material?

| Answer | Colorbond corrugated metal roofing |  |
| --- | --- | --- |

Where will you find information about the footings?

| Answer | Engineers drawing |  |
| --- | --- | --- |

What do the letters ‘DAR’ refer when timber ordering?

| Answer | Dressed-all round |  |
| --- | --- | --- |

Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | Starting Joist40x63 LVL LVL means laminated veneer lumber |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

What is the Job Number?

| Answer | 00R-112 |  |
| --- | --- | --- |

What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

What does the drawing show?

| Answer | The drawing shows the East, West, North & South Elevations and the Section of the building |  |
| --- | --- | --- |

Which view shows the front of the house?

| Answer | East elevation |  |
| --- | --- | --- |

How many motor vehicles can be parked in the garage?

| Answer | 2 motor vehicles |  |
| --- | --- | --- |

What are the window and doors made from?

| Answer | Aluminium framed windows, aluminium framed sliding door & windows, selected timber door |  |
| --- | --- | --- |

What is the size of the eaves?

| Answer | 150 |  |
| --- | --- | --- |

What are the eaves lined with?

| Answer | Hardiflex |  |
| --- | --- | --- |

What type of roof structure is the roof built from?

| Answer | Hip & Valley Roof |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about roof structure in Q59. ?

| Answer | NCC Volume 2, Part 3.5.1 |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | AS 1562. |  |
| --- | --- | --- |

What is the roof cladding materials made from?

| Answer | Concrete roof tiles |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | NCC Volume 2, Part 3.5.1.2 Roof Tiling |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | AS 2049 and AS 2050 |  |
| --- | --- | --- |

What is the roof pitch?

| Answer | 22.5° slope |  |
| --- | --- | --- |

Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | City Council |  |
| --- | --- | --- |

Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | The City Council assesses the application for building approvals to ensure it complies with all other legislative requirements When these requirements have been met, the permit is issued. The timeframes for an approval through City Council can take anywhere between 2 weeks and 4 months. |  |
| --- | --- | --- |

What is a prime cost (PC) item? Give an example.

| Answer | A PC item is: - An amount of money included in a contract sum to purchase a specified item such as tiles, taps, doors or bathroom fittings - An agreed estimated amount is included at contract signing but the specific products are not selected until a later stage - A builder must estimate the cost of such items at or above the lowest amount these items could reasonably cost For example if a client selects a more expensive door than allowed by the PC item, say a $500 door compared with a $440 PC amount, a variation will be required to cover the additional $60. If the PC item varies, so will the final contract sum. |  |
| --- | --- | --- |

What is a provisional sum (PS) item? Give an example.

| Answer | A provisional sum is: - An amount of money included in contract sum to cover work and/or materials, not detailed when entering the contract - Typically builders will include a PS for site work costs - The builder is legally required to take reasonable steps to ensure an accurate site works cost estimate but unforeseen events mean that the provisional sum is exceeded which is added to the contract sum For example a building site may seem sandy and clean with minimal siteworks required, however, there may be large limestone rocks/tree trunks concealed below the surface which only emerge once siteworks commence, causing cost overrun. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify two (2) materials being used for the floors?

| Answer | Carpet, Vinyl & Floating Floors |  |
| --- | --- | --- |

True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

How much does the contracted plasterer cost?

| Answer | $12,900.00 |  |
| --- | --- | --- |

How much do the kitchen appliances cost?

| Answer | $2,700.00 |  |
| --- | --- | --- |

What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 1 each |  |
| --- | --- | --- |

If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $2.75 x 1 each = $2.75 |  |
| --- | --- | --- |

How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 2400x1200 4mm ply = 3 sheets 2700x1200 4mm ply = 6 sheets 3000x1200 4mm ply = 4 sheets |  |
| --- | --- | --- |

What material has the highest total cost?

| Answer | Pine 90x45 mgp10 (long lengths) studs over 2.7 |  |
| --- | --- | --- |

How many metres are required of the material in Q80. ?

| Answer | 322.5 m |  |
| --- | --- | --- |

What beam is identified as being a better option than F17?

| Answer | E14 beams |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter (Road Construction) |  |
| --- | --- | --- |

If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $107.40 x 2 hours = $214.80 |  |
| --- | --- | --- |

Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar Bender and Fixer |  |
| --- | --- | --- |

If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $214.30 x 6 hours = $1285.80 |  |
| --- | --- | --- |

Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | - Building Act 1975 (Qld) - Building Regulation 2006 (Qld) |  |
| --- | --- | --- |