| Organisation Name | Matt Greenwood Constructions | ABN | 92163040186 |
| --- | --- | --- | --- |
| Organisation Address | Lot 202 Gillies Range Rd, Gordonvale 4865 |  |  |

| Project | Sarah Close, Gordonvale 4865 |
| --- | --- |

| Task / Operation / High Risk Activity | Install Roof Trusses |  |  |  |  |  |  | Number |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Applicable Legislation, Standards or References | QLD WH&S Act, QLD WH&S Regulation, QLD Mobile Crane COP, QLD Electrical Safety Act, QLD Electrical Safety Regulation, QLD How to Manage WH&S Risks COP, AS 4440 Installation of Nailplated Timber Trusses, AS 1576 Scaffolding, AS 2550 Cranes, Hoists and Winches |  |  |  |  |  |  |  |  |  |
| High Risk Construction Work | - Falls of more than 2 metres - Powered mobile plant |  |  |  |  |  |  |  |  |  |
| Required Licences, Certificates, Competencies, Permits or OHS Authorities | DG ( Dogger ), CN ( Non Slew Crane) or C2 ( 20t Slew Crane ) |  |  |  |  |  |  |  |  |  |
| Personal Protective Equipment (Tick PPE required) Ensure that all workers are trained in the use and maintenance of all PPE required for task by site supervisor | Safety Footwear | Safety Helmet |  | Respirator* | * Provide specific details of PPE required Harness and Lanyard as required. |  |  |  |  |  |
|  | Hearing Protection | High Vis |  | Other* |  |  |  |  |  |  |
|  | Eye Protection* | Gloves |  |  |  |  |  |  |  |  |
| Plant, Equipment or Tools | Crane |  |  | Maintenance Checks |  |  | Responsibility of subcontractor |  |  |  |
| Special/Hazardous Materials | (Ensure MSDS for hazardous Materials are available to workers) Mobile Scaffold, Bond Beam Trestles. |  |  |  |  |  |  |  |  |  |
| Emergency Procedures/Resources (Specific to this JSA-SWMS) Ensure that all workers are trained in these emergency procedures by site supervisor | General Emergency procedures are available in the Site Health and Safety Plan. First Aid procedures are as per site specific induction and Health and Safety Plan. |  |  |  |  |  |  |  |  |  |
| Environmental Impacts and Controls | Impacts |  | Controls ( If there is a potential impact define controls) |  |  |  |  |  |  |  |
|  | Hydro Carbons (oil and fuel spills) |  | Spill Kits |  |  | Designated Refueling and Servicing Areas |  |  | Other (specify) |  |
|  | Sediment Run Off |  | Silt Fences (Erection and Monitor) |  |  | Drainage |  |  | Containment Ponds |  |
|  | Dust |  | Watering |  |  | Site Speed Limits |  |  | Dust Containment Fences |  |

Job Sequence

| Risk levels: VH – Very High, H – High, M – Medium, L- Low |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence of job steps (What to do in the right order) |  | Potential hazards to people or the environment for each step (eg. Manual Handling) | Risk Level (each hazard) | Control Measures (How to do it) Elimination Substitution Engineering Administration PPE | Risk Level after Control | Person Responsible |
|  | Truss delivery truck access and truss unloading. | Blocking Traffic and Pedestrian Flow outside site. | M | Traffic controllers and signage are positioned in accordance with the MUTCD Part 3. | L | Site Supervisor |
|  |  | Persons being stuck by falling objects. | M | The unloading area is to be barricaded to deny access of other workers and pedestrians. Hard Hats are to be worn by all workers involved in the unloading process. High Vis clothing is worn by all workers. | L | Site Supervisor |
|  | Truss storage | Stored trusses blocking pedestrian flow and site access / egress. | M | Trusses stored in suitable area, if trusses block footpaths suitable barricading and signage to be erected. | L | Site Supervisor |
|  | Crane set up | Blocking Traffic and Pedestrian Flow outside site. | H | Traffic controllers and signage are positioned in accordance with the MUTCD Part 3. | L | Site Supervisor |
|  |  | Soft ground/underground services. | H | Assessment of the ground condition and location of underground services to be undertaken prior to crane set up. Suitable outrigger packing to be positioned on set up. | L | Site Supervisor/ Crane operator |
|  |  | Proximity to Power Lines. | VH | The location of power lines to be identified prior to operation. Authorised spotters or visual markers to be used after consultation with Power entity if the operation is closer than specified in the Code of Practice (SWNELP) 3m LV, 6m HV, 8m HV. Dry natural fibre rope tag lines are to be used when working near powerlines. | M | Site Supervisor/ Crane operator |
|  | Slinging of trusses | Faulty or damaged lifting gear. | M | All lifting gear to be inspected by a competent person (dogger, rigger) prior to use. | L | Dogger |
|  |  | Incorrect slinging method. | M | A licenced dogger or rigger must sling the truss. Slinging points must be in accordance with the manufactures’ recommendations. | L | Dogger |
|  | Lifting of trusses | Falling trusses | M | The work area must be barricaded and signed off to deny access to all other workers. Workers must not stand under load where possible | L | Site Supervisor |
|  |  | Controlling the truss | M | A tag line must be used to control the truss as it is lifted. | L | Dogger |
|  |  | Incorrect crane signals whilst directing the truss | M | A licenced dogger or rigger must direct the crane. | L | Dogger |
|  | Access work area | Persons falling from heights | H | All workers to maintain three points of contact when climbing ladders. | L | Site Supervisor, Carpenter |
|  | Position truss | Persons falling from heights | H | Workers to remain on bond beam scaffold when landing roof trusses. | M | Site Supervisor, Carpenter |
|  | Conduct work at height | Persons falling from heights | H | The following controls will be used either together or individually Where the trusses are erected at up to 900mm centres work can be performed off the bottom cord if; - The trusses are braced - Only competent, trained persons to perform work. - No damage to bottom cord - Person completing work take extra precaution And/ OR | M | Site Supervisor, Carpenter |
|  |  |  | H | A mobile scaffold is used as a catch platform below the work area or as a work platform where ever possible. OR | L | Site Supervisor, Carpenter |
|  |  |  | H | Scaffold planks are laid across the bottom cord to create a work platform | L | Site Supervisor, Carpenter |
|  | Fix Truss | Use of Electric drill | M | Only trained and competent workers to use. Drill lead checked for any damage prior to use. Correct safety features to be correctly installed. i.e drill handle | L | Site Supervisor, Carpenter |
|  |  | Truss collapse. | M | Install temporary bracing | M | Site Supervisor, Carpenter |
|  |  | Falling objects | M | Barricade & sign post area below when working at height above others. Hard hats to be worn | L | Site Supervisor, Carpenter |
|  | Un-slinging of crane | Persons falling from heights | M | Access lifting sling by either; - Standing on scaffolding - Standing on bottom cord if the cord has been inspected for defects and a catch platform (mobile scaffold) is used. - Use a step ladder. | L | Dogger |
|  | Filling in remaining trusses | Persons falling from heights | M | Workers to remain on bond beam scaffold supplied where possible Correct footwear to be worn Extra precaution when climbing around on top of trusses | L | Site Supervisor, Carpenter |
|  | Strapping, Batten screwing and speedy bracing to complete job | Persons Falling from heights | M | Correct footwear to be worn Extra precaution when climbing around trusses Use mobile scaffold where possible | L | Site Supervisor, Carpenter |
|  |  | Use of coil gun | M | Ensure correct PPE are worn. (Safety Glasses, Ear protection) Only trained and competent workers to use Precaution taken when using coil gun on top cords of trusses Straps to be shot on when standing on scaffold or bottom cord where possible | L | Site Supervisor, Carpenter |
|  |  | Use of electric drill | M | Only trained and competent workers to use Screw Batten screws whilst standing on bottom cord or scaffold where possible | L | Site Supervisor, Carpenter |
|  | Exiting the roof | Persons falling from height | M | Ensure workers use ladders supplied with bond beam scaffold Maintain three points of contact when climbing down the ladder | L | Site Supervisor, Carpenter |

| LIKELIHOOD How likely could it happen? | CONSEQUENCES: How severely could it affect health and safety? |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Insignificant No medical treatment required | Minor Minor lost time injury or illness | Moderate Moderate lost time injury or illness | Major Serious lost time injury or illness | Catastrophic Death or permanent disablement |
| Almost Certain Is expected to occur in most circumstances | Medium | High | Very High | Very High | Very High |
| Likely Will probably occur in most circumstances | Medium | Medium | High | Very High | Very High |
| Possible Might occur at some time | Low | Medium | High | High | Very High |
| Unlikely Could occur but doubtful | Low | Low | Medium | Medium | High |
| Rare May occur but only in exceptional circumstances | Low | Low | Low | Medium | Medium |

Based on Table 6.6 HB436-2004 Risk Management Guidelines Companion to AS/NZS4360:2004

| Risk Level | Action |
| --- | --- |
| VH - Very High | ACT NOW – Urgent - Do something about these risks immediately |
| H – High | Highest management action required urgently |
| M – Medium | Manage by specific monitoring or management procedures |
| L – Low | These risks may not need immediate action – manage by routine procedures |

Consequence scales for environmental hazards and risks

| Consequence | Description |
| --- | --- |
| Catastrophic | Catastrophic long term environmental impairment of eco systems functions |
| Major | Very serious long term environmental impairment of eco systems functions |
| Moderate | Serious medium term environmental effects |
| Minor | Moderate short term effects not affecting eco system functions |
| Insignificant | Minor effects on biological or physical environment |