CPCCBC4012

Read and interpret plans and specifications

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4012 Read and interpret plans and specifications cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Micheal Dawkins |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. | Satisfactory | Not Satisfactory |
| Workbook | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for CPCCBC4012 | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Theory Exam Workbook Hand out 3 - Slab Plan for Activity 3 | Practical Assessment Cost estimation sheet (HO5 template available) |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

CPCCBC4012

Read and interpret plans and specifications

THEORY EXAM

| Participant name | Micheal Dawkins |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within two (2) hours, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

What is the significance of concept drawing/sketches?

| Answer | 2d drawing usually done as a freehand sketch |  |
| --- | --- | --- |

Discuss when you may come across concept drawings/sketches.

| Answer | In the beginning stages of the project |  |
| --- | --- | --- |

Explain what is meant by architectural plans.

| Answer | These are drawings that can be used to obtain details about the architectural features of a building |  |
| --- | --- | --- |

Provide four (4) plans that are considered part of the architectural plans.

| Answer 1. 2. 3. 4. | Section View Prospective View Site Plan Schedule of windows and doors |  |
| --- | --- | --- |

Describe what structural plans are.

| Answer | These are drawings that can be used to acquire information about the structural aspects of the building IE specifications of load-carrying members, dimensions of structural components . |  |
| --- | --- | --- |

How is a structural plan different from an architectural plan?

| Answer | Architectural plans are concerned with information regarding the aesthetic, function, and general feel of the building, structural plans are concerned with elements that make the building strong and stable against expected external forces |  |
| --- | --- | --- |

Explain what a civil plan is.

| Answer | These drawings show an overview of all construction site elements and how these are combined into each other during the construction phase. Civil plans generally contain details that can be found on both architectural plans and structural plans. |  |
| --- | --- | --- |

Discuss how civil plans may be similar to other plans.

| Answer | Like architectural plans, civil plans contain the contours of the landscape. You can refer to civil plans if you intend to identify the reduced level of a footing element at a specific area in the site |  |
| --- | --- | --- |

What is the purpose for mechanical plans?

| Answer | These are drawings that contain the specifications related to mechanical equipment and processes, such as heating, cooling, ventilation, and transportation |  |
| --- | --- | --- |

Discuss three (3) details you may find in mechanical plans.

| Answer 1. 2. 3. | Size, type, and layout of ducting HVAC layout Thermostat type and location |  |
| --- | --- | --- |

When would electrical plans be used?

| Answer | To get information about the building essentials related to electrical systems or circuits, such as power, lighting, and communication |  |
| --- | --- | --- |

Provide three (3) examples of details contained in electrical plans.

| Answer 1. 2. 3. | Wiring layout for lighting Location of panels and switches Schedule of loads |  |
| --- | --- | --- |

What are plumbing and drainage plans used for?

| Answer | These are drawings that provide visual representations of elements related to plumbing systems |  |
| --- | --- | --- |

Discuss what may be needed when plans display council pipework.

| Answer | Council approval |  |
| --- | --- | --- |

Why is it important to check and ensure that the plans you are using are the current version?

| Answer | It is important that all changes are documented, and everyone involved is inform as this will potentially lead to the development of building defects, non-compliance with the building and construction regulations, standards and codes and other building project-related issues |  |
| --- | --- | --- |

What can occur if an outdated version of the plans is used?

| Answer | development of building defects, non-compliance with the building and construction regulations, standards and codes and other building project-related issues |  |
| --- | --- | --- |

Discuss what site plans are drawn to reflect.

| Answer | A site plan is a drawing which provides an overview of the entire property. It shows all site elements to be considered in the construction of a building and gives an idea of the entire scope of work involved in the project. |  |
| --- | --- | --- |

Discuss the purpose of reading the floor plan.

| Answer | A floor plan is a graphical representation of a building section’s top view. It shows the arrangement of elements such as walls, columns, doors, and windows and defines the limits of the building’s different spaces. |  |
| --- | --- | --- |

Discuss two (2) key differences between elevation view and section view.

| Answer 1. 2. | Elevations are views that show the vertical projection of the building as it can be seen on the outside. In other words, it shows how the building façade looks like at the front (front view), back (rear view), and sides (right side and left side view). Sections are views showing how a structure looks like with respect to an imaginary vertical plane that cuts through it. This imaginary cutting plane allows one to see through the interior of the structure, which is otherwise concealed by the exterior walls |  |
| --- | --- | --- |

Discuss the purpose of reading detail drawings.

| Answer | These are drawings that provide large scale views of parts or components of a building for one to examine and analyze intricate details and any complex configuration |  |
| --- | --- | --- |

Why is it important to measure from a datum point?

| Answer | A datum can be an horizontal plane or a point outside or within the property, either real or imaginary, that is used as a reference for all vertical measurements made in the construction project. |  |
| --- | --- | --- |

Discuss what reduced levels (RL) are and how these are established in construction drawings.

| Answer | a height above (or below) a nominated datum |  |
| --- | --- | --- |

Describe the purpose of the title block and what it contains.

| Answer | A title block is a portion of the drawing sheet that provides general information about the drawings and specific information about the particular drawing. 1. Project Title 2. Project Address 3. Draftsman name 4. Amendments 5. Date of Publication 6. Sheet Number 7. Scale 8.Client Name |  |
| --- | --- | --- |

Where is the title block usually located on the plans?

| Answer | Lower right hand corner |  |
| --- | --- | --- |

Describe the purpose of using scale drawings in construction.

| Answer | Scale refers to the ratio that dictates the size of scale drawings, which are drawings used for accurately illustrating real-life building components or items at fixed, reduced and enlarged sizes |  |
| --- | --- | --- |

Describe each ratio of standard scale ranges below.

| Answer | Ratio | Description |  |
| --- | --- | --- | --- |
|  | 1:100 | - For one unit of the drawing, the real size of the object is 100 times bigger; or - Given the real size of the object, the drawing is 100 times smaller. |  |
|  | 1:2 | - For one unit of the drawing, the real size of the object is two times bigger; or - Given the real size of the object, the drawing is two times smaller. |  |
|  | 1:50 | - For one unit of the drawing, the real size of the object is 50 times bigger; or - Given the real size of the object, the drawing is 50 times smaller. |  |

Describe why examining detail drawings is important.

| Answer | In examining detail drawings, you need to look at the corresponding dimensions of the building component. These dimensions provide the necessary measurements such as size, thickness, diameter, etc |  |
| --- | --- | --- |

What is the difference between utilities plans and specifications and services plans and specifications in the context of building construction?

| Answer | .The utilities plan is concerned with providing information regarding the supply for electricity and water, lifts, gas supply, sewage, and other considerations necessary for the construction phase. The services plan is concerned with systems necessary for the smooth flow of construction operations, including fire safety detection and protection, lighting, and security and alarm. |  |
| --- | --- | --- |

Describe what is meant by each of the following drawing descriptions.

| Answer | Drawing description | Meaning |  |
| --- | --- | --- | --- |
|  | Isometric drawings | are 3D drawings on a 2D surface where the same scale is used for all axes. |  |
|  | Oblique drawings | are the 2D projection of 3D drawings that represent an object accurately by showing the true shape and form of the object based on the point of view of the observer. |  |
|  | Perspective drawings | These are 3D drawings which show an approximate representation of an object as it is perceived by the eye |  |
|  | Orthographic drawings | These are drawings wherein 3D images are presented in 2D, in which three views are generally produced |  |

What are CAD drawings? Explain how CAD is used in building and construction?

| Answer | CAD drawings are drawings created and modified with the use of computer software. These drawings may be 2D or 3D illustrations or models created and used by engineers, architects, and construction managers to create precise drawings of the building and its components |  |
| --- | --- | --- |

Provide a brief description of the given classes of buildings according to the National Construction Code.

| Answer | Building Class | Description |  |
| --- | --- | --- | --- |
|  | Class 1a building | one or more buildings forming a single dwelling |  |
|  | Class 2 building | apartment buildings where people live above and below each other |  |
|  | Class 4 building | a part of a building that is: - The only dwelling or residence within the building - Inside any Class 5, 6, 7, 8 or 9 buildings |  |
|  | Class 5 building | an office building used for professional or commercial purposes, except for buildings of Class 6, 7, 8 or 9 |  |
|  | Class 7b building | warehouses, storage buildings, buildings used to display or produce wholesale goods |  |
|  | Class 10a building | A private garage, carport, shed |  |

What is building information modelling (BIM)?

| Answer | BIM is a 3D model-based process that utilizes tools and technologies that enable the creation and management of digital representations of physical and functional characteristics of structures/buildings/places |  |
| --- | --- | --- |

Explain how the NCC is relevant to construction drawings.

| Answer | The National Construction Code (formerly Building Code of Australia – BCA) sets out the technical provisions for the design, construction, and performance of buildings and plumbing and drainage systems. |  |
| --- | --- | --- |

Discuss the importance of complying with standards during a building and construction project.

| Answer | All aspects of the building, including structural details, must comply with the NCC requirements, which are: General Requirements – contain the rules and instructions for using and complying with the NCC. Performance Requirements – refer to requirements that state the minimum level of performance for buildings, as well as plumbing and drainage installations |  |
| --- | --- | --- |

Discuss how you would identify wind bracing and tie-down requirements from construction plans.

| Answer | The requirements for wind bracing can be found at the right half of the sheet. |  |
| --- | --- | --- |

Discuss why you would access AS 1684.2 Residential timber-framed construction Non-cyclonic areas, when reading plans and specifications.

| Answer | It is a document by Standards Australia that contains the minimum standards for the construction of timber-framed residential buildings. |  |
| --- | --- | --- |

Reasonable allowances are made by builders for the different construction items in a building project. Discuss what Prime Cost (PC) items are.

| Answer | These are reasonable allowances for the supply and installation of items whose prices cannot be determined at the time the contract is entered. |  |
| --- | --- | --- |

Explain what Provisional Sum (PS) items are.

| Answer | These are reasonable allowances for the supply and installation of items whose prices cannot be determined at the time the contract is entered. |  |
| --- | --- | --- |

Explain how to locate installation points of the services to the building from the plans.

| Answer | you need to refer to the construction plans and take note of the labels and legends |  |
| --- | --- | --- |

Explain how to locate connection points of services to the building from the plans.

| Answer | To locate the connection points, you need to look at the labels in the plans and schedule of components attached at the upper right portion of the sheet |  |
| --- | --- | --- |

Explain how to locate fixtures from the plans.

| Answer | are objects or assets installed in a building that are considered to be fixed and irremovable. Examples of common building fixtures include the following electrical installations HAVC systems fire and smoke detectors |  |
| --- | --- | --- |

CPCCBC4012

Read and interpret plans and specifications

WORKBOOK

| Participant name | Micheal Dawkins |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written activities as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - The timeframe for these tasks will be established by your Assessor or Training Plan, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

NOTE

At times, you will be provided with a specific business context to which you must respond, otherwise it is anticipated that you will use a workplace of your choosing (your own, or hypothetical) to provide the context to the task.

At all times, the responses in your assessment must reference Australian (Queensland or Federal) documentation, including legislation, regulations, codes of practice, standards and guidance publications.

For the following activities, name the State/Territory you will base your research on.

| State/ Territory | Queensland |
| --- | --- |

Research and document your findings regarding legislation and codes that apply in your state/territory relevant to the following three (3) items. Summarise the requirements relevant to building drawings and sketches you identified and provide the URL where you sourced your responses. - Smoke alarms in a new building - Pool fences - Protection of waterways

- Smoke alarms

Name of Legislation (Commonwealth or State)

Smoke Detector Legislation

Summary of relevant excerpt/requirement

New smoke alarm legislation means all homes must install interconnected photoelectric smoke alarms in every bedroom, in hallways, and on every level by 2027. Photoelectric smoke alarms respond to a wide range of fires but are particularly responsive to smouldering fires and dense smoke, the most common type of household fire. Having interconnected alarms means when one alarm is activated, all alarms will sound. The extra time that gives you can make all the difference.

Source (URL or web source)

https://www.qld.gov.au/about/newsroom/new-smoke-alarm-laws#:~:text=New%20smoke%20alarm%20legislation%20means,on%20every%20level%20by%202027.

- Pool fence installation

Name of Legislation (Commonwealth or State)

Swimming pool barriers

Summary of relevant excerpt/requirement

To help make your pool fence or barrier compliant: - replace, tighten or adjust the hinges on your gates - ensure the pool barrier height is at least 1200mm from bottom to top - trim back any vegetation or branches that a child could use to climb over the pool barrier - shield or remove climbable objects within 900mm of the pool barrier - install fixed security screens on windows that open into the pool enclosure - remove climbable objects from the pool barrier and surrounding areas.

Source (URL or web source)

https://www.qld.gov.au/housing/buying-owning-home/pool-safety/pool-laws-and-standards/fences-and-barriers#:~:text=If%20your%20pool%20fence%20or,modifies%20the%20Australian%20Standard%20AS1926.

- Protection of waterways

Name of Legislation (Commonwealth or State)

Water Act 2000

Summary of relevant excerpt/requirement

25E : Requirement to comply with water supply emergency declaration. (1)A service provider to whom a direction is given under a water supply emergency declaration must comply with the direction. Maximum penalty: (a)for a direction mentioned in section 25C(1)(d)(i)—1,665 penalty units; or (b)for a direction mentioned in section 25C(1)(d)(iii) or (v)—1,000 penalty units. Note: See also the Water Supply Act, section 43(4) to (6).

Source (URL or web source)

https://www.legislation.qld.gov.au/view/html/inforce/current/act-2000-034#sec.25E

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Access Handout 1 (HO1- Floor Plan with dimensions) and respond to the following: - What is used for the internal wall lining? - What is used for the external cladding? - What roofing cladding is required? - How do you identify the smoke alarm installation points?

- What is used for the internal wall lining?

Clay & Brick

- What is used for the external cladding?

Painted render finish to external walls

- What roof cladding is required?

R2.5 insulation

- How do you identify the smoke alarm installation points?

Symbolizes smoke detector

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Access Handout 2 (HO2- Footing Plan) and Handout 3 (HO3- Slab Plan) and respond to the following: - Specify the size and type of reinforcing to be used for BP.1 in Handout 2 (HO2-footing plan) - How do you determine where each BP.1 footing pier should be placed? - Mark each of the load bearing points in Hand out 3 (HO3- Slab Plan)

- Specify the size and type of reinforcing to be used for BP.1

450mm Dia concrete bored pier

- How do you determine where each BP.1 footing pier should be placed?

By referring to the construction issue footing plan

- Mark each of the load bearing points in Hand out 3 (HO3- Slab Plan)

Please ref to HO3 Mark up - MD in submissions

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Access Handout 4 (HO4- Residential Floor Plan) and respond to the following: - Which drawings are shown in these plans? - Which Australian Standard must be referenced when installing termite protection? - Make a list of the finishes noted on the document.

- Which drawings are shown in these plans?

Floor plans and Elevation & Section plans

- Which Australian Standard must be referenced when installing termite protection?

AS 3660 SE

- Make a list of the finishes noted on the document.

Roof – Custom ORB Deep Ocean Colourbond Gutter / Fascia – Deep Ocean Colourbond Walls – Dulux Cargo River Corbells – Dulux Calf Skin Window Frames – Powder coat anotech grey Paving – Lifestyle paving liquid limestone

| Satisfactory Outcome |  |  |
| --- | --- | --- |

CPCCBC4012

Read and interpret plans and specifications

PRACTICAL ASSESSMENT

| Participant name | Micheal Dawkins |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel, unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 | 14/02/2024 | Workplace Simulation | Project Office |  |  |

ASSESSOR COMMENTS: GENERAL

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 1 |  |
| --- | --- |
| x Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Provide detail |
| 1.1A. 1.1B. | Specify type of plan and NCC class determined: Warehouse plans Class 7b |
| 1.1A. 1.1C. | Specify how currency of documents was confirmed: Jeffery Hill & Associates |
| 1.1D. | Specify environmental controls determined from plans/specifications. |
| 1.2B. 1.2C. | Specify how building layout, material requirements spaces and dimensions were determined from plans and specifications. |
| 1.2G. | Specify services identified on site: WM, SWM, MDB, SDB, TELSTRA, |
| 1.2H. | Specify site features located to be removed/ retained: |
| 1.2J. | Specify variations found: |
| 1.3A. | Specify concrete footing and slab information identified: On the Footing Plan |
| 1.3B. | Specify how load bearing points were recognised: General Notes under Project Loads |
| 1.3C. | Specify how wind bracing and tie down requirements were confirmed: Rood framing plan |
| 1.4B. 1.4C. | Specify how installation points and fixtures were determined: |
| 1.4B. | Specify how connection points were determined: |
| 1.4C. | Specify where finishing details were located: |

CPCCBC4012

Read and interpret plans and specifications

| Practical Task 1 |  |
| --- | --- |
| Description of task: The participant must demonstrate skills and knowledge to identify and interpret building and construction information from plans and specifications in a workplace (or a similar environment) in the context of a construction project including: - Identify types of plans. - Check and confirm that the current version of plans, specifications and amendments is being used. - Use the appropriate plans and aspects of drawings for relevant purposes. - Confirm that the details on plans comply with the specifications. - Identify environmental controls from plans/specifications. - Read and interpret the plans. - Read and confirm standards of work and tolerances from job specifications. - Determine building layout, spaces and dimensions from floor plan. - Determine material sizes, thickness and construction methods from detail drawings for estimates. (Template available for use HO5_ Cost Estimation) - Recognise and confirm job specifications located in drawings, notes, and descriptions. - Determine internal linings, external cladding and roof materials needed for estimates from plans and specifications. .(Template available for use HO5_ Cost Estimation) - Apply information from plans to estimation, planning and supervisory tasks.(Template available for use HO5_ Cost Estimation) - Gain access to the site, achieve orientation and identify services, datum and reduced levels. - Locate site features to be removed or retained. - Determine the building location from the site plans to actual. - Verify plans recognising variations between the plans and the actual site, reporting findings as required. - Appropriately communicate with others to be understood. - Adhere to site safety requirements, including requirements relating to hazard identification. - Identify structural details. - Identify concrete footing and slab sizes and position and type of reinforcing required. - Identify structural load bearing points of the building and compare information from specification. - Identify wind bracing and tie-down requirements checking for accuracy against the wind bracing table. - Identify non-structural details. - Identify provisional sum (PS) and prime cost (PC) items when estimating costs.(Template available for use HO5_ Cost Estimation) - Locate installation and connection points of services to the building. - Identify location of fixtures from plans and details of finishes from specifications. These tasks may be simulated or real. The Participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. The Observer is to provide detailed comments on the tasks completed by the Participant (on the Observer Comments, overleaf). The Assessor is to verify the Observer’s comments, noting their findings in the Assessor Comments (on the back of the Practical Assessment cover page) and provide the assessment outcome. |  |
| Resources required: Work site; plans and specifications, quality documentation; access to the internet/access legislation, current building and construction codes and standards and the NCC. PC applications and software to access CAD. Templates for estimating. |  |
| Observer details |  |
| Name of observer: |  |
| Role of observer (Confirm suitability to sign) |  |
| Email address of observer |  |
| Signature of observer: |  |
| Date: | Select Date. |

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |
| --- | --- |
| 1 Identify types of plans |  |
| Check and confirm that the current version of plans, specifications and amendments is being used. |  |
| Use the appropriate plans and aspects of drawings for relevant purposes. |  |
| Confirm that the details on plans comply with the specifications. |  |
| Identify environmental controls from plans/specifications. |  |
| 2 Read and interpret the plan |  |
| Read and confirm standards of work and tolerances from job specifications. |  |
| Determine building layout, spaces and dimensions from floor plan. |  |
| Determine material sizes, thickness and construction methods from detail drawings.(Template available for use HO5_ Cost Estimation) |  |
| Recognise and confirm job specifications located in drawings, notes, and descriptions. |  |
| Determine internal linings, external cladding and roof materials from plans and specifications. (Template available for use HO5_ Cost Estimation) |  |
| Apply information from plans to estimation, planning and supervisory tasks.(Template available for use HO5_ Cost Estimation) |  |
| Gain access to the site, achieve orientation and identify services, datum and reduced levels. |  |
| Locate site features to be removed or retained. |  |
| Determine the building location from the site plans to actual. |  |
| Verify plans recognising variations between the plans and the actual site, reporting findings as required. |  |
| Appropriately communicate with others to be understood. |  |
| Adhere to site safety requirements, including requirements relating to hazard identification. |  |
| 3 Identify structural details |  |
| Identify concrete footing and slab sizes and position and type of reinforcing required. |  |
| Identify structural load bearing points of the building and compare information from specification. |  |
| Identify wind bracing and tie-down requirements checking for accuracy against the wind bracing table. |  |
| 4 Identify non-structural details |  |
| Identify provisional sum (PS) and prime cost (PC) items when estimating costs. |  |
| Locate installation and connection points of services to the building. |  |
| Identify location of fixtures from plans and details of finishes from specifications. |  |

Observer comments: