BSBOHS201A

Participate in OHS processes

ASSESSMENT SUMMARY

The assessment activities for BSBOHS201A Participate in OHS processes cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Chantal Nix |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

BSBOHS201A

Participate in OHS processes

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name |  |
| --- | --- |
| Participant signature |  |
| Date | Click here to enter a date. |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 24 |  | Satisfactory | Not Yet Satisfactory |

ASSESSOR Comments

Match the terms and definitions by writing the appropriate letter next to each number in the centre column.

| Answer | Terms |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | PRA | 1 - F | A | Outline the requirements of the work to be undertaken. |  |
|  | 2 | JSA | 2 - E | B | Outline what to do in the event of fires and other accidents |  |
|  | 3 | Plans and specifications | 3 - A | C | Contains important information about hazardous materials or chemicals. |  |
|  | 4 | Manufacturer specifications | 4 - D | D | Outlines how tools and equipment should be used (e.g. lubricant levels, maintenance requirements). |  |
|  | 5 | SDS | 5 - C | E | Identifies hazards and potential hazards that may be encountered while working. |  |
|  | 6 | Emergency procedures | 6 - B | F | A personal assessment of risk |  |

For each of the hazards identified below, describe the injuries/damage they may cause (i.e. to fellow workers, other people and/or animals) and suggest a suitable control measure.

| Answer | Hazard | Injuries or damage | Control measure |  |
| --- | --- | --- | --- | --- |
|  | Hazardous chemical | Respiratory Damage, skin rashes, chemical burns | PPE: respiratory masks, gloves, safety glasses |  |
|  | Noise | Hearing damage, headaches, fatigue | PPE: earmuffs |  |
|  | Manual handling | Strains, sprains | Procedures, signage, Risk assessments, |  |

Who should you speak to if you find problems (e.g. hazards, accidents and emergencies)?

| Answer | Supervisor, site/project manager, health & safety representative |  |
| --- | --- | --- |

What action should be taken if plant, tools and/or equipment are found to be defective? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Report, Record |  |
|  |  | Stop, Report, Record |  |
|  |  | Stop, Tag, Report, Record |  |
|  |  | Stop, Fix, Report, Record |  |

Which legislation relates to safety in the workplace? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Occupational Health and Safety Act 2015 |  |
|  |  | Work Health and Safety Act 2011 |  |
|  |  | Safety Act 1900 |  |
|  |  | Safety and Health Act 2011 |  |

True or False (Tick your response): Employers are required by legislation to implement safety measures which are reasonably practicable.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): Workers are required by legislation to follow reasonable employer directives (e.g. to undertake first aid and/or fire safety training).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

The following symbols appear under the heading ‘Required PPE’ on an SDS. What do the symbols represent?

| Answer | Symbol | Meaning |  |
| --- | --- | --- | --- |
|  |  | Personal protective clothing required |  |
|  |  | PPE – gloves required |  |
|  |  | PPE – Safety glasses required |  |
|  |  | PPE – respiratory mask required |  |

Describe the meaning and use of the signs displayed below.

| Answer | Sign | Meaning |  |
| --- | --- | --- | --- |
|  |  | First aid kit location |  |
|  |  | Warning sign – authorised personnel only |  |
|  |  | Fire extinguisher location |  |

Refer to the Risk Matrix below when responding to Q10. to Q12.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

Which rating is designated for the highest risk and which rating is used for the lowest risk?

| Answer | Highest Risk: A1 Lowest Risk: E5 |  |
| --- | --- | --- |

What is the risk rating for the following circumstances: An event which is possible and would require low-level hospital treatment.

| Answer | D3 |  |
| --- | --- | --- |

What are the factors that would lead to a risk rating of E: Extreme?

| Answer | Death or permanent disability or serious injury |  |
| --- | --- | --- |

When should safety checks be undertaken on machinery and equipment? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Before operation |  |
|  |  | After operation |  |
|  |  | Both before and after operation |  |

Order the elements of the hierarchy of control, from 1 to 6 (i.e. most effective to least effective).

| Answer | Label (1 – 6) | Element |  |
| --- | --- | --- | --- |
|  | 3 | Isolate the hazard |  |
|  | 6 | Provide and use Personal Protective Equipment (PPE) |  |
|  | 1 | Eliminate the hazard |  |
|  | 4 | Control the hazard by engineering means |  |
|  | 5 | Control the hazard by administrative means |  |
|  | 2 | Substitute the hazard |  |

Why is elimination the best method of hazard control and PPE considered the least effective?

| Answer | It is the least effective as it doesn’t remove the hazard and potential risk, only reduces the severity of the risk such as burns from chemicals. The PPE only protects the skin from the hazardous chemicals but doesn’t remove the possibility of spilling or exposure. |  |
| --- | --- | --- |

If you were to replace a flammable solvent with a water-based solvent, what category of control measure would that be under the hierarchy of control?

| Answer | 1 – Eliminate the hazard |  |
| --- | --- | --- |

In the diagram below a person is lifting a tool box.
Filling in the table, indicate how each body part should be used/placed.

F. Hands

E. Knees

D. Shoulders

C. Feet

B. Back

A. Neck

| Answer | Area of body (Letter) | How it should be placed |  |
| --- | --- | --- | --- |
|  | D | Level |  |
|  | A | Upright |  |
|  | F | Good grip |  |
|  | B | Natural curve |  |
|  | E | Bent |  |
|  | C | Flat, facing load |  |

In a medical emergency, what is the first step in the DRSABCD action plan that you need to follow?

| Answer | Danger check |  |
| --- | --- | --- |

In the event of a fire emergency, what should you do? 
Fill in the table below to explain the acronym R.A.C.E.

| Answer | Letter | Meaning |  |
| --- | --- | --- | --- |
|  | R | Remove people from immediate danger |  |
|  | A | Alert people of emergency |  |
|  | C | Confine fire and smoke |  |
|  | E | Evacuate to the assembly area |  |

What should you do as part of the clean up process? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Store all plant, tools and equipment without cleaning |  |
|  |  | Maintain plant, tools and equipment |  |
|  |  | Dispose of materials that cannot be reused or recycled |  |
|  |  | Report faults in logbook and/or site records |  |
|  |  | Clean all plant, tools and equipment before storage |  |

Read the scenario below and complete the Hazard Report Form.

SCENARIO

Today at 10:45am a delivery truck reversed into a car park (located at 54 Standish Street, Salisbury) to deliver gardening materials. There was nobody in the car park to guide the driver and he backed into a number of drums of diesel located to the left of the photograph (below).  One drum was pierced and its contents erupted from the container.

The marked area on the photo represents the diesel spill, measuring 3m x 4m.

HAZARD REPORT FORM

| GENERAL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date | 08/02/2021 |  |  |  |  |  |
| Submitted by | Chantal Nix |  |  |  |  |  |
| HAZARD DETAILS |  |  |  |  |  |  |
| Location | 54 Standish Street, Salisbury QLD |  |  |  |  |  |
| Description of issue | On Monday 8th February 2021 at 10:45 a delivery truck reversed into a car park at 54 Standish Street Salisbury to deliver gardening material. No one was present on his arrival and he reversed into 6 drums containing diesel, pictured in the photo. One drum was pierced, and the diesel contests erupted from the container spilling on the car park. The area on the photo provided shows the size of the diesel spill, measuring 3mx4m. |  |  |  |  |  |
| Risk class | Extreme | High | Medium |  |  | Low |
| CONTROL MEASURES |  |  |  |  |  |  |
| Corrective action required | Clean up the diesel spill Remove all remaining drums. Create designated delivery/drop off area in the carpark Create signage for delivery area |  |  |  |  |  |
| Person to action | Site Manager |  |  | When | Immediately Within 24 hours Within 7 days |  |

| Answer |  |  |
| --- | --- | --- |

Who would you submit a hazard report form to?

| Answer | Health & Safety Representative and the Project/site manager |  |
| --- | --- | --- |

Who would approve proposed risk controls and organise the required resources for implementation of the risk control?

| Answer | Health and safety representative and project/site manager |  |
| --- | --- | --- |

Identify one (1) way you can assess if risk controls have been effective.

| Answer | Review assessment regularly |  |
| --- | --- | --- |