Handout 1 - Compliance Review

| Participant name | Michael Trott |
| --- | --- |
| Date | 17/12/23 |

| Building and construction project | RTO Facility |
| --- | --- |
| Project overview | Office Refurb |
| Building classification | Class 1 – 10 buildings, max two storeys Class 2 – 9 building, Type C only constructions Specify class: class 5 |
| Project site address | 878 Beaudesert Road, Coopers plains |
| State/territory where project is undertaken | QLD |
| Construction plan documents reviewed | Sections/Elevations/Floor plans/site plan |

Building Regulations

List relevant regulations

| Building regulations | Plans comply |  | Explanation |
| --- | --- | --- | --- |
| Example: Smoke alarms | Yes | No | Smoke alarm locations shown in floor plans - photoelectric (AS 3786-2014) hardwired and interconnected. |
| Plumbing and drainage act 2022 | Yes | No | Shown in plumbing layout (AS/NZS.3500) |
| Structural timber sizes | Yes | No | Shown in section a (AS1684) |
| Wet areas | Yes | No | Shown in floor plans (AS3740) |
| Mechanical ventilation | Yes | No | Shown in floor plans (AS1668.1) |
|  | Yes | No |  |
|  | Yes | No |  |

Building Standards

Provide relevant standards

| Building Standards | Plans comply |  | Explanation |
| --- | --- | --- | --- |
| AS1684 | Yes | No | Structural timber sizes |
| AS/NZS.3500 | Yes | No | Plumbing and drainage works installation |
| AS3740 | Yes | No | Wet area compliance |
| AS1668.1 | Yes | No | Mechanical ventilation compliance |
| AS3786 | Yes | No | Smoke alarm installation |
| AS1562.1 | Yes | No | Roof installation |

NCC Requirements

Fire Safety

| Requirements | Plans comply |  | Explanation |
| --- | --- | --- | --- |
| Smoke alarms connected to mains power installed in accordance with AS-3786 and NCC PART 3.7.2 | Yes | No | Electrical layout shows smoke detectors |
| Lighting to assist evacuation 3.7.2.5 | Yes | No | Electrical layout shows emergency exit light wired to mains power |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |

Health and amenity

| Requirements | Plans comply |  | Explanation |
| --- | --- | --- | --- |
| There should be a clear space of 1.2m between the closest pan within the sanitary compartment and the doorway p3.8.3.3 | Yes | No | Door way opening has clear 1.2m space away from toilet components |
| ### Artificial lighting provided in sanitary compartments with no windows NCC3.8.4.3 ### , | Yes | No | As shown in Electrical layout |
| Provide mechanical ventilation NCC 3.8.5 | Yes | No | Not shown in electrical layout |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |

Safe movement and access

| Requirements | Plans comply |  | Explanation |
| --- | --- | --- | --- |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |

Energy efficiency

| Requirements | Plans comply |  | Explanation |
| --- | --- | --- | --- |
| External windows and door to be fitted with a seal NCC 3.12.33 | Yes | No | As shown in floor plans |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |
|  | Yes | No |  |