CPCCBC4004A

Identify and produce estimated costs for building and construction projects

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Mohamed Elbagoury |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 23/04/2017 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 37 |  | Yes | No |

Assessor Comments

Refer to the plans and specifications below to respond to Q1. to Q9.

- Match the sheet numbers and document types by writing the appropriate letter next to each number in the centre column.

| Answer | Sheet Number |  | Answers | Document Type |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Sheet 4 | 1 - A | A | Floor plan |  |
|  | 2 | Sheet 5 | 2 - B | B | Section plan |  |
|  | 3 | Sheet 6 | 3 - C | C | Site plan |  |

- Which sheet number of Drawing WD 9733 identifies the storm water drainage for the proposed structure? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Sheet 4 |  |
|  |  | Sheet 5 |  |
|  |  | Sheet 6 |  |

- Identify the Volume and Parts of the NCC that outlines the requirements for storm water drainage.

| Answer | ## NCC 2015 VOL TWO --- part 3 ---- 3.1.2.5 Stormwater drainage |  |
| --- | --- | --- |

- Which Australian Standard is referenced in the NCC with regards to storm water drainage?

| Answer | AS/NZS 3500 Part Three --- 2003 --- Plumbing and drainage |  |
| --- | --- | --- |

- What is the area of the verandah? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | 76m2 |  |
|  |  | 92m2 |  |
|  |  | 135m2 |  |
|  |  | 211m2 |  |
|  |  | 243m2 |  |

- What is the pitch of the verandah roof?

| Answer | 15° |  |
| --- | --- | --- |

- What size are the verandah beams?

| Answer | 150*50 |  |
| --- | --- | --- |

- Which of the following Jamb Studs are to be used for both the upper and lower storeys? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | 90x45 |  |
|  |  | 90x70 |  |
|  |  | 90x90 |  |

- Identify three (3) subcontractors that may work on this construction project.

| Answer | Plumber Tiler Electrician Painting and paper hanging |  |
| --- | --- | --- |

- Identify three (3) items of plant and/or equipment that may be used on this construction project.

| Answer | ### Earth Moving Equipment excavator ### Construction vehicle like tanker # Forklift |  |
| --- | --- | --- |

- Identify the Volume and Part of the NCC that outlines the requirements for the timber weatherboards identified in Sheet 5.

| Answer | NCC 2015 VOLUME TWO – Part 3.5 – Cl. 3.5.3.2 Timber weatherboard cladding |  |
| --- | --- | --- |

- Which Australian Standard is referenced in the NCC with regards to timber structures?

| Answer | AS 1562 3.5.3 Wall Cladding |  |
| --- | --- | --- |

Refer to the contour plan below to respond to Q13. to Q15.

- What is the datum reading at the tip of the pit?

| Answer | 0.00 |  |
| --- | --- | --- |

- What is the level at the kerb invert?

| Answer | Georgeeson rd 0.55 - Barnaby rd 0.79 |  |
| --- | --- | --- |

- True or False (Tick your response): The lowest point on the site is found at the northernmost point.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): A Gantt Chart may assist with determining the numbers of on-site personnel required and the time requirements for a project.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify labour that may be required for non-contract elements of a construction project.

| Answer | receptionist recruiter payroll accountant construction manager |  |
| --- | --- | --- |

- Explain how the following methods may be used to calculate the costs for on-site work.

| Answer | Method | Explanation |  |
| --- | --- | --- | --- |
|  | Per hour | Per hour is labour hour cost and it involves many aspects, such as the trade and the award related to. So it is used to count the hours and multiply it by the cost per hour. |  |
|  | Per unit | Per unit method of calculation is usually used for jobs like windows and doors. This method is used a long experience in doing the same job over and over, which creates a knowledge of how much the door will cost from A to Z, taking the number of units such as doors and multiply it by the cost of each unit. |  |
|  | Per metre | The meter method such as the square meter and is used for areas jobs like floors. Also cubic meter for volume and is used for concrete, linear meters which is primarily used for building materials, as a unit of amount (not distance), particularly in contrast with square meter. For example, 2 linear meters of 50 centimetre wide board is 1 square meter of board. |  |

- Which of the following documents may you refer to when establishing material quantities for a construction project? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Plans |  |
|  |  | Project contract |  |
|  |  | Specifications |  |
|  |  | Standard construction contracts |  |

- True or False (Tick your response): The fast way to locate suppliers, such as manufacturers and wholesalers, is to find them online.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Explain how each of the following factors affects supplier selection.

| Answer | Factor | Impact |  |
| --- | --- | --- | --- |
|  | Location | Dealing with distant suppliers might mean longer delivery times and extra freight costs |  |
|  | Price | Affordability is a key factor, but cheap does not always equate with quality. |  |
|  | Reliability | Suppliers should be able to deliver the right goods and on time |  |
|  | Stability | Look for experienced suppliers who have been in business a long time. This illustrates financial stability. |  |
|  | Supplier groups | While using a single supplier has its advantages, it also has risks. It might be better to use a carefully selected group of suppliers as insurance against any problems occurring. |  |

- Read the following information to respond to Q22. to Q27.

- The diagram below shows formwork that is required for a concrete slab.

The table below outlines the costs for this task.

| Item | Cost |
| --- | --- |
| Timber | $2.50 per metre |
| Concrete | $50.00 per cubic metre (excluding GST) |
| Labour | $49.50 per hour |

- Calculate the lineal meters of timber (150mm wide x 25mm thick) required to form up the edge of a concrete slab which is 6.0m x 2.5m x 150mm.  Provide no allowance for waste. Show your answer to two decimal places.

| Answer | 17 L m |  |
| --- | --- | --- |

- Use your response to Q22. to calculate how much the timber will cost. 
Show your workings and answer to two decimal places.

| Answer | $42.5 |  |
| --- | --- | --- |

- Calculate how many cubic metres of concrete are required. 
Show your workings and answer to two decimal places.

| Answer | 2.25 m³ |  |
| --- | --- | --- |

- Use your response to Q24. to calculate the cost of the required concrete. Show your answer to two decimal places and ensure it includes GST.

| Answer | $112.5 exc GST |  |
| --- | --- | --- |

- Calculate how much the labour will cost if the job takes 6 ½ hours. 
Show your workings and answer to two decimal places.

| Answer | $321.75 |  |
| --- | --- | --- |

- Use your responses for Q23. Q25. and Q26. to calculate how much this project will cost. Show your workings and answer to two decimal places.

| Answer | $476.75 |  |
| --- | --- | --- |

Refer to the electrical plan below to respond to Q28. and Q29.

Source: https://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1944_CCBY.PDF

- Measure the required quantities for the items listed below.

| Answer | Quantity | Item |  |
| --- | --- | --- | --- |
|  | 26 | Ceiling light |  |
|  | 1 | Wall light |  |
|  | 1 | External light |  |
|  | 3 | Exhaust fan |  |
|  | 2 | Smoke detector |  |

- The costs for each of the items are provided in the table below.

| - Cost | - Item |
| --- | --- |
| - $49 each | - Ceiling light |
| - $29 each | - Exhaust fan |
| - $82 each | - External light |
| - $39 each | - Smoke detector |
| - $59 each | - Wall light |

- Determine the total costs for each item (using the numbers from Q28. ) and the total cost for all items.

| Answer | Cost | Item |  |
| --- | --- | --- | --- |
|  | 1274 | Ceiling light |  |
|  | 59 | Wall light |  |
|  | 82 | External light |  |
|  | 87 | Exhaust fan |  |
|  | 78 | Smoke detector |  |
|  | 1580 | TOTAL |  |

- Name one (1) organisation that provides workers’ compensation insurance.

| Answer | ### Workers Compensation Insurance ¦ GIO - SunCorp |  |
| --- | --- | --- |

- Which of the following environmental costs should be considered when determining project costs? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Approvals |  |
|  |  | Environmental protection |  |
|  |  | Permits |  |
|  |  | Waste management |  |

- List three (3) statutory costs that should be considered when determining project costs.

| Answer | planning applications, building regulations applications, licensing, Land Registry fees |  |
| --- | --- | --- |

- Which two (2) of the following methods are commonly used when determining the profit on a project? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Double the builder’s cost |  |
|  |  | Lump sum based on market conditions |  |
|  |  | Percentage on builder’s total cost |  |

- True or False (Tick your response): Usually a percentage is added to the estimate to cover company overheads.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify one (1) example of company overheads for each of the listed categories.

| Answer | Factor | Impact |  |
| --- | --- | --- | --- |
|  | Business premises | Receptionist |  |
|  | Staff and professional fees | Recruiter |  |
|  | Transport | travel expenditures |  |
|  | Finance | Payroll |  |

- Fill in the blanks: Complete the following chart to identify what is included in the final cost for a project.

Plant

Material

Building

Project cost

Overheads

| Choose from | Builder’s total cost ¦ Company overheads ¦ Direct costs ¦ Materials ¦ Plant and equipment |  |
| --- | --- | --- |

- Which of the following should be included in a letter of quotation? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Conditions to your price |  |
|  |  | Discrepancies |  |
|  |  | Final cost |  |
|  |  | Invitation to proceed |  |