RIIMPO402D

Apply the principles of earthworks construction

ASSESSMENT SUMMARY

The assessment activities for RIIMPO402D Apply the principles of earthworks construction cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name |  |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for RIIMPO402D | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Theory Exam | Practical Assessment Copy of Job Plan completed by participant |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |

RIIMPO402D

Apply the principles of earthworks construction

THEORY EXAM

| Participant name | Robert Bolton |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within a two (2) hour period, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

What information can you obtain by reading the project plans, surveys, drawings and specifications during planning for earthworks?

| Answer | You can gather multiple information from reading the plans and specs like quantity/volume of material need, heights and finish surface levels you need to build too. The full scope of works that you are required to build, specifications you need to adhere too and methodology on how to build the certain task. Existing services so you can put together a service control plan to work to, so you don’t clash with any services. |  |
| --- | --- | --- |

Why is project site hydrological data important to earthworks planning and where this can be accessed?

| Answer | It’s important because it identifies areas of the work site that have potential flood risks of ground conditions that are unsuitable to build on etc and allows us to plan to put in possible temporary measures during construction like bio-basins etc. This is the purpose of pre-construction testing like Geotech reports etc. We can access this on the Geotech reports/ dilapidation reports pre survey reports and data that the local council has already established that we can access. |  |
| --- | --- | --- |

What is the purpose and procedure for conducting a proof roll test to the subgrade?

| Answer | The purpose of a proof roll is to make sure there’s no soft spots or unsuitable ground prior to building on top of it. It is a lot easy to fix NOW before building on top and fixing it after. Once subgrade has been built and rolled and is ready to go you book in a pre-inspection with the superintendent which is normally a hold point in the ITP quality document and get the water truck with a full tank to drive slowly up the road prep and watch for any heaving or soft spots that you need to mark, dig up and rectify before you proof roll again. |  |
| --- | --- | --- |

What is the purpose of geotechnical investigation reports and how they inform earthworks planning?

| Answer | To see if the ground is suitable to build on in a nutshell! They also help in a financial sense, so the contractor doesn’t get caught out during construction. For example, if the Geotech report comes back saying the ground is rock then the estimators need to allow a rock clause in the contract so when we hit said rock, our rate/price can factor this in. |  |
| --- | --- | --- |

What information would you expect to see in a job plan?

| Answer | Human resource requirements Scope of works Risks and plan for contingencies Monitoring requirements Management requirements Materials needed for the job etc |  |
| --- | --- | --- |

How would you identify if any drainage or dewatering issues need to be addressed at the site and how you would promptly and effectively manage these once identified?

| Answer | Hydrological data and Geotech reports will help you find this information, and a couple effective ways of managing these issues would be temporary bio-basins and bunds or channelling watering and easing erosion with sandbags. |  |
| --- | --- | --- |

What is the importance of understanding the relationship between the different areas of civil work when planning and sequencing work tasks, providing examples?

| Answer | It’s important to plan and programme the job so that you can get the sequencing right with the build to avoid delays, subbies working on top of each other and to avoid re-works just to name a few. - Prepping and pouring a concrete footpath before the electrical service needs to be re-located underneath it would make you have to rip up the footpath again! - Building the new road before doing the underground service would cause re-works and a big financial hit if this was done as well. |  |
| --- | --- | --- |

Before commencing a project, what would you estimate to effectively determine resource and sequencing requirements?

| Answer | You would need to estimate scope of work when it comes to earth works volume so you can procure materials such as gravels, sand, asphalt etc. Also, when you estimate the scope, you can the allocate resources like labour, plant, engineers etc. Also, you can do a programme up so you can determine how long a programme will take and you can plan accordingly. |  |
| --- | --- | --- |

What is the definition of the following material properties and test results that are applicable to earthworks?

| Answer | Test | Definition |  |
| --- | --- | --- | --- |
|  | Compaction test | A compaction test is a laboratory or field test used to determine the relationship between moisture content and dry density of a soil, in order to establish the optimum moisture content (OMC) at which the soil can be compacted to achieve its maximum dry density (MDD).\ |  |
|  | Soil density/ moisture | Soil density is the mass of soil per unit volume. See’s how well the soil is compacted |  |
|  | Plasticity | Plasticity is the property of a fine-grained soil that allows it to be moulded or deformed without cracking or changing volume when it has sufficient moisture. See’s how easy the soil is to work with and shrinkage and swell behaviour |  |
|  | Particle size distribution | Particle size distribution (PSD) describes the proportions of different-sized soil particles (gravel, sand, silt, and clay) present in a soil. Helps you see if the soil is suitable and the strength and compaction of drainage materials |  |

What resources must you confirm are available before the job plan can be implemented?

| Answer | Human resources- what labour, plant and other materials. Monitoring and testing requirements-what testing needs to be done and monitored post construction. Procurement-what needs to be procured just in case there are long lead times for certain items. Health & Safety- quality docs, SWMNS and other items such as a operational management plan that need to be created prior to construction. |  |
| --- | --- | --- |

Outline the information you will be discussing with the work team during the pre-start meeting.

| Answer | Scope of works, what person is doing what tasks. Site conditions and any change of conditions if there had been night works the night before. Risks and hazards, Toolboxes talk if there is a talk box to be given. Details of the task and resource/material requirements per task. Ask them if there’s any questions they have before they start the day. |  |
| --- | --- | --- |

What methods can you apply to confirm instructions provided have been understood?

| Answer | Follow up questions so your can see if your team understands the assignment. Pre-works toolbox so you can go through with the team in detail and make sure everyone is on the same page. |  |
| --- | --- | --- |

What would you do if you come across conflict between sub-contractors?

| Answer | Address it immediately! \ Separate the two parties. Follow the policy and procedures of the company when it comes to conflict resolution. Begin the process of finding out all details of the conflict etc. |  |
| --- | --- | --- |

Discuss what you would do if you noticed a client walking onto the site with no PPE and they haven’t signed in.

| Answer | I would approach the client, respectfully remind them about the PPE requirements on site and sign in process. And insist that they head to the main office, sign in and put on the correct PPE before heading back out on site. |  |
| --- | --- | --- |

Discuss the importance of accurate setting out of offsets.

| Answer | Reworks and financial implications! Offsets need to be 100% accurate otherwise the time and money we put into building these structures, if set out wrong will have to be re-worked and all cost associated with this re-works will be our own fault. |  |
| --- | --- | --- |

You have noticed a worker who is assisting during setting out, doesn’t seem to know what they are doing. How will you respond to this situation?

| Answer | I will head over there and stop and take 5 with them. Bring over the plans and go through with them what needs to be done and ask questions, so I know they are on the same page as me. Then when work starts back again, I will hang around a bit while I watch them setting back out and make sure, they are doing it right before I carry on to the next task. I will do more frequent visits to keep check on them, so I know it’s getting done correctly. |  |
| --- | --- | --- |

After speaking with the worker from the previous question, you establish that this worker has difficulty calculating percentage of slope, as well as other things. Provide a simple equation you can provide to assist the workers learning.

| Answer | Slope % is Rise divided by Run x100 =……….. So, if the Rise is 2.5 and the Run is 10 2.5 divided by 10 x100 = 25% |  |
| --- | --- | --- |

What tool and equipment maintenance would direct the team about before setting out the site?

| Answer | - Survey equipment ie: laser levels, dumpys total station. - We need to make sure we have the callobration certificates etc prior to commencement of works otherwise all set what be insured and could come back and make us liable. - Also maintain in accordance with manufacturers instructions |  |
| --- | --- | --- |

Describe how you ensure all workers understand the communication methods used on site and all appropriate equipment before beginning work tasks.

| Answer | This would form part of there onsite induction and orientation of the new site. It can also be relayed through a toolbox talk and Jha process. There will also be signage erected on site to show what UHF channel the site is on etc. |  |
| --- | --- | --- |

Calculate the amount of road base required for an area of 200m in length, 12.4m in width and a depth of 250mm. Answer using cubic metres (m³). Allow an extra 20% for compaction.

| Answer | 200x12.4x0.250=620(m3) x.2=124(m3) Total= 744(M3) |  |
| --- | --- | --- |

Calculate the following to determine the percentage of grade. A rise of 820 millimetres and a run of 60 metres (Round up to two (2) decimal points).

| Answer | 0.820 divided by 60x100= 1.3667 =1.37% |  |
| --- | --- | --- |

How do supervisors know that they are working according to client and organisational requirements when conducting excavation work?

| Answer | You will always go off a set of IFC drawings (issued for construction) We also check its in accordance with SWMNS operational policies and procedures and to the local council specifications. During the works we would carry out inspections during witness points and hold points to make sure we are building to the plan and the client is happy with said work! |  |
| --- | --- | --- |

Briefly describe two (2) situations that would require supervisors to make changes to the work practices or job plan.

| Answer 1. | Latent conditions can be a factor when it requires the site supervisor to make changes to the work practices or job plan. |  |
| --- | --- | --- |
| 2. | Safety risks are another. New hazard might arise where it requires to change the methodology or bring in additional controls etc. |  |

What are two (2) types of reports an organisation could use to record how installation against required outcomes is progressing and identify opportunities for improvement in relation to safety, efficiency, and effectiveness.

| Answer 1. | Job Dairy |  |
| --- | --- | --- |
| 2. | Reporting through correspondence |  |

Why should written information be clearly expressed?

| Answer | It because a factual reliable record of correspondence so there is no misunderstanding. It ensure quality compliance and reduces any risk or error or mistakes. |  |
| --- | --- | --- |

What is the purpose of levelling equipment and why is routine inspection, proper handling and regular calibration important?

| Answer | Levelling equipment is what we build our whole job off so this is critically important. Maintenance and calibration in the periods designated by the manufacturers is important for insurance liability. It must be handled with care to make sure it has longevity and can used to it’s maximum ability. |  |
| --- | --- | --- |

RIIMPO402D

Apply the principles of earthworks construction

PRACTICAL ASSESSMENT

| Participant name |  |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 (Occasion 1) | Select Date. | Workplace Simulation |  |  |  |
| Task 1 (Occasion 2) | Select Date. | Workplace Simulation |  |  |  |

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 1 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Provide detail |
| 1.1A. | Specify () documents read and confirmed: Plans/ Drawings/Maps Specifications TMP EMP Site Safety Plan SWMS Engineering survey data Site cultural and heritage information Sources of material Labour resources required, including sub-contractors Schedule Costing requirements Other (please specify): |
| 1.1B. | Specify () specifications clarified: Scheduling requirements Milestones Testing requirements Materials Site soil/rock and geological data Other (please specify): |
| 1.1D. | Specify () job plan inclusions: HR requirements Plant and machinery requirements Construction materials requirements Sub-contractor support requirements Waste disposal requirements Coordination requirements Activity scheduling Materials delivery scheduling Risk assessment and management requirements WHS requirements Quality management requirements, including testing scheduling requirements Traffic management requirements Environmental requirements Task monitoring requirements Task performance monitoring requirements Communication requirements Reporting requirements Other (please specify): |
| 1.2K. | Specify () test results read and correctly understood: Compaction test results Soil/ density/ moisture relationship Plasticity index Particle size distribution Other (please specify): |
| 1.3B. | Specify adjustments made: |
| 1.4B. | Specify recommendations made: |

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 2 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Details |
| 1.1A. | Specify () documents read and confirmed: Plans/ Drawings/Maps Specifications TMP EMP Site Safety Plan SWMS Engineering survey data Site cultural and heritage information Sources of material Labour resources required, including sub-contractors Schedule Costing requirements Other (please specify): |
| 1.1B. | Specify () specifications clarified: Scheduling requirements Milestones Testing requirements Materials Site soil/rock and geological data Other (please specify): |
| 1.1D. | Specify () job plan inclusions: HR requirements Plant and machinery requirements Construction materials requirements Sub-contractor support requirements Waste disposal requirements Coordination requirements Activity scheduling Materials delivery scheduling Risk assessment and management requirements WHS requirements Quality management requirements, including testing scheduling requirements Traffic management requirements Environmental requirements Task monitoring requirements Task performance monitoring requirements Communication requirements Reporting requirements Other (please specify): |
| 1.2K. | Specify () test results read and correctly understood: Compaction test results Soil/ density/ moisture relationship Plasticity index Particle size distribution Other (please specify): |
| 1.3B. | Specify adjustments made: |
| 1.4B. | Specify recommendations made: |

RIIMPO402D

Apply the principles of earthworks construction

Practical Task 1 (2X)

Description of task: The participant must demonstrate the ability to apply the principles of earthworks construction while undertaking the role of a civil construction supervisor on two (2) occasions including demonstration of: - During planning and preparation of work: - Reading and applying requirements set out in project plans and specifications - Complying with site safety plan and organisational policies and procedures when confirming resource availability - Complete a job plan showing best use of available resources to meet task requirements - Prepare to lead pre-start meeting - When implementing the job plan: - Confirm availability of resources at site - Lead daily pre-start meeting providing clear instructions to meet schedule and safety requirements - Set out the job and allocate tasks to others - While supervising the execution of tasks: - Monitor the installation, checking against performance requirements and identifying gaps - Initiate adjustments to process or work practice to ensure achievement of expected outcomes - Oversee plant, equipment and tool maintenance procedures and ensure written documentation is completed - During reporting: - Complete all written reporting requirements in a timely manner - Make recommendations for changes to improve efficiency, effectiveness, and safety during the execution of tasks Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Project site, project documents, labour, plant, equipment and tools, construction materials.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |  |
| --- | --- | --- |
|  | 1st Occasion | 2nd Occasion |
| 1 Plan and prepare for earthworks construction |  |  |
| - Locate and read all appropriate earthworks construction documentation to confirm compliant activities. |  |  |
| - Read and clarify specific task information and requirements from technical specifications and notes. |  |  |
| - Meet with project management as required to confirm client requirements are met. |  |  |
| - Develop a job plan that meets task requirements and makes best use of resources. Assessor attach a copy of the job plan with participant name and date showing as permitted by workplace. Alternatively note job plan observed. |  |  |
| - Recognise training requirements for team and allocate tasks to ensure safety. |  |  |
| - Prepare for site meetings (toolbox/ pre-start) by preparing notes and resources required. |  |  |
| 2 Implement the job plan |  |  |
| - Confirm availability of all resources required for safe and effective task completion. |  |  |
| - Lead pre-start meeting providing clear, legible and timely instructions from job plan to team members and others both verbal and in written form. |  |  |
| - Allocate tasks to others in a manner to be understood. |  |  |
| - Manage team and other relationships in a manner to diffuse friction and induce work production. |  |  |
| - Monitor checking of plant tools and equipment for serviceability. |  |  |
| - Read and interpret project engineering surveys. |  |  |
| - Undertake set out of offsets according to plans and drawings. |  |  |
| - Adhere to manufacturer and workplace requirements for use of levelling tools and equipment to achieve accurate set out and offset marking. |  |  |
| - Correctly calculates resource consumption, supply rate, volumes, areas, grades and percentages |  |  |
| - Communicate offset requirements using approved workplace methods of marking. |  |  |
| - Interpret information from site soil/rock, hydrological and geological data test results. |  |  |
| - Recognise and address drainage issues according to site requirements. |  |  |
| 3 Oversee the execution of tasks |  |  |
| - Apply performance monitoring skills, checking measurements, making calculations and assessing performance against schedule and requirements for any gaps. |  |  |
| - Make adjustments, or recognise adjustments made to work practices or the job plan and implements these to ensure achievement of the required outcomes |  |  |
| - Monitor performance, recognising and acting on training needs of workers. |  |  |
| - Demonstrate knowledge of plant and equipment capabilities and limitations. |  |  |
| - Adhere to or alter materials delivery scheduling according to requirements and procedures. |  |  |
| - Oversee plant, equipment and tool maintenance procedures on completion of tasks including documentation of maintenance records |  |  |
| - Inspect work to ensure outcomes conform with requirements, applying testing where required. |  |  |
| - Organise or apply mentoring to workers requiring training and monitoring. |  |  |
| 4 Report on task execution |  |  |
| - Complete and submits reports including all test results. |  |  |
| - Adhere to continuous improvement by making reasonable recommendations for improvement of processes or procedures. |  |  |