RIIMPO402D

Apply the principles of earthworks construction

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Michael Moore |
| --- | --- |
| Participant signature |  |

| Assessor name | John Bell |
| --- | --- |
| Assessor signature |  |

| Date | 21/02/2021 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 45 |  | Yes | No |

Assessor Comments

- Compliance documentation and reporting:

- Give one (1) example of each of the following types of compliance documentation relating to earthworks construction:

- Legislation.

- Organisational requirements.

- Client requirements.

- Site operational requirements.

- What are two (2) government agencies or statutory authorities you may have to report to?

| Answer | - Legislation - Occupational Health and Safety Organisational requirements - Workplace relations Client requirements - Client standards and expectations Site operational requirements - Operational procedures eg: Safe Operating Procedures - Workcover EPA |  |
| --- | --- | --- |

- Administrative documentation and project quality management:

- List three (3) types of administrative documentation that you may have to complete.

- What are three (3) factors of project quality management?

| Answer | - Daily Activities - Ordering and reordering - General administrative tasks - Documentation - Work quality and standards - Dimension tolereances of tasks |  |
| --- | --- | --- |

- What is the aim of OHS/WHS legislation?

| Answer | To ensure safety of everyone on site so everyone can go home safely to there loved ones. |  |
| --- | --- | --- |

- List three (3) requirements that may be covered by the environmental management plan for your worksite?

| Answer | Noise and Vibration eg Keep noise to minimum before we start works. Dust management eg Water truck on site for dust suppression Water quality eg test water water before you pump out to make sure ph is all good. |  |
| --- | --- | --- |

- What are the three (3) plans that outline the requirements and procedures for controlling traffic and people around and through the work zone?

| Answer | TMP (Traffic management plan)eg Deals with traffic moving through site Vehicle management plan eg haul circuits Appropriate signage to be put up Site isolation plan eg Temp fencing site signage with supervisor name barricades |  |
| --- | --- | --- |

- Why is it important to consult and communicate with others during the risk management process?

| Answer | This will help ensure that risks and hazards are identified so workers on site can control these hazards. By doing this everyone on site goes home safe each day. |  |
| --- | --- | --- |

- What should you do if you don’t understand the meaning of a particular term, phrase or word used on your worksite?

| Answer | Speak to the supervisor or the peers above to explain the term properly. Otherwise it could disrupt operations and cause a safety issue. |  |
| --- | --- | --- |

- Geotechnical, geological, survey and engineering data:

- List three (3) ways that geotechnical data is used in earthworks.

- List three (3) uses of soil or geological data.

- What is survey and engineering information used for?

| Answer | Identify soil and rock types Check ground conditions and materials under ground surface Determine their physical mechanical and chemical properties Defining material types ,Locating water tables, compaction levels - Define work areas and zones. Helps to determine thrFloor heights, road profile requirements, voids and underground situations |  |
| --- | --- | --- |

- Hydrological and meteorological data:

- Why is hydrological data important?

- List three (3) factors that may be included in meteorological data.

| Answer | - To determine the properties, distribution and effects of water, moisture content site soil and rocks. This will ensure we are using correct materials and processes. - Rainfall, temperature and Humidity |  |
| --- | --- | --- |

- Project plans and specifications:

- What are three (3) things provided in project plans and drawings?

- What is the purpose of project and task specifications?

| Answer | - Location of site and earthworks in relation to the area, Structures roads, Layout of drainage - Lists the types and characteristics of particular earthworks tasks to be done. - And presented in the quality requirements, project drawings. |  |
| --- | --- | --- |

- Drainage and dewatering:

- List two (2) examples of drainage and dewatering requirements.

- Why must any drainage and dewatering issues be managed promptly and effectively once you have identified them?

| Answer | - Pumping out of basins, any form of erosion or sediment controls - No dirty or contaminated water to leave site. Penalties could occur. Must abide with EPA requirements. |  |
| --- | --- | --- |

- Why is it important that earthworks construction activities are scheduled within a job plan?

| Answer | Earthworks projects have strict timelines to be met. Make the most efficient use of resources on site. |  |
| --- | --- | --- |

- Soil density and testing:

- Why do you need to test the relationship between soil density and moisture when scheduling activities?

- What does ‘plasticity index’ mean?

- Why is it important for materials to have the right balance of particle sizes?

| Answer | - To gain adequate compaction as it is necessary to have moisture and density balanced. Eg if moisture is high can have softies in material which will have to re worked and if moisture is low it won’t compact. - Determines how the materials can be shaped and relate to the clay content in material. Eg high clay content can be difficult to work - To ensure correct construction of layers. Must be correct balance of particle sizes to meet specs. It is important when backfilling structures so no damage occurs to structure. |  |
| --- | --- | --- |

- Earthwork calculations:

- How do you calculate the following in earthworks construction:

- Grade or slope.

- Volume.

- Area.

- What is percentage or cross fall?

- How do you work out the rates of resource consumption?

| Answer | - Rise divide Run, Length x width x Depth, Rise divide Run - Slope placed on earthworks to ensure water can drain of work area - Daily production figures will be used as a guide line. |  |
| --- | --- | --- |

- Sequencing of civil works task:

- How does an understanding of the relationship between the different areas of civil works help in planning work?

- What is the sequence of tasks for drainage works?

| Answer | - Always ensure that all base activities or sub tasks are completed. These are the things that need to be completed prior to commencement of other works. - Survey to peg lDrainage line, excavate drainage line , bedding,lay pipe, culverts, headwalls, manholes,, backfill to required earthworks level. |  |
| --- | --- | --- |

- Job planning and administration:

- List four (4) requirements that should be included when drafting a job plan.

- What are three (3) examples of administration procedures and requirements?

| Answer | - Human resources, risk management, task and performance monitoring, activity scheduling - Sub contractors, supply agreements, Accounts procedures |  |
| --- | --- | --- |

- Interpret requirements and provide estimates:

- Why is it important to correctly interpret the requirements of the project contract?

- What are two (2) factors you may have to provide estimates for, and how do you go about it?

| Answer | - To ensure that all earthworks plans, client requirements and quality are known, understood and achieved. - Cost of production to cost of materials, How many activities need to be completed. Total of these two figures will give an estimated total of hours. |  |
| --- | --- | --- |

- List four (4) types of resources that will need to be available and ready for earthworks construction tasks.

| Answer | Labour, Plant and Equipment, construction materials, subbies. |  |
| --- | --- | --- |

- How can you be sure that you have followed the relevant resource requirements and procedures?

| Answer | Checking daily plant inspection checklists, Project and task documentation Workers know how to use machinery or tools Resource requirements can change throughout the day so may need additional labour operators to get job done. |  |
| --- | --- | --- |

- How do you confirm that plant and equipment is suitable for the task?

| Answer | Daily plant inspection checklists. Operators notify if any damage or any faults asap. |  |
| --- | --- | --- |

- Provide instructions and supervise team members:

- List three (3) ways you can provide clear and timely instructions to the work team.

- List three (3) team members and others who may be involved in the earthworks activities you are supervising.

| Answer | - Toolbox, pre starts, requirements and procedures - Workers directly involved in the task, suppliers representatives, subbies |  |
| --- | --- | --- |

- List two (2) factors that may be detailed in your worksite communication requirements and procedures?

| Answer | Two way radios, hand signals |  |
| --- | --- | --- |

- Prepare messages and instructions:

- Why is it important to prepare for and conduct briefings, toolbox and site meetings?

- What should you keep in mind when preparing and delivering short messages or instructions?

| Answer | - So the appropriate and relevant information is conveyed to the team. Allows interaction between management supervisory staff , workers , subbies to be involved in activities - To be clear , precise to be easy understood. |  |
| --- | --- | --- |

- Why is it important for a leader/supervisor to develop and maintain workplace relationships?

| Answer | It promotes a good work environment and encourages effective interactions between team members,supervisors,management. |  |
| --- | --- | --- |

- What factors will determine the leadership style and techniques you adopt?

| Answer | Depends on the nature and dynamics of the group and experience of the other workers. Skills and knowledge of the team and ensure that everybody can contribute. Give feedback to individuals and team as a whole and be constructive and helpful. |  |
| --- | --- | --- |

- Supervisor techniques:

- What are four (4) things you need to be able to do as a supervisor?

- List the five (5) techniques commonly used in mentoring.

| Answer | - Communicate, organise, plan, manage people and resources - Accompanying, sowing,catalysing, showing, harvesting |  |
| --- | --- | --- |

- What do set out requirements and procedures include?

| Answer | Establishing control lines – base line for measuring from Establishing cleared width – indicates the level of clearing that must be undertaken Determining batters – are slopes that may be generally formed Establishing construction offsets – before a project begins boundaries are pegged and flagged. |  |
| --- | --- | --- |

- What are construction offsets and how are they established?

| Answer | Survey pegs or bench marks which can tell how far from a control line the works is to take place or an area to be cleared Survey places Survey pegs with offsets written on pegs. |  |
| --- | --- | --- |

- List three (3) factors of road geometry.

| Answer | Grade or slope Crests structures |  |
| --- | --- | --- |

- What are levelling devices used for in the setting out process?

| Answer | Determining Drainage grades Measuring offsets Road geometry |  |
| --- | --- | --- |

- What are the four (4) aspects of project and task monitoring that you must carry out?

| Answer | Engineering survey Risk assessment and management Sampling and testing General supervision, observation and recording |  |
| --- | --- | --- |

- Engineering principles and interpret test results:

- Why should earthworks operators be aware of the engineering survey principles for the job?

- Why do you need to interpret material properties and compaction test results?

| Answer | - The works is being done correctly, with correct layer formation and within required tolerances. Earthworks operators are aware of the relevant engineering survey principles that apply. - Testing should be undertaken so the properties are known and understood to allow for other treatment where necessary. Test results are also used to ensure levels of compaction are achieved based on specs. |  |
| --- | --- | --- |

- Monitor earthworks tasks:

- List four (4) criteria that may relate to performance of the tasks you are monitoring.

- What are three (3) methods you might use for monitoring earthworks tasks?

| Answer | - Compaction, Layer thickness, clay content, particle size - Field checklists, survey and level checks, Geo tech testing. |  |
| --- | --- | --- |

- Why should routine inspections of earthworks operations be carried out?

| Answer | To ensure compliance with OHS legislation also complying with contract obligations.and requirements specific to the project. |  |
| --- | --- | --- |

- List four (4) methods and operational techniques involved in earthworks construction practice.

| Answer | Site prep Load and haulage Placement Surface finishing |  |
| --- | --- | --- |

- How will you find out the operational techniques for the earthworks job you are working on?

| Answer | Outlined in the relevant procedures for the work activity. Project and task specs also indicate operational techniques for various types of plant , tools and equipment to be used. |  |
| --- | --- | --- |

- In ground surface treatment, what is proof rolling?

| Answer | Proof rolling shows any movement in the ground. Any movement indicates unsuitable material in the ground which can come from ground not compacted properly and will have to be removed. |  |
| --- | --- | --- |

- Give two (2) examples of how you should inform operators and other personnel/workers in writing and verbally that changes to the job plan or relevant earthworks construction practice have been initiated?

| Answer | Written communication- plans drawings sketches Oral- Briefings, toolbox meetings, verbal, team meetings |  |
| --- | --- | --- |

- What are four (4) maintenance activities that may be carried out by earthworks construction operators?

| Answer | Cleaning and check for damage before and after use Minor repairs eg replacing ripping boots Check oils Record any damage |  |
| --- | --- | --- |

- How can you ensure that your team carries out maintenance activities in line with site procedures and the manufacturer’s specifications?

| Answer | Daily plant checklists Create a schedule to ensure at regular times are allocated for cleaning checking repairs and any other maintenance tasks. |  |
| --- | --- | --- |

- List four (4) reasons why you should follow organisational and/or site requirements and procedures for preparing and presenting reports.

| Answer | How and when reports to be completed Who completes report How the report is submitted Supporting documentation |  |
| --- | --- | --- |

- What are three (3) guidelines you should keep in mind when preparing reports?

| Answer | Keep clear and precise Proofread Present most important info first |  |
| --- | --- | --- |

- What are logbooks and diaries used for?

| Answer | To keep track of machinery maintenance |  |
| --- | --- | --- |

- List four (4) things that you need to do when recommending changes or improvements to the safety, effectiveness or efficiency of tasks.

| Answer | Summarise the situation Clarify why you are providing advice Outline all the considerations Explain the risks or hazards |  |
| --- | --- | --- |

- How do you develop individual workers or the team as a whole?

| Answer | Team building exercises can be created in any situation where change is needed. By listening and considering ideas and suggestions as a supervisor are building the individual and team .to function at a higher level and team morale will be better. |  |
| --- | --- | --- |