# Documentation of design and construction (suitability or evidence of suitability)

| # Project Name: Multi-Residential Apartment Building |  |  |  |
| --- | --- | --- | --- |
| # Project address: 12 Example Street, Brisbane, QLD |  | # Site Plan attached: No |  |
| Prepared by: Abbie Heaslip |  |  |  |
| # Date: 8/7/25 |  |  |  |
| # Description of project: # This project is a multi-storey apartment building with 6 sole-occupancy units (Class 2), two units per storey. Reinforced concrete floors, walls, and columns are required for structural stability and fire separation. The building is located in Climate Zone 2 and must comply with energy efficiency, accessibility, and fire safety requirements as outlined in the NCC. |  |  |  |
| # Site Classification/s: # M (Moderately Reactive Clay) as per AS 2870. |  |  |  |
| Documentation of design and construction (suitability or evidence of suitability) |  |  |  |
| Site preparation |  |  |  |
| Site preparation | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Earthworks | Earthworks must stabilize the site and prevent soil erosion (AS 3798). |  | Earthworks were conducted to ensure stable foundations for construction. |
| Site preparation |  |  |  |
| Site preparation | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Earth retaining structures | Retaining walls must resist lateral soil forces and manage hydrostatic pressure (AS 4678). |  | Reinforced concrete retaining walls were designed with drainage systems to reduce hydrostatic pressure. |
| Drainage | Drainage systems must prevent water ingress into habitable areas (NCC Volume 1, Part F1). |  | Surface drains and stormwater management systems were installed to prevent water pooling. |
| Site preparation | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Termite risk management | Termite barriers are required for Class 2 buildings (AS 3660.1). |  | Physical termite barriers were installed beneath slabs and around service penetrations to prevent infestations. |
| Materials |  |  |  |
| Footings and slabs |  |  |  |
| Site classification |  |  |  |
| Footings and slabs | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Footings and slabs | Footings and slabs must be designed to resist soil movement (AS 2870). |  | Reinforced concrete slab-on-ground was designed for Class M soil reactivity and includes proper reinforcement. |
| Footings and slabs | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Moisture Barrier | Moisture barriers are required to prevent dampness (AS 2870). |  | A polyethylene membrane was installed beneath slabs to protect against rising moisture. |
| Materials |  |  |  |
| Concrete and reinforcement |  |  |  |
| Concrete and reinforcement | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Concrete and reinforcement | Concrete and reinforcement must meet structural and durability requirements (AS 3600). |  | Concrete mix was designed for durability, with adequate cover to reinforcement for fire resistance. |
| Materials |  |  |  |
| Masonry |  |  |  |
| Masonry | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Masonry | Masonry walls must meet structural and fire resistance requirements (AS 3700). |  | Fire-rated masonry was used for internal walls, achieving FRLs of 60/60/60 and 90/90/90. |
| Moisture Barrier |  |  |  |
| Materials |  |  |  |
| Framing/Structure |  |  |  |
| Framing/Structure | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Framing | Framing must achieve structural adequacy (AS 1684). |  | Reinforced concrete columns and beams were installed to meet NCC structural requirements and FRLs. |
| Materials |  |  |  |
| Roof and Wall Cladding |  |  |  |
| Roof and wall cladding | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Roof and wall cladding | Cladding must be weatherproof and non-combustible (NCC Volume 1, Part F1). |  | External cladding used was non-combustible and compliant with fire safety regulations. |
| Materials |  |  |  |
| Glazing |  |  |  |
| Glazing | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Glazing | Glazing must meet energy efficiency and safety requirements (AS 1288 and AS 2047). |  | Double-glazed windows were installed for energy compliance, and safety glazing was used for balcony doors. |
| Materials |  |  |  |
| Fire Safety |  |  |  |
| Fire Safety | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Fire Safety | Fire resistance levels (FRLs) must meet NCC Part C1. |  | Fire-rated walls, doors, and floors were installed with FRLs of 90/90/90 and 60/60/60 where required. |
| Access/Stairs/Lifts/Ramps |  |  |  |
| Access/Stairs/Lifts/Ramps | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Access | Access must meet AS 1428.1 for accessibility. |  | Continuous accessible paths and lifts were provided for all levels. |
| Materials |  |  |  |
| Ancillary Provisions |  |  |  |
| Ancillary Provisions | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Energy Efficiency |  |  |  |
| Energy Efficiency | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Energy Efficiency | Energy efficiency must comply with NCC Section J for Climate Zone 2. |  | Insulation, energy-efficient lighting, and glazing were installed. |
| Health and Safety |  |  |  |
| Safe Work Method Statements (SWMS) prepared for high-risk activities, including concrete pouring, structural framing, and roofing. Regular site inspections conducted to identify and mitigate hazards. Weekly toolbox talks held to address site-specific safety concerns and fire safety updates. Emergency response plan developed, including evacuation routes and first aid procedures. Personal Protective Equipment (PPE) policy enforced, requiring hard hats, high-visibility clothing, gloves, and safety boot |  |  |  |

| Consultation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Consultation with fire safety engineers to ensure fire-rated walls meet the NCC’s specified Fire Resistance Levels (FRLs). Accessibility consultant engaged to confirm compliance with AS 1428.1 for tactile indicators and continuous accessible paths. Structural engineer consulted to verify site grading and drainage systems to prevent water ingress. Pest control specialist engaged to oversee termite barrier installation as per AS 3660.1. |  |  |  |  |
| Policies |  |  |  |  |
| Work Health and Safety (WHS) Policy: Ensures compliance with safety regulations, providing a safe working environment for all workers. Environmental Management Policy: Minimizes site impact, addressing waste management and erosion control. Fire Safety Policy: Ensures fire protection systems and fire-rated materials are compliant with NCC requirements. |  |  |  |  |
| Procedures |  |  |  |  |
| Site Inductions: All workers and subcontractors complete site-specific safety inductions. SWMS and Toolbox Talks: High-risk activities are documented in SWMS, and weekly toolbox talks address ongoing safety concerns. Inspections and Maintenance: Regular inspections of fire-rated materials and accessibility features during construction. |  |  |  |  |
| Issues to be addressed and consultation |  |  |  |  |
| Issue | Who to Consult | Improvements for Consideration | Feedback Following Consideration | Issue |
| Accessibility compliance for tactile indicators. | Accessibility consultant. | Ensure tactile indicators and compliant handrails are installed as per AS 1428.1. | Feedback confirmed compliance with tactile indicator placement and handrail design. | Accessibility compliance for tactile indicators. |
| Fire safety compliance for fire-rated walls and doors. | Fire safety engineer and certifier. | Conduct inspections during installation to ensure FRLs of 60/60/60 and 90/90/90 are achieved. | Adjustments to door frames were made to meet fire safety standards. | Fire safety compliance for fire-rated walls and doors. |
| Preventing water ingress through proper drainage. | Structural engineer and plumber. | Install perimeter drains and ensure proper grading to direct stormwater away from the building foundations. | Drainage systems passed inspection, grading near the rear entrance was improved to manage water flow. | Preventing water ingress through proper drainage. |
| Termite prevention. | Pest control specialist. | Install physical termite barriers beneath slabs and around penetrations as per AS 3660.1. | Termite barriers were installed successfully and verified by pest control specialist. | Termite prevention. |
| Issue | Who to Consult | Improvements for Consideration | Feedback Following Consideration | Issue |