CPCCBC4012B

Read and interpret plans and specifications

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4012B Read and interpret plans and specifications cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Danielle Henderson |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Danielle Henderson |
| --- | --- |
| Participant signature |  |
| Date | 28/03/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 87 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

- Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

- Which compass direction is always shown on plans and drawings?

| Answer | North |  |
| --- | --- | --- |

- What information is provided on a location plan?

| Answer | A location plan is required for large developments to ensure that the location of the site in a particular street is clear which will aid in deliveries and access. The plan will identify the building site and have the following details shown: Lot number/s Deposit plan numbers North point Scale |  |
| --- | --- | --- |

- What detail does a site or block plan contain?

| Answer | A site or block plan, as a general rule, will cover the following specific detail of the block for the proposed development: Outline of site boundaries showing location of proposed building and existing dwellings Orientation to north point Street frontage names Major geological and topographical features (trees and vegetation) Datum points, contours and slopes (with reference height to other site levels) Driveways, stormwater drainage, easements and right of carriage Soil and surface water drains, pipe sizes Service runs from the house to mains - utility services location (sewer, water, gas, electricity) Details of paving and retaining walls |  |
| --- | --- | --- |

- What do floor plans identify?

| Answer | A floor plan is a scaled diagram and shows all detail of the proposed structure. This includes, however not limited to, rooms, positions of fixtures, floor coverings. These plans are drawn showing a view from above and can include dimensions between walls to note room sizes and wall lengths. Additional detail to be shown can include: - Symbols of electrical items - Construction methods - Finishes & fixtures Scaling for floor plans of a large building would generally be drawn at 1:200, whereas floor plans for standard building sizes would be drawn at 1:50 and 1:100. Floor plans are important to be provided to subcontractors for accurate estimation. |  |
| --- | --- | --- |

- What is a landscape plan and what information might it provide?

| Answer | Landscape plans are similar to floor plans however they show details for an outdoor area and can include for natural elements such as grass, flowers and trees as well as paths, furniture, sheds and retaining walls. These plans also use scaled dimensions and may include an overlay for services contractors (irrigation and lighting). |  |
| --- | --- | --- |

- What are elevation views? What do they provide to the plan reader?

| Answer | Elevation views differ from floor plans in two distinctive ways; - They can show multi-storey as well as single-storey details - They show a view of the proposed structure from its sides (i.e. from north, south, east and west as if the viewer was standing in front of that wall). Each of these side views is what is called an elevation view and this is indicated on the plan (if the wall shown is on the south side of the house, this will be nominated as the south elevation.). These views are important to show further detail of finishes and sizing of external walls, windows, doors and balconies |
| --- | --- |

- What is a section?

| Answer | Even though an elevation plan can include more information for specific finishes, this won’t always allow a builder or tradesman to determine the exact construction for many structural parts to be built or assembled internally. To allow for this information, a section is a view of the proposed construction after it has been cut vertically at some point, this being shown on the plan to assist with interpretation. Like floor plans it shows the size and thickness of structural members and relationships between spaces. Included on section plans are the following key features: - Height of building - Position of piers, beams, openings - Floor to ceiling heights - Distance between finished floor and existing or proposed ground levels - Construction details |  |
| --- | --- | --- |

- What are detailed drawings?

| Answer | Detailed drawings are provided by and for many contractors on site to give detail of a particular method of construction or finish. These drawings can be used to demonstrate compliance with relevant regulations and have the ability to be a large-scale drawing that shows parts in detail that are otherwise unable to be included on a general arrangement drawing. They can include: - Dimensions - Specification information - Symbols - Tolerances It is important to ensure that duplicate information found in other specifications is not included as amendments or contradictory details can cause confusion. |  |
| --- | --- | --- |

- What are specifications?

| Answer | Specifications are details that should be read in conjunction with plans, drawings, building permits, contract documentation, engineers’ details, and any other legal documentation applicable to the building. Specifications support these documents by providing details of the materials and techniques to be used, details of fixtures and fittings, and standards to be observed to achieve the client’s final aesthetic expectations as well as meeting all regulations for a safe and occupiable building. The specifications will set out any minimum standards or codes (which you should be aware of), however it is important to read in conjunction with the contract documentation to ensure these meet any obligations to go above and beyond the minimum requirements. Key features included within building specifications include: - Levels and survey information Including the current ecological value of the site and any impact by the construction, recommendations to enhance the site for ecology and any requirements to protect ecological features through impact mitigation. - Materials lists Including type of finishes, colour codes, fixtures & fittings, make and model information. Performance data and material technical data - This data includes types, brands, grades, thickness, colour, models, and detailed information for the materials. Schedules of quantities - A bill of quantities should be used during the estimation stage to itemise materials, parts and labour. Stress, load and bearing calculations - These calculations are crucial to the design and must identify and meet all building code requirements. |  |
| --- | --- | --- |

- Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standards |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bull Nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Block |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Glass |  |
|  | CJ | Ceiling Joist |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold Water Tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Proof Course |  |
|  | DSB | Distribution Switch Board |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion Joint |  |
|  | FPBD | Fibrous Plaster Board |  |
|  | FFL | Finished Floor Level |  |
|  | FHR | Fire Hose Reel |  |
|  | GPO | General Purpose Outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

- Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Wall – Cavity Brick |  |  |
|  | Wall – Brick Veneer |  |  |
|  | Wall – Timber Stud |  |  |
|  | Window – In Cavity Wall |  |  |
|  | Window – In Single Wall |  |  |
|  | Door – Swing Single 90 |  |  |
|  | Door – Swing Double 90 |  |  |
|  | Door – Sliding into pocket |  |  |
|  | Door – Sliding exposed on wall |  |  |
|  | Socket Outlet (Power) – General |  |  |

- Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Hinged at frame |  |  |
|  | Hinged at top |  |  |
|  | Sliding horizontally |  |  |
|  | Fixed |  |  |

- What does a clouded area on a plan indicate?

| Answer | A revision or amendment |  |
| --- | --- | --- |

- An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

- On what date was an amendment made to the window details?

| Answer | 05/10/2011 |  |
| --- | --- | --- |

- True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What amendment occurred on 08/11/2011?

| Answer | A robe was deleted from Bed 3 |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

- What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

- What is the Lot number of this site?

| Answer | 16 |  |
| --- | --- | --- |

- What is the size of this lot?

| Answer | 3653m2 |  |
| --- | --- | --- |

- Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

- True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What size is the easement on the eastern boundary of the site?

| Answer | 5 metres |  |
| --- | --- | --- |

- True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

- What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

- Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

- Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26. ?

| Answer | The cross-section plan above shows the proposed residence from Q19. to Q26. from the north elevation view, showing further detail on the sizes and installation of the particular building materials listed. It shows details for a corrugated colorbond metal roof installation, the house to be constructed of 10mm plasterboard walls and ceilings (with W.R. board in all wet areas), 15mm plywood flooring, timber weatherboards to the external and various other fixings. It also indicates verandahs and numerous rooms. The list of materials in the top left of the detail has relevant reference codes that would relate back to the final specification document. |  |
| --- | --- | --- |

- Who is the client?

| Answer | H&M Smith |  |
| --- | --- | --- |

- Where is this project to be built?

| Answer | Lot 16 Glenvalley Road, Richmond |  |
| --- | --- | --- |

- What are the contact details for the draftsman?

| Answer | Joe Foreman (03 9514 2311 / 0413 113 113) 18 Fairview Road, Richmond 3216 |  |
| --- | --- | --- |

- What sheet number is this drawing?

| Answer | Sheet 5 of 7 |  |
| --- | --- | --- |

- What timber is required for loading and non-load bearing walls?

| Answer | F5 pine 45x90 for load bearing, and 35x90 for non-load bearing |  |
| --- | --- | --- |

- What spacing is recommended for common studs?

| Answer | 600 maximum spacing |  |
| --- | --- | --- |

- What type of roof truss is to be installed?

| Answer | Gang Nail roof trusses. |  |
| --- | --- | --- |

- What centres are the trusses positioned at?

| Answer | Trusses are to be installed in a scissor truss application at 900 spacing, with a top chord at 30° and a bottom chord at 20°. Every truss must be strapped to a wall plate |  |
| --- | --- | --- |

- What size are the veranda posts?

| Answer | 90x90 |  |
| --- | --- | --- |

- What timber species are the veranda posts to come from?

| Answer | Oregon (Douglas Fir) |  |
| --- | --- | --- |

- What pitch is the roof?

| Answer | 30° |  |
| --- | --- | --- |

- What size are the noggings and at what spacing?

| Answer | The noggings are 35x70 and spaced at a maximum of 1350 |  |
| --- | --- | --- |

- What size are the ceiling joists?

| Answer | 240x63 LVL |  |
| --- | --- | --- |

- What timber is used for roof battens and what spacing?

| Answer | FG pine battens (35x70) at 900 spacing |  |
| --- | --- | --- |

- What thickness is the plasterboard sheeting?

| Answer | 10mm |  |
| --- | --- | --- |

- What type of flooring is to be installed?

| Answer | 15mm plywood |  |
| --- | --- | --- |

- What is the roof cladding material?

| Answer | Colorbond corrugated metal roofing |  |
| --- | --- | --- |

- Where will you find information about the footings?

| Answer | Refer to the engineers drawing for all footing details |  |
| --- | --- | --- |

- What do the letters ‘DAR’ refer when timber ordering?

| Answer | Dressed All Round, this refers to the finish of the timber and is the result of the timber being machined on all sides, which leaves a smooth surface finish. |  |
| --- | --- | --- |

- Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | This nominates the width (240mm) and height (63mm) of the starting joists. LVL refers to the type of building material; Laminated Veneer Lumber is a high-strength engineered wood product used primarily for structural applications. It is comparable in strength to solid timber, concrete and steel and is manufactured by bonding together rotary peeled or sliced thin wood veneers under heat and pressure. |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

- What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

- What is the Job Number?

| Answer | OOR-112 |  |
| --- | --- | --- |

- What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

- What does the drawing show?

| Answer | The drawing shows each Elevation (North, South, East & West), and a selected section drawing and some basic information about the building materials, to be cross referenced with the specification and materials list. It also shows reduced levels detailed around the construction |  |
| --- | --- | --- |

- Which view shows the front of the house?

| Answer | East Elevation |  |
| --- | --- | --- |

- How many motor vehicles can be parked in the garage?

| Answer | 2 |  |
| --- | --- | --- |

- What are the window and doors made from?

| Answer | Timber entrance door and external door, and aluminium framed sliding windows and doors. |  |
| --- | --- | --- |

- What is the size of the eaves?

| Answer | 450 |  |
| --- | --- | --- |

- What are the eaves lined with?

| Answer | Hardiflex |  |
| --- | --- | --- |

- What type of roof structure is the roof built from?

| Answer | Timber framed roof trusses at 600 centres (refer to manufacturers design & detail). |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the requirements for the roof structure in Q59. ?

| Answer | Volume 2, Section 2.1 |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | AS 1684 Residential Timber Framed Construction |  |
| --- | --- | --- |

- What is the roof cladding materials made from?

| Answer | Concrete roof tiles |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Volume 2, Section 3.5 |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | AS 4055 Wind loads for housing |  |
| --- | --- | --- |

- What is the roof pitch?

| Answer | 22.5° slope |  |
| --- | --- | --- |

- Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | Power authority – Energex l |  |
| --- | --- | --- |

- Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | The timeframes for power connection differ depending on if an existing supply is available. If so, a Network Connection Agreement can be submitted and will be assessed within 10 business days. If supply isn’t available on the site, there will be investigation and an estimate of costs provided. The client then has 20 business days to provide a response. From this point, works on site by the electrician can take place and an EWR will be submitted once pre-connection works are finalised. Energex will attend within 5 business days of an approved request. |  |
| --- | --- | --- |

- What is a prime cost (PC) item? Give an example.

| Answer | Prime Costs are nominated in an estimate for items that the specific type has not yet been determined. It is up to the contractor to allow for the minimum (or higher) cost for the reasonable costs of this item. For example, a client has advised that they haven’t selected the technical information for downlights, however they need 10 installed. The contractor can allow a Prime Cost of $12.50 per downlight to cover the minimum they will incur per light. Should the $12.50 downlights selected by the contractor not be approved, and the client wishes for more expensive lighting to be installed, they would need to approve the variation costs. It is important to remember that this may not only affect the materials, and there may be labour variation costs to install the final material. |  |
| --- | --- | --- |

- What is a provisional sum (PS) item? Give an example.

| Answer | A Provisional Sum is detailed in an estimate to allow for a sum to complete works that cannot be determined at the time of quoting or signing the contract. This is usually surrounding site works, however the contractor is legally required to take all reasonable steps to ensure an accurate site works costing estimate, as increases to this sum will apply to unforeseen circumstances. For example, a site that is relatively clear but requires excavation works may encounter large rocks or tree roots underground that would involve more machinery and labour on site. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

- True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify two (2) materials being used for the floors?

| Answer | Carpet and vinyl |  |
| --- | --- | --- |

- True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- How much does the contracted plasterer cost?

| Answer | $12,900.00 + GST |  |
| --- | --- | --- |

- How much do the kitchen appliances cost?

| Answer | $2,700.00 + GST |  |
| --- | --- | --- |

- What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

- What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

- How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

- If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $275.00 |  |
| --- | --- | --- |

- How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 |  |
| --- | --- | --- |

- What material has the highest total cost?

| Answer | Pine 90 x 45 mgp10, 80 @ 3m lengths and 25 @ 3.3m lengths, total cost $322.50 |  |
| --- | --- | --- |

- How many metres are required of the material in Q80. ?

| Answer | 322.5m |  |
| --- | --- | --- |

- What beam is identified as being a better option than F17?

| Answer | E14 beams |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

- Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter (Road Construction) |  |
| --- | --- | --- |

- If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $214.80 |  |
| --- | --- | --- |

- Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar Bender and Fixer |  |
| --- | --- | --- |

- If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $1,285.80 |  |
| --- | --- | --- |

- Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | Building Act 1975 (Qld) Building Regulation 2006 (Qld) Queensland Building and Construction Commission Act 1991 Queensland Building and Construction Commission Regulation 2018 |  |
| --- | --- | --- |

CPCCBC4012B

Read and interpret plans and specifications

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name |  |
| --- | --- |
| Participant signature |  |
| Date | Click here to enter a date. |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4012B

Read and interpret plans and specifications

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to read and interpret plans and specifications in order to inform estimation, planning and supervisory activities. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - Any drawing with a cloud or mark-up you have inserted. - Copy of the correspondence regarding mark up and any site management/authorising engineer response. The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Drawings (including site plan), Specifications, Tender documentation, Telephone, Facsimile machine, Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: |  |  |  |
| Name of observer: |  |  |  |
| Role of observer (Confirm suitability to sign) |  |  |  |
| Email address of observer |  |  |  |
| Signature of observer: |  |  |  |
| Date: | Click here to enter a date. |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| 1 Interpret drawings |  |  |  |
| - Identify purpose and advantage of different types of drawings. |  |  |  |
| - Identify different aspects of drawings. |  |  |  |
| - Identify, understand and apply: - Commonly used symbols and abbreviations - Common building and construction terms on drawings |  |  |  |
| - Identify building site from location drawings. |  |  |  |
| - Identify on the site plan: - True north - Building orientation - Key features in relation to building and other structures - Levels - Contours - Service entry points - Site features to be removed or retained |  |  |  |
| - Identify key features of plans, elevations and sections. |  |  |  |
| - Identify client requested variations to standard plans on drawings. |  |  |  |
| 2 Interpret specifications |  |  |  |
| - Identify and correctly apply Provisional Sum (PS) and Prime Cost (PC) values. |  |  |  |
| - Identify and confirm customer variations to standard specifications and communicate changes to organisational personnel. |  |  |  |
| - Apply and communicate correct interpretations of essential elements to estimation, planning and supervisory tasks (including labour hours/costs and material quantities/costs). |  |  |  |
| - Identify Building Codes or standards affecting the work to be undertaken (including references to Australian Standards and the National Construction Code). |  |  |  |
| - Identify key features of products included in the specification, including the design, purpose, aesthetics and cost relationships. |  |  |  |
| 3 Communicate with others |  |  |  |
| - Consult with industry professionals, clients, organisational personnel and appropriate local authorities. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Using technology (including telephone, facsimile, email) |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments