# Documentation of design and construction (suitability or evidence of suitability)

| # Project Name: Aurora on Thynne |  |  |  |
| --- | --- | --- | --- |
| # Project address: 109 Thynne road, Morningside |  | # Site Plan attached: |  |
| Prepared by: Michael Trott |  |  |  |
| # Date: 21/07/24 |  |  |  |
| # Description of project: # Town house project |  |  |  |
| # Site Classification/s: # Class 1a building |  |  |  |
| Documentation of design and construction (suitability or evidence of suitability) |  |  |  |
| Site preparation |  |  |  |
| Site preparation | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Earthworks | Site preparation (Part 3.1.1) |  | Site preparation (Part 3.1.1) Excavation and Filling: The site must be excavated and filled to provide a stable foundation. This involves removing any unsuitable material (e.g., vegetation, organic matter) and properly compacting the fill material |
| Site preparation |  |  |  |
| Site preparation | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Earth retaining structures | AS 4678-2002 |  | Earth-retaining structures must be designed to ensure structural stability and safety. |
| Drainage | Part 3.3 Drainage |  | The site must be graded to direct surface water away from the building's foundation. This involves sloping the ground away from the building to prevent water from pooling around the foundation |
| Site preparation | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Termite risk management | AS 3660.1-2014: Termite Management - New Building Work AS 4312-2019 |  | Termite-Resistant Materials: Use termite-resistant materials for parts of the building that are at risk, such as timber used in structural elements. Install physical or chemical termite barriers around footings and slabs to prevent termite access and damage |
| Materials | AS 2660.1-2019: Timber |  | Use timber that is graded and treated according to AS 2660.1 to ensure it is suitable for its intended use and resistant to pests and decay. |
| Footings and slabs |  |  |  |
| Site classification: |  |  |  |
| Footings and slabs | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Footings and slabs | AS 2870-2011: Residential Slabs and Footings |  | Ensure that concrete used in structural elements meets strength requirements and is properly reinforced as per AS 3600 |
| Footings and slabs | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Moisture Barrier | AS 3740-2010: Waterproofing of Wet Areas within Residential Buildings |  | In wet areas such as bathrooms, kitchens, and laundry rooms, moisture barriers must be installed to prevent water ingress. This includes the use of waterproof membranes or coatings. |
| Materials | AS 2870-2011: Residential Slabs and Footings |  | Footings should be constructed from concrete that meets the strength and durability requirements specified in the standard. The concrete must be mixed, placed, and cured to achieve the desired properties. |
| Concrete and reinforcement |  |  |  |
| Concrete and reinforcement | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Concrete and reinforcement | AS 3600-2018: Concrete Structures |  | Concrete must meet the specified compressive strength, which is typically measured in megapascals (MPa). Common grades of concrete include C25/30, C30/37, and C40/50, with the numbers indicating the compressive strength in MPa. |
| Materials | AS 4671-2001 |  | Reinforcing steel must comply with AS 4671-2001, which specifies requirements for tensile strength, ductility, and corrosion resistance. Common grades include Grade 500N. |
| Masonry |  |  |  |
| Masonry | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Masonry | AS 3700-2018: Masonry Structures |  | Load-Bearing Walls: Design masonry walls to support loads, including vertical loads from floors and roofs and lateral loads such as wind. The design should account for factors like wall thickness, reinforcement, and bonding patterns. |
| Moisture Barrier | AS 4773 – Masonry in Small Buildings |  | Proper installation of moisture barriers and DPCs is crucial. They should be installed at the correct level and be continuous to be effective |
| Materials | AS 3700 – Masonry Structures |  | AS 4773 – Masonry in Small Buildings Part 1: Specifies the materials and methods for masonry in small buildings, including requirements for masonry units (bricks, blocks), mortar, and reinforcement. |
| Framing/Structure |  |  |  |
| Framing/Structure | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Framing | Design of timber structures: AS 1720.1 AS 4055 – Wind Loads for Housing |  | Framing must be designed to support the expected loads, including live loads, dead loads, wind loads, and seismic forces. |
| Materials | AS 1684 – Residential Timber-Framed Construction AS 4055 – Wind Loads for Housing |  | Material Performance: Requires that all materials used in construction, including framing materials, must be fit for purpose and capable of withstanding loads and environmental conditions. Durability: Framing materials must be durable enough to resist decay, corrosion, and other forms of degradation. This includes ensuring that timber framing is protected from moisture and pests, and metal framing is protected from corrosion. Fire Resistance: Framing materials must comply with fire resistance requirements, especially if they are part of a fire-rated assembly or located near boundaries. |
| Roof and Wall Cladding |  |  |  |
| Roof and wall cladding | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Roof and wall cladding | AS 1562 – Design and Installation of Sheet Roof and Wall Cladding AS 2728 – Prefinished Metal Roofing and Wall Cladding |  | Part 2: Deals with the design and installation of roof and wall cladding using other materials, such as plastic or fibrous cement Specifies requirements for prefinished metal cladding materials, including durability, corrosion resistance, and performance criteria. |
| Materials | Clause B1.1 Clause B2.1 |  | Cladding materials must be designed to withstand expected loads and environmental conditions, including wind, rain, and thermal stress. Cladding materials must be durable, resisting weathering, decay, and other forms of degradation. |
| Glazing |  |  |  |
| Glazing | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Glazing | AS/NZS 2208 – Safety Glazing Materials for Use in Buildings AS 1288 – Glass in Buildings – Selection and Installation |  | Safety Glass: Specifies performance requirements for safety glazing materials to reduce the risk of injury from broken glass. It covers different types of safety glass, including laminated and toughened glass. |
| Materials | AS 1170 – Structural Design Actions AS/NZS 2208 – Safety Glazing Materials for Use in Buildings |  | Ensure that glazing materials meet the specifications set out in Australian Standards for their intended application Safety Glass: Defines performance standards for safety glazing materials to minimize injury from broken glass. Covers requirements for toughened and laminated glass. |
| Fire Safety |  |  |  |
| Fire Safety | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Fire Safety | AS 1530 – Methods for Fire Tests on Building Materials, Components, and Structures |  | Part 1: Provides methods for determining the fire-resistance of materials and structures. Part 2: Describes methods for assessing the fire-resistance of components like walls and floors. Part 3: Covers fire-resistance tests for structural systems |
|  | AS 1851 – Maintenance of Fire Protection Systems and Equipment |  | General Maintenance: Outlines the requirements for the regular maintenance and testing of fire protection systems and equipment, including smoke alarms and fire extinguishers. |
|  | AS 3786 – Smoke Alarms |  | Installation and Performance: Specifies the performance and installation requirements for smoke alarms in residential buildings, including placement and maintenance. |
| Access/Stairs/Lifts/Ramps |  |  |  |
| Access/Stairs/Lifts/Ramps | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Access | AS 1428. AS 1428 – Design for Access and Mobility |  | Doorways must be wide enough to accommodate wheelchairs and other mobility devices |
| Materials | AS 4586 |  | Use materials with appropriate non-slip properties to ensure safety on ramps, stairways, and paths. This includes textured or slip-resistant finishes for floors and ramps to prevent slipping. |
| Ancillary Provisions |  |  |  |
| Ancillary Provisions | NCC / Australian Standards requirements |  | Compliant Performance solution |
|  | AS/NZS 3500 – Plumbing and Drainage |  | Part 1: Covers the design and installation of plumbing systems, including water supply, drainage, and sanitary facilities. It ensures that plumbing works meet safety and performance standards. Part 2: Addresses the installation of sanitary and drainage systems, including requirements for maintenance and performance. Part 3: Provides requirements for stormwater drainage and management system |
|  | AS 4505 – Garage Doors |  | Safety and Performance: Specifies requirements for garage doors, including safety features, durability, and performance criteria. |
|  | AS 3671 |  | Noise Control: Incorporate acoustic treatments and design measures to manage external noise intrusion and improve internal sound quality, as outlined in AS 3671 |
| Energy Efficiency |  |  |  |
| Energy Efficiency | NCC / Australian Standards requirements |  | Compliant Performance solution |
|  | AS/NZS 4859 – Materials for the Thermal Insulation of Buildings |  | General Requirements: Specifies requirements for insulation materials used in building construction to ensure effective thermal performance. |
|  | 3.12.5 |  | Requires the use of energy-efficient hot water systems, such as solar hot water systems or heat pumps. |
|  | Clause 3.12.6 |  | Mandates the use of energy-efficient lighting systems, including LED or compact fluorescent lights, and effective lighting controls |
| Health and Safety |  |  |  |

| Consultation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Policies |  |  |  |  |
| Procedures |  |  |  |  |
| Issues to be addressed and consultation |  |  |  |  |
| Issue | Who to consult | Improvements for consideration | Feedback following consideration | Supporting Resources |