Booking Sheet

| Candidate name | Dawei Yang |
| --- | --- |
| Work role/title |  |
| Organisation/workplace | Dawn construction |
| Date | 08/08/23 |
| Instrument no. |  |

| Backsight | Intermediate sight | Foresight | Rise | Fall | Reduced level | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 2.544 |  |  |  |  | 50.00 |  |
|  | 1.783 |  | 0.771 |  | 50.771 |  |
|  | 0.926 |  | 0.857 |  | 51.628 |  |
|  | 1.963 |  |  | 1.037 | 50.591 |  |
| 1.305 |  | 3.587 |  | 1.624 | 48.967 |  |
|  | 1.432 |  |  | 0.127 | 48.840 |  |
| 3.250 |  | 0.573 | 0.859 |  | 49.699 |  |

| Backsight | Intermediate sight | Foresight | Height of collimation | Reduced level | Top | Bottom | Dist | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.554 |  |  | 52.554 | 50 |  |  | 0 | DatumRL+50m |
|  | 1.783 |  |  | 50.771 |  |  | 14.990 | A |
|  | 0.926 |  |  | 51.628 |  |  | 29.105 | B |
|  | 1.963 |  |  | 50.591 |  |  | 48.90 | C |
| 1.305 |  | 3.587 | 50.272 | 48.967 |  |  | 63.540 | D/change point one |
|  | 1.432 |  |  | 48.840 |  |  | 87.665 | E |
| 3.250 |  | 0.573 | 52.949 | 49.699 |  |  | 102.050 | F/change point two |

End of Booking Sheet