RIICPL401E

Supervise the installation of underground services using open excavation

ASSESSMENT SUMMARY

The assessment activities for RIICPL401E Supervise the installation of underground services using open excavation cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Luke Wallace |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for RIICPL401E | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Theory Exam | Practical Assessment |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |

RIICPL401E

Supervise the installation of underground services using open excavation

THEORY EXAM

| Participant name | Luke wallace |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within two (2) hours, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

You have been assigned the task of supervising open excavation for installing services. Complete the table below describing documents you need to review to confirm the work activity and compliance.

| Answer | Type of document | Documents required |  |
| --- | --- | --- | --- |
|  | Project documents | Project plans and drawings Scope of works Permits and service drawings |  |
|  | Guidance documents | Work method statements Risk assessments Site safety plans |  |
|  | Legislation | Work health and safety regulations Excavation code of practice Work method statement |  |

What type of information are you expecting to locate from the documents listed in Q1. ?

| Answer | Location of existing services and potential clashes Sequence of work and time frame Specific excavation areas and boundaries Required permits and approvals Identified hazards and associated risks Control measures in place Safe work procedures for excavations PPE and emergency procedures Legal duties and responsibilities Mandatory safety practices |  |
| --- | --- | --- |

What are three (3) considerations you make as part of risk assessment processes during the ‘walk-through’ of the work area?

| Answer | - Identify hazards in the area - Consider the ground conditions - Assess the access and egress in case of emergency |  |
| --- | --- | --- |

How would you divert runoff around stockpiles unavoidably located in drainage paths?

| Answer | Create drains or bunds and utilise environment controls to mitigate sediment and silt into the drainage paths |  |
| --- | --- | --- |

All workers must wear the PPE required by the site. Why is it particularly important for supervisors?

| Answer | It is important for supervisors to wear PPE to set a standard and ensure compliance among all workers. |  |
| --- | --- | --- |

Where does the use of PPE fit in the risk management process?

| Answer | It is considered the last line of control and should be combined with other control measures |  |
| --- | --- | --- |

What would you read in the job plan to confirm the plan meets task requirements?

| Answer | Review the scope of works, specific project requirements, resource and material allocation, safety requirements |  |
| --- | --- | --- |

Why is it important to understand the relationship of the job to other jobs within the project?

| Answer | This understanding helps with effective resource use, reduces disputes and interruption between various work teams, and guarantees task coordination and sequencing |  |
| --- | --- | --- |

How do you confirm that required resources are available to meet job requirements?

| Answer | Check plans, inventory, deliveries, equipment, and Labor availability to confirm resources are ready. Communicate with suppliers and the team. |  |
| --- | --- | --- |

Two (2) labourers assigned to your team have called in sick and are unable to attend site as allocated. How would you respond?

| Answer | I would assess the task schedule, prioritise urgent work, and reassign tasks to available workers. If needed, I would contact subcontractors or other teams for temporary support to maintain progress. |  |
| --- | --- | --- |

Once your team members have arrived on-site, how do you confirm that they understand their roles and all requirements?

| Answer | conduct a pre-start meeting, go over the job plan, outline safety requirements, explain tasks, and confirm understanding by asking questions or having them repeat key points. |  |
| --- | --- | --- |

What emergency response information should you be sure your team understands before undertaking work tasks?

| Answer | Make sure the team knows emergency contacts, evacuation routes, assembly points, first aid locations, fire safety and how to report incidents |  |
| --- | --- | --- |

How do you encourage workers to adhere to work zone traffic management controls such as exclusion zones and traffic routes?

| Answer | use clear signage, regular reminders, and supervision to ensure workers follow traffic management controls for safety. |  |
| --- | --- | --- |

How can you support sub-contractors to achieve their responsibilities?

| Answer | provide clear instructions, resources, and support to help subcontractors meet their responsibilities. |  |
| --- | --- | --- |

How do supervisors determine human resources and sub-contractors needed for the job plan?

| Answer | assess project scope and tasks to determine the required workforce and subcontractors for efficient completion. |  |
| --- | --- | --- |

What is the relationship between various areas of civil work and how are these relationships relevant when developing job plans and work schedules?

| Answer | Civil work areas depend on each other, so understanding these relationships helps with proper sequencing and avoids delays. |  |
| --- | --- | --- |

Elaborate on four (4) different types of estimates used when developing job plans.

| Answer | - Preliminary estimate – a rough cost for early budgeting. - Detailed estimate – an accurate cost based on full plans. - Unit rate estimate – cost per unit of work. - Quantity estimate – total materials, labor, and equipment needed. |  |
| --- | --- | --- |

Refer to the following image and respond to Q18. Q19. and Q20.

Depth 1.85m

Length 3.725m

Width 1.56m

What is the total volume of the shape?

| Answer | Total volume = 3.725m × 1.56m × 1.85m = 10.75m³ |  |
| --- | --- | --- |

Calculate the surface area of the shape.

| Answer | Surface area = 2 × (5.811 + 6.891 + 2.886) = 2 × 15.588 = 31.176m² |  |
| --- | --- | --- |

Calculate the percent grade of the dotted line.

| Answer | Percent grade = (1.85m ÷ 3.725m) × 100 = 0.496 × 100 = 49.6% |  |
| --- | --- | --- |

Refer to the following image and respond to Q21. and Q22.

Depth 1.58m

Length 3.545m

Width 1.34m

Calculate the volume of the shape.

| Answer | Total volume = 3.545m × 1.34m × 1.58m = 7.51m³ |  |
| --- | --- | --- |

Calculate the grade of the dotted line as a percentage.

| Answer | Percent grade = (1.58m ÷ 3.545m) × 100 = 0.445 × 100 = 44.5% |  |
| --- | --- | --- |

You need an excavator for 3 days and have been asked to make sure there is enough diesel to last the whole 3 days.

- The excavator uses 15 litres per hour and will be operating for approximately 6 hours per day.  The company buys its diesel at cost ($1.10 p/l) and it has a policy that requires all orders to include a contingency of 10%.

- How much diesel will you need to order?

| Answer | multiply 15 litres by 6 hours, then by 3 days, and add a 10% contingency: (15 × 6 × 3) × 1.1 = 297 litres. |  |
| --- | --- | --- |

Clearly explain the steps you undertake when setting out the site for a trench.

| Answer | Mark the trench location, measure and level the area, set out reference points, check alignment and depth, and verify with plans before excavation begins. |  |
| --- | --- | --- |

A hold point may be required following the set out. Why would this be necessary?

| Answer | A hold point ensures the trench layout is correct before excavation, verifying measurements, alignment. |  |
| --- | --- | --- |

Write a description for each of the following trench collapse prevention methods (1) shoring, (2) benching (3) battering.

| Answer | - Shoring: Using supports like timber or metal to prevent trench walls from collapsing. - Benching: Creating steps in the trench walls to reduce the risk of collapse. - Battering: Sloping the trench walls to a safe angle to prevent failure. |  |
| --- | --- | --- |

Why is it important that the slope ratio not exceed the prescribed maximum value?

| Answer | Exceeding the maximum slope ratio increases the risk of collapse, compromising safety and causing damage. |  |
| --- | --- | --- |

What does it mean, to monitor the excavation and installation process?

| Answer | Monitoring means ensuring excavation and installation follow plans, safety standards, and are done correctly. |  |
| --- | --- | --- |

What methods do you use to confirm the outcomes are being achieved according to the job requirements?

| Answer | Use inspections, check measurements, verify against plans, and monitor progress to ensure outcomes meet job requirements. |  |
| --- | --- | --- |

What is one (1) reason you may have to adjust the work practices and (2) what effect can this have on your work plan?

| Answer | - Unexpected weather conditions may require adjusting work practices. - This can delay progress, requiring adjustments to the work schedule and resource allocation. |  |
| --- | --- | --- |

How do you record the changes and adjustments made to work practices mentioned in Q30. ?

| Answer | Record changes in a log, noting reasons, impacts on the schedule, and revised procedures. |  |
| --- | --- | --- |

If you come across an unidentified existing service during excavation work, which methods or procedures would you use to identify the service?

| Answer | Use utility maps, call a service locator, conduct non-invasive scanning, and contact the relevant utility providers to identify the service. |  |
| --- | --- | --- |

What is the BYDA system and how would you go about getting the information sent out to you and by what means?

| Answer | The Before you dig system provides underground service info. Request it online or by phone, and they’ll send the details via email or post. |  |
| --- | --- | --- |

What are some characteristics, technical capabilities and limitations related to one (1) item of plant or equipment you should be monitoring during installation works?

| Answer | For an excavator, monitor lifting capacity, digging depth, fuel efficiency, and range. Limitations include ground conditions, weight capacity, and fuel usage, affecting performance. |  |
| --- | --- | --- |

How do you, as the supervisor, confirm plant, equipment and tools maintenance requirements are carried out and recorded according to job requirements and manufacturer specifications?

| Answer | check logs, schedule inspections, verify compliance with specs, and maintain detailed maintenance records. |  |
| --- | --- | --- |

RIICPL401E

Supervise the installation of underground services using open excavation

PRACTICAL ASSESSMENT

| Participant name | Luke wallace |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 (Occasion 1) | Select Date. | Workplace Simulation |  |  |  |
| Task 1 (Occasion 2) | Select Date. | Workplace Simulation |  |  |  |

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 1 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Provide detail |
| 1.1A. | Specify documents located, read and applied: |
| 1.1B. | Describe job to be undertaken: |
| 1.2J. | Specify what is set out for the tasks: |
| 1.3D. | Specify where measurements are recorded: |
| 1.3E. | Specify method of communication used with operators: |
| 1.3F. | Specify adjustments made: |
| 1.4A. | Specify reports submitted: |
| 1.4C. | Specify recommended changes: |

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 2 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Provide detail |
| 1.1A. | Specify documents located, read and applied: |
| 1.1B. | Describe job to be undertaken: |
| 1.2J. | Specify what is set out for the tasks: |
| 1.3D. | Specify where measurements are recorded: |
| 1.3E. | Specify method of communication used with operators: |
| 1.3F. | Specify adjustments made: |
| 1.4A. | Specify reports submitted: |
| 1.4C. | Specify recommended changes: |

RIICPL401E

Supervise the installation of underground services using open excavation

Practical Task 1 (2X)

Description of task: The participant must demonstrate the ability to supervise the installation of underground service using open excavation on two (2) occasions including: - plan and prepare for underground service installation - locate job plan and all associated documentation to confirm work requirements and compliance - determine all risk controls required to address identified risks and hazards, including wearing PPE - prepare team members to execute tasks - check and confirm resource availability including human resources, organising as required - prepare for and facilitate a pre-start meeting and walk the site with team to confirm roles and responsibilities, provide instruction and clarify questions - set out task from plans and specifications using control lines, cleared width, batters and off-sets as required - oversee the execution of tasks - monitoring, measuring, testing according to task requirements to confirm outcomes are achieved according to job requirements - perform calculations to confirm volumes, grades, percentages, area and resource consumption figures and supply rates - initiate adjustments to work practice and job plans as required to meet job requirements - confirm plant, equipment and tools maintenance requirements are carried out and recorded according to job requirements and manufacturer specifications - report on execution of tasks - complete and submit reports as required by organisational, authority and site requirements - make recommendations related to required changes to confirm task execution meets job requirements Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Work site, work team, job plan and all associated documents (e.g. SWMS, permit to excavate) all plant, tools and equipment required for the task.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |  |
| --- | --- | --- |
|  | 1st Occasion | 2nd Occasion |
| 1 Plan and prepare for underground service installation |  |  |
| Locate the job plan and all associated documentation to confirm work requirements and compliance. |  |  |
| Check contents of job plan against related documentation to confirm the installation of underground services using open excavation and any related pavement maintenance meets legislated and industry requirements. |  |  |
| Read and confirm contents of SWMS and permit (as required) and facilitate implementation of required controls. |  |  |
| Meet site requirements for personal protective equipment. |  |  |
| 2 Prepare team members to execute tasks |  |  |
| Prepare notes or presentation material for prestart meeting. |  |  |
| Check availability of plant, equipment and tools for the tasks against schedule and potential conflicting requirements. |  |  |
| Check human resources for availability of competent workers and confirm availability. |  |  |
| Estimate material requirements using specifications for work and confirm availability onsite when required. |  |  |
| Facilitate a pre-start meeting and walk the site with team to confirm roles and responsibilities and check for new or unexpected hazards. |  |  |
| Use appropriate questioning and answering techniques to confirm worker understanding of tasks, roles and responsibilities. |  |  |
| Clarify emergency response procedures including emergency rescue as required. |  |  |
| Discuss and confirm worker understanding of work zone traffic management and safety |  |  |
| Have workers read and sign understanding of SWMS related to tasks as applicable. |  |  |
| Set out tasks from plans and specifications. |  |  |
| 3 Oversee execution of tasks |  |  |
| Confirm workers understand how to respond and refer client or public query. |  |  |
| Watch work in progress, measuring consistently to ensure outcomes are achieved. |  |  |
| Ensure trench collapse prevention methods are applied as required. |  |  |
| Record measurements as taken. |  |  |
| Assess operational techniques and apply approved method of communication to operators to ensure outcomes are achieved according to job requirements. |  |  |
| Initiate adjustments to work practice and job plans as required to meet job requirements. |  |  |
| Adhere to measuring and testing requirements, hold and witness points, recording and reporting as applicable in specifications. |  |  |
| Check that plant, equipment and tools maintenance requirements are carried out and recorded as required by site and manufacturer requirements. |  |  |
| 4 Report on execution of tasks |  |  |
| Complete and submit reports according to site requirements. |  |  |
| Use sector-specific vocabulary, grammatical structures and conventions when writing reports. |  |  |
| Recommend changes as required to confirm task execution meets job requirements. |  |  |