CPCCBC4029B

Apply construction information to the sales process

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4029B Apply construction information to the sales process cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Christine Muter |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4029B

Apply construction information to the sales process

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Christine Muter |
| --- | --- |
| Participant signature |  |
| Date | 12/05/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 18 |  | Satisfactory | Not Yet Satisfactory |

ASSESSOR Comments

Outline the circumstances in which a building licence must be obtained before carrying out building work.

| Answer | In Queensland any Individual or company is required to hold a QBCC licence if they are carrying out any building work valued over $3300. A QBBC licence is also required to carry out any hydraulic work over $1100 for example; sanitary drainage, soil waste, venting trade waste, plumbing and venting. Building works such as Gas Fitting, plumbing and drainage, Termite barriers involving chemicals, fire protection installations as well as building inspectors, Building designers and those carrying out site classifications also require a QBBC license. Any sub trade contractor that is carrying out any works for a licensed trade contractor employed to carry out any works such as: plumbing drainage, gas fitting, chemical termite treatment or fire protection work needs to hold an occupational licence. |  |
| --- | --- | --- |

Describe the steps involved in organising electricity supply to site. Provide approximate timeframes for this process.

| Answer | 1.The client should discuss directly with the electrical contractor to see if supply is available for the site. If not the electrical contractor can enquire on behalf of the client regarding the supply costs and timeline. From there the Distribution supplier (eg. Energex in Qld) will commence investigations and present the potential cost and timeframe to have supply for the client available. This timeframe is approx 20 days. If the is electricity supply is available then the electrician/contractor will need to enter into a Network connection agreement with the Distribution supplier, if the contractor agrees to the Terms and conditions set out in the connection contract the distributor will advise of the connection offer within 10 business days. The Contractor can then prepare the property for connection. Once the required works are completed the electrical contractor will submit an Electrical Work Request (EWR) to the Distribution Service provider with a copy this is then sent on to the clients preferred electricity retailer. Once all the forms are received confirming works are complete the Distribution supplier (ie Energex) will attend and inspect the property within 5 business days. All times frames are only estimates and can be dependent on other factors such as access requirements, network availability or other impacts that may fall outside the control of the electricity supplier. |  |
| --- | --- | --- |

True or False (Tick your response): Staged approval is one type of approval that may be sought for the building approval stage.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): Building provision approvals are issued by the local government (e.g. Brisbane City Council).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Describe the steps involved in organising water supply to site. Provide approximate timeframes for this process.

| Answer | As the approval for water connections are different dependant on the scale/ size of the development, time frames can differ from a standard new home build to a multi stage residential subdivision. Standard connections are usually completed in 5 days but larger scale developments will need additional information submitted to councils, they will determine if a connection is available. For larger developments you can refer to the Qld Urban Utilities website under Non-standard connections and review their PDF link in their “Our Service Commitments” (under their assessments section). |  |
| --- | --- | --- |

Identify the Volume and Parts of the NCC which outline energy efficiency requirements for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume 1 Section J- B1.4 Volume 2 Parts 3.12 | Volume 1 Section J - JP1 Energy use Volume 2 Parts 2.6 |  |

Describe the long-term benefits of sustainable building solutions. List one (1) environmental benefit and one (1) benefit for the client.

| Answer | Topics | Definition |  |
| --- | --- | --- | --- |
|  | Environmental benefit | Using recyclable materials such as glass, reconstituted wood materials and engineered materials helps to reduce our environmental footprint |  |
|  | Client benefit | Long term saving options – ie. Installation of Solar power, glazing of windows, artificial lighting etc, all saves costs on electricity consumption |  |

Match the insurance types and definitions by writing the appropriate letter next to each number in the centre column.

| Answer | Insurances |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Builder’s All Risk | 1 -B | A | Covers personal injury or property damage. |  |
|  | 2 | Indemnity | 2 -C | B | Covers accidental risks of physical loss or damage during construction, third party liabilities and advance loss of profits. |  |
|  | 3 | Public Liability | 3 -A | C | Covers costs where a contractor has caused building errors or omissions. |  |
|  | 4 | Workers’ Compensation | 4 -D | D | Covers death, injury or illness of employees. |  |

True or False (Tick your response): Insurance should be taken out by the contractor prior to starting construction.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Select and describe five (5) trades involved in the construction process.

| Answer | Trade | Description |  |
| --- | --- | --- | --- |
|  | Plasterer | Installs and prepares internal wall and ceiling linings prior to painting |  |
|  | Concreter | Installs formwork and boxing to facilitate the pouring of the concrete slab in accordance with the design plans |  |
|  | Painter | Paints internal ( and external where required) walls, ceilings doors and trims |  |
|  | Plumber | Installs and connects pipes and drains, internal fixings such as taps and toilets, ensures the downpipes are fitted correctly and that the sewer is connected to either the mains or waste water treatment such as a bio-cycle |  |
|  | Carpenter | Installs wall and door frames, roof trusses, doors, wall trims such as skirting boards, cabinetry, decks, steps etc |  |

Order the following steps from 1 to 10 to identify an appropriate construction sequence.

| Answer | Step Number | Steps |  |
| --- | --- | --- | --- |
|  | 3 | Floor frame |  |
|  | 5 | Windows and external door frames |  |
|  | 7 | Wall insulation |  |
|  | 1 | Set out |  |
|  | 9 | Painting |  |
|  | 6 | Electrical wiring |  |
|  | 2 | Plumbing of waste pipes |  |
|  | 8 | Internal linings |  |
|  | 10 | Landscaping |  |
|  | 4 | Wall frames |  |

For each of the main stages of construction listed below, identify the main trades involved and an approximate duration of each stage for a standard.

| Answer | Stage | Description |  |  |
| --- | --- | --- | --- | --- |
|  | 1: Clearing of the site | Trades | Soil testing may need to be carried out by a licensed company to determine the type of soil on site. Operators of heavy earth moving machinery such as excavators, Arborist for tree removal above a certain height for land clearing and grubbing. Surveyor |  |
|  |  | Duration | The duration on this will vary dependant on weather and the time lag on soil testing and their results |  |
|  | 2: Slab or base stage | Trades | Builder – measure out the design on the cleared site. Plumber - to arrange the under slab drainage Steel fixing- to reinforce the concrete Concreter – to pour the slab. Termite contractor – to install the termite barrier. |  |
|  |  | Duration | Approx 1 -2 weeks weather dependant |  |
|  | 3: Frame stage | Trades | Bricklayer – to lay/install the brick walls. Carpenter - to install the building framework and roof trusses, door and window frames. Electrician – install the wiring for electrical requirements as per the design plan. Plumber - - to install the roof sheets, gutters, downpipes and to connect the water supply to the taps, toilets, hot water system etc. Plasterer - to install the interior wall linings. |  |
|  |  | Duration | Approx 3-4 weeks weather dependant |  |
|  | 4: Lockup stage | Trades | Cabinet maker, Glazier, Plasterer, locksmith to install locks on doors and windows, Garage door installer. |  |
|  |  | Duration | Approx 4 weeks to complete walls, install doors and windows |  |
|  | 5: Fit-out or fixing stage | Trades | Plasterer for cornices, cabinet maker for cabinets and architraves, electrician for lights and power points etc, plumbing installations, tiler for all the wet areas , backsplashes and other areas not being carpeted |  |
|  |  | Duration | Approx 5-6 weeks |  |
|  | 6: Practical completion stage | Trades | Painter, flooring installation (ie carpet) final fit outs, painting touch ups, gardener/ landscaper to complete the gardens/ turf etc, |  |
|  |  | Duration | Approx 7-8 weeks |  |

Identify five (5) factors that can affect the duration of each construction stage.

| Answer | - Progress payments not being made by the client as per the contract - Material availability - Labour/ trade availability - Inclement weather conditions - If the requirements of the job changes – ie if the clients requests a change and additional labour or materials are required |  |
| --- | --- | --- |

Describe how each of the following factors can affect construction times.

| Answer | Factor | Effect |  |
| --- | --- | --- | --- |
|  | Changed conditions | Unknown field conditions , design changes or owner requests that have to be reviewed by an architect can affect the contract and costings when it comes to additional labour and materials (variations) |  |
|  | Labour availability | If the correct trade sequence isn’t arranged or available in the correct order, this will delay the completion of the project. Each trade is required to perform certain tasks and these can be dependant on the success of the trades that are to carry out work prior to them arriving to site – eg the painter wouldn’t be scheduled to arrive to paint the internal walls and ceiling before the plasterer puts the wall sheets up |  |
|  | Material availability | If the materials are not ordered in advance and booked to arrive when the trades are due to commence work , this will delay the completion time for the project especially if the client has requested specific items such as floor tiles for the tiler or a specific vanity for the plumber |  |
|  | Weather | Inclement weather / poor weather conditions can affect any and all trades in the initial construction phase – even landscapers towards the end of a project. Roofers cant install roof tiles or metal roofing materials in wet weather or in high winds for safety reasons, Concreters cant lay the concrete in pouring rain as it may affect the foundations |  |

Describe the site establishment requirements for each of the points listed.

| Answer | Requirement | Description |  |
| --- | --- | --- | --- |
|  | Earthworks | Contact Dial Before You Dig DBYD ¦ Dial Before You Dig (1100.com.au) OR 1300 340 017 Before undertaking any excavation work to ensure that any hidden services (electricity etc) will not be damaged during construction |  |
|  | Electricity | Electricity supply may need o be disconnected for certain work zones prior to construction and temporary utilities (such as generators) installed for the duration of the construction process |  |
|  | Neighbours | Provide neighbours / adjoining land owners with advance notice of intended start dates for construction and to provide them with contact details. |  |
|  | Waste disposal | Set up and maintain designated areas for rubbish removal and to ensure relevant permits are obtained if rubbish bins are required to be placed on the street |  |
|  | Water connection | A plumber to be appointed to perform any required disconnection of existing services before demolition takes place |  |

Identify the section of the Work Health and Safety Regulations that outlines the site establishment requirements listed.

| Answer | Regulation section | Requirement |  |
| --- | --- | --- | --- |
|  | Division 2 - s40 (a) through to (g) inclusive | The layout of the work area must allow for persons to enter, exit and move about without risk to health and safety |  |
|  | Division 2 – S 41 (1) through to (3) inclusive | Adequate facilities must be provided for workers, including toilets, drinking water, washing facilities and eating facilities. |  |
|  | Division 2– S 308 (a) through to (c) inclusive | A principal contractor for a construction project must install signs outside the work area that shows their name and telephone contact numbers and site office address. |  |

True or False (Tick your response): The owner is able to access the site at any time.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following parties should the client contact to access the site? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contractor |  |
|  |  | Local Government |  |
|  |  | Industry Regulator (e.g. Queensland Building and Construction Commission) |  |
|  |  | Subcontractors |  |

CPCCBC4029B

Apply construction information to the sales process

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Christine Muter |
| --- | --- |
| Participant signature |  |
| Date | 12/05/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4029B

Apply construction information to the sales process

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to apply construction information during the sales process in order to develop the customer’s knowledge of construction processes and the effects on the contract and building timelines. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - RFI and relevant correspondence with client - A variation, price and acceptance by the client The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Two (2) sales contracts. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Christine Muter |  |  |
| Name of observer: | John Gammon |  |  |
| Role of observer (Confirm suitability to sign) | Construction Manager – QLD |  |  |
| Email address of observer | john.gammon@tbs.com.au |  |  |
| Signature of observer: |  |  |  |
| Date: | 12/05/2021 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Identify and explain to the customer the approvals required before starting construction, including: - Statutory approvals - Sustainability requirements - Insurance covers |  |  |  |
| - Identify and explain to the customer the stages and timeframes of construction, including: - Main trade components - Construction process sequence (including approximate duration) |  |  |  |
| - Identify and explain to the customer potential industry conditions that can affect construction times. |  |  |  |
| - Identify and explain to the customer site establishment requirements before starting construction. |  |  |  |
| - Establish site access arrangements with the client. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Active listening and responding to client questions |  |  |  |
| The participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments

I have observed Christine’s approach in handling all relevant correspondence pertaining to Our Companies’ contracts and the process in arranging trades to attend site and complete the agreed works. Christine has also been involved in presenting variations and handling the paperwork through to the variation approvals within our Business Model. Christine’s approach is professional when dealing not only with the requirements under our Building contracts but also when dealing with our clients and their needs. Christine displays an understanding of the steps necessary and requirements regarding legislation in order to obtain Building approvals and the Building process.