CPCCBC4007A

Plan building or construction work

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Emma Carew |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 23/05/2018 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 29 |  | Yes | No |

Assessor Comments

Who is responsible for supervision on a building construction site?

| Answer | The Builder is responsible for supervision. Under the QBCC Act 1991, a licensed contractor must ensure that building work done under their licence is adequately supervised. If this obligation is not met, the licensed contractor will be charged with an offence. |  |
| --- | --- | --- |

Who is responsible for supervision when a Builder has a Construction Manager on site?

| Answer | The Construction Manager has the same obligations as a licenced contractor. They must ensure that the work done by licenced contractors for a principal is supervised by an appropriately licensed person |  |
| --- | --- | --- |

Who is responsible to supervise and coordinate tradespeople working on site?

| Answer | The Site Manager is responsible to supervise and coordinate trades on site |  |
| --- | --- | --- |

Is it likely that particular requirements for quality and finish would be included in the contract (e.g. the type, style and number of coats of paint to be applied or the quality of bricklaying and finish to be achieved or the style of cladding or lining to be applied etc.)? 

How should the builder encourage the client with checking the implementation of such details?

| Answer | The Builder needs to advise the client to refer to the Building Specification which describes the materials used and quality, characteristics and performance of the work. |  |
| --- | --- | --- |

What might the client do if the client is unsure about conducting on-site inspections of work in progress or completed?

| Answer | The client should refer to the construction program and then liaise with the builder to organise suitable times for any on-site inspections. The construction program will show the sequence of events and activities for the build and monitor the overall progress so the client will know which stage the project is up at all times. |  |
| --- | --- | --- |

Explain the role of the Building Certifier.

| Answer | Building Certifiers are responsible for ensuring specific compliance is achieved and they do this by conducting visual inspections to confirm that the details of the documentation are reflected in the actual on-site construction. Building Certifier roles: - Check that the builder has complied with the approved building plans and appropriate building standards. - Work with local government, or in a private practice. - Assess & approve plans relating to new or altered buildings - Inspect construction work at mandatory stages - Provide certificates of inspection to the homeowner, builder and local authority - Issue enforcement notices where required - Giving final approval for a building to be occupied |  |
| --- | --- | --- |

Explain what a Building Certifier is not responsible for.

| Answer | Building Certifiers are not responsible for the following: - The quality of workmanship within the construction process - They do not supervise a build or intervene as this is the responsibility of the builder and client - Ensuring the builder is complying with the contract |  |
| --- | --- | --- |

Name at least eight (8) things you can do to help avoid problems escalating into dispute.

| Answer | - Keep the client and subcontractors well informed, notifying them when problems, delays and variations arise. - Carefully read and understand the contract - Weigh up the cost of the dispute escalating as legal costs may be more than the outcome is worth - Check any associated plans and specifications before signing - Discuss any questions with your contractor - Seek legal or expert advice if you have any concerns - Keep accurate records - Work with suppliers and subcontractors fairly and resolve claims when they arise - Administer the construction contract in a spirit of honesty and cooperation by all parties |  |
| --- | --- | --- |

What is the Building Code of Australia (BCA)?

| Answer | The Building Code of Australia (BCA) is a uniform set of technical provisions for the design and construction of buildings and other structures throughout Australia. It is a performance-based document that is referenced so that your building system or materials can demonstrate that it meets certain performance requirements in order to meet legislative requirements. |  |
| --- | --- | --- |

List the number and name and descriptions of the Building Code of Australia (BCA).

| Answer | There are two BCA Volumes: Volume One - Class 2-9 buildings - Access requirements for people with disability in Class 1b and 10a buildings - Certain Class 10b structures including access requirements for people with a disability in class 10b swimming pools. Volume Two - Class 1 and 10a buildings - Certain Class 10b structures (other than access requirements for people with a disability in Class 10b - Class 10c |  |
| --- | --- | --- |

What are the key sections of the structure of the Building Code of Australia (BCA)?

| Answer | - Building Structure - Fire Resistance - Access and Egress - Services and Equipment - Health and Amenity |  |
| --- | --- | --- |

Explain the term ‘Deemed to Satisfy’ with respect to the provisions in the BCA.

| Answer | Deemed-to-satisfy provisions – are prescriptive and include materials, components, design factors and construction methods that, if used, will result in compliance with the performance requirements of the BCA |  |
| --- | --- | --- |

What is a Standard?

| Answer | A Standard is a published document which sets out specifications and procedures designed to ensure products, services and systems are safe, reliable and consistently perform the way they were intended to. A Standards establishes a common language which defines quality and safety criteria |  |
| --- | --- | --- |

Describe the purpose of the following Australian Standards.

| Answer | Standard | Purpose |  |
| --- | --- | --- | --- |
|  | AS1684 | Residential Timber-Framed Construction This Standard is a four-part Australian Standard covering design criteria, building practices, tie-downs, bracing and span tables for timber framing members. It is also referred to as the Timber Framing Code. By complying with this Standard, users are Deemed to Satisfy the requirements of the Building Code of Australia |  |
|  | AS2870 | Residential Slabs and Footings The purpose of this Standard is to establish performance requirements and specific designs for footing systems for foundation conditions commonly found in Australia and to provide guidance on the design of footing systems by engineering principles. This Standard places particular emphasis on the design for reactive clay sites susceptible to significant ground movement due to moisture changes. The Standard takes account of the following: (a) Swelling and shrinkage movements of reactive clay soils due to moisture changes. (b) Settlement of compressible soils or fill. (c) Distribution to the foundation of the applied loads. (d) Tolerance of the superstructure to movement |  |
|  | AS3000 | Electrical Installations This Standard sets out requirements for the design, construction and verification of electrical installations, including the selection and installation of electrical equipment forming part of such electrical installations. These requirements are intended to protect persons, livestock, and property from electric shock, fire and physical injury hazards that may arise from an electrical installation that is used with reasonable care and with due regard to the intended purpose of the electrical installation. The Standard comprises two parts: Part 1 provides uniform essential elements that constitute the minimum regulatory requirements for a safe electrical installation. It also provides an alternative regulatory vehicle for Australian and New Zealand regulators seeking to move from the present prescription of AS/NZS 3000 in electrical installation safety and licensing legislation. Part 2 provides installation practices that achieve certainty of compliance with the essential safety requirements of Part 1. It is primarily a revision of the 2000 edition to provide work methods and installation practices that are 'deemed to comply' with the associated performance outcomes/safety conditions. |  |
|  | AS3500 | Plumbing and Drainage AS 3500.1 – Plumbing and Drainage – Water Services This Standard sets out requirements for the installation of water services from the point of connection to the network utility’s water supply or from an alternative drinking water supply to the points of discharge. This Standard applies to new installations as well as alterations, additions and repairs to existing installations. AS 3500.2 – Plumbing and Drainage – Sanitary Plumbing and Drainage This Standard specifies requirements for the design and installation of sanitary plumbing and drainage from fixtures to a sewer, common effluent system or an on-site wastewater management system. Applies to new installations, additions or repairs to existing installations. AS 3500.3 – Plumbing and Drainage – Stormwater Drainage This Standard sets out materials and products, design and installation requirements for stormwater plumbing and stormwater drainage up to the Regulatory Authority's point(s) of connection. The Standard applies to new installations as well as alternations, additions, and repairs to existing installations. AS 3500.4 – Plumbing and Drainage – Heated Water Services This Standard sets out requirements for the design, installation and commissioning of heated water services using drinking water, rainwater or a combination thereof. It applies to new installations as well as alterations, additions and repairs to existing installations. |  |
|  | AS3600 | Concrete Structures This Standard sets out minimum requirements for the design and construction of concrete structures and members which contain reinforcing steel, or tendons, or both. It also sets out minimum requirements for plain concrete members. |  |
|  | AS3660 | Termite Management AS3660.1 - Termite Management – New Building Work Sets out requirements for the design and construction of subterranean termite management systems for new buildings and new building work. AS3660.2 – Termite Management - In and around existing buildings and structures Provides a guide for building owners and others involved in the management of subterranean termites within and around existing buildings and structures. AS3660.3 – Termite Management - Assessment criteria for termite management systems Specifies the criteria for assessing the effectiveness of termite management systems intended for use in buildings and structures as required by AS 3660.1 or AS 3660.2. It also outlines procedures to assess the ability of a system and its components to manage termite activity in and around buildings and structures |  |

What is the difference between the Prime Cost Item Schedule and the Provisional Sum Schedule in a contract? Give one example to help explain your answer.

| Answer | Prime cost items include tiles, taps, doors and bathroom fittings. An amount for these items is included in the contract sum. This is an agreed estimated amount as the specific products are not selected until a later stage. The estimate for the PC items must be above or at what they could reasonably cost A provisional sum is an amount included in the contract to cover work and/or materials note detailed when entering the contact. For example Site works, removal of trees. The builder legally needs to take steps to ensure an accurate cost estimate but if unforeseen event occur and the provisional sum is exceeded, the additional costs can be added to the contract sum. |  |
| --- | --- | --- |

What does Section 109 of the Queensland Building and Construction Commission Act 1991 provide to the consumer/client?

| Answer | Section 109 - Access to building sites A building contractor must, at the request of a consumer allow the consumer reasonable access to a building site at which building work is being carried out for the consumer. |  |
| --- | --- | --- |

What are some of the preliminary contracts that a builder could ask a client to sign prior to signing the main contract?

| Answer | - Pre-construction contract - Preparatory work agreement - Quote or estimate - Order - Preliminary contract - Provisional quote - Authorised tender acceptance - Contract request |  |
| --- | --- | --- |

What is the ‘cooling-off’ period once a contract is signed?

| Answer | When referencing the QBCC New Home Construction Contract – You may withdraw from the contract in writing within 5 business days of receiving copies of the signed contract and the Consumer Building Guide. Costs for home owners to withdraw are generally $100 plus any out of pocket expenses reasonably incurred by the contractor. |  |
| --- | --- | --- |

If a client withdraws during the ‘cooling-off’ period how must they present that withdrawal to the builder?

| Answer | You must advise the Builder in writing if you withdraw from the contract during the ‘cooling off’ period. |  |
| --- | --- | --- |

What are liquidated damages in terms of a signed building contract?

| Answer | When the contractor is unable to finish the works agreed to in the contract, the owner is entitled to claim for liquidated damages – which is a pre-arranged amount agreed to at the start of the contract. |  |
| --- | --- | --- |

What is the maximum deposit allowed for the total contract price under $20,000? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | 5% |  |
|  |  | 15% |  |
|  |  | 12% |  |
|  |  | 10% |  |

What is the maximum allowable deposit when the contract price is $20,000 or more? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | 15% |  |
|  |  | 5% |  |
|  |  | 8% |  |
|  |  | 10% |  |

Name the five (5) stages to build a house.

| Answer | - Base Stage - Frame Stage - Enclosed Stage - Fixing Stage - Practical Completion |  |
| --- | --- | --- |

What contract documentation would you review to identify any unusual aspects for a low rise construction projects related to materials and installations?

| Answer | Project Specifications contain all the written information that accompanies the plans. |  |
| --- | --- | --- |

If you were looking to identify any possible penalties associated with a build where would you find it?

| Answer | Liquidated damages – Condition 24 Item 10 – QBCC New Home Construction Contract |  |
| --- | --- | --- |

What operational requirements need to be established when planning building/construction work? Consider the supply of labour, materials and relevant documentation for your residential build.

| Answer | Operational requirements that need to be established: - Identify any unusual aspects of construction, materials or penalties within the contract documentation - Determine the availability of selected subcontractors to suit the job requirements - Assess and confirm the availability of materials with suppliers - Identify site access requirements, limitations and actions taken to facilitate entry - Prepare documentation for authorities controlling construction work and determine project commencement date - Implement the procedures for controlling and recording site deliveries |  |
| --- | --- | --- |

In what document/s would you find organisational strategies and procedures for applying work health and safety?

| Answer | Workplace Health and Safety Management System The WHSMS should include procedures for: - Recoding the hire of plant and equipment - Other purchasing and receiving policies and procedures - Implementing hazard and risk management procedures in accordance with local and state government and environmental protection agency guidelines - Procedures for the removal of existing services and hazardous materials in accordance with environmental protection agency and requirements - Procedures for the control of multiple projects (i.e. subcontractors) to be followed |  |
| --- | --- | --- |

How would you prepare your on-site personnel receiving plant, equipment and materials to site for the build? What documentation can be used to identify defective plant, equipment and materials and ensure only non-defective materials are used for all installations of the build?

| Answer | To improve the reliability of documentation on site and in the office you should: - Keep an accurate up to date inventory of material that is delivered and stored on site. - Ensure that employees and subcontractors are trained in materials handling and storage practices - Cross check purchase orders against what is actually received on site - Do not accept / sign for goods that are defective |  |
| --- | --- | --- |

You are to sequence the construction operations for the residential construction project shown below and outlined overleaf. Operational details are to be entered into a manually prepared project schedule or computer-based software program and the critical path of the project is to be defined and revised as required. 

Your project timeframes are to allow for adjustments to account for time of year, anticipated delays, inclement weather and the possibility of variations called for by the client. 

In further preparation you are to also determine and document the following:

- Installation of temporary services and any site accommodation requirements

- Any plant and other heavy equipment requirements

- The availability of subcontractors and materials, scheduling the on-site labour requirements with reference to the contract documentation

- A site condition report to notify neighbours, authorities and others

- A site delivery checklist

- A facsimile order get a site survey done and to fine accurate heights of house pad above check level

- A communication (e.g. email) with the relevant authorities for serves (i.e. electrical, drainage plumbing, sewerage/septic, gas and telecommunications).

You have been approached as a local builder and have since met on site with Errol and Elizabeth McKendry on their 6 hectares of accessible land at 1416 Chambers Flat Road, Logan Village, Queensland.

They have provided you a sketch floor plan which is for a slab on ground home with pitched roof of three bedrooms and study (see previous page). The home has “Scyon Linea Weatherboard” (James Hardie Product) external cladding with plasterboard internal linings to walls and ceilings with a feature rail at 1100mm on all internal vertical walls and partitions. There is a coloured concrete tile roof over sealed 600mm eaves and a two car garage and attached roofed alfresco (loggia) off internal living areas at the rear. The owners have not chosen any colour schemes nor any electrical/gas appliances or wet area installations at this time. They have requested builder supplied at this stage.

You are provided a geotechnical report and some photos of the property. Nothing untoward is mentioned in the geotechnical report. The block is heavily timbered at the rear and backs on to the Logan River at its rearmost boundary. The owners advise you the location of the house pad is 11 metres above all known flood levels of the past 50 years for the area. There are no trees which are likely to affect the slab on ground location or any work or workers on or around the immediate building to be constructed.

There is a property access gate and approximately 310 metres of 5 metre wide ant bed road into the property with the block partially fenced with slab posts and four strand barbed wire. The entry gates are solar powered with reinforced concrete block curved feature walls 2.5 metres high and each 5 metres long on either side of the gate with the property title “Little Tuscany” emblazoned on the right hand side concrete block wall. The gate when fully open provides 8 metres wide access with bitumen road from all access routes to the property and up to the boundary at the entry gate. The nearest major community hubs to the location are Logan Village, Loganlea, Loganholme, Beenleigh and Yatala.

Following a site survey you have identified that minimal pad levelling is required other than a general scrape of the area to clean it of grass and other plant matter. Power from the mains is already established on site and there is a property pole established approximately 30 metres from the house slab location. You note the mains power is underground and will need to liaise with the power authority and Council for other possible underground services that may traverse the property. You should also satisfy yourself that the landed property is above the flood plain by gaining access to recent flood reports for the area from the Local Authority.

The nearest neighbours and any other buildings that could be impacted are 800 and 900 metres away respectively to either side boundaries.

| Answer |  |  |
| --- | --- | --- |