RIIMPO402D

Apply the principles of earthworks construction

ASSESSMENT SUMMARY

The assessment activities for RIIMPO402D Apply the principles of earthworks construction cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Brent Johnson |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for RIIMPO402D | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Theory Exam | Practical Assessment Copy of Job Plan completed by participant |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |

RIIMPO402D

Apply the principles of earthworks construction

THEORY EXAM

| Participant name | Brent Johnson |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within a two (2) hour period, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

What information can you obtain by reading the project plans, surveys, drawings and specifications during planning for earthworks?

| Answer | By reviewing project plans, surveys, drawings, and specifications during earthworks planning, you can determine site layout and boundaries, existing conditions such as topography and soil data, design requirements for grading and drainage, material and compaction standards, locations of underground utilities, environmental and safety constraints, and construction details like sequencing and equipment needs. This information ensures accurate excavation, compliance with regulations, and effective risk management. |  |
| --- | --- | --- |

Why is project site hydrological data important to earthworks planning and where this can be accessed?

| Answer | Project site hydrological data is crucial for earthworks planning because it provides information on water movement, drainage patterns, groundwater levels, and flood risks. This data helps determine how water will interact with the site during and after construction, influencing decisions on cut and fill, slope stability, erosion control, and drainage design. Without accurate hydrological data, earthworks can lead to waterlogging, structural failures, or environmental damage. It can be accessed in a few places Survey reports and geotechnical investigations Project-specific hydrological reports |  |
| --- | --- | --- |

What is the purpose and procedure for conducting a proof roll test to the subgrade?

| Answer | The purpose of a proof roll is to make sure there are no soft spots in the subgrade by slowly driving backwards and forward with a ¾ fill water cart watching the wheels for any deflection or heaving in the subgrade |  |
| --- | --- | --- |

What is the purpose of geotechnical investigation reports and how they inform earthworks planning?

| Answer | Geotechnical investigation reports are important because they provide detailed information about the physical properties of the soil and rock at the project site. Their purpose is to assess ground conditions to ensure the design and construction of earthworks are safe, stable, and cost-effective. |  |
| --- | --- | --- |

What information would you expect to see in a job plan?

| Answer | A Job Plan should include - Labour, Plant and Machinery - Construction Materials - Sub-Contractors - Waste Disposal - Material scheduling - Risk assessments - Test Requirements - Environmental Requirements - Health and Safety Requirements - Traffic control requirements - Quality management - Communication - Performance reporting - The scopes of work |  |
| --- | --- | --- |

How would you identify if any drainage or dewatering issues need to be addressed at the site and how you would promptly and effectively manage these once identified?

| Answer | To identify any drainage issues regular inspections of the site especially after a rain event for pooling water. Checking any open excavations, saturated ground or scouring of batters If issues are found install a sump in the lowest point of the excavation to be able to pump the water out after proper water test and de-watering permits have been organised as per our ESCP Create diversion channel to protect the jobsite, set up sediment controls around stockpiled materials. |  |
| --- | --- | --- |

What is the importance of understanding the relationship between the different areas of civil work when planning and sequencing work tasks, providing examples?

| Answer | Understanding the relationship between different areas of civil work is essential for effective planning and sequencing, as tasks often rely on the completion of others. Coordinating these activities prevent delays, reworks, and safety hazards. Drainage must be installed after excavation but before road construction to avoid water damage, Utilities should be laid before paving to prevent cutting into finished surfaces. Proper sequencing ensures efficiency, cost control and compliance with project requirements |  |
| --- | --- | --- |

Before commencing a project, what would you estimate to effectively determine resource and sequencing requirements?

| Answer | Before starting a project, you would estimate Scope of work, Productivity Rates, Methodology, Constraints and Risks then develop a Work Breakdown Schedule. These combined allow you accurately determine resource needs and task sequencing |  |
| --- | --- | --- |

What is the definition of the following material properties and test results that are applicable to earthworks?

| Answer | Test | Definition |  |
| --- | --- | --- | --- |
|  | Compaction test | Measures the density and moisture in the material to make sure it has been compacted to the specified rate for stability and load bearing capacity |  |
|  | Soil density/ moisture | Soil Density = How solid and stabile the ground is after compaction Soil Moisture = How much water is present in the soil |  |
|  | Plasticity | Plasticity is how easily fine grained soil can be moulded / compacted when water is added |  |
|  | Particle size distribution | PSD is the proportion on different sized soil particles like gravels, sand, silt or clay within a sample |  |

What resources must you confirm are available before the job plan can be implemented?

| Answer | Labour, Plant and Equipment, Tools, Trucks Construction Materials Sub Contractors needed for the job |  |
| --- | --- | --- |

Outline the information you will be discussing with the work team during the pre-start meeting.

| Answer | Any issues raised from the previous shift Any safety or environmental updates The works for the day split up in our different areas |  |
| --- | --- | --- |

What methods can you apply to confirm instructions provided have been understood?

| Answer | Face to face Ask if anyone doesn’t understand what their doing to come and see me after pre-start and I will Explain more clearly Over the 2 Way Radio explaining the work instruction while on out onsite |  |
| --- | --- | --- |

What would you do if you come across conflict between sub-contractors?

| Answer | I would step in calmly and address the issue immediately to prevent it affecting safety, productivity and project timelines. I would speck to each party to understand the cause of the issue. I would then bring both parties together to see if we can rectify the issue. If I can not resolve the issue myself then I would raise it with my supervisor or project manager |  |
| --- | --- | --- |

Discuss what you would do if you noticed a client walking onto the site with no PPE and they haven’t signed in.

| Answer | I would stop them immediately, ask if they know what PPE is required onsite and ask them why they are not wearing the correct PPE. Then escort back to the compound, Have them sign in correctly and put on the correct PPE |  |
| --- | --- | --- |

Discuss the importance of accurate setting out of offsets.

| Answer | It is important to accurately set out offsets as these define your boundary lines, the control line, levels of each section of the job as per the plans and specifications |  |
| --- | --- | --- |

You have noticed a worker who is assisting during setting out, doesn’t seem to know what they are doing. How will you respond to this situation?

| Answer | I would first stop the task straight away to prevent any errors, I would then speak to the worker and assess their level of understanding and clarify any instructions or drawings. If needed I would provide guidance or demonstrate the correct procedure, I would then pair them up with someone who has more experience to learn off |  |
| --- | --- | --- |

After speaking with the worker from the previous question, you establish that this worker has difficulty calculating percentage of slope, as well as other things. Provide a simple equation you can provide to assist the workers learning.

| Answer | Slope % = (Rise / Run) x 100 |  |
| --- | --- | --- |

What tool and equipment maintenance would direct the team about before setting out the site?

| Answer | Before setting out the site I would direct the team to inspect, maintain and calibrate all necessary tools and equipment to ensure accuracy, Safety and reliability. This would include all levelling and survey equipment as well like total stations, Laser levels, Dumpy levels, Staffs for calibration. Check all tape measures, string lines and lumpy hammers for any defects. Make sure all batteries are charged. |  |
| --- | --- | --- |

Describe how you ensure all workers understand the communication methods used on site and all appropriate equipment before beginning work tasks.

| Answer | Before starting work, I make sure everyone understands how communication will work on site by holding a pre-start or toolbox talk. I explain the main communication methods, such as verbal instructions, hand signals, radios, and signage, and who they should report to if there are issues. I also check that workers know how to use the required tools, equipment, and PPE. I encourage questions to confirm understanding so everyone can work safely and effectively. |  |
| --- | --- | --- |

Calculate the amount of road base required for an area of 200m in length, 12.4m in width and a depth of 250mm. Answer using cubic metres (m³). Allow an extra 20% for compaction.

| Answer | 200m x 12.4m x 0.25m = 620m3 620m3 x 1.2 = 744m3 including 20% for compaction |  |
| --- | --- | --- |

Calculate the following to determine the percentage of grade. A rise of 820 millimetres and a run of 60 metres (Round up to two (2) decimal points).

| Answer | 0.82 / 60 x 100 = 1.37% Grade |  |
| --- | --- | --- |

How do supervisors know that they are working according to client and organisational requirements when conducting excavation work?

| Answer | By regular meetings (Daily 1pm program meeting) with the project manager and other supervisors on the project. Following program Regular inspections of the work being produced (proof rolls, Compaction tests Following plans, drawings and specifications |  |
| --- | --- | --- |

Briefly describe two (2) situations that would require supervisors to make changes to the work practices or job plan.

| Answer 1. | Safety Risks If a new hazard is identified—such as unsafe ground conditions, plant issues, or weather changes—the supervisor must adjust the work method to maintain safety. |  |
| --- | --- | --- |
| 2. | Unexpected Site Conditions If unexpected site conditions arise, such as services not shown on drawings or material delays, the job plan may need to be revised to ensure the work is completed safely, efficiently, and in line with project requirements. |  |

What are two (2) types of reports an organisation could use to record how installation against required outcomes is progressing and identify opportunities for improvement in relation to safety, efficiency, and effectiveness.

| Answer 1. | A Progress Report A progress report tracks installation work against planned outcomes, timelines, and quality requirements, helping identify delays, inefficiencies, or areas needing improvement. |  |
| --- | --- | --- |
| 2. | A Safety Performance Report A safety performance report (such as incident, hazard, or inspection reports) records safety observations, near misses, and compliance issues, allowing the organisation to improve safety practices while also increasing overall efficiency and effectiveness on site. |  |

Why should written information be clearly expressed?

| Answer | So it can be easily understood by all readers and interpreted in the same way. Clear writing reduces confusion, mistakes, and misunderstandings, especially in work environments where safety, quality, and efficiency are critical. It ensures instructions, procedures, and requirements are followed correctly, supports effective communication, and helps maintain consistency and accountability |  |
| --- | --- | --- |

What is the purpose of levelling equipment and why is routine inspection, proper handling and regular calibration important?

| Answer | The purpose of levelling equipment is to accurately measure and establish heights, levels, and gradients so work is set out correctly and meets design requirements. Routine inspection, proper handling, and regular calibration are important to ensure the equipment remains accurate and reliable. |  |
| --- | --- | --- |

RIIMPO402D

Apply the principles of earthworks construction

PRACTICAL ASSESSMENT

| Participant name |  |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 (Occasion 1) | Select Date. | Workplace Simulation |  |  |  |
| Task 1 (Occasion 2) | Select Date. | Workplace Simulation |  |  |  |

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 1 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Provide detail |
| 1.1A. | Specify () documents read and confirmed: Plans/ Drawings/Maps Specifications TMP EMP Site Safety Plan SWMS Engineering survey data Site cultural and heritage information Sources of material Labour resources required, including sub-contractors Schedule Costing requirements Other (please specify): |
| 1.1B. | Specify () specifications clarified: Scheduling requirements Milestones Testing requirements Materials Site soil/rock and geological data Other (please specify): |
| 1.1D. | Specify () job plan inclusions: HR requirements Plant and machinery requirements Construction materials requirements Sub-contractor support requirements Waste disposal requirements Coordination requirements Activity scheduling Materials delivery scheduling Risk assessment and management requirements WHS requirements Quality management requirements, including testing scheduling requirements Traffic management requirements Environmental requirements Task monitoring requirements Task performance monitoring requirements Communication requirements Reporting requirements Other (please specify): |
| 1.2K. | Specify () test results read and correctly understood: Compaction test results Soil/ density/ moisture relationship Plasticity index Particle size distribution Other (please specify): |
| 1.3B. | Specify adjustments made: |
| 1.4B. | Specify recommendations made: |

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 2 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Details |
| 1.1A. | Specify () documents read and confirmed: Plans/ Drawings/Maps Specifications TMP EMP Site Safety Plan SWMS Engineering survey data Site cultural and heritage information Sources of material Labour resources required, including sub-contractors Schedule Costing requirements Other (please specify): |
| 1.1B. | Specify () specifications clarified: Scheduling requirements Milestones Testing requirements Materials Site soil/rock and geological data Other (please specify): |
| 1.1D. | Specify () job plan inclusions: HR requirements Plant and machinery requirements Construction materials requirements Sub-contractor support requirements Waste disposal requirements Coordination requirements Activity scheduling Materials delivery scheduling Risk assessment and management requirements WHS requirements Quality management requirements, including testing scheduling requirements Traffic management requirements Environmental requirements Task monitoring requirements Task performance monitoring requirements Communication requirements Reporting requirements Other (please specify): |
| 1.2K. | Specify () test results read and correctly understood: Compaction test results Soil/ density/ moisture relationship Plasticity index Particle size distribution Other (please specify): |
| 1.3B. | Specify adjustments made: |
| 1.4B. | Specify recommendations made: |

RIIMPO402D

Apply the principles of earthworks construction

Practical Task 1 (2X)

Description of task: The participant must demonstrate the ability to apply the principles of earthworks construction while undertaking the role of a civil construction supervisor on two (2) occasions including demonstration of: - During planning and preparation of work: - Reading and applying requirements set out in project plans and specifications - Complying with site safety plan and organisational policies and procedures when confirming resource availability - Complete a job plan showing best use of available resources to meet task requirements - Prepare to lead pre-start meeting - When implementing the job plan: - Confirm availability of resources at site - Lead daily pre-start meeting providing clear instructions to meet schedule and safety requirements - Set out the job and allocate tasks to others - While supervising the execution of tasks: - Monitor the installation, checking against performance requirements and identifying gaps - Initiate adjustments to process or work practice to ensure achievement of expected outcomes - Oversee plant, equipment and tool maintenance procedures and ensure written documentation is completed - During reporting: - Complete all written reporting requirements in a timely manner - Make recommendations for changes to improve efficiency, effectiveness, and safety during the execution of tasks Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Project site, project documents, labour, plant, equipment and tools, construction materials.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |  |
| --- | --- | --- |
|  | 1st Occasion | 2nd Occasion |
| 1 Plan and prepare for earthworks construction |  |  |
| - Locate and read all appropriate earthworks construction documentation to confirm compliant activities. |  |  |
| - Read and clarify specific task information and requirements from technical specifications and notes. |  |  |
| - Meet with project management as required to confirm client requirements are met. |  |  |
| - Develop a job plan that meets task requirements and makes best use of resources. Assessor attach a copy of the job plan with participant name and date showing as permitted by workplace. Alternatively note job plan observed. |  |  |
| - Recognise training requirements for team and allocate tasks to ensure safety. |  |  |
| - Prepare for site meetings (toolbox/ pre-start) by preparing notes and resources required. |  |  |
| 2 Implement the job plan |  |  |
| - Confirm availability of all resources required for safe and effective task completion. |  |  |
| - Lead pre-start meeting providing clear, legible and timely instructions from job plan to team members and others both verbal and in written form. |  |  |
| - Allocate tasks to others in a manner to be understood. |  |  |
| - Manage team and other relationships in a manner to diffuse friction and induce work production. |  |  |
| - Monitor checking of plant tools and equipment for serviceability. |  |  |
| - Read and interpret project engineering surveys. |  |  |
| - Undertake set out of offsets according to plans and drawings. |  |  |
| - Adhere to manufacturer and workplace requirements for use of levelling tools and equipment to achieve accurate set out and offset marking. |  |  |
| - Correctly calculates resource consumption, supply rate, volumes, areas, grades and percentages |  |  |
| - Communicate offset requirements using approved workplace methods of marking. |  |  |
| - Interpret information from site soil/rock, hydrological and geological data test results. |  |  |
| - Recognise and address drainage issues according to site requirements. |  |  |
| 3 Oversee the execution of tasks |  |  |
| - Apply performance monitoring skills, checking measurements, making calculations and assessing performance against schedule and requirements for any gaps. |  |  |
| - Make adjustments, or recognise adjustments made to work practices or the job plan and implements these to ensure achievement of the required outcomes |  |  |
| - Monitor performance, recognising and acting on training needs of workers. |  |  |
| - Demonstrate knowledge of plant and equipment capabilities and limitations. |  |  |
| - Adhere to or alter materials delivery scheduling according to requirements and procedures. |  |  |
| - Oversee plant, equipment and tool maintenance procedures on completion of tasks including documentation of maintenance records |  |  |
| - Inspect work to ensure outcomes conform with requirements, applying testing where required. |  |  |
| - Organise or apply mentoring to workers requiring training and monitoring. |  |  |
| 4 Report on task execution |  |  |
| - Complete and submits reports including all test results. |  |  |
| - Adhere to continuous improvement by making reasonable recommendations for improvement of processes or procedures. |  |  |