CPCCBC4012B

Read and interpret plans and specifications

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4012B Read and interpret plans and specifications cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Ryan Readdy |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Ryan Readdy |
| --- | --- |
| Participant signature |  |
| Date | 29/03/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 87 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

- Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

- Which compass direction is always shown on plans and drawings?

| Answer | North |  |
| --- | --- | --- |

- What information is provided on a location plan?

| Answer | Lot number Deposit plan numbers North Point |  |
| --- | --- | --- |

- What detail does a site or block plan contain?

| Answer | Outline of site boundaries showing location of proposed building and existing dwellings Street frontage names Major geological and topographical features (trees and vegetation) Datum points, contours and slopes Driveways, stormwater drainage, easements and right of carriage Soil and surface water drains, pipe sizes Service runs from the house to mains - utility services location (sewer, water, gas, electricity) Details of paving and retaining walls |  |
| --- | --- | --- |

- What do floor plans identify?

| Answer | Shows detail of the proposed structure, such as rooms, positions of fixtures, floor coverings |  |
| --- | --- | --- |

- What is a landscape plan and what information might it provide?

| Answer | Shows details of plants, paths, garden layouts and retaining walls |  |
| --- | --- | --- |

- What are elevation views? What do they provide to the plan reader?

| Answer | Views of the proposed structure from its sides (i.e. from north, south, east and west). Each side is what is called an elevation view. |  |
| --- | --- | --- |

- What is a section?

| Answer | In most cases elevation and plan views cannot show sufficient information to enable a builder or tradesman to see how various structural parts are to be built or assembled. The section is a view of the house after it has been cut vertically at some point. Like the floor plan it shows the size and thickness of structural members and relationships between spaces. |  |
| --- | --- | --- |

- What are detailed drawings?

| Answer | Illustrate a particular method of construction or finish |  |
| --- | --- | --- |

- What are specifications?

| Answer | General items contained within Building Specifications include a: Full description of work Precise list of materials Details of all fittings, fixtures and finishes (appliances – make and model) Precise list of products (e.g. types, brands, grades, thickness, colour, model) |  |
| --- | --- | --- |

- Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bull Nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Block |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Gloss |  |
|  | CJ | Ceiling Joist |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold Water Tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Proof Course |  |
|  | DSB | Distribution Switch Board |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion Joint |  |
|  | FPBD | Fibrous Plasterboard |  |
|  | FFL | Finished Floor Level |  |
|  | FHR | Fire Hose Reel |  |
|  | GPO | General Purpose Outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

- Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Double masonry wall |  |  |
|  | Cavity wall 1x Masonry, 1x Stud wall |  |  |
|  | Stud Wall |  |  |
|  | Window |  |  |
|  | Window with sill |  |  |
|  | Door- Single Leaves opening |  |  |
|  | Door- 2 leaves opening |  |  |
|  | Door- Cavity Slider |  |  |
|  | Door- Fascia Slider |  |  |
|  | Electrical GPO |  |  |

- Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Hinged at side |  |  |
|  | Hinged at top |  |  |
|  | Sliding Horzinotally |  |  |
|  | Fixed |  |  |

- What does a clouded area on a plan indicate?

| Answer | That area has been revised |  |
| --- | --- | --- |

- An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

- On what date was an amendment made to the window details?

| Answer | 5/10/2011 |  |
| --- | --- | --- |

- True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What amendment occurred on 08/11/2011?

| Answer | Bed 3 robe deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

- What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

- What is the Lot number of this site?

| Answer | Lot 16 |  |
| --- | --- | --- |

- What is the size of this lot?

| Answer | 3653m sq |  |
| --- | --- | --- |

- Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

- True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What size is the easement on the eastern boundary of the site?

| Answer | 5m |  |
| --- | --- | --- |

- True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

- What is the level at the northernmost point of the site?

| Answer | 3.94m |  |
| --- | --- | --- |

- Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

- Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26. ?

| Answer | It is the same structure proposed in Q19 – Q26. It is a 2-storey residence, lower floor cavity brick with verandah on either side, upper floor timber weather boards, Colorbond metal roof. |  |
| --- | --- | --- |

- Who is the client?

| Answer | H&M Smith |  |
| --- | --- | --- |

- Where is this project to be built?

| Answer | Lot16 Glenvalley Road, Richmond |  |
| --- | --- | --- |

- What are the contact details for the draftsman?

| Answer | Joe Foreman Ph: 03 9514 2311 Mob: 0413 113 113 18 Fairview Road, Richmond 3216 |  |
| --- | --- | --- |

- What sheet number is this drawing?

| Answer | 5 of 7 |  |
| --- | --- | --- |

- What timber is required for loading and non-load bearing walls?

| Answer | F5 45x90 & F5 35x90 |  |
| --- | --- | --- |

- What spacing is recommended for common studs?

| Answer | 600mm Max spacing |  |
| --- | --- | --- |

- What type of roof truss is to be installed?

| Answer | Scissor trusses at 900mm spacing Top choard 30deg Bottom cord 20deg |  |
| --- | --- | --- |

- What centres are the trusses positioned at?

| Answer | 900mm spacing |  |
| --- | --- | --- |

- What size are the veranda posts?

| Answer | 90mm x 90mm |  |
| --- | --- | --- |

- What timber species are the veranda posts to come from?

| Answer | Oregon |  |
| --- | --- | --- |

- What pitch is the roof?

| Answer | 30deg |  |
| --- | --- | --- |

- What size are the noggings and at what spacing?

| Answer | 35mm x 70mm at 1350 spacing |  |
| --- | --- | --- |

- What size are the ceiling joists?

| Answer | 120x45 |  |
| --- | --- | --- |

- What timber is used for roof battens and what spacing?

| Answer | Pine, 900mm spacing |  |
| --- | --- | --- |

- What thickness is the plasterboard sheeting?

| Answer | 10mm |  |
| --- | --- | --- |

- What type of flooring is to be installed?

| Answer | 15mm plywood floor |  |
| --- | --- | --- |

- What is the roof cladding material?

| Answer | Colorbond corrugated metal roofing |  |
| --- | --- | --- |

- Where will you find information about the footings?

| Answer | Engineering drawings |  |
| --- | --- | --- |

- What do the letters ‘DAR’ refer when timber ordering?

| Answer | Dressed All Round |  |
| --- | --- | --- |

- Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | 240mm x 63mm Laminated Veneer Lumber |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

- What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

- What is the Job Number?

| Answer | OOR-112 |  |
| --- | --- | --- |

- What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

- What does the drawing show?

| Answer | North, South, East & West Elevations and Cross Section |  |
| --- | --- | --- |

- Which view shows the front of the house?

| Answer | East Elevation |  |
| --- | --- | --- |

- How many motor vehicles can be parked in the garage?

| Answer | 2 |  |
| --- | --- | --- |

- What are the window and doors made from?

| Answer | Windows- Aluminium Framed glass, Doors- Timber |  |
| --- | --- | --- |

- What is the size of the eaves?

| Answer | 450mm |  |
| --- | --- | --- |

- What are the eaves lined with?

| Answer | Hardiflex |  |
| --- | --- | --- |

- What type of roof structure is the roof built from?

| Answer | Timber framed roof trusses at 600 centers. |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the requirements for the roof structure in Q59. ?

| Answer | NCC Volume2 Part 3.4 Framing |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | AS1684 |  |
| --- | --- | --- |

- What is the roof cladding materials made from?

| Answer | Concrete Roof Tiles |  |
| --- | --- | --- |

- Which Part of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | NCC Volume2 Part 3.5 Roof and Wall Cladding |  |
| --- | --- | --- |

- Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | AS2050 |  |
| --- | --- | --- |

- What is the roof pitch?

| Answer | 22.5deg |  |
| --- | --- | --- |

- Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | Brisbane City Council |  |
| --- | --- | --- |

- Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | Step 1 – Prepare building plans, seek advice from architecture and/or engineering firms (e.g. survey, architectural engineering plans). Step 2 – Apply for BA to your local government authority or by engaging an accredited Building Certifier. Building Certifiers are familiar with the Building Act and Regulations and the Queensland Development Code. An internet search will give you a list of Building Certifiers in your local area. Step 3 – Building Approval will be granted by the building certifier to allow construction to commence. Inspections are undertaken by the building certifier during the course of the construction and various forms are completed that recognise compliance. This process can take between 9-55 days. If this application can be assessed through the RiskSMART process it is guaranteed to take 5 days. |  |
| --- | --- | --- |

- What is a prime cost (PC) item? Give an example.

| Answer | PC- Prime Cost A prime cost is an amount of money included in the contract to purchase a specific item. The amount included is an agree estimated amount that is at least the lowest amount the item could reasonably cost. A more expensive item may be requested at a later stage and this difference would be a variation. Eg. A PC of $20.00 was included for Internal door handles. After signing the contract the owner found a handle they liked that cost $25.00 each. The $5.00 difference will be a variation. |  |
| --- | --- | --- |

- What is a provisional sum (PS) item? Give an example.

| Answer | PS- Provisional Sum A Provisional Sum is an amount of money allowed for in the contract to cower work and/or materials that could not be reasonably detailed when entering into the contract. Eg. A provisional Sum of $10,000.00 was included in the contract for excavation of the footings based upon the site appearing sandy and clean. Once excavation commenced, large limestone rocks were found on site and additional excavation and filling was required to complete the works. The total cost for the works was $12,000.00. The $2,000.00 difference will be a variation to the contract. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

- True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify two (2) materials being used for the floors?

| Answer | Carpet Vinyl |  |
| --- | --- | --- |

- True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- How much does the contracted plasterer cost?

| Answer | $12,900.00 |  |
| --- | --- | --- |

- How much do the kitchen appliances cost?

| Answer | $2,700.00 |  |
| --- | --- | --- |

- What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

- What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

- How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

- If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $275.00 |  |
| --- | --- | --- |

- How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 |  |
| --- | --- | --- |

- What material has the highest total cost?

| Answer | 920x45 MGP10 Long Lengths |  |
| --- | --- | --- |

- How many metres are required of the material in Q80. ?

| Answer | 322.5 |  |
| --- | --- | --- |

- What beam is identified as being a better option than F17?

| Answer | E14 beams |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

- Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter (road construction) |  |
| --- | --- | --- |

- If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $214.80 |  |
| --- | --- | --- |

- Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar bender and fixer |  |
| --- | --- | --- |

- If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $1,285.80 |  |
| --- | --- | --- |

- Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | Building Act 1975 Building Regulation 2006 |  |
| --- | --- | --- |

CPCCBC4012B

Read and interpret plans and specifications

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Ryan Readdy |
| --- | --- |
| Participant signature |  |
| Date | 29/03/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4012B

Read and interpret plans and specifications

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to read and interpret plans and specifications in order to inform estimation, planning and supervisory activities. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - Any drawing with a cloud or mark-up you have inserted. - Copy of the correspondence regarding mark up and any site management/authorising engineer response. The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Drawings (including site plan), Specifications, Tender documentation, Telephone, Facsimile machine, Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: |  |  |  |
| Name of observer: |  |  |  |
| Role of observer (Confirm suitability to sign) |  |  |  |
| Email address of observer |  |  |  |
| Signature of observer: |  |  |  |
| Date: | Click here to enter a date. |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| 1 Interpret drawings |  |  |  |
| - Identify purpose and advantage of different types of drawings. |  |  |  |
| - Identify different aspects of drawings. |  |  |  |
| - Identify, understand and apply: - Commonly used symbols and abbreviations - Common building and construction terms on drawings |  |  |  |
| - Identify building site from location drawings. |  |  |  |
| - Identify on the site plan: - True north - Building orientation - Key features in relation to building and other structures - Levels - Contours - Service entry points - Site features to be removed or retained |  |  |  |
| - Identify key features of plans, elevations and sections. |  |  |  |
| - Identify client requested variations to standard plans on drawings. |  |  |  |
| 2 Interpret specifications |  |  |  |
| - Identify and correctly apply Provisional Sum (PS) and Prime Cost (PC) values. |  |  |  |
| - Identify and confirm customer variations to standard specifications and communicate changes to organisational personnel. |  |  |  |
| - Apply and communicate correct interpretations of essential elements to estimation, planning and supervisory tasks (including labour hours/costs and material quantities/costs). |  |  |  |
| - Identify Building Codes or standards affecting the work to be undertaken (including references to Australian Standards and the National Construction Code). |  |  |  |
| - Identify key features of products included in the specification, including the design, purpose, aesthetics and cost relationships. |  |  |  |
| 3 Communicate with others |  |  |  |
| - Consult with industry professionals, clients, organisational personnel and appropriate local authorities. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Using technology (including telephone, facsimile, email) |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments